# Submission ID #: 68296

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=20830328

# Title: Design of Solid-State Fermentation Systems for Polymer Hydrolytic Extracellular Enzyme Production by Filamentous Fungi

# Authors and Affiliations:

# Cynthia Lizbeth López-García, Adriana Jazmín Legorreta-Castañeda, Raúl Sansón-Temich, Guadalupe Guerra-Sánchez, Dario Rafael Olicón-Hernández

# Instituto Politécnico Nacional, Escuela Nacional de Ciencias Biológicas, Departamento de Microbiología, Laboratorio de Bioquímica y Biotecnología de Hongos, Carpio y plan de Ayala s/n, Col. Santo Tomás

# Corresponding Authors:

# Guadalupe Guerra-Sánchez 			mguerras@ipn.mx

# Dario Rafael Olicón-Hernández		doliconh@ipn.mx

# Email Addresses for All Authors:

Cynthia Lizbeth López-García			cynthiallg021599@gmail.com

Adriana Jazmín Legorreta-Castañeda		adri0294@hotmail.com

# Raúl Sansón-Temich				raulsanson8@gmail.com

# Guadalupe Guerra-Sánchez 			mguerras@ipn.mx

# Dario Rafael Olicón-Hernández		doliconh@ipn.mx

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  19

Number of Shots:  35

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

  - Dario Rafael Olicón-Hernández: We design versatile solid-state fermentation systems to enhance enzyme production by fungi, exploring how different inducers, inoculum types, and rotary systems improve scalability and yield [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.3.1

What are the current experimental challenges?

  - Cynthia Lizbeth López-García: Scaling up rotary SSF systems while maintaining homogeneous conditions, oxygen transfer, and consistent enzyme yields remains a key experimental challenge [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.5.1

What significant findings have you established in your field?

  - Raúl Sansón-Temich: We demonstrated that rotary SSF systems produce 4–6 times higher enzyme yields than submerged fermentation, showcasing their versatility and scalability for multiple enzymes [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 4.1.3

What advantage does your protocol offer compared to other techniques?

  - Adriana Jazmín Legorreta-Castañeda: Our protocol integrates rotary mixing, diverse inoculum types, and a variety of specific inducers, ensuring high enzyme yields, homogeneous growth, and applicability across multiple enzyme systems [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 5.1.1

What research questions will your laboratory focus on in the future?

  - Dario Rafael Olicón-Hernández: We will focus on scaling up rotary SSF systems and exploring their application for producing novel enzymes in food, bioenergy, and environmental sectors [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.5.3

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

- Preparation of Substrate and Inoculum

Demonstrators: Cynthia Lizbeth López-García, Adriana Jazmín Legorreta-Castañeda, Raúl Sansón-Temich

  - To begin, wash the wheat bran three times with sterile distilled water [1] to remove organic matter residues, debris, and dust [2].

    - WIDE: Talent adding wheat bran with sterile distilled water using a spatula.

    - Talent draining the water through a sieve after each wash cycle.

  - Spread the washed wheat bran on an aluminum tray [1] and dry it in an oven set at 60 degrees Celsius for 24 hours [2].

    - Talent evenly spreading washed bran on an aluminum tray using a spatula.

    - Talent placing the tray inside a laboratory oven and setting the temperature to 60 degrees Celsius.

  - To prepare the spore suspension, transfer a 5-millimeter-diameter agar disk saturated with mycelium onto a fresh potato dextrose agar plate [1]. Incubate the plate at 28 degrees Celsius for 5 to 7 days, or until the mycelium saturates the medium [2].

    - Talent placing the disk onto the center of a fresh potato dextrose agar plate.

    - Talent placing the plate in incubator.

  - Then, add 5 milliliters of sterile distilled water to the plate [1] and, using a sterile loop, detach the spores from the surface [2].

    - Talent pipetting 5 milliliters of sterile distilled water onto the mycelium surface.

    - Talent swirling a sterile loop across the surface to detach spores into suspension.

  - Now, prepare a 1 to 100 dilution of the spore suspension using sterile distilled water [1]. Add 10 microliters of the suspension to the Neubauer chamber [2] and for counting the spores using a microscope [3].

    - Talent pipetting 1 part of spore suspension into 99 parts sterile distilled water in a sterile test tube.

    - Talent adding suspension to Neuberg chamber.

    - Talent placing the chamber under a microscope.

- Cultivation of Mycelium for Fermentation

  - For liquid culture, use sterile forceps to transfer a mycelium-saturated agar disk onto a fresh potato dextrose agar plate [1]. Incubate the plate at 28 degrees Celsius until the medium is fully saturated [2].

    - Talent transferring the agar disk to a fresh PDA plate.

    - Talent placing the plate in an incubator.

  - Prepare 25 milliliters of potato dextrose broth in a sterile 125 milliliter Erlenmeyer flask [1] and autoclave the flask to sterilize the medium [2].

    - Talent measuring 25 milliliters of broth into a flask using a sterile measuring cylinder.

    - Talent loading the flask into an autoclave and initiating the sterilization cycle.

  - Next, transfer a 5-millimeter disk of mycelium from the saturated PDA plate into the sterile broth [1].

    - Talent placing the mycelium disk into the flask with sterile broth.

  - Incubate the flask on a shaker at 125 rpm (R-P-M) for 24 to 48 hours, adjusting time based on fungal strain [1].

    - Talent placing the flask on a shaking incubator.

  - Collect 2 milliliters of the cultured broth for inoculum preparation [1] and another 2 milliliters for dry weight determination [2].

    - Talent drawing 2 milliliters of broth into a sterile syringe or pipette. Note: 3.5.1 and 3.5.2 are merged

    - Talent transferring another 2 milliliters into a labeled container for dry weight measurement.

  - For direct inoculation of mycelium disks, place the mycelium-saturated agar disk onto a fresh potato dextrose agar plate [1] and incubate it at 28 degrees Celsius until the medium is fully saturated [2].

    - Talent placing the agar disk on a fresh PDA plate using forceps.

    - Talent incubating the plate by placing it into an incubator set to 28 degrees Celsius.

- Solid-State Fermentation (SSF) Procedure and Enzyme Extraction

  - Prepare the substrate tube with 5 grams of wheat bran, 0.5 grams of the inducer and 5.5 milliliters of water [1] and autoclave the tube at 15 pounds per square inch for 15 minutes [2]. After cooling, insert the electrode probe directly into the reactor at varying depths to measure the relative humidity using an electrode-based hygrometer [3].

    - Talent adding the components to the tube.

    - Talent placing the tube into the autoclave. NOTE: 4.1.2 mistakenly slated as 4.11

    - Talent inserting the electrode probe into the reactor.

  - Inoculate the substrate with 1 milliliter of spore suspension containing 106 to 107 spores per milliliter [1-TXT].

    - Talent pipetting inoculum into the cooled tube containing substrate. TXT: Alternatively, inoculate 2 mL of cellular suspension; Or add a 5 mm mycelium disk

  - Vortex the tubes at maximum speed for 5 minutes in 1-minute cycles to avoid substrate clumping [1].

    - Talent placing the tubes on a vortex mixer.

  - Then, place the tubes in a rotary mixer with a horizontal axis [1]. Incubate the rotary mixer in an incubator set to the microorganism’s optimal growth temperature [2].

    - Talent arranging the tubes horizontally in the rotary mixer and starting rotation.

    - Talent placing the rotary mixer with tubes in the incubator.

  - For enzyme extraction, resuspend the substrate in 20 milliliters of pre-chilled buffer after the desired fermentation period [1].

    - Talent pipetting pre-chilled buffer into the tube containing fermented substrate.

  - Vortex the tubes in cycles of 1 minute at maximum speed [1] followed by 1 minute on ice [2].

    - Talent vortexing of the tube.

    - Talent placing the tube on ice.

  - Filter the suspension using paper filters [1] and press to mechanically extract the supernatant [2].

    - Talent adding the suspension on paper filter. Note: 4.7.1 and 4.7.2 are merged

    - Talent pressing the unit to extract the liquid supernatant.

  - Finally, centrifuge the supernatant at 3000 g for 15 minutes at 4 degrees Celsius to clarify the liquid [1].

    - Talent placing the sample tubes in a centrifuge.

# Results

- Results

  - After 6 days of solid-state fermentation, the substrate showed visible fungal colonization with a fibrous, mycelial network covering the surface [1].

    - LAB MEDIA: Figure 2B and C.

  - Glucosamine formation through the hydrolysis of chitosan was significantly higher in Trichoderma harzianum under solid-state fermentation, indicating that chitinase activity in this case was much higher [1] than that in submerged fermentation [2].

    - LAB MEDIA: Figure 3A. Video editor: Highlight the taller bar labeled "SSF" with the value 321.96.

    - LAB MEDIA: Figure 3A. Video editor: Highlight the shorter bar labeled "SmF" with the value 50.64.

  - Similarly, amylase activity by Aspergillus lentulus was significantly elevated in solid-state fermentation [1], compared to that in submerged fermentation [2].

    - LAB MEDIA: Figure 3B. Video editor: Highlight the taller bar labeled "SSF" with the value 7.754.

    - 3.5.2 LAB MEDIA: Figure 3B. Video editor: Highlight the shorter bar labeled "SmF" with the value 1.2179

# Pronunciation Guide:

# 1. mycelium

# Pronunciation link:
https://www.merriam-webster.com/dictionary/mycelium youtube.com+12merriam-webster.com+12openmd.com+12
IPA: /maɪˈsiː.li.əm/ 
Phonetic Spelling: my-SEE-lee-um

# 2. Neubauer

# (as in Neubauer chamber / counting chamber name)
Pronunciation link:
https://www.howtopronounce.com/neubauer en.wikipedia.org+11howtopronounce.com+11youtube.com+11
IPA: /ˈnuː.baʊ.ər/ 
Phonetic Spelling: NOO-bow-er

# 3. Erlenmeyer (as in “Erlenmeyer flask”)

# Pronunciation link:
https://www.merriam-webster.com/dictionary/Erlenmeyer%20flask merriam-webster.com+1yourdictionary.com+1
IPA: /ˈɜrlənˌmaɪər/
Phonetic Spelling: ER-len-my-er

# 4. hygrometer

# Pronunciation link:
https://www.merriam-webster.com/dictionary/hygrometer merriam-webster.com+15merriam-webster.com+15merriam-webster.com+15
IPA: /haɪˈɡrɑmətər/
Phonetic Spelling: hy-GRAH-muh-ter

# 5. vortex

# Pronunciation link:
https://www.merriam-webster.com/dictionary/vortex merriam-webster.com+10merriam-webster.com+10merriam-webster.com+10
IPA: /ˈvɔrˌtɛks/
Phonetic Spelling: VOR-teks