# Submission ID #: 68332

# Scriptwriter Name: Sulakshana Karkala

# Project Page Link: https://review.jove.com/account/file-uploader?src=20841308

# Title: In Vivo Imaging of Neural Activity in Unanesthetized Drosophila Adult Flies

# Authors and Affiliations:

# Prachi Shah1, Isaac Cervantes-Sandoval1,2

# 1Department of Biology, Georgetown University
2Interdisciplinary Program in Neuroscience, Georgetown University

# Corresponding Authors:

Isaac Cervantes-Sandoval	(ic400@georgetown.edu)

# Email Addresses for All Authors:

Prachi Shah			(ps1246@georgetown.edu)

Isaac Cervantes-Sandoval	(ic400@georgetown.edu)

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  YES

Authors: Please create scope videos of the shots labeled as SCOPE and upload the files to your project page as soon as possible : https://review.jove.com/account/file-uploader?src=20841308

SCOPE: 3.4.1-3.4.2, 3.5.1, 3.6.2, 3.7.3,3.9.3,3.10.2,3.11.2,3.12.2,3.12.3

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=20841308

SCREEN: 4.1.2, 4.2.1-4.2.3, 4.3.1-4.3.2,4.4.1-4.4.2

Current Protocol Length

Number of Steps:  26

Number of Shots:  57

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

AUTHORS: Please note that only 5 interview statements may be presented. Some statements have been edited for brevity

REQUIRED: What is the scope of your research? What questions are you trying to answer?

  - Isaac Cervantes-Sandoval: We are trying to understand the molecular, cellular, and circuit basis of natural memory forgetting, aiming to uncover how the brain actively erases or suppresses memories to maintain cognitive flexibility.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:4.3

What are the most recent developments in your field of research?

  - Isaac Cervantes-Sandoval: Recent research has shown that forgetting is not merely a passive decay of memories, but rather a highly regulated and active biological process that requires specific patterns of neuronal activity.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What are the current experimental challenges?

  - Prachi Shah: A major challenge is linking specific circuit manipulations to dynamic memory processes, while integrating connectomic, genetic, and behavioral data in a rigorous and interpretable way.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What significant findings have you established in your field?

  - Prachi Shah: We helped establish that forgetting is an active, biologically regulated process. Our work identified specific dopaminergic neurons and molecular pathways required for normal forgetting in the Drosophila brain.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:5.5

What research gap are you addressing with your protocol?

  - Prachi Shah: Our protocol enables functional imaging in flies without anesthesia, preventing unwanted non-specific effects by the anesthetics. We use this approach to investigate the neural correlates underlying memory formation and active memory forgetting.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:5.6

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

- Assembly of Behavioral Experimentation Tools for Drosophila Neurobiology Studies

Demonstrator: Prachi Shah

  - To begin, use Dremel tools and a diamond saw blade to cut a 22-gauge hypodermic metal tubing to a length of approximately 10 centimeters [1]. With a Dremel 420 cut-off wheel, buff both ends of the tubing to create a smooth and clean opening that can accommodate the proboscis of the fly [2].

    - WIDE: Talent using a Dremel tool and diamond saw blade to cut the 22-gauge tubing to 10 centimeters.

    - Talent using a Dremel 420 cut-off wheel to polish the tubing ends.

  - Wrap the cut tubing around a 15-milliliter centrifuge tube to form the desired curved shape [1]. Then cut a 7-centimeter-long piece of 12-gauge hypodermic metal tubing [2].

    - Talent manually bending the metal tubing around the 15-milliliter centrifuge tube.

    - Talent cutting the 12-gauge tubing with Dremel tools.

  - Now use a razor blade to trim the end of a 2-microliter pipette tip to fit the 22-gauge metal tubing [1]. Fit the 12-gauge tubing into the other end of the pipette tip [2].

    - Talent trimming the pipette tip with a razor blade.

    - Talent inserting the 12-gauge tubing into one end of the pipette tip.

  - Next, mix a small amount of epoxy resin and hardener together [1]. Apply the epoxy to the junctions where the small metal tubing meets the pipette tip and where the larger tubing connects to the other end [2]. Allow the epoxy to fully cure overnight before connecting the assembly to a micromanipulator holder and adjusting the angle as necessary [3].

    - Talent mixing epoxy resin and hardener.

    - Talent applying epoxy to the joints at both ends of the pipette tip.

    - Talent adjusting angle on micromanipulator.

  - To build a shock and odor delivery pipette, cut 1 milliliter off a 1 by 100 glass pipette at the 3-milliliter mark, using a Dremel diamond tool [1]. Then cut a small rectangular acrylic sheet measuring 24.5 millimeters by 8 millimeters with a thickness of one-eighth inch [2].

    - Talent cutting the pipette with the Dremel diamond tool.

    - Talent cutting the acrylic sheet to specified dimensions.

  - Cut a copper shock grid to fit onto the rectangular acrylic piece [1]. Solder two electrical wires to opposite ends of the copper grid [2].

    - Talent trimming the copper grid to fit the acrylic.

    - Talent soldering wires onto the copper grid.

  - Now, place the copper grid on the acrylic piece [1] and bend it slightly to accommodate the fly’s abdomen and legs [2].

    - Talent inserting the copper grid onto the acrylic grid.

    - Talent positioning and curving the copper grid into the acrylic.

  - Use electrical tape to attach the copper grid to the acrylic piece [1]. Then use a hot glue gun to attach the glass pipette to the shock grid, ensuring it is straight and centered [2].

    - Talent taping the grid to the acrylic.

    - Talent hot gluing the glass pipette in place.

  - To build the recording chamber, take a glass microscope slide as the chamber base [1]. Mix resin and epoxy glue together [2]. Using the epoxy glue, attach neodymium magnets onto all four corners of a black acrylic chamber [3-TXT].

    - Talent placing the microscope slide on the work surface.

    - Talent mixing resin and epoxy.

    - Talent applying glue to attach magnets to the acrylic chamber. TXT: Magnet dimensions: 3 mm x 2 mm

  - Place an additional magnet on top of each glued magnet then glue the newly placed magnets to a glass slide using epoxy [1]. Hold the assembly in place with paper clips while curing [2].

    - Talent placing magnets on top of existing corner magnets and glueing them to the glass slide.

    - Talent securing the assembly with paper clips for curing.

- Preparation and Dissection of Drosophila for Neural Imaging

  - Remove the 200-microliter pipette tip from the aspirator [1]. Insert the aspirator into a vial containing the Drosophila and aspirate a single fly into the 1,000-microliter pipette tip [2].

    - Talent removing the 200 µL pipette tip from the aspirator.

    - Talent aspirating a fly from a vial into a 1,000 µL pipette tip.

  - Replace the 200-microliter pipette tip back onto the aspirator [1]. Then gently blow and flick the aspirator so the fly is immobilized headfirst at the top of the 200-microliter pipette tip [2].

    - Talent replacing the 200-microliter pipette tip onto the aspirator.

    - Talent blowing and flicking the aspirator to position the fly headfirst.

  - Next, place the dissection chamber onto the manipulator holder [1]. Connect the vacuum to the fly-holding tubing and adjust the flow rate to approximately 500 milliliters per minute [2-TXT].

    - Talent placing the dissection chamber onto the manipulator holder.

    - Talent connecting vacuum tubing to the fly-holding tubing and adjusting flow rate. TXT: Use a fast connector to facilitate quick connection and disconnection of the vacuum

  - Now move the vacuum metal tubing to the center of the microscope's field of view [1]. Gently aspirate the fly’s proboscis into the vacuum holder [2]. 
Authors: Please create scope videos of the shots labeled as SCOPE and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=20841308

    - SCOPE: The vacuum tubing position is being adjusted.

    - SCOPE: The fly’s proboscis is being aspirated into the vacuum holder.

  - Adjust the manipulator to align the fly’s head with the chamber opening [1].

    - SCOPE: The manipulator is being moved to align the fly’s head with the chamber.

  - Turn on the direct current power supply [1]. Using platinum resistance wire, apply melted myristic acid to glue the eyes and thorax to the chamber [2].

    - Talent switching on the direct current power supply.

    - SCOPE: Platinum resistance is being used to apply melted myristic acid on the eyes and thorax.

  - Once secured, disconnect the vacuum tubing [1]. Remove the recording chamber from the vacuum connection using the manipulator and turn the chamber upside down [2]. Then glue the proboscis from below using platinum resistance [3].

    - Talent disconnecting vacuum tubing.

    - Talent detaching and inverting chamber.

    - SCOPE: The proboscis is being glued from below.

  - When everything has been glued, turn off the direct current power supply [1]. Then turn the chamber upright [2].

    - Talent turning off power.

    - Talent orienting the chamber upright.

  - Attach the chamber to the glass slide base [1]. Cut a small piece of tape with scissors [2] and place it in front and behind the fly’s head [3].

    - Talent placing the chamber onto the glass slide base.

    - Talent cutting  tape with scissors.

    - SCOPE: Tape is being applied in front and behind the fly’s head.

  - Rotate the chamber so the fly’s head faces the experimenter at a 90-degree angle [1]. With a dissecting needle, make vertical incisions along the sides of the eyes [2].

    - Talent turning the chamber to face the fly’s head toward the experimenter.

    - SCOPE: Vertical cuts are being made along the eyes with a dissecting needle.

  - Rotate the chamber horizontally [1]. Then make a horizontal cut across the cuticle [2].

    - Shot of the chamber being rotated horizontally.

    - SCOPE: A horizontal cut is being made across the cuticle.

  - Now add 100 microliters of saline to the top of the fly’s head [1]. Using sharp forceps, remove the cuticle window [2]. Then, remove any remaining fat or trachea with the forceps [3].

    - Talent pipetting 100 microliters of saline onto the fly’s head.

    - SCOPE: The cuticle window is being removed with sharp forceps.

    - SCOPE: Fat and trachea remnants are being removed using forceps.

- Visualization Of Learning-Induced Plasticity Through Aversive Olfactory Conditioning

Demonstrator: Click here to enter name of demonstrator(s)

  - Place a prepared fly onto the microscope stage of a confocal microscope equipped with a laser and a water immersion objective [1]. With a micromanipulator, adjust the position of the shock grid and odor pipette so the fly is correctly positioned on the shock grid [2].
Authors: Please create SCREEN CAPTURE videos of the shots labeled as SCREEN and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=20841308

    - Talent placing the prepared fly chamber on the stage of the confocal microscope.

    - SCOPE/SCREEN: The fly is being aligned with the shock grid.

  - Use the coarse Z adjustment knob to scan through the Z-axis of the brain [1] and locate the brain region of interest [2]. Set the frame size to 512 by 512 pixels [3].

    - SCOPE/SCREEN: Live Z-stack scan through the brain is being done using the coarse Z adjustment.

    - SCOPE/SCREEN: Brain ROI is being located. 
AUTHORS: For this shot, please hover the mouse over the ROI

    - SCOPE/SCREEN: The frame size is being set to 512 x 512 px.

  - Begin recording from the neuron of interest using a custom-made or commercially available odor delivery system [1]. Simultaneously initiate a “pre-training” odor delivery protocol. Set the recording duration to 2 minutes [2].

    - SCREEN: Neural recording is being started.

    - SCREEN: Odor delivery via the odor delivery interface is being initiated and recording duration is being set to 2 minutes.

  - Initiate the training protocol using the odor delivery system, 5 minutes after collecting pre-training responses [1]. Then record post-training responses about 5 to 15 minutes after training [2].

    - Talent initiating training odor delivery from the software or hardware control.
AND
SCREEN: Data is being recorded for the training protocol.

    - SCREEN: Post-training responses are being seen.

# Results

- Representative Results

  - The calcium indicator GCaMP6f (G-Camp-six-F) and the red fluorescent protein TdTomato (T-D-tomato) were selectively expressed in the Mushroom Body Output Neuron, with dendrites projecting into the γ (Gamma) and α′ (alpha-dash) lobes of the mushroom body [1-TXT], and the neuron was visualized using the MB077C(M-B-Zero-Seven-Seven-C)  split-Gal4 (Gal-four) driver line [2].

    - LAB MEDIA: Figure 2A. TXT: Mushroom Body Output Neuron: MBON γ2α′1  Video editor: Please highlight the green fluorescent signal

    - LAB MEDIA: Figure 2B. Video editor: Zoom in on the black-and-white image showing the dendritic arbor and cell body labeled ROI

  - Calcium responses in the Mushroom Body Output Neuron to 3-octanol were significantly reduced 5 minutes after aversive conditioning without anesthesia [1] and remained suppressed at 15 minutes [2].

    - LAB MEDIA: Figure 2D and F  Video Editor: Show the bottom panel images of D and F

    - LAB MEDIA: Figure 2D and F. Video Editor: Please highlight OCT (CS+) of 15 min in both  images.

  - In contrast, calcium responses to 4-methyl cyclohexanol, were significantly enhanced 5 minutes post-training [1], and remained elevated at 15 minutes [2].

    - LAB MEDIA: Figure 2D and F. Video Editor: Show the top panel images of D and F

    - LAB MEDIA: Figure 2D and F. Video Editor: Please highlight OCT (CS+) of 15 min of both images.

  - Pseudo-color images demonstrated distinct fluorescence changes pre- and post-training [1].

    - LAB MEDIA: Figure 2E. Video editor: Please sequentially highlight the images of top and bottom panel from Pre, 5 min to 15 min

  - In anesthetized flies, post-training calcium responses to CS+ (C-S-plus) were only partially reduced and responses to CS− (C-S-minus) were not significantly different from baseline [1].

    - LAB MEDIA: Figure 2G. Video editor: Please emphasize 5 min post training

  - Quantitative analysis confirmed that CS+ response was significantly depressed post-training in anesthetized flies [1] but CS− responses remained statistically unchanged [1].

    - LAB MEDIA: Figure 2H. Video editor: Please highlight the light blue column corresponding to “Post” in the bottom graph

    - LAB MEDIA: Figure 2H. Video editor: Please highlight the light green column corresponding to “Post” in the top graph

  - Plasticity was significantly higher in non-anesthetized flies compared to anesthetized ones [1].

    - LAB MEDIA: Figure 2I.