# Submission ID #: 68340

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=20843748

# Title: Using the Droplet Transfer Method to Reliably Prepare Giant Unilamellar Vesicles

# Authors and Affiliations:

# Dario Cecchi, Elisa Roberti, Eugenia De Remigis, Stefano Palagi

# The Biorobotics Institute, Sant’Anna School of Advanced Studies

# Department of Excellence in Robotics & AI, Sant'Anna School of Advanced Studies

# Corresponding Authors:

# Stefano Palagi					stefano.palagi@santannapisa.it

# Dario Cecchi					dario.cecchi@santannapisa.it

# Email Addresses for All Authors:

Elisa Roberti					elisa.roberti@santannapisa.it

Eugenia De Remigis				eugenia.deremigis@santannapisa.it

Stefano Palagi					stefano.palagi@santannapisa.it

Dario Cecchi					dario.cecchi@santannapisa.it

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar? NO

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  NO

3. Filming location: Will the filming need to take place in multiple locations?   YES, same building

Current Protocol Length

Number of Steps:  25

Number of Shots:  55

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

REQUIRED: What is the scope of your research? What questions are you trying to answer?

  - Stefano Palagi: We aim at cell-like microrobots that navigate through body tissues for locally administering therapies. They are based on Giant Unilamellar Vesicles, have a phospholipidic membrane and contain active particles.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.5.1

What research gap are you addressing with your protocol?

  - Dario Cecchi: This protocol offers a guide for beginners who want to adopt, and adapt, the droplet transfer method in synthetic biology and in other fields (for example in microrobotics).

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.9.1

What advantage does your protocol offer compared to other techniques?

  - Dario Cecchi: Compared to other GUV preparation techniques, this protocol offers high encapsulation efficiency and straightforward execution, making it accessible to any research group with basic laboratory equipment.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.4.1

What research questions will your laboratory focus on in the future?

  - Elisa Roberti: We will assess GUV-based microrobots’ navigation in tissue-like settings and implement sensing strategies to enable autonomous movement towards diseased cells.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 5.1.1

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

- Preparing the Lipid Solution (LS)

Demonstrator: Dario Cecchi

  - To begin, move the glove chamber either inside or beside a chemical hood [1]. Open the glove chamber and place a crystallizer filled with silica particles at the bottom of the chamber [2]. Cover the crystallizer with the white support surface [3] and then layer it with paper towels [4].

    - WIDE: Talent positioning the glove chamber beside a chemical hood.

    - Talent opening the glove chamber and inserting a crystallizer with silica particles inside.

    - Talent placing a white support surface over the crystallizer.

    - Talent covering the surface with paper towels.

  - Insert all required materials into the glove chamber [1] and place the glass vial containing the chloroform solution on a cork ring holder or a 150 milliliter beaker positioned as close to the operator’s side as possible [2].

    - Shot of all listed materials placed into the glove chamber in an organized manner.

    - Talent positioning the vial with chloroform solution securely in a cork ring or beaker near the front of the chamber.

  - Now, close the glove chamber, ensuring that the two halves are securely aligned and cannot slide [1]. Use the provided plastic screws to completely seal the chamber [2].

    - Talent aligning the glove chamber halves precisely.

    - Talent fastening the plastic screws to seal the chamber.

  - Close the outlet valve and submerge pipe number 5 into a 500-milliliter beaker that is half-filled with water [1].

    - Talent sealing the outlet valve and placing the pipe into the water-filled beaker.

  - Next, connect the valve-regulated end of pipe number 2 to the vacuum line [1]. Then, connect the other end to the nitrogen line using pipe number 3 [2].

    - Talent attaching one end of the pipe to the vacuum line.

    - Talent connecting the opposite end to the nitrogen supply using pipe number 3.

  - Open the valve and turn on the vacuum line until the gloves inflate and become too rigid to bend [1]. Then, close both the vacuum line and the valve [2].

    - Talent opening the valve and activating the vacuum line; gloves visibly inflating.

    - Talent turning off the vacuum line and closing the valve.

  - Next, open the nitrogen line until the gloves deflate and extend out from the glove chamber [1-TXT].

    - Talent opening the nitrogen valve to allow gloves to deflate and become flexible. TXT: Repeat the vacuum and nitrogen cycle until the internal humidity is <40%

  - After donning gloves, use a Hamilton syringe to transfer the specified volume of DOPC (D-O-P-C) in chloroform solution into a dark glass vial [1-TXT]. Seal the vial with the cap containing the needles [2].

    - Talent using a Hamilton syringe to draw the chloroform solution. TXT: DOPC: 1,2-Dioleoyl-sn-glycero-3-phosphocholine

    - Talent capping the dark glass vial with the preassembled needle-bearing cap.

  - Now, connect pipe number 6 to pipe number 8 and pipe number 7 to pipe number 9 using the two-way junctions [1].

    - Talent attaching pipes 6 and 7 to pipes 8 and 9 through the junction connectors.

  - Remove the gloves from the glove chamber and open the outlet valve [1].

    - Talent taking off the gloves and turning the outlet valve to open it.

  - Then, open the nitrogen line slowly and monitor the gas flow for bubbles in the beaker [1]. Gradually increase the nitrogen flow until a steady but observable bubbling rate is reached [2]. After 3 minutes of flow, close the nitrogen line [3].

    - Talent gently opening the nitrogen valve and looking at the beaker for bubble formation.

    - Talent adjusting the flow to achieve a stable, visible stream of bubbles.

    - Talent turning off the nitrogen line after 3 minutes.

  - Once the final bubble exits the beaker, close the outlet valve [1]. Put the gloves back on and disconnect pipes 8 and 9 from the two-way junctions [2].

    - Talent observing the beaker and sealing the outlet valve after the last bubble.

    - Talent wearing the gloves and detaching pipes 8 and 9 from the junctions.

  - Next, open the vial and move the cap back to the 50-milliliter tube [1]. Then open the oil bottle and transfer the required amount of oil into the dark vial [2]. Seal the vial with the cap again, remove the gloves, and open the glove chamber [3].

    - Talent removing the cap and repositioning it on the 50 milliliter tube.

    - Talent pipetting oil into the dark vial and resealing it with the cap.

    - Talent removing gloves and opening the glove chamber.

  - Vortex the sealed lipid suspension vial vigorously for at least 30 seconds [1], then incubate it at 80 degrees Celsius in a water bath for 30 minutes [2]. After vortexing the vial again, incubate it overnight at room temperature [3-TXT].

    - Talent vortexing the vial firmly.

    - Talent placing the vial in the 80 degrees Celsius water bath.

    - Talent placing the vial on the bench. TXT: Store the lipid solution at 4 °C for up to 1 week

- GUVs (Giant Unilamellar Vesicles) Production: Setting up a Container for Easy Removal of Oil

  - Using scissors, cut off the cap from a 5-milliliter tube [1]. Drill an approximately 8-millimeter hole in the center of the cap to fit a P-1000 pipette tip [2].

    - Talent cutting the cap from a 5 milliliter tube using scissors.

    - Talent drilling a central hole and testing fit with a P1000 pipette tip.

    - Talent discarding an ill-fitting cap and starting again with a new one.

  - With a 2 by 5-centimeter strip of sandpaper, smoothen the edges of the drilled hole [1]. Place the cap back on the 5-milliliter tube and centrifuge at 3,000 g for 5 minutes to eliminate any residual plastic particles [2]. Then, remove the cap and discard the tube [3].

    - Talent sanding the drilled hole using a prepared strip of sandpaper.

    - Talent securing the cap on the tube and placing it in the centrifuge.

    - Talent removing and discarding the centrifuged tube, retaining only the cap.

  - Now, cut the cap from a new 5-milliliter tube and replace it with the prepared holed cap [1]. Insert a P-1000 pipette tip through the hole and place an O-ring (Oh-ring) on top of the cap, locking it between the tip and the cap to prevent further downward movement [2]. Fill the tube with 5.5 milliliters of oil solution through the tip [3].

    - Talent swapping the cap of a new tube with the previously holed cap.

    - Talent placing an O-ring on the cap and inserting a P1000 tip to secure it.

    - Talent pipetting oil solution into the tube through the mounted tip.

  - Vortex the lipid suspension vial prepared earlier for at least 30 seconds [1]. Then, incubate it in the 50 degrees Celsius water bath for 30 minutes [2].

    - Talent vigorously vortexing the vial.

    - Talent placing the vial into the 50 degrees Celsius water bath for incubation.

  - Allow the water solutions prepared earlier to equilibrate at room temperature for at least 10 minutes [1].

    - Talent placing water solution tubes on the bench and letting them rest at room temperature.

  - Now, pipette 300 microliters of oil solution into a 1.5-milliliter tube or the previously modified 5 milliliter tube [1]. Slowly layer 100 microliters of lipid suspension on top of the oil solution, ensuring the formation of two distinct and continuous phases [2]. Let the interface stand undisturbed for 10 minutes to equilibrate [3].

    - Talent adding 300 microliters of oil solution to the selected tube.

    - Talent carefully pipetting 100 microliters of lipid suspension on top to form the interface.

    - Talent setting the tube aside .

- GUVs (Giant Unilamellar Vesicles) Production: Water-in-Oil Emulsion Formation

Demonstrator: Elisa Roberti

  - In a 2-milliliter tube, pipette 250 microliters of lipid suspension followed by 6.25 microliters of inner solution to prepare a 2.5 percent water-in-oil emulsion [1].

    - Talent pipetting lipid suspension and inner solution in the specified order into a 2 milliliter tube.

  - Mix the two phases until the emulsion appears homogeneous and stable [1]. Transfer 200 microliters of the prepared emulsion onto the oil solution-lipid suspension interface formed earlier [2].

    - Talent mixing the contents by vortexing.

    - Talent carefully pipetting the emulsion over the interface in the tube.

  - If using a 1.5-milliliter tube, simply close the tube [1]. If using the modified 5 milliliter tube, place a rubber cap on the pipette tip and gently transfer the entire tube into a 50-milliliter conical tube [2]. Centrifuge the tubes at room temperature at appropriate speed [3]. After centrifugation, verify that an opaque pellet is present at the bottom and that the oil phase is clear at the top [4].

    - Talent sealing a 1.5 milliliter tube with its cap.

    - Talent capping the tip with rubber and positioning the 5 milliliter tube inside a larger 50 milliliter tube.

    - Talent placing the tubes in the centrifuge and running the spin.

    - Close up of the pellet at the bottom and clarity of the oil layer.

  - To remove oil from the sample container used in earlier steps, remove the pipette tip from the tube while ensuring the rubber cap remains in place [1]. Discard the tip and retain the rubber cap, O-ring, and holed cap for future use [2].

    - Talent detaching the pipette tip from the modified container, keeping the rubber cap secure.

    - Talent discarding the used tip and storing reusable parts.

  - Using a P-1000 micropipette, remove the oil solution from the tube, leaving behind approximately 50 microliters and making sure not to disturb the pellet [1]. Change the pipette tip, then resuspend the pellet in the remaining 50 microliters of solution [2]. Finally, transfer the resuspended material to a clean 2 milliliter tube [3].

    - Talent carefully aspirating the oil phase, ensuring that the pellet remains intact.

    - Talent pipetting to mix the pellet into the residual volume.

    - Talent transferring the resuspended GUVs into a fresh tube.

# Results

- Results

  - Lipid solutions prepared in environments with high humidity produced giant unilamellar vesicles with markedly lower yield and greater presence of disordered aggregates [1] compared to those prepared in a humidity-controlled chamber [2].

    - LAB MEDIA: Figure 6C. Video editor: Point to the large dark shapes with arrows indicating phospholipid aggregates.

    - LAB MEDIA: Figure 6A.

  - At 3,300 g for 20 minutes, the yield and quality of sucrose-loaded vesicles were optimal [1], whereas both lower and higher centrifugation speeds led to unfavorable outcomes such as reduced yield or aggregate formation [2].

    - LAB MEDIA: Figure 7B. Video editor: Highlight the clearly visible round vesicles scattered across the image.

    - LAB MEDIA: Figure 7A and C.

  - Mineral oil resulted in vesicles with a larger average diameter distribution [1], while silicone oil AR20 generated a significantly higher overall vesicle count [2].

    - LAB MEDIA: Figure 8A. Video editor: Highlight the peak of the mountain like curve.

    - LAB MEDIA: Figure 8B. Video editor: Highlight the peak of the mountain like curve.