# Submission ID #: 68369

# Scriptwriter Name: Pallavi Sharma

# Project Page Link: https://review.jove.com/account/file-uploader?src=20852593

# Title: Utilizing a 3D Printed Laparoscopic Nissen Fundoplication Model to Shorten a Resident’s Learning Curve

# Authors and Affiliations:

# Sidney Moses Amadi1,5#, Zhu Zhihao2#, Jinlei Mao5, Shijie Yu1, Chen Qi3, Ketrine Dessery5, Muhammad Osama Zada4, Menghui Zhou2, Mingzhun Xia2, Junfeng Hu1*, Zhifei Wang1*

# 1 General Surgery, Cancer Center, Department of Hernia Surgery, Zhejiang Provincial People’s Hospital

# 2The Second School of Clinical Medicine, Zhejiang Chinese Medical University

# 3School of Basic Medicine and Law, Hangzhou Medical College

# 4School of Basic Medical Sciences and Forensic Medicine, Hangzhou Medical College

# 5 International Education College Zhejiang Chinese Medical University

# Corresponding Authors:

# Zhifei Wang 			(zhifei1973@126.com)

# Junfeng Hu			(hjfeng234@126.com)

# Email Addresses for All Authors:

# Sidney Moses Amadi  		(sidneyogah@icloud.com)

# Zhu Zhihao  			(zhuzhihao1103@163.com)

# Jinlei Mao  			(maojl9817@163.com)

# Shijie Yu  			(yusj19981120@163.com)

# Chen Qi  			(1284861554@qq.com)

# Ketrin Desdery  		             (ketrindesdery@gmail.com)

# Muhammad Osama Wali Zada  	(osamawalizadaa@gmail.com)

# Menghui Zhou  		              (menghuizhou@126.com)

# Mingzhun Xia  				(xiaminjun0318@163.com)

# Zhifei Wang 				(zhifei1973@126.com)

# Junfeng Hu				(hjfeng234@126.com)

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon: https://review.jove.com/account/file-uploader?src=20852593

3. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  23

Number of Shots:  45

# Introduction

REQUIRED: What is the scope of your research? What questions are you trying to answer?

  - Zhifei Wang: Our research evaluates whether a realistic 3D-printed model can help surgical residents master complex laparoscopic surgery skills faster and more safely before they enter the operating room.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B roll: 3.2

What research gap are you addressing with your protocol?

  - Sidney Moses Amadi: Residents often lack confidence in advanced procedures after standard training. We address the gap between basic simulation and the complexities of real operating room performance and suturing.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

What advantage does your protocol offer compared to other techniques?

  - Ketrin Desdery: Our protocol uses a low-cost, reusable, and anatomically accurate model. It allows for risk-free practice of critical surgical steps, which isn’t always feasible with other methods.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B roll: 2.5

How will your findings advance research in your field?

  - Muhammad Osama Wali Zada: Our findings provide a validated training pathway that shortens the learning curve. This can directly lead to improved surgical proficiency, reduced complications, and ultimately, better patient outcomes.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Ethics Title Card

This research has been approved by the Zhejiang Provincial People's Hospital Institutional Review Board

# Protocol

- Preparing the 3D Printed LNF Model

Demonstrators: Shijie Yu, Muhammad Osama Wali Zada

  - After performing 3D computed tomography [1], convert the DICOM (Die-Com) profiles into STL (S-T-L) format [2]. Import the STL files into Magics 24 software by selecting File and then Import [3]. Run automatic fixing through the Menu bar, select Fix, and choose Auto Fix [4].

    - WIDE: Talent at a computer with the CT images displayed on the monitor. This shot was not taken.

    - SCREEN: The DICOM files are being converted to STL.

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=20852593

    - SCREEN; Show Magics 24 interface with the File > Import menu option being clicked to load the STL files.

    - SCREEN: Show the Fix > Auto Fix option being selected from the Menu bar.

  - To perform partial manual fixing, select Menu bar, Fix, and Manual Fixing [1]. Then, select Menu bar and Fix followed by Remove Noise Shells to remove noise shells and debris [2].

    - SCREEN: Show the Fix > Manual Fixing option being selected from the Menu bar and the interface for manual model adjustments.

    - SCREEN: Show the Fix > Remove Noise Shells option being selected and debris being removed from the model.

  - Afterward, select Menu bar, Orientation, and Create Support to add support structures [1]. Next, to export the repaired and supported model, select the Menu bar, File, Export, and STL [2].

    - SCREEN: Show Orientation > Create Support being selected and support structures appearing on the model.

    - SCREEN: Show the File > Export > cbddlp menu path being clicked.

  - Using a LCD 3D printer, Use the chitubox software settings: Click Slice Settings > Resin, layer thickness set to 0.1 mm, number of bottom layers set to 8, bottom layer lift speed set to 90 mm/min, and lift speed set to 80 min/min [1].

    - SCREEN: In the Chitubox Resin settings window, show; layer thickness to 0.1 mm, number of bottom layers to 8, bottom layer lift speed to 90 mm/min, and lift speed to 80 mm/min.

    - SCREEN: Show the Z-axis speed being set to 2 millimeters per second and the temperature control being set to 30 degrees Celsius.

    - SCREEN: Show the curing speed being set to 0.2 millimeters per second, the spot size being adjusted to 0.07 millimeters and the layer thickness being adjusted to 0.1 millimeters.

  - Now, clean the printed mold thoroughly in an ultrasonic cleaner using ethanol solvent at a frequency of 40 kilohertz for 3 to 5 minutes [1].

    - Talent placing the printed mold into an ultrasonic cleaner filled with ethanol solvent.

  - Complete the secondary curing of the mold in a UV chamber [1-TXT]. After curing, apply a thin layer of petroleum jelly to the inner surfaces of the mold components to act as a release agent.

    - Talent placing the mold inside a UV curing chamber and starting the curing process. TXT: Wavelength: 405; Intensity: 30 mW/cm²; Curing time: 30 min

  - Next, prepare two-component zero-degree silicone [1]. Place the mixture in an environment maintained at an ambient temperature of 22 to 25 degrees Celsius to prevent high temperatures from affecting the operational time [2].

    - Talent pouring Component A and 750 Component B into a mixing container.

    - Talent placing the mixed silicone in a temperature-controlled workspace at 22 to 25 degrees Celsius.

  - Place the silicone mixture into a vacuum chamber set to a vacuum level of minus 0.09 megapascal for a defoaming time of 8 minutes [1]. Pour the prepared silicone from the vacuum chamber into the organ mold [2] and observe to ensure that the silicone fills the mold evenly and that no large air bubbles are visible on the surface [3].

    - Talent positioning the silicone mixture inside a vacuum chamber and starting the defoaming process.

    - Talent pouring the degassed silicone into the organ mold.

    - Close-up of the silicone filling the mold evenly with no visible large air bubbles.

  - Allow the silicone to fully cure in an airtight environment maintained at approximately 25 degrees Celsius for 1 hour [1].

    - Talent placing the filled mold inside an airtight curing chamber with a visible temperature display.

  - After curing,. Carefully remove the mold to obtain the final silicone organ model [2].

    - Talent applying petroleum jelly evenly along the mold’s inner surfaces. (Please move shot 2.10.1 Applying petroleum jelly evenly after 2.6.1)

    - Talent separating the mold components to reveal the silicone liver model.

  - Examine the surface of the silicone model to ensure completeness and clarity of anatomical structures [1].

    - Close-up shot of the silicone liver model showing intact anatomical details.

- Assembling the LNF model

  - To begin, place the 3D printed organ models according to the corresponding anatomical layout [1].

    - WIDE: Talent arranging the 3D printed organ models on the workspace in the correct anatomical order.

  - Lay out the oesophagus attached to the stomach so that it enters the mediastinum through the opening between the crura and the gastric fundus [1].

    - Talent positioning the oesophagus and stomach, aligning them with the opening between the crura and gastric fundus.

  - Now, place the omentum, liver, and bile duct adjacent to the stomach, securing them in position with pins [1]. Then, place the skin onto the plastic platform, securing it with velcro and pins [2]. Connect the internal platform light to a power source to enhance visibility [3].

    - Talent positioning the omentum, liver, and bile duct next to the stomach and fixing them in place with pins.

    - Talent stretching and attaching the skin to the platform using velcro and pins. (Velcro and Pins not needed)

    - Talent plugging in the platform light to a power source and switching it on.

  - Next, make 3 incisions on the skin for ergonomic trocar positioning [1]. Insert a 5-millimeter trocar for the 30-degree laparoscope at the center to establish the triangle of vision [2]. Position two 10-millimeter trocars on either side for laparoscopic needle drivers, atraumatic graspers, and laparoscopic scissors [3].

    - Talent making 3 measured incisions on the skin.

    - Talent inserting a 5-millimeter trocar at the central incision.

    - Talent positioning two 10-millimeter trocars on each side of the central trocar.

  - Then, secure the laparoscope using a clamp [1] and connect it to the USB port of a high-definition display screen on either a television or laptop [2].

    - Talent fixing the laparoscope in place with a clamp.

    - Talent connecting the laparoscope cable to a laptop USB port and displaying the live feed.

  - Place a 2-0 silk suture with a CT-1 needle inside the model [1].

    - Talent positioning the 2-0 silk suture with a CT-1 needle within the surgical model.

- Training Curriculum

  - Gently pass a Penrose drain behind the already mobilized posterior esophagus to serve as a retractor [1]. Apply gentle traction to the drain to retract the esophagus, ensuring clear visualization of the right and left crura of the diaphragm [2].

    - Talent using a grasper to guide the Penrose drain into position behind the esophagus..

    - Talent applying gentle traction on the Penrose drain, retracting the esophagus to clearly expose both crura of the diaphragm.

  - Then, identify the bilateral diaphragmatic crura and place five interrupted sutures to approximate them securely without tension [1]. Verify that the crura are firmly closed and adequately aligned to ensure proper approximation of the diaphragmatic pillars [2].

    - Talent shows the bilateral diaphragmatic crura and placing the first of five interrupted sutures between the crura.

    - Close-up of the closed crura showing even approximation without tension.

  - Next, place the mesh symmetrically on the diaphragm [1]. Suture the sides of the mesh evenly to the diaphragm [2] to ensure it lies flat without wrinkling while maintaining a clear margin between the mesh and the esophageal wall [3].

    - Talent positioning the mesh evenly over the diaphragm.

    - Talent suturing one side of the mesh to the diaphragm.

    - Close-up showing the mesh secured flat with an even margin from the esophagus.

  - Pass the bougie through the esophagus into the stomach, [1] Perform the ‘shoe shine manoeuvre’ to mobilize and align the gastric fundus [1]. Wrap the fundus over 2 to 3 centimeters of the oesophagus, ensuring a floppy, tension-free, 360-degree fundoplication wrap no longer than 2 centimeters [2]. (We pass the bougie pass through the esophagus then we proceed to performing the ‘shoe shine manouvre’)

    - Talent demonstrating the ‘shoe shine manoeuvre’ with the gastric fundus.

    - Close-up of the gastric fundus wrapping evenly around the oesophagus with no visible tension.

  - Then, close the wrap with three interrupted sutures over a 1.5-centimeter length [1-TXT] and pass a bougie to confirm the wrap is not too tight [2].

    - Talent placing the first of three interrupted sutures on the fundoplication wrap. TXT: Ensure secure, even sutures to maintain wrap integrity

    - Talent passing a bougie through the esophagus to the stomach to confirm appropriate looseness. (Please move shot 4.5.2 Passing a bougie through the esophagus to the stomach together with 4.4.1 )

    - Added Shot: Final inspection to ensure the wrap is firmly done.

  - Conduct a final assessment of live operating room performance under expert surgeon supervision, ensuring all steps meet procedural standards [1-TXT].

    - Expert surgeon observing and evaluating the trainee performing the final steps of the procedure in a simulated OR setting. TXT: Assess the 3D printed LNF model training platform

# Results

- Results

  - The experimental group achieved significantly higher OSATS (O-Sats) scores compared to the control group [1], while also completing the procedure in a notably shorter duration [2].

    - LAB MEDIA: Figure 3A. Video editor: Highlight the taller purple bar labeled "Experimental Group

    - LAB MEDIA: Figure 3B. Video editor: Highlight the shorter purple bar labeled "Experimental Group"

  - Training session performances showed a consistent increase in total scores from session 1 to session 6 [1], reaching values close to the expert benchmark [2].

    - LAB MEDIA: Figure 4A.

    - LAB MEDIA: Figure 4A. Video editor: Highlight the similar height of the blue and orange dots labeled "OR" and "Expert"

  - Median performance scores improved steadily across training sessions [1], starting from 4.5 in session 1 and reaching 9.0 in session 6 [2].

    - LAB MEDIA: Table 1. Video editor: Highlight the row “Training Session 1” under “Median IQR” showing 4.5 (3.0–9.75).

    - LAB MEDIA: Table 1. Video editor: Highlight the row “Training Session 6” under “Median IQR” showing 9.0 (6.75–18.75).

  - A strong inverse relationship was observed between procedure completion time and total score, with completion time decreasing as scores increased [1].

    - LAB MEDIA: Figure 4B.

  - Expert evaluations confirmed the model was easy to use [1], highly suitable for surgical training [2], and effective in improving surgical skills [3].

    - LAB MEDIA: Table 2. Video editor: Highlight the expert rating value of 4.52 ± 0.50 for “The model is easy to use.”

    - LAB MEDIA: Table 2. Video editor: Highlight the expert rating value of 4.80 ± 0.45 for “Using model in surgical training is reasonable.”

    - LAB MEDIA: Table 2. Video editor: Highlight the expert rating value of 4.28 ± 0.08 for “The model can help improve surgical skills.”