# Submission ID #: 68462

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=20880958

# Title: Development of an Innovative LED-Based Illumination Device for In Vitro Application of Photodynamic Therapy with Rose Bengal

# Authors and Affiliations:

# Clément Bouchez1*, Pascal Deleporte1*, Gregory Baert1, Anthony Lefebvre1, Olivier Moralès1,2, Nadira Delhem1, Guillaume Paul Grolez1*, Anne-Sophie Dewalle1*

# 1Inserm, CHU Lille, U1189—ONCO-THAI—Assisted Laser Therapy and Immunotherapy for Oncology, University of Lille

# 2CNRS, Inserm, CHU Lille, UMR9020-U1277—CANTHER—Cancer Heterogeneity Plasticity and Resistance to Therapies, University of Lille

*These authors contributed equally

# Corresponding Authors:

# Nadira Delhem		nadira.delhem@inserm.fr

# Email Addresses for All Authors:

# Clément Bouchez		clement.bouchez1@inserm.fr

# Pascal Deleporte		pascal.deleporte@inserm.fr

# Gregory Baert			gregory.baert@inserm.fr

# Anthony Lefebvre		anthonylefebvre53@gmail.com

# Olivier Moralès		olivier.morales@cnrs.fr

# Nadira Delhem		nadira.delhem@inserm.fr

# Guillaume Paul Grolez		 guillaume.grolez@inserm.fr

# Anne-Sophie Dewalle		anne-sophie.dewalle@inserm.fr

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  22

Number of Shots:  43

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

REQUIRED: What is the scope of your research? What questions are you trying to answer?

  - Guillaume Grolez: The scope of our research is to develop fundamental and translational projects aiming to characterize the effects of Photodynamic Therapy in the treatment of cancers with no effective therapeutic options.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.3.1

What are the current experimental challenges?

  - Anne-Sophie Dewalle: The current experimental challenges are to develop the technologies necessary for the implementation of Photodynamic Therapy from in vitro to clinic applications, including new photosensitizers and new illumination devices.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 4.2.1

What advantage does your protocol offer compared to other techniques?

  - Anne-Sophie Dewalle: Compared to other techniques, our protocol involves a low-cost and homemade device enabling homogeneous illumination of a 96-well plate under physiological conditions inside a cell culture incubator.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.3.1

What research questions will your laboratory focus on in the future?

  - Guillaume Grolez: In the future, one of our main objectives is to develop new PDT packages each comprising both a photosensitive compound enabling specific cancer cells targeting and an associated illumination device.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 5.2.1

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

- Seeding the Cells before Treatment

Demonstrator: Clément Bouchez

  - To begin, remove the cultured HepG2 (hep-G-2) cells from the incubator [1] and place the flask under the pre-cleaned microbiological safety station [2].

    - WIDE: Talent opening the incubator and retrieving the culture flasks.

    - Talent placing the culture flask in the microbiological safety station.

  - Using a pipette, remove the culture medium from the culture flask [1] and wash it with phosphate buffer solution to remove residual medium [2].

    - Talent aspirating the medium from the culture flask inside the safety cabinet.

    - Talent pipetting phosphate buffer solution into the flask and gently swirling before aspirating it out.

  - Then, apply 3 milliliters of trypsin solution containing 0.25 percent trypsin and 0.53 millimolar EDTA to the cells [1] and place the culture back in the incubator at 37 degrees Celsius for 5 minutes to detach the cells [2].

    - Talent adding trypsin to the culture flask under the safety station.

    - Talent placing the treated flask inside the incubator and closing the door.

  - After 5 minutes, add 7 milliliters of culture medium to the flask to neutralize the trypsin and suspend the cells [1]. Now, transfer the trypsin, medium, and detached cells into a 15-milliliter centrifuge tube [2].

    - Talent pipetting medium into the flask and mixing gently.

    - Talent transferring the cell suspension into labeled 15 milliliter tubes.

  - To count the cells, pipette 20 microliters of the cell solution into a clean tube [1] and add 20 microliters of trypan blue solution [2]. After mixing, load the stained mixture onto the counting slide [3] and insert it into the cell counter [4].

    - Talent dispensing 20 microliters of cell suspension into a microtube.

    - Talent adding 20 microliters of trypan blue and mixing with a pipette.

    - Talent loading the stained sample onto the slide.

    - Talent inserting the slide into the cell counter.

  - Next, dilute the cells in culture medium to achieve a concentration of 150,000 cells per milliliter [1]. Seed 1.5 × 10⁴ cells per well into a white 96-well plate with a clear flat bottom [2]. Prepare two plates for each viability reading time [3].

    - Talent pipetting culture medium into a tube containing cells to dilute them to the target concentration.

    - Talent dispensing 1.5 × 10⁴ cells into each well of the 96-well plate.

    - Talent labeling and setting aside two plates for each time point.

  - Incubate the plates for 24 hours before treatment to let the cells attach [1].

    - Talent placing the 96-well plates into the incubator and closing the door.

- Rose Bengal Treatment of the Cells

  - To prepare a stock solution of Rose Bengal, dissolve it in 10 percent saline solution [1].

    - WIDE: Talent adding Rose Bengal powder to a tube containing 10 percent saline solution.

  - Dilute the Rose Bengal treatment solutions at concentrations ranging from 0 to 100 micromolar in cell culture medium [1-TXT].

    - Talent pipetting stock solution into multiple tubes arranged on the bench. TXT: Concentrations: 0, 5, 10, 25, 50, 75 and 100 µM

  - Now, remove the culture medium from the cell plates [1] and add 100 microliters of the prepared Rose Bengal treatment solutions to each well [2].

    - Talent aspirating the medium from each well of the 96-well plate.

    - Talent pipetting 100 microliters of each Rose Bengal concentration into the corresponding wells.

  - Incubate the cells with Rose Bengal for 2 hours to allow internalization [1].

    - Talent placing the treated microplates into the incubator and closing the door.

  - After 2 hours, remove the Rose Bengal solution from each well [1], wash the cells two times with PBS [2], and add 100 microliters per well of Rose Bengal-free culture medium [3]. Cover the microplates with aluminum foil to protect them from light [4] and set aside half of the plates for illumination during the photodynamic therapy assay [5].

    - Talent aspirating Rose Bengal solution from the well.

    - Talent adding phosphate-buffered saline to a well.

    - Talent adding fresh medium to the wells.

    - Talent covering microplates with aluminum foil.

    - Talent labeling and grouping plates for photodynamic therapy and dark conditions.

- Operating Procedure for the CELL-LED Device

  - Connect the male connector on the light distributor to the female connector of the light source [1].

    - Talent aligning and connecting the light distributor cable to the LED light source.

  - Remove the photodynamic therapy and dark condition microplates from the incubator [1]. Unwrap the photodynamic therapy plate [2] and place it on the light distributor [3].

    - Talent retrieving the plates from the incubator.

    - Talent unwrapping the plate that was covered with foil.

    - Talent placing the photodynamic therapy plate onto the aligned light distributor panel.

  - Then, set the dimmer on the LED driver to the maximum level to deliver an average irradiance of 0.62 milliwatt per square centimeter across the 96 wells [1]. Illuminate the microplate until the desired light dose on the cells is achieved [2].

    - Show the LED driver control panel with settings being adjusted.

    - Shot of the plate being illuminated.

  - Once the desired light dose is reached, turn off the device [1]. Rewrap the photodynamic therapy microplate in aluminum foil [2] and return it along with the control plate to the incubator until the viability test is performed [3].

    - Talent switching off the LED system.

    - Talent covering the photodynamic therapy plate again with foil.

    - Talent placing both plates back into the incubator.

- Cell Viability Assay After Treatment

  - After completing the photodynamic therapy assay, place the illuminated plate in the incubator for 24 hours to allow for post-treatment cellular response [1].

    - Shot of the plate lying inside the incubator.

  - After the incubation period, retrieve one illuminated and one non-illuminated plate [1]. Add 100 microliters of reagent from the cell viability assay kit into each well to measure mitochondrial metabolism and ATP production [2].

    - Talent removing both microplates from the incubator.

    - Talent using a multichannel pipette to dispense 100 microliters of reagent into every well of both plates.

  - Incubate the plates in the dark for 10 minutes before proceeding with luminescence measurement [1].

    - Talent placing the foil-covered plates into a dark incubation chamber.

  - After 10 minutes, read the luminescence in each well using a multimodal reader [1].

    - Talent loading the plate into the luminescence reader.

  - Consider the luminescence from the untreated control wells as representing 100 percent viability [1]. Normalize the luminescence values from the treated wells to this control to calculate the percentage of viability for each treatment condition [2].

    - Talent working at a computer entering values.

    - Shot of the computer screen displaying the resulting percentage viability values, grouped by treatment concentration and condition.

  - Repeat the same protocol for cell viability measurement at additional post-treatment time points such as 24 hours, 48 hours, and 72 hours [1].

    - Talent removing the plate from the reader and placing another one.

# Results

- Results

  - Illumination alone, without Rose Bengal, did not alter HepG2 cell viability at any of the tested light doses [1], and Rose Bengal alone, without light, also caused no significant change across all concentrations [2].

    - LAB MEDIA: Figure 5. Video editor: Highlight the three blue BARS under “Light”.

    - LAB MEDIA: Figure 5. Video editor: Highlight the seven grouped bars under the “RB” label .

  - Rose Bengal-mediated photodynamic therapy using CELL-LED-550/3 (cell-L-E-D-five-fifty-bar-3) significantly reduced HepG2 cell viability at all light doses, showing a strong cytotoxic effect [1].

    - LAB MEDIA: Figure 5. Video editor: Highlight the bars corresponding to 25,50, 75 and 100 µM under the label “PDT” for 0.30 and 0.60 J/cm2 and highlight all bars for 1.22 J/cm2 (These bars have ** on them).

  - The overlap between the LED emission profile and the absorption spectrum of Rose Bengal confirmed the spectral compatibility of the CELL-LED-550/3 device with the photosensitizer [1].

    - LAB MEDIA: Figure 6. Video editor: Highlight the peak of the green line (LED spectrum) overlapping with the peak of the pink line (Rose Bengal spectrum) in the 550 nanometer region.