# Submission ID #: 68470

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=20883398

# Title: Upper-Extremity Approach for Secondary Access in Transfemoral Transcatheter Aortic Valve Implantation

# Authors and Affiliations:

# Geert A.A. Versteeg1, Maxim J.P. Rooijakkers1, Niels A. Stens1,2, Marleen H. van Wely1, Robert-Jan M. van Geuns 1, Leen A.F.M. van Garsse3, Guillaume S.C. Geuzebroek3, Michel W.A. Verkroost3, Dick H.J. Thijssen2,4, Robin H. Heijmen3, Lokien X. van Nunen1, Niels van Royen1

# 1Department of Cardiology, Radboud University Medical Center

# 2Department of Medical BioSciences, Radboud University Medical Center

# 3Department of Cardiothoracic Surgery, Radboud University Medical Center

# 4Research Institute for Sport and Exercise Sciences, Liverpool John Moores University

# Corresponding Authors:

# Geert A.A. Versteeg				Geert.versteeg@radboudumc.nl

# Email Addresses for All Authors:

# Maxim J.P. Rooijakkers			max.rooijakkers@radboudumc.nl

# Niels A. Stens					niels.stens@radboudumc.nl

# Marleen H. van Wely				marleen.vanwely@radboudumc.nl

# Robert-Jan M. van Geuns			robertjan.vangeuns@radboudumc.nl

# Leen A.F.M. van Garsse			leen.vangarsse@radboudumc.nl

# Guillaume S.C. Geuzebroek			guillaume.geuzebroek@radboudumc.nl

# Michel W.A. Verkroost			michel.verkroost@radboudumc.nl

# Dick H.J. Thijssen				dick.thijssen@radboudumc.nl

# Robin H. Heijmen				robin.heijmen@radboudumc.nl

# Lokien X. van Nunen				lokien.vannunen@radboudumc.nl

# Niels van Royen				niels.vanroyen@radboudumc.nl

# Geert A.A. Versteeg				Geert.versteeg@radboudumc.nl

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of computer/software usage?  Yes

Videographer: Please record the computer screen for the shots involving fluoroscope and ultrasound

3. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  24

Number of Shots:  43 (11 SC)

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

REQUIRED: What is the scope of your research? What questions are you trying to answer?

  - Geert Versteeg: Our goals include refining our TAVI practices and improve overall procedural outcomes. One of our aims was to reduce access site complications and facilitate early mobilization and quick rehabilitation.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.3.1

What advantage does your protocol offer compared to other techniques?

  - Geert Versteeg: Inserting a temporary pacing lead in a manner that allows for early patient mobilization does not only enhance clinical outcomes but also increases satisfaction among both patients and nursing staff.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 4.2.1

What research questions will your laboratory focus on in the future?

  - Geert Versteeg: Our current goal is to enhance diagnostic accuracy for paravalvular regurgitation. By utilizing invasive hemodynamic measurements and developing a real-time algorithm, we aim to more effectively detect and reduce paravalvular regurgitation.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 5.1.1

Videographer: Obtain headshots for all authors available at the filming location.

Ethics Title Card

This research has been approved by the Medical Research Ethics Committee Oost-Nederland and the review boards at each participating site

# Protocol

- Patient and the Access Site Preparation

Demonstrators: Niels van Royen and Guillaume Geuzebroek

  - To begin, instruct the patient to lie in a supine position on the catheterization table [1]. Fully extend the patient’s left arm at a 90-degree angle relative to the thorax, maintaining the arm in the same vertical plane [2].

    - WIDE: Talent guiding the patient to lie on the catheterization table in a supine position.

    - Talent positioning the patient’s left arm at a 90-degree angle, aligned with the thorax.

  - Then, support the patient’s extended arm using an armrest [1]. Instruct the patient to overextend and supinate the arm to present the medio-ventral side of the upper arm to the operator [2].

    - Talent placing an armrest under the patient’s extended arm.

    - Talent demonstrating and the patient supinating and overextending the arm to expose the medio-ventral upper arm.

  - Disinfect the entire upper arm using either chlorhexidine or povidone-iodine to prepare the access site [1] and wrap a tourniquet around the upper arm as proximally as possible to enhance vein visibility [2].

    - Talent swabbing the patient’s upper arm with disinfectant in a circular motion.

    - Talent tying a tourniquet high on the upper arm and tightening it to make the veins more visible.

  - Place a sterile cover that exposes only the disinfected upper arm to create a sterile field [1]. Now, prepare for ultrasound-guided puncture by using a vascular probe, covering it with a sterile probe cover, and applying ultrasound gel [2].

    - Talent laying down a sterile drape and adjusting it to reveal only the disinfected area of the arm.

    - Talent fitting a sterile cover over the ultrasound probe and applying ultrasound gel to the patient’s upper arm.

- Access Site Puncture for Upper-Extremity Temporary Pacing Lead Placement

  - Use ultrasound to locate a suitable vein for puncture, typically found 5 to 10 centimeters proximal to the elbow crease [1]. Identify the basilic vein on the medio-ventral side or the cephalic vein on the lateral side as common choices [2-TXT].

    - Talent scanning the upper arm with ultrasound to locate veins.

    - Talent pointing to the ultrasound view showing vein options. TXT:  Alternatively, locate the brachial vein if necessary

  - To evaluate the vein’s suitability, check for proximity to critical structures that could lead to complications [1]. If the vein lies close to an artery, trace the vein proximally or distally to locate a safer puncture site [2].

    - Shot of the Ultrasound monitor showing a vein adjacent to an artery.

    - Shot of the Ultrasound monitor showing vein being followed proximally to find a safer position.

  - Choose a vein based on depth, giving preference to superficial veins and select a vein with a larger diameter for easier puncture [1]. Ensure that the vein’s location allows arm flexion without interfering with the pacing lead, avoiding sites too close to the elbow crease [2].

    - Shot of the Ultrasound monitor showing the scan and pointing to a shallow vein.

    - Talent moving ultrasound probe slightly more proximal to the elbow crease.

  - After selecting a suitable vein, use the ultrasound probe to compress it and confirm it is a vein by ensuring it collapses under pressure [1]. Infiltrate the subcutaneous tissue at the desired access site with 1 to 2 milliliters of 1 percent lidocaine solution [2].

    - Talent compressing the vessel, showing vein collapse.

    - Talent injecting lidocaine at the selected puncture site using a syringe.

  - Using direct ultrasound guidance, puncture the vein with a hollow needle from the 6 French sheath kit and confirm backflow [1].

    - Talent puncturing the vein under ultrasound guidance with the hollow needle and confirming backflow through the hollow needle.

  - Immediately insert a guidewire through the hollow needle and advance it [1]. Then, remove the tourniquet and withdraw the hollow needle over the wire [1].

    - Talent feeding the guidewire through the needle and advancing it further.

    - Talent removing the tourniquet and then withdrawing the needle while leaving the guidewire in place.

  - Next, flush the sheath and side port thoroughly to ensure no air remains in the system [1]. Introduce the 6 French sheath over the guidewire and then remove the guidewire and dilator, creating a 6 French lumen [1].

    - Talent flushing both the sheath and the side port with saline using a syringe.

    - Talent advancing the sheath over the wire, then removing the wire and dilator to establish the lumen.

  - Introduce a 5 French flow-directed pacing catheter with the balloon deflated through the sheath and advance the pacing lead further [1].

    - Talent inserting the pacing catheter into the sheath with the balloon tip deflated and beginning to advance the lead.

- Imaging Guidance Set-Up and Lead Advancement

  - Use fluoroscopy in an anterior-posterior 0-degree view to guide the pacing lead toward the right ventricle [1]. Continue advancing the pacing lead so it naturally follows the vascular path without resistance [2].

    - Shot of fluoroscopy monitor with pacing lead advancing toward the right ventricle under anterior-posterior view. Authors, videographer will capture the monitor as you perform these steps

    - Shot of Fluoroscopy monitor showing the pacing lead smoothly progressing through the vasculature.

  - Once the lead tip reaches the subclavian vein, inflate the balloon to help steer it into the right ventricle [1]. If the pacing lead veers into the contralateral subclavian vein or jugular vein, withdraw it slightly [2] and rotate the angulated tip before re-advancing [3].

    - Talent inflating the balloon on the pacing catheter.

    - Optional: Talent withdrawing the lead. Authors: Please let us know if these optional steps were filmed or not after the shoot. If not filmed, then we can delete these shots.

    - Optional: Talent adjusting and rotating the tip before readvancing.

  - To pass through the tricuspid valve, rotate the pacing lead and guide the balloon tip through the valve while inflated [1].

    - Talent rotating the lead.

  - After the lead contacts the right ventricular apex, deflate the balloon [1].

    - Shot of Fluoroscopy screen showing lead tip at the apex.

    - Talent deflating the balloon tip of the catheter.

- Connecting, Testing and Fixing the Pacing Lead

  - When the pacing lead is properly positioned in the right ventricle, connect the electrode plugs to their corresponding ports on the adapter [1].

    - Talent connecting the pacing lead’s electrode plugs to the appropriate adapter ports.

  - Perform a capture threshold test by pacing at 20 beats per minute above the intrinsic heart rate at maximum pacemaker output [1]. Gradually reduce the output amperage until malcapture is observed, indicated by irregular heartbeat or a drop in heart rate below the pacemaker setting [2]. Ensure that the pacemaker output setting exceeds at least twice the measured threshold value [3].

    - Talent showing external pacemaker and adjusting pacing rate to 20 beats per minute above intrinsic heart rate and output at maximum.

    - Shot of the monitor showing the graph with irregular heart activity as the output is reduced, indicating malcapture.

    - Talent shows the pacemaker setting adjusted to exceed twice the identified threshold.

  - If the threshold exceeds 3 milliamperes, reposition the pacing lead to achieve a lower threshold [1].

    - Optional: Fluoroscopy view showing pacing lead being slightly adjusted or repositioned within the right ventricle.

  - Now, fixate the temporary pacing lead using a large transparent film dressing that covers the access site and approximately 10 centimeters of the external portion of the pacing lead [1].

    - Talent positioning the external part of the pacing lead and applying a transparent film dressing over the access site and external pacing lead. TXT: Lead fixation: Rolled up fashion

  - Ensure proper fixation of the pacing lead to support extended pacing or as a backup, allowing the patient to move while keeping the lead in place [1].

    - Talent checking lead stability by gently manipulating the arm and verifying the lead remains secured.

  - If the pacing lead needs to be removed, confirm the balloon is deflated and gently retrieve the lead using slight traction [1].

    - Talent checking if the balloon is deflated and pulling the pacing lead out slowly.

  - Keep the 6 French sheath in place until the activated clotting time returns to normal levels [1]. Once normalized, manually remove the sheath [2].

    - Talent pointing to the the ACT measurements.

    - Talent manually withdrawing the sheath after ACT is confirmed normal.

  - Finally, apply manual pressure on the access site for 2 minutes [1]. Afterward, place a sterile bordered gauze or island dressing [2] and use a compression bandage if necessary [3].

    - Talent compressing the access site with gloved fingers while monitoring time.

    - Talent placing sterile dressing.

    - Optional: Talent applying a light compression bandage.

# Results

- Results

  - In the trial, 238 patients were randomized between the upper-extremity and the lower-extremity cohort [1]. Clinically relevant bleeding events were significantly lower in the upper-extremity group [2] compared to the lower-extremity group [3].

    - LAB MEDIA: Figure 5

    - LAB MEDIA: Figure 5. Video editor: Highlight the two "upper extremity" bars.

    - LAB MEDIA: Figure 5. Video editor: Highlight the two bars for "lower extremity".

  - When the upper-extremity approach was used, time to mobilization was significantly shorter in patients requiring extended temporary pacing lead placement at around 530 minutes [1] versus 1415 minutes for the lower-extremity group [2].

    - LAB MEDIA: Figure 6. Video editor: Highlight the triangle in the BLUE bar in the "Pacing lead maintained" section corresponding to “upper extremity”

    - LAB MEDIA: Figure 6. Video editor: Highlight the triangle in the RED bar in the "Pacing lead maintained" section corresponding to “lower extremity”