Submission ID #:68476

Scriptwriter Name: Sulakshana Karkala

Project page Link: https://review.jove.com/account/file-uploader?src=20885228

Title: Modification of a 3D Printed Flexipeg Device to Model Catheter-Associated Urinary Tract Infection

Q: Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  19

Number of Shots:  41

# Protocol

Protocol

  - To begin, obtain pooled human urine sample [1]. Aliquot 135 microliters of pooled human urine into each on-test well of a 96-well plate following the designated layout [2-TXT].

    - WIDE: Talent holding a tube with pooled human urine sample.

    - Talent pipetting pooled human urine into the wells of a 96-well plate as per the layout. TXT: Maintain 3 technical replicates per biological replicate

  - Inoculate each on-test well with 15 microliters of normalized overnight bacterial culture to achieve a final volume of 150 microliters per well [1]. Reserve at least 100 microliters of this culture for subsequent serial dilution validation steps [2].

    - Talent adding overnight culture to each well using a micropipette.

    - Talent transferring 100 microliters of overnight culture for later use.

  - Prepare negative control wells by adding 150 microliters of sterile pooled human urine into designated wells [1].

    - Talent filling control wells with sterile urine.

  - Now, insert the pegs into their corresponding wells [1]. Press down the peg lid until it sits flush on the plate [2] and secure it with the microplate lid, taping it down if necessary [3].

    - Talent inserting the peg lid into the wells of the microplate.

    - Talent pressing down on the peg lid.

    - Talent covering the peg lid with the microplate lid and securing it with tape.

  - Then place the peg models in a sealed plastic tub or bag containing a damp blue roll to prevent evaporation [1]. Incubate statically for 24 hours at 37 degrees Celsius [2].

    - Talent placing the sealed plate setup into a humidified container.

    - Talent placing the setup in an incubator.

  - For inoculum validation, aliquot 180 microliters of PBS into rows 2 through 6 of the first column of a 96-well plate [1]. Add 100 microliters of reserved overnight culture to the first well in the column [2].

    - Talent pipetting phosphate-buffered saline into designated wells of a new microplate.

    - Talent adding reserved overnight culture to the first well.

  - Now, transfer 20 microliters from the first well into the second well, mixing gently by pipetting up and down [1]. Continue serial dilutions until the initial inoculum has been diluted to the final well [2].

    - Talent performing serial dilution by transferring and mixing the culture in successive wells

    - Talent completing the dilution series up to the final well.

  - Then plate 10 microliter spots from each dilution onto appropriate solid media in triplicate [1]. After overnight incubation at 37 degrees Celsius, count the colonies from each spot [2-TXT].

    - Talent transfer 10 µL spots from each dilution onto appropriate solid media.

    - Talent counting colonies on the agar plate. TXT: Calculate CFU/mL inoculum

  - For biofilm quantification, remove the peg model from the microplate [1] and invert it under a flame or inside a microbiological safety cabinet, with pegs facing upwards [2]. Discard the used microplates [3].

    - Talent removing the peg lid from the microplate.

    - Talent inverting it under a flame.

    - Talent disposing of the used microplate in a designated biohazard bin.

  - Transfer the peg lid to a fresh microplate containing 180 microliter aliquots of sterile PBS to rinse off unadhered cells [1-TXT].

    - Talent placing peg lid into a plate with 180 µL PBS. TXT: Repeat wash in fresh PBS 2x

  - Next, working aseptically, place a sterile glass test tube rack onto the bench [1] and attach the fitted rack top that holds the peg lid [2].

    - Talent placing sterile glass test tube rack onto the bench.

    - Talent attaching the rack top under aseptic conditions.

  - With a micropipette, aliquot 600 microliters of sterile PBS into each test tube [1]. Now, insert the peg lid into the rack, ensuring the pegs are aligned with the test tubes [2]. Remove any autoclave tape from the lid [3].

    - Talent pipetting sterile phosphate-buffered saline into the glass tubes.

    - Talent placing the peg lid over the rack and aligning the pegs with tubes.

    - Talent peeling off autoclave tape from the peg lid.

  - Use sterile forceps to push each peg through the lid into the test tubes directly below [1]. Then replace the test tube caps [2] and vortex each tube for 60 seconds to dislodge biofilm-associated cells [3].

    - Talent using forceps to insert pegs into tubes

    - Talent capping the tubes.

    - Talent vortexing each tube.

  - Remove 100 microliters from each vortexed tube [1] and perform serial dilutions in sterile PBS as done previously [2].

    - Talent pipetting out 100 µL from a vortexed tube.

    - Talent performing serial dilution of the vortexed biofilm samples.

  - Now use a serological pipette to spot 10 microliters of each dilution on solid media in triplicate and incubate [1-TXT]. The next day, count the colonies on each plate and calculate the colony-forming units per milliliter for biofilm-associated cells [2].

    - Talent plating dilution spots. TXT: Incubation: 37 °C, overnight

    - Talent counting colonies on biofilm dilution plates.

  - To stain the cells with crystal violet, pipette 180 microliters of 0.1 percent weight per volume crystal violet in sterile distilled water into the 24 on-test wells of a 96-well plate [1]. Insert the rinsed peg model into the corresponding wells and incubate at room temperature for 20 minutes [2].

    - Talent pipetting crystal violet solution into designated wells of the microplate.

    - Talent placing the rinsed peg lid into the wells.

  - After staining, remove the peg model from the plate and discard the used plate [1]. In a clean tub, rinse the entire peg model in running tap water with gentle agitation [2].

    - Talent lifting peg model and discarding the stained plate.

    - Talent gently swirling the peg lid in a container of tap water.

  - After 2 more washes, invert the peg model and let it dry completely at room temperature [1]. Then add 180 microliters of 30% acetic acid to a new 96-well plate [2. Insert the dried peg model into the corresponding wells [3-TXT].

    - Talent placing the peg lid inverted on a clean surface to dry.

    - Talent pipetting acetic acid solution into a fresh plate.

    - Talent placing the dry peg lid into the wells and setting a timer. TXT: Incubation: RT, 30 min

  - When incubation is complete, remove the peg model from the microplate [1]. Measure the optical density of the stained wells at 540 nanometers using a microplate reader or spectrophotometer [2-TXT].

    - Talent removing the peg model from the microplate.

    - Talent placing the plate in a microplate reader. TXT: Subtract OD540nm of stained negative control pegs from average on-test pegs for biofilm formation