# Submission ID #: 68498

# Scriptwriter Name: Sulakshana Karkala

# Project Page Link: https://review.jove.com/account/file-uploader?src=20891938

# Title: A Multimodal Imaging Framework to Advance Phenotyping of Living Label-Free Breast Cancer Cells

# Authors and Affiliations:

# Leonardo Bianchi1,2,3, Arianna Bresci2, Koseki J. Kobayashi-Kirschvink2,4, Gabriela Paroni5, Paola Saccomandi3, Peter T. C. So1,2,6, Jeon Woong Kang2*

# 1Department of Mechanical Engineering, Massachusetts Institute of Technology

# 2Laser Biomedical Research Center, G. R. Harrison Spectroscopy Laboratory, Massachusetts Institute of Technology

# 3Department of Mechanical Engineering, Politecnico di Milano

# 4Klarman Cell Observatory, Broad Institute of MIT and Harvard

# 5Department of Molecular Biochemistry and Pharmacology, Istituto di Ricerche Farmacologiche Mario Negri IRCCS

# 6Department of Biological Engineering, Massachusetts Institute of Technology

# Corresponding Authors:

Leonardo Bianchi			(lbianchi@mit.edu)

# Email Addresses for All Authors:

Leonardo Bianchi			(lbianchi@mit.edu)

Arianna Bresci				(abresci@mit.edu)

Koseki J. Kobayashi-Kirschvink	(kjkk@mit.edu)

Gabriela Paroni			(gabriela.paroni@marionegri.it)

Paola Saccomandi			(paola.saccomandi@polimi.it)

Peter T. C. So				(ptso@mit.edu)

Jeon Woong Kang			(jwkang76@mit.edu)

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes, all done

3. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  29

Number of Shots:  59

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

  - Leonardo Bianchi: We present multimodal imaging using confocal Raman micro spectroscopy and tomographic phase microscopy for rapid and unbiased morpho chemical phenotyping of live breast cancer cells in their native environment.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Video editor: Please include the affiliation of the author as given:
Leonardo Bianchi

Massachusetts Institute of Technology and Politecnico di Milano

What advantage does your protocol offer compared to other techniques?

  - Leonardo Bianchi: Combining Raman spectroscopy and tomographic phase microscopy enables label-free, physiological-condition imaging, offering detailed biochemical and morphological cell data while avoiding perturbations from fluorescence labeling or chemical fixation.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:6.4

How will your findings advance research in your field?

  - Leonardo Bianchi: Our multimodal approach enables detailed morpho-chemical profiling of cancer cells. Its versatility, reproducibility, and non-invasiveness support broad biomedical applications, from basic cell biology to diagnostic research.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:6.3

What research questions will your laboratory focus on in the future?

  - Jeon Woong Kang: We aim to create an atlas of optical biomarkers for non-invasive embryonic quality control, detecting cellular senescence, and drug screening in organoid models to advance regenerative medicine and disease research.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. 
Video editor: Please include the affiliation of the author as given:

Jeon Woong Kang

# Massachusetts Institute of Technology

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

- Raman Imaging of MDA-MB-231 Cells

Demonstrator: Leonardo Bianchi

  - To begin, place the MDA-MB-231 (M-D-A-M-B-Two-Three-One) human breast cancer cells sample into the onstage incubator [1-TXT]. Add water for the water immersion objective [2].

    - WIDE: Talent placing the biological sample into the onstage incubator. TXT: Incubation: 37 °C, Humidity > 95%, 5% CO2

Added shot: Talent pipetting water for the water immersion objective.

  - Switch on the automated water-immersion feeder to supply water to the objective lens [1]. Regularly check the water levels to avoid evaporation and ensure consistent imaging conditions throughout the measurements [2].

    - Talent pressing the power button on the water-immersion feeder.

    - Talent checking and refilling the water reservoir.

  - Now, turn on the pump laser and adjust its output to attain a laser power of 75 milliwatts at the sample plane [1]. Open the Micro-manager (Micro-manager) microscope control software [2].

    - Talent turning on the pump laser and adjusting power output dial.

    - SCREEN: 68498_screenshot_1.mp4	00:00-00:08.

  - In the Configuration settings, select BF (B-F) and click Live on the left side of the software window [1]. Choose the desired single cell, then click Stop to end the bright-field visualization [2].

    - SCREEN: 68498_screenshot_2.mp4.	00:00-00:06

    - SCREEN: 68498_screenshot_2.mp4.	00:07-00:14

  - Open the CCD camera control software [1]. In the Experiment Setting, set the Exposure Time to 1.5 seconds [2]. Click Run on the Experiment toolbar [3] and turn on the laser beam [4].

    - SCREEN: 68498_screenshot_3.mp4	00:00-00:08

    - SCREEN: 68498_screenshot_3.mp4	00:10-00:18

    - SCREEN: 68498_screenshot_3.mp4	00:19-00:22

    - Talent manually turning the laser on.
AND

    - Added shot: SCREEN: 68498_screenshot_4.mp4.	00:00-00:11 
TXT: Laser ON
Video editor: Author requests to overlay text as a short fade, appearing for 2 seconds at 00:06 before fading out. Also play both shots side by side

  - Next, adjust the Z position to maximize the signal counts in the fingerprint region from 600 to 1800 inverse centimeters [1]. Then turn off the laser [2], click Stop on the Experiment toolbar, and exit the software [3-TXT].

    - Talent manually adjusting Z-axis to optimize signal peaks.

Added shot: 2.6.1b Close up on the microscope controller used to adjust the Z-axis.
Added shot: 2.6.1c SCREEN: 68498_screenshot_5.mp4	00:00-00:25 
Video editor: Please play 2.6.1 a to 2.1.c side by side

    - Talent manually turning the laser off.

    - SCREEN: 68498_screenshot_6.mp4	00:05-00:15
TXT: Laser OFF
Video editor: Author requests to overlay text as a short fade at 00:06, appearing for 2 seconds before fading out.

  - Start the data acquisition process using the MATLAB script, which automatically manages Raman channels in multi-dimensional measurements and saves the data [1].

    - SCREEN: 68498_screenshot_7.mp4	00:00-00:11

  - Set the field of view to 50 by 50-micrometer square, with a resolution of 40 by 40 pixels, each pixel covering 1.25 by 1.25-micrometer square [1]. Set the exposure time for each Raman spectrum to 1.5 seconds and run the MATLAB script [2].

    - SCREEN: 68498_screenshot_8.mp4	00:00-00:16

    - SCREEN: 68498_screenshot_8.mp4	00:17-00:26

- Raman Spectroscopy Data Analysis

  - For the post-processing of Raman hyperspectral maps, launch the RamApp (Rahm-App)  web based tool [1]. Import the hyperspectral data by clicking on New in the My Analyses tab, and upload the .mat (dot-mat) file containing the spatial and spectral data variables along with the x-axis variable [2].

    - SCREEN: 68498_screenshot_9.mp4	00:00-00:05

    - SCREEN: 68498_screenshot_9.mp4	00:06-00:37

  - To truncate the spectrum, in the Crop and rotate menu, choose Spectral crop [1].  Define the inferior and superior borders of the Raman shift region as 600 and 1800 inverse centimeters respectively [2].

    - SCREEN: 68498_screenshot_10.mp4	00:00-00:06

    - SCREEN: 68498_screenshot_10.mp4	00:07-00:23

  - To correct for cosmic rays, go to Denoising, click Despike, select Z-score as Method, and set Threshold to 8 [1]. Check the options Use first difference and Correct spikes [2]. Then click Run to apply the correction [3].

    - SCREEN: 68498_screenshot_11.mp4	00:00-00:08

    - SCREEN: 68498_screenshot_11.mp4	00:15-00:17

    - SCREEN: 68498_screenshot_11.mp4	00:18-00:22

  - To remove noise from the maps, click Smooth Map , choose the Median filter, set the size of the square to 3 pixels, then click Run [1].

    - SCREEN: 68498_screenshot_12.mp4	00:00-00:12

  - To smooth the spectrum of each pixel, click on Smooth spectrum under Denoising, choose the Savitzky-Golay (sævɪtski ˈɡoʊleɪ) filter [1]. Set filter window length to 7 and polynomial order to 2, then click Run [2].

    - SCREEN: 68498_screenshot_13.mp4	00:00-00:06

    - SCREEN: 68498_screenshot_13.mp4	00:07-00:15

  - To subtract the average background signal, open Optical substrate removal and click Identify substrate [1]. Set k-means as the Method with 2 clusters, then click on advanced options [2]. Choose Morphological cleaning to remove small blobs from the foreground then click Run [3].

    - SCREEN: 68498_screenshot_14.mp4	00:00-00:05

    - SCREEN: 68498_screenshot_14.mp4. 	00:06-00:09

    - SCREEN: 68498_screenshot_14.mp4. 	00:10-00:20

  - To remove the average background signal, click on Remove substrate, then choose Global average (mean of the central 95% of values) (mean-of-the-central-95-percent-of-values) method and click Run [1].

    - SCREEN: 68498_screenshot_15.mp4	00:00-00:09

  - To correct for baseline fluorescence, open the Baseline panel, click on Correct baseline, select Adaptive Smoothness PLS (P-L-S) then set lambda to 5,000,000, and click Run [1]. For the normalization of each Raman hyperspectral map, go to Miscellaneous, select Normalize, choose Frobenius method, and click Run [2].

    - SCREEN: 68498_screenshot_16.mp4	00:00-00:16

    - SCREEN: 68498_screenshot_16.mp4	00:17-00:27

  - To generate a false-color image showing subcellular distribution, on Images panel, click the Open menu symbol under Intensity Image, then click Edit image [1]. On the editing panel, select Single band and define the spectral range [2]. Now choose Double color on the Color map then define the color, intensity threshold, and opacity, before clicking Confirm [3].

    - SCREEN: 68498_screenshot_17.mp4	00:00-00:04

    - SCREEN: 68498_screenshot_17.mp4	00:05-00:19

    - SCREEN: 68498_screenshot_17.mp4	00:20-00:40

  - For the download of a single-point spectrum, choose a pixel of interest, then on the Spectral legend panel, click Download spectrum [1-TXT].

    - SCREEN: 68498_screenshot_18.mp4	00:00-00:11
TXT: Repeat as needed for other regions

  - Finally, to export foreground and substrate spectra, navigate to Spectral legend then click Download spectrum for both Foreground and Substrate sections [1].

    - SCREEN: 68498_screenshot_19.mp4	00:00-00:12

- Tomographic Phase Microscopy (TPM) Measurement Protocol

  - Place a drop of distilled water onto the objective lens of the microscope [1]. Position the sample on the microscope's translation stage [2]. Then adjust the sample position to align it with the objective lens [3].

    - Talent pipetting distilled water onto the microscope objective.

    - Talent placing the sample onto the translation stage.

    - Talent fine-tuning the position to align the sample with the objective.

  - Turn on the microscope and launch the imaging software [1]. Click the microscope icon on the toolbar [2]. After initialization, click Configuration, select LiveCell in the Job panel, and PBS as the Medium [3].

    - SCREEN: 68498_screenshot_20.mp4.	00:00-00:06

    - SCREEN: 68498_screenshot_20.mp4.	00:07-00:14

    - SCREEN68498_screenshot_20.mp4.	00:27-00:40

  - Next, access the Calibration tab in the control panel of the imaging software [1]. Set the axial positions of the objective and condenser lenses by selecting Focus and Surface, respectively [2].

    - SCREEN: 68498_screenshot_21.mp4	00:00-00:03

    - SCREEN: 68498_screenshot_21.mp4	00:04-00:17

  - Now click Scanning mode to manually adjust the lenses and ensure the illumination patterns are centered and nearly static. Choose Normal mode [1], then adjust the translation stage to bring the cell into the field of view and focus on the sample [2].

    - SCREEN: 68498_screenshot_22.mp4	00:00-00:30

    - SCREEN: 68498_screenshot_23.mp4	00:00-00:14

  - Adjust the translation stage to locate a region without cells [1]. Choose Calibrate to capture multiple 2D holograms at varying illumination angles [2].

    - Talent moving the stage to an acellular region.

4.5.1b Close-up on the stage controller.

    - SCREEN: 68498_screenshot_24.mp4	00:00-00:10

  - Adjust the translation stage to center the cell within the field of view . Then navigate to the Acquisition tab and select 3D Snapshot to capture the tomogram of the cell [1].

4.6.1.Talent repositioning stage to center the cell.
NOTE: Shot deleted at author’s request

4.6.2  SCREEN: 68498_screenshot_25.mp4	00:00-00:18

- Tomographic Phase Microscopy Data Analysis

5.1 To visualize the holographic tomograms, select the data on the Data Navigation panel, right-click and click Open [1]. On the Data Manager panel, click on RI (R-I) Tomogram [2].

    - SCREEN: 68498_screenshot_26.mp4	00:00-00:11

    - SCREEN: 68498_screenshot_26.mp4	00:12-00:15

  - In the Volume Visualization panel, select RI and draw four rectangular color boxes within the RI canvas [1]. Set the minimum and maximum values of the RI range for each color box then associate an opacity and color with each box [2]. Click Save to store the defined RI Ranges and color boxes [3].

    - SCREEN: 68498_screenshot_27.mp4	00:00-00:11

    - SCREEN: 68498_screenshot_27.mp4	00:12-00:34

    - SCREEN: 68498_screenshot_27.mp4	03:41-03:54

  - To attain quantitative descriptors of cell morphology, utilize the Analysis interface [1]. Select Manual under the Segmentation panel then set 1.3450 as the RI Threshold, and click Apply [2]. Click Save to save the calculated morphological indexes [3].

    - SCREEN:68498_screenshot_28.mp4.	00:00-00:04

    - SCREEN: 68498_screenshot_28.mp4	00:08-00:16,00:35-00:37

    - SCREEN: 68498_screenshot_28.mp4	00:41-00:53

# Results

- Results

  - Raman spectral mapping revealed the spatial distribution of cytoplasm [1], nucleus [2], phenylalanine [3], and lipids within individual MDA-MB-231 cells [4].

    - LAB MEDIA: Figure 2A. Video editor: Highlight the green-labeled image under “CYTOPLASM” showing bright green color in the cytoplasmic region.

    - LAB MEDIA: Figure 2A. Video editor: Highlight the blue-labeled image under “NUCLEUS” showing bright blue signal localized in the nuclear region.

    - LAB MEDIA: Figure 2A. Video editor: Highlight the red-labeled image under “PHENYLALANINE” showing red fluorescence in specific intracellular zones.

    - LAB MEDIA: Figure 2A. Video editor: Highlight the brown-labeled image under “LIPIDS” showing golden-brown intensity in the lipid-rich areas.

  - Distinct Raman spectral peaks corresponding to cytoplasm [1], nucleus [2], phenylalanine [3], and lipids were confirmed in the fingerprint region between 600 and 1800 inverse centimeters [4].

    - LAB MEDIA: Figure 2B. Video editor: Highlight the green vertical band at 715–725 on the x-axis representing cytoplasm peak.

    - LAB MEDIA:  Figure 2B. Video editor: Highlight the blue vertical band at 780–790 on the x-axis representing nuclear peak.

    - LAB MEDIA: Figure 2B. Video editor: Highlight the narrow brown vertical band at 1004 on the x-axis indicating phenylalanine signal.

    - LAB MEDIA: Figure 2B. Video editor: Highlight the orange vertical band at 1440–1450 on the x-axis indicating lipid peak.

  - Refractive index heat maps of single cells revealed the internal distribution of subcellular components in the axial XY plane [1], sagittal YZ plane [2], and coronal XZ planes [3].

    - LAB MEDIA: Figure 4. Video editor: Show the leftmost XY PLANE heatmap.

    - LAB MEDIA: Figure 4. Video editor: Highlight the YZ PLANE middle panel

    - LAB MEDIA: Figure 4. Video editor: Highlight the XZ PLANE rightmost panel

  - A 3D tomographic reconstruction identified regions of volumetric refractive index corresponding to internal substructures [1]. Quantitative morphological measurements of a single MDA-MB-231 cell showed different descriptors of cell morphology, with a total volume of 4024.189 cubic micrometers and surface area of 2383.707 square micrometers [2], with a mean refractive index of 1.356 [3].

    - LAB MEDIA: Figure 5A.

    - LAB MEDIA: Figure 5B. Video editor: Highlight the values for “VOLUME” and “SURFACE AREA” on the right panel.

    - LAB MEDIA: Figure 5B. Video editor: Highlight the “MEAN RI” value in the list on the right.

Pronunciation Guide:

1. Confocal

- Pronunciation link: https://www.merriam-webster.com/dictionary/confocal

- IPA: /kənˈfoʊkəl/

- Phonetic Spelling: kuhn-FOH-kuhlmerriam-webster.com+6merriam-webster.com+6merriam-webster.com+6

2. Raman

- Pronunciation link: https://www.merriam-webster.com/dictionary/Raman

- IPA: /ˈrɑːmən/

- Phonetic Spelling: RAH-muhnmerriam-webster.commerriam-webster.com+1merriam-webster.com+1

3. Spectroscopy

- Pronunciation link: https://www.merriam-webster.com/dictionary/spectroscopy

- IPA: /spɛkˈtrɑːskəpi/

- Phonetic Spelling: spek-TRAH-skuh-peemerriam-webster.com+14merriam-webster.com+14merriam-webster.com+14

4. Tomographic

- Pronunciation link: https://www.merriam-webster.com/dictionary/tomography

- IPA: /ˌtoʊməˈɡræfɪk/

- Phonetic Spelling: TOH-muh-GRAF-ikmerriam-webster.com

5. Microscopy

- Pronunciation link: https://www.merriam-webster.com/dictionary/microscopy

- IPA: /maɪˈkrɑːskəpi/

- Phonetic Spelling: my-KRAH-skuh-peemerriam-webster.com+15merriam-webster.com+15merriam-webster.com+15

6. Phenylalanine

- Pronunciation link: https://www.merriam-webster.com/dictionary/phenylalanine

- IPA: /ˌfɛnɪlˈæləniːn/

- Phonetic Spelling: FEN-il-AL-uh-neen

7. Hyperspectral

- Pronunciation link: https://www.merriam-webster.com/dictionary/hyperspectral

- IPA: /ˌhaɪpərˈspɛktrəl/

- Phonetic Spelling: HY-per-SPEK-truhl

8. Savitzky-Golay

- Pronunciation link: https://www.howtopronounce.com/savitzky-golay

- IPA: /ˈsævɪtski ˈɡoʊleɪ/

- Phonetic Spelling: SAV-its-kee GOH-lay

9. Frobenius

- Pronunciation link: https://www.howtopronounce.com/frobenius

- IPA: /froʊˈbiːniəs/

- Phonetic Spelling: froh-BEE-nee-uhsmerriam-webster.com+1merriam-webster.com+1

10. Morphological

- Pronunciation link: https://www.merriam-webster.com/dictionary/morphological

- IPA: /ˌmɔːrfəˈlɑːdʒɪkəl/

- Phonetic Spelling: mor-fuh-LOJ-ih-kuhl