# Submission ID #: 68539

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=20904443

# Title: Real-Time Metabolic Detection in Living Cells Using Hyperpolarized ¹³C NMR

# Authors and Affiliations:

# Tomoto Ura1,2, Keita Saito1, Ryoma Kobayashi1, Natsuko Miura3, Yoichi Takakusagi1,4

# 1Institute for Quantum Life Science, National Institutes for Quantum Science and Technology

# 2Institute of Pure and Applied Sciences, University of Tsukuba

# 3Department of Applied Biological Chemistry, Graduate School of Agriculture, Osaka Metropolitan University

# 4Center of Quantum Life Science for Structural Therapeutics (cQUEST), Chiba University

# Corresponding Authors:

# Yoichi Takakusagi	takakusagi.yoichi@qst.go.jp

# Email Addresses for All Authors:

Tomoto Ura		ura.tomoto.gn@u.tsukuba.ac.jp

Keita Saito		saito.keita@qst.go.jp

Ryoma Kobayashi	kobayashi.ryoma@qst.go.jp

# Natsuko Miura	miuran@omu.ac.jp

# Yoichi Takakusagi	takakusagi.yoichi@qst.go.jp

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes, all done

3. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  21

Number of Shots:  53 (10 SC)

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

  - Tomoto Ura: We develop non-invasive methods to measure real-time cellular metabolism using advanced hyperpolarized NMR techniques. Our goal is creating accessible tools for various biological research applications.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.3.1

What research gap are you addressing with your protocol?

  - Tomoto Ura: Several hyperpolarized NMR systems for cellular metabolism have been reported. However, standardized and reproducible protocols for broader research implementation have not been available until now.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.5.1

What advantage does your protocol offer compared to other techniques?

  - Natsuko Miura: Our protocol enhances accessibility using commercially available components. We also provide comprehensive validation including in-situ mixing advantages and repeated measurement capabilities for broader adoption.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 4.1.2

What new scientific questions have your results paved the way for?

  - Natsuko Miura: Our results enable real-time tracking of dynamic metabolic changes in living cells without destruction. This opens new possibilities for long-term drug studies and diverse metabolic probe applications.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 5.1.2

What research questions will your laboratory focus on in the future?

  - Yoichi Takakusagi: Our approach has limitations including probe options and automation requirements. Solving these challenges will establish cellular metabolic measurement techniques and bridge our laboratory's DNP-MRI research efforts in the future.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.8.1

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

- Alginate Gel Preparation

Demonstrator: Tomoto Ura

  - To begin, add 2 milliliters of 0.25 percent volume-to-volume trypsin-EDTA solution to a 10-centimeter dish [1]. Tilt the dish gently to ensure the entire surface is covered [2], then aspirate the excess trypsin-ethylenediaminetetraacetic acid solution [3].

    - WIDE: Talent adding 2 milliliters of trypsin-ethylenediaminetetraacetic acid to the 10 centimeter dish using a pipette.

    - Talent gently tilting the dish to evenly coat the surface with the solution.

    - Talent aspirating the excess trypsin-ethylenediaminetetraacetic acid using a pipette or vacuum aspirator.

  - Place the dish in an incubator set at 37 degrees Celsius and incubate for 2 minutes [1].

    - Talent placing the dish inside a 37 degrees Celsius incubator and closing the door.

  - Then, add 10 milliliters of culture medium to the dish [1]. Using a pipette, gently pipette up and down to detach the cells from the surface [2] and collect the entire cell suspension into a 15-milliliter conical tube [3].

    - Talent adding 10 milliliters of culture medium to the dish. NOTE: The shots 2.3.1, 2.3.2 and 2.3.3 may have been shot together

    - Talent pipetting up and down gently to detach the cells.

    - Talent transferring the cell suspension into a 15 milliliter conical tube using a pipette.

  - Now, take approximately 10 microliters of the cell suspension and load it into a cell counter to determine the number of cells [1]. Ensure that the measured count confirms a yield of approximately 10 million cells, or adjust as needed [2].

    - Talent pipetting 10 microliters of cell suspension into the cell counter.

    - Display of cell count results on the cell counter monitor.

  - Centrifuge the remaining cell suspension at 120 g for 5 minutes [1]. After centrifugation, aspirate and discard the supernatant [2]. Resuspend the resulting cell pellet in 4 milliliters of 2 percent weight-to-volume alginate solution [3] and pipette slowly to minimize bubble formation [4].

    - Talent placing the conical tube into the centrifuge.

    - Talent removing the supernatant from the centrifuged tube using a pipette.

    - Talent adding 4 milliliters of 2 percent alginate solution to the pellet.

    - Talent pipetting the mixture slowly and carefully to avoid bubbles.

  - Now, add 10 milliliters of 50 millimolar calcium chloride solution to a 50-milliliter centrifuge tube [1].

    - Talent pipetting 10 milliliters of calcium chloride solution into a labeled 50 milliliter centrifuge tube.

  - Secure the syringe to the cap of the centrifuge tube containing calcium chloride solution [1]. Draw 300 microliters of the cell-alginate mixture into a 0.5-milliliter syringe fitted with a 3- gauge, 10 millimeter-needle [2].

2.7.2   Talent attaching and securing the syringe to the cap of the centrifuge tube

    - Talent filling a 0.5 milliliter syringe with 300 microliters of the cell-alginate mixture. NOTE: The shots and VO are inverted. Show 2.7.2 first

  - Then, close the centrifuge tube tightly with the syringe fixed to the cap [1] and place the assembly into the centrifuge to spin gently at 200 g for 5 minutes [2]. Remove the supernatant after centrifugation [3] and resuspend the resulting gel in culture medium [4].

    - Talent sealing the centrifuge tube with the syringe in place. NOTE: The shots 2.8.1, 2.8.2 were shot together

    - Talent placing the assembly into the centrifuge and initiating the spin at 200 g.

    - Talent removing the supernatant.

    - Talent gently resuspending the gel in culture medium by pipetting up and down.

  - Next, using a Pasteur pipette, transfer the cell-encapsulated gel into an NMR tube [1] and insert the sponge fixture to ensure a tight seal [2].

    - Talent transferring the gel from the tube to the NMR tube using a Pasteur pipette.

    - Talent inserting the sponge fixture into the NMR tube and securing the seal.

  - Pre-warm the medium reservoir containing 5 percent volume-to-volume deuterium oxide in a 37 degrees Celsius water bath [1]. Begin circulation at a rate of approximately 500 microliters per hour to stabilize the conditions [2].

    - Talent placing the medium reservoir in the water bath set at 37 degrees Celsius.

    - Talent activating the circulation system.

  - Inspect the setup to ensure there are no leaks before proceeding [1].

    - Talent examining all connections in the setup.

- Preparation of Hyperpolarized Probe using a DNP Polarizer

Demonstrator: Keita Saito

  - Add 18 microliters of Carbon 13 labeled pyruvic acid at a concentration of 14.2 molar, doped with 25 millimolar OX063 (ox-63), to a sample vial [1]. Connect the vial to the fluid path of the dynamic nuclear polarization polarizer system [2].

    - Talent pipetting 18 microliters of [1-13C] pyruvic acid doped with OX063 into a labeled sample vial.

    - Talent connecting the sample vial to the tubing of the dynamic nuclear polarization polarizer system.

  - Next, insert the connected vial into the bore unit of the dynamic nuclear polarizer [1-TXT]. Apply microwave irradiation at approximately 188 gigahertz with a power of 22 milliwatts for a duration of 60 to 80 minutes [2].

    - Talent placing the vial assembly into the bore unit of the polarizer system. TXT: Polarizer settings: 6.7 T; 1.25 K; 1.2 mbar NOTE: 2 takes were shot

    - Talent adjusting settings for 188 gigahertz, 22 milliwatts.  NOTE: 2 takes were shot

  - Use the solid-state nuclear magnetic resonance spectrometer integrated into the dynamic nuclear polarization system to monitor the carbon-13 signal intensity [1]. Record the signal data every 5 minutes using the system's control software [2].

    - Talent switching on SPINit software and setting up the monitoring time for the polarization build-up.  NOTE: 2 takes were shot

    - SCREEN: 3.3.2. 13:00-13:10 NOTE: Screen recordings are on the project page

  - Now, rapidly dissolve the polarized sample in 3.2 milliliters of dissolution buffer preheated to biological temperature [1-TXT]. Ensure the final solution reaches a temperature between 308 and 313 kelvin and a pH of approximately 7 before transferring for NMR measurement [2].

    - Talent dissolving the sample in 3.2 milliliters of heated dissolution buffer quickly after polarization. TXT: Dissolution buffer: 40 mM Tris; 50 mM NaCl; 80 mM NaOH; 100 mg/L EDTA Videographer’s NOTE: This shot covers all talent shots from 3.4.1 to 4.5.2. This is to show how quickly they need to inject the solution into the NMR tube. The shot includes a pan to follow Tomoto. We have closer shots of all of the steps that can also be used. This shot starts by adding the buffer then 3-4 minute wait for it to be ready. The main shot is at the end of the clip.

    - Talent measuring the pH of the solution using pH test paper.

- Nuclear Magnetic Resonance (NMR) Setup and Measurement

Demonstrator: Tomoto Ura

  - Load the prepared sample into an NMR tube and insert the tube into the spectrometer [1]. On the NMR console, initiate the automated procedure to perform locking, tuning, matching, and shimming with the sample in place to ensure optimal spectral resolution and signal stability [2].

    - Talent adding the sample to the NMR tube.  NOTE: Delete the shot. VO merged with the next shot

    - Talent inserting the sample-filled NMR tube into the spectrometer slot.

    - SCREEN: 4.1.3 .01:45-01:59

  - Load a carbon-13 pulse-acquire sequence, with proton decoupling enabled using a setting like 13-CPD [1-TXT]. Set the flip angle to 90 degrees with a 1 microsecond pulse labeled as P1 [2].

    - SCREEN: 4.2.1. TXT: ¹³C pulse-acquire sequence example: zg2d

    - SCREEN: 4.2.2.

  - Next, define the acquisition time as 1.376 seconds, spectral width as 23,663 hertz, and acquisition points or TD as 65,536 [1]. Set the relaxation delay D1 to zero seconds [2]. Assign 150 time increments to TD1 for acquiring 150 sequential spectra in pseudo two-dimensional mode [3] and adjust the receiver gain to a low value, such as approximately 1, to avoid signal saturation caused by the hyperpolarized samples [4].

    - SCREEN: 4.3.1. 00:15-00:30.

    - SCREEN: 4.3.2.

    - SCREEN: 4.3.3.

    - SCREEN: 4.3.4.

  - Now, stop the peristaltic pump to temporarily halt the circulation of the medium before injecting the hyperpolarized solution through the inlet tubing [1]. Begin the spectral acquisition approximately 10 seconds before dissolving the hyperpolarized substrate to ensure signal capture upon arrival at the detection site [2]. When the solution exits the dynamic nuclear polarization polarizer, draw 1 milliliter of it into a syringe [3].

    - Talent stopping the peristaltic pump.

    - Talent injecting the hyperpolarized solution through the inlet tubing. NOTE: Delete the shot. VO merged with the next shot

    - SCREEN: 4.4.3-4.6.1.  00:00-00:15.

    - Talent drawing 1 milliliter of hyperpolarized solution into a syringe from the polarizer outlet.

  - Switch the three-way valve at the bioreactor inlet to Position B to direct the hyperpolarized solution into the NMR tube [1]. After the injection is complete, switch the valve back to Position A to resume medium circulation and prevent any backflow [2].

    - Talent switching the valve to Position B and injecting the solution into the NMR tube.

    - Talent switching the valve back to Position A to restore medium circulation.

  - Finally, confirm that the injection was successful after noting an increase in the Free Induction Decay signal on the acquisition software [1].

    - SCREEN: 4.4.3-4.6.1.  00:30-00:35 and 05:04-05:08.

# Results

- Results

  - Hyperpolarized carbon-13 NMR spectra of the same SCCVII (S-C-C-7) cell-encapsulated sample were acquired at 1.5-hour intervals to evaluate the feasibility of repeated non-destructive measurements [1]. The first NMR measurement showed a markedly increased lactate signal, indicating active pyruvate-to-lactate metabolic conversion [2].

    - LAB MEDIA: Figure 5.

    - LAB MEDIA: Figure 5. Video editor: Highlight the data points for lactate in A.

  - Subsequent measurements at 1.5-hour intervals revealed a progressive reduction in lactate signal intensity, indicating decreased metabolic activity over time [1].

    - LAB MEDIA: Figure 5. Video editor: Highlight the data points for lactate in B and C.

  - In HeLa cells, initial measurements confirmed successful pyruvate-to-lactate conversion with clearly discernible signal peaks [1].

    - LAB MEDIA: Figure 6. Video editor: Highlight the data points for lactate between 5 to 50 seconds on X axis

  - Although the signal decreased gradually due to T₁ relaxation, the signal-to-noise ratio remained adequate throughout the 60-second acquisition, suggesting that the current protocol is broadly applicable and can serve as a versatile tool for real-time metabolic profiling in various cell types [1].

    - LAB MEDIA: Figure 6.

Pronunciation guides:

### 1. trypsin

Pronunciation link: https://www.merriam-webster.com/dictionary/trypsin
IPA: /ˈtraɪpˌsɪn/
Phonetic spelling: TRYPE-sin

### 2. ethylenediaminetetraacetic

Pronunciation link: https://www.merriam-webster.com/dictionary/ethylenediaminetetraacetic
IPA: /ˌɛθəˌliːn daɪˌæmᵻnɛˌtɛtrəəˈsɛtɪk/
Phonetic spelling: ETH-uh-leen dye-am-in-uh-NEH-tet-ruh-uh-SET-ik

### 3. aspirate

Pronunciation link: https://www.merriam-webster.com/dictionary/aspirate
IPA: /ˈæs.pəˌreɪt/
Phonetic spelling: AS-puh-rayt

### 4. conical

Pronunciation link: https://www.merriam-webster.com/dictionary/conical
IPA: /ˈkɑ.nɪ.kəl/
Phonetic spelling: KAH-ni-kuhl

### 5. alginate

Pronunciation link: https://www.merriam-webster.com/dictionary/alginate
IPA: /ˈæl.dʒɪˌneɪt/
Phonetic spelling: AL-jih-nayt

### 6. Pasteur (as in Pasteur pipette)

Pronunciation link: https://www.merriam-webster.com/dictionary/Pasteur
IPA: /pɑːˈstɜr/
Phonetic spelling: pah-STUR

### 7. deuterium

Pronunciation link: https://www.merriam-webster.com/dictionary/deuterium
IPA: /duːˈtɪə.ri.əm/
Phonetic spelling: doo-TEER-ee-um

### 8. hyperpolarized

Pronunciation link: https://www.merriam-webster.com/dictionary/hyperpolarize
IPA: /ˌhaɪ.pərˈpoʊ.ləˌraɪzd/
Phonetic spelling: HY-per-poh-luh-ryzd

### 9. pyruvic

Pronunciation link: https://www.merriam-webster.com/dictionary/pyruvic
IPA: /ˌpaɪˈruː.vɪk/
Phonetic spelling: pie-ROO-vik

### 10. polarization

Pronunciation link: https://www.merriam-webster.com/dictionary/polarization
IPA: /ˌpoʊ.lə.rɪˈzeɪ.ʃən/
Phonetic spelling: poh-luh-ruh-ZAY-shuhn

### 11. peristaltic

Pronunciation link: https://www.merriam-webster.com/dictionary/peristaltic
IPA: /ˌpɛr.ɪˈstæl.tɪk/
Phonetic spelling: per-ih-STAHL-tik