# Submission ID #: 68577

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=20916033

# Title: Overexpressing and Purifying a Toxic Nuclease from Escherichia coli

# Authors and Affiliations:

# Erik J. Daquilanea, Meredith N. Frazier

# Department of Chemistry and Biochemistry, College of Charleston

# Corresponding Authors:

# Meredith N. Frazier      fraziermn@cofc.edu

# Email Addresses for All Authors:

# Erik J. Daquilanea          daquilaneaej@cofc.edu

# Meredith N. Frazier      fraziermn@cofc.edu

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  19

Number of Shots:  41

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

REQUIRED: What is the scope of your research? What questions are you trying to answer?

  - Erik Daquilanea: Our lab aims to structurally and biochemically characterize a conserved endonuclease found in nidoviruses, including coronaviruses, to develop an evolutionary model and provide a basis for therapeutic targets.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 4.3.1

What are the current experimental challenges?

  - Erik Daquilanea: Nucleases can be difficult to express in E. coli systems due to enzymatic activity on cellular DNA or RNA, which can result in slow growth and poor protein yields.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.3.1

How will your findings advance research in your field?

  - Erik Daquilanea: Our finding will allow us the purify toxic nucleases from other nidoviruses for further downstream biochemical and structural studies.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.2.1

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

- Culturing the Cells and Inducing Protein Expression with IPTG

Demonstrator: Erik Daquilanea

  - To begin, use one 50-milliliter sterilized Erlenmeyer flask for each liter of culture, and prepare 2 to 3 additional flasks as starter cultures [1]. Label one of the flasks with a star to designate it for optical density checks, as its measurements will represent the entire growth unless there is a visual difference among flasks [2].

    - WIDE: Talent placing sterilized 50 milliliter Erlenmeyer flasks on the workbench.

    - Talent labeling one flask with a star using a lab marker.

  - Using a graduated cylinder, prepare a master mix by combining 60 milliliters of 2x TY medium and 60 microliters of ampicillin stock solution thawed from minus 20 degrees Celsius [1]. Swirl the mixture gently to combine [2] and aliquot 10 milliliters of the master mix into each Erlenmeyer flask [3].

    - Talent pouring 2xTY medium into a cylinder and adding ampicillin stock with a micropipette.

    - Talent swirling the container to mix the master solution.

    - Talent pipetting 10 milliliters of the mix into each flask.

  - Now, remove the transformed agar plate from the incubator [1]. Using a sterile toothpick or pipette tip, pluck a single, isolated colony from the plate [2] and transfer it directly into a flask containing media [3]. Repeat this procedure until each flask contains a colony-inoculated toothpick [4].

    - Talent opening the incubator and removing the bacterial plate.

    - Talent isolating a single colony using a sterile tool from the plate.

    - Talent dropping the tool into a flask containing media.

    - Shot of the remaining flasks with the toothpick.

  - Place the inoculated flasks into a shaking incubator set to 210 revolutions per minute and 37 degrees Celsius for approximately 5 to 7 hours [1].

    - Talent loading the inoculated flasks into the incubator and adjusting the settings.

  - To check the optical density at 600 nanometers, use a serological pipette to remove 1 milliliter of media from the starred flask [1]. Continue checking periodically until the optical density reaches between 0.8 and 1.0 [2].

    - Talent removing 1 milliliter of media from the starred flask using a serological pipette.

    - Talent placing the sample in a spectrophotometer.

  - Once the target optical density is achieved, add 1 milliliter of 1 molar IPTG solution thawed from minus 20 degrees Celsius to each flask to induce protein overexpression [1].

    - Talent pipetting 1 milliliter of IPTG into each culture flask.

  - Then, transfer the 2-liter flasks to a shaking incubator set to 210 revolutions per minute and 16 degrees Celsius for overnight induction for 14 to 16 hours [1].

    - Talent placing the flasks in a large incubator.

- Cell Lysis and Lysate Clarification

  - Resuspend the bacterial pellet by adding 2 milliliters of lysis buffer for every 1.2 grams of pellet [1]. Add 100 microliters of 1 molar AEBSF for every 10 milliliters of lysis buffer as a protease inhibitor [2]. If using a pellet combined from 4 liters of culture, mix it with 100 microliters of DNase I to achieve a final concentration of 200 units [3].

    - Talent pipetting lysis buffer into the tube containing the bacterial pellet.

    - Talent adding AEBSF using a micropipette.

    - Talent inverting the tube to mix.

  - After vortexing for 30 seconds, transfer the vortexed mixture into a Dounce tissue grinder [2] and use a loose pestle to homogenize the sample with approximately 10 strokes [3].

    - Talent pouring the vortexed solution into a Dounce grinder.

    - Talent performing about 10 strokes with a loose pestle.

  - Then, transfer the homogenized sample into a metal beaker for sonication [1].

    - Talent pouring the homogenized solution into a clean metal beaker.

  - To maximize sample recovery, rinse the original pellet tube with 5 milliliters of lysis buffer [1] and vortex briefly [2]. Pour the rinse into the Dounce homogenizer, apply approximately 5 strokes with the pestle [3], and transfer the contents to the sonication beaker [4].

    - Talent adding lysis buffer to the pellet tube.

    - Talent holding the tube on a vortex mixer.

    - Talent transferring the rinse to the Dounce grinder and performing about 5 strokes.

    - Talent pouring the second homogenate into the metal beaker.

  - Add 75 microliters of Triton X-100 to the metal beaker to aid in membrane lysis and solubilization [1]. Place the metal beaker into an ice bath to keep the sample cold during sonication [2].

    - Talent adding Triton X-100 to the beaker using a micropipette.

    - Talent placing the beaker into a pre-prepared ice bath.

  - Now, insert the sonication probe into the metal beaker [1] and sonicate the sample for 6 minutes and 30 seconds, with pulses every 2 seconds and amplification set to 70 percent [1].

    - Talent positioning the sonication probe into the sample.

    - Talent operating the sonicator with specified parameters and starting the cycles.

  - Then, using a serological pipette, transfer the lysate into a centrifuge tube [1]. Add 1 molar AEBSF at a 1 to 100 dilution to the same tube as a fresh protease inhibitor [2] and centrifuge the sample at 26,915 g for 50 minutes to clarify the lysate [3].

    - Talent pipetting the sonicated lysate into a centrifuge tube.

    - Talent adding AEBSF to the centrifuge tube using a micropipette.

    - Talent placing the tube in the centrifuge.

- Protein Elution and Tag Cleavage

  - After performing affinity chromatography by gravity filtration, elute the His-tagged Nsp15 protein in three stages, with the first and second elutions each using 2 milliliters of elution buffer, and the third using 1 milliliter [1].

    - Talent adding the elution buffer to the column.

  - Prepare a 1 to 1 dilution of Bradford reagent in water for a quick qualitative protein assay [1]. Transfer 10 microliters from each elution into separate tubes and invert gently to mix [2]. Assess the color change to blue to estimate protein content and decide how to combine the elutions accordingly [3].

    - Talent vortexing the reagent mixed with water.

    - Talent pipetting 10 microliters from each elution into separate tubes and inverting to mix.

    - Talent looking at tubes with blue colours.

  - Next, using a serological pipette, apply the cleavage reaction mix containing the previous eluate directly onto the resin [1] and collect the eluate into a 15-milliliter conical tube [2].

    - Talent carefully pipetting the cleavage reaction onto the resin.

    - Talent collecting flow-through in a conical tube.

  - Wash the resin twice with 2 milliliters of cleavage buffer, collecting both washes into the same 15 milliliter tube [1]. Add AEBSF to this tube to quench thrombin and reach a final concentration of 10 millimolar [2].

    - Talent pouring cleavage buffer on the resin.

    - Talent adding AEBSF to the combined eluate using a micropipette.

  - Finally, transfer the entire repass sample to a new 30-kilodalton molecular weight cutoff concentrator [1]. Centrifuge the sample at 3,000 g for 10 minutes to reduce the volume to 500 microliters or less [2] and transfer the concentrated protein sample into a 0.5-milliliter microcentrifuge tube [3].

    - Talent loading the repass sample into a fresh 30 kilodalton concentrator.

    - Talent placing the concentrator in the centrifuge and setting time and speed.

    - Talent pipetting the final concentrated sample into a microcentrifuge tube.

# Results

- Results

  - Expression of wild-type Nsp15 in Escherichia coli resulted in slow cell growth with an approximate doubling time of 1 hour [1], whereas the catalytic-dead mutant showed a normal doubling time of approximately 20 minutes [2].

    - LAB MEDIA: Figure 1A. Video editor: Highlight the blue curve labelled “WT Nsp15 PI”.

    - LAB MEDIA: Figure 1A. Video editor: Highlight the green curve labelled “H223A Nsp15 PI” .

  - During affinity purification, a clear band corresponding to wild-type Nsp15 appeared in the elution lane, indicating successful isolation despite low expression [1].

    - LAB MEDIA: Figure 2A. Video editor: Highlight the dark band in lane 7 labelled “Elution 1” at approximately between 50 and 37 marks.

  - The catalytic-dead Nsp15 also eluted as a strong single band indicating high yield [1].

    - LAB MEDIA: Figure 3A. Video editor: Highlight the intense band in lane 7 labelled “Elution 1” at approximately 42 kilodaltons.

  - Thrombin cleavage reduced the molecular weight of Nsp15 by approximately 2 kilodaltons, confirming successful His-tag removal [1].

    - LAB MEDIA: Figure 2B. Video editor: Highlight the lane 3 “Post-Cleave”.

  - Size exclusion chromatography of wild-type Nsp15 showed two peaks, with the 11 milliliter peak corresponding to active hexameric form [1] and the 15 milliliter peak to inactive monomeric form [2].

    - LAB MEDIA: Figure 4A. Video editor: Highlight the first peak with the “hexamer” icon.

    - LAB MEDIA: Figure 4A. Video editor: Highlight the second peak with the “monomer” icon.

  - In a fluorescence-based RNA cleavage assay, only the hexameric wild-type Nsp15 showed visible degradation of RNA over time [1], while the monomeric wild-type and all forms of catalytic-dead Nsp15 remained inactive [2].

    - LAB MEDIA: Figure 5. Video editor: Highlight the long lanes in “WT” hexamer (0 to 60 min) in 5A.

    - LAB MEDIA: Figure 5. Video editor: Highlight B.