# Submission ID #: 68579

# Scriptwriter Name: Pallavi Sharma

# Project Page Link: https://review.jove.com/account/file-uploader?src=20916643

Title: Multilevel Microdissection and Functional-Structural Profiling of Human Renal Arterial Branches

# Authors and Affiliations:

Xuya Kang1,2,*，Yingjia Li1,2,*，Junxia Zhang1,2,3， Xinying Wang1,2， Lin Yao4，Yan Zhang1,2,5，Yahan Liu1,2

1School of Basic Medical Sciences, Peking University Health Science Center, State Key Laboratory of Vascular Homeostasis and Remodeling

2Beijing Key Laboratory of Cardiovascular Receptors Research, Research Unit of Medical Science Research Management/Basic and Clinical Research of Metabolic Cardiovascular Diseases, Chinese Academy of Medical Sciences, Haihe Laboratory of Cell Ecosystem

3Department of Cardiology and Institute of Vascular Medicine, Peking University Third Hospital, State Key Laboratory of Vascular Homeostasis and Remodeling, Peking University

4Department of Urology, Peking University First Hospital, Peking University

5Institute of Cardiovascular Diseases, First Affiliated Hospital of Dalian Medical University

*These authors contributed equally

# Corresponding Authors:

Yahan Liu                                                   lyhcnc@bjmu.edu.cn

Yan Zhang                                             zhangyan9876@pku.edu.cn

Lin Yao                                                      poparies@163.com

# Email Addresses for All Authors:

Yingjia Li                                                    liyingjia@bjmu.edu.cn

Junxia Zhang                                              zhangjunxia@pku.edu.cn

Xinying Wang                                          2411110076@stu.pku.edu.cn

Lin Yao                                                       poparies@163.com

Yan Zhang                                              zhangyan9876@pku.edu.cn

Yahan Liu                                                  lyhcnc@bjmu.edu.cn

## Author Questionnaire

- Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  Yes.

If Yes, can you record movies/images using your own microscope camera?

Yes.

LEICA S9D

2.3.1, 2.4.1, 3.1.1, 3.2.1, 3.2.2, 3.3.1, 3.3.2, 3.3.4, 3.4.1, 3.4.2.

Authors: Please use your microscope camera to film the scope shots and upload the file to your project page as soon as possible:  https://review.jove.com/account/file-uploader?src=20916643

- Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes.

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=20916643

3. Filming location: Will the filming need to take place in multiple locations?   No.

Current Protocol Length

Number of Steps:  11

Number of Shots:  26

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

REQUIRED: What is the scope of your research? What questions are you trying to answer?

  - Xuya Kang: Our research aims to establish standardized methods to isolate and functionally assess human renal arterial branches, uncovering mechanisms of vascular dysfunction and guiding targeted therapies.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.  Suggested B roll: Figure 1

What technologies are currently used to advance research in your field?

  - Yahan Liu: Previous research relied on animal models or indirect imaging, while our wire myography directly measures human renal artery function in vitro.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

How will your findings advance research in your field?

  - Xuya Kang: Our method enables precise, human-specific vascular analysis, overcoming species limitations and improving translational drug development.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

# Ethics Title Card

This research has been approved by the Urology Department of Peking University First Hospital and conducted following the Helsinki Declaration

# Protocol

- Renal Artery Isolation for Functional Assessment

Demonstrator: Xuya Kang

  - To begin, ensure the kidney tissue remains fully submerged in liquid during transport to maintain tissue viability and structural integrity [1-TXT].

    - Establishing shot of the talent with kidney placed in a transport container filled with liquid. TXT: Kidney tissue was obtained from renal carcinoma patients

  - Visually identify the coronal plane by locating the renal hilum and aligning the cut to pass through both the renal pelvis and the lateral convex border [1]. Using a sterile scalpel, bisect the kidney along this coronal plane to create two symmetrical halves [2] and expose internal structures such as the renal pyramids and columns [3].

    - Talent pointing to the renal hilum and aligning the coronal plane.

    - Talent using a sterile scalpel to cut the kidney along the identified coronal plane.

    - Talent exposes the internal structures such as the renal pyramids and columns.

  - Under a stereomicroscope, use micro-dissection scissors and forceps to meticulously separate the interlobar, arcuate, and interlobular arteries in a 10-centimeter black-bottomed culture dish [1].

    - SCOPE: View of talent dissecting the kidney and separating arteries under the stereomicroscope.

Authors: Please use your microscope camera to film the scope shots and upload the file to your project page as soon as possible:  https://review.jove.com/account/file-uploader?src=20916643

  - Gently remove any surrounding tissue and fat from the dissected arteries in the same 10-centimeter black-bottomed culture dish to clean them thoroughly [1].

    - SCOPE: Talent carefully trimming fat and connective tissue from the arteries using forceps.

- Arterial Ring Mounting

  - Using micro-dissection scissors, section the cleaned arteries into rings approximately 2 millimeters in length for vascular function studies [1].

    - SCOPE: Talent using micro-dissection scissors to cut the arteries into 2-millimeter-long rings.

  - Carefully insert the first guide wire into an arterial ring in the dish [1]. Bend one side of the guide wire at a 90-degree angle [2] and transfer the arterial ring with the wire into the chamber [3].

    - SCOPE: Talent inserting a guide wire into the arterial ring inside the dish.

    - SCOPE: Talent bending one side of the guide wire at a 90-degree angle.

    - SCOPE: Talent placing the ring onto the chamber.

  - Before fixing the arterial ring on the sample holder, record the vessel length [1]. Place the ring between the two holders and read the micrometer scale, where 1 scale division equals 10 micrometers [2]. Subtract the initial scale value measured when the holders just touch each other to calculate the arterial ring's length and width [3]. Then, fix the arterial ring onto the clamp-type sample holder using the instrument-provided screws, tightening them in a clockwise direction [4].

    - SCOPE: Talent noting the arterial ring's initial placement and preparing to measure.

    - SCOPE: Talent positioning the ring between the holders and reading the micrometer scale.

    - Talent subtracting the initial value to determine vessel dimensions.

    - SCOPE: Talent tightening the screws clockwise.

  - Then, thread a second guide wire through the arterial ring [1]. Wind the wire clockwise around the fixing screws on both ends, securing it tightly to the surface of the sample holder [2].

    - SCOPE: Talent inserting the second guide wire through the arterial ring.

    - SCOPE: Talent winding the guide wire around the screws on both ends and securing it.

- Vessel-Specific Normalization for Optimal Initial Tension Determination

  - Select Normalization Settings from the DMT (D-M-T) menu and set the Eyepiece calibration as 1 millimeter per division, target pressure as 13.3 kilopascal, IC1/IC100 (I-C-One-By-I-C-Hundred) as 0.9, online averaging time as 3 seconds, and delay time as 60 seconds [1].

    - SCREEN: Show the DMT software interface as the talent selects Normalization Settings, then enters the specific parameter values in each corresponding field.

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=20916643

  - Select the channel corresponding to the target artery and open the Normalization screen from the DMT menu [1]. In the appropriate fields, enter the tissue endpoints as a1 equals 0 and a2 equals the measured vessel length in millimeters [2]. Input the wire diameter as 40 micrometers and enter the micrometer reading from the scale [3]. Click Add point to save the data [4].

    - SCREEN: Show talent selecting the correct channel and opening the Normalization screen from the DMT menu.

    - SCREEN: Talent entering tissue endpoints: a1 = 0 and a2 = vessel length.

    - SCREEN: Talent inputting the wire diameter and micrometer scale reading.

    - SCREEN: Talent clicking Add point to record the data.

  - Now apply passive stretch and wait for 3 minutes [1]. Enter the new micrometer reading as the next point and click Add point [2]. Add 5 milliliters of 60 potassium ion solution to the chamber to induce a potassium-mediated contraction of the vessel [3]. To wash out the 60-potassium ion solution, add 5 milliliters of Krebs solution three times [4]. Finally, add 5 microliters of phenylephrine stock solution to the chamber containing 5 milliliters of Krebs buffer [5].

    - Talent initiating passive stretch and setting a timer for 3 minutes.

    - SCREEN: Talent entering the updated micrometer reading and clicking Add point.

    - Talent pipetting 5 milliliters of 60K+ solution into the chamber.

    - Talent washing out the chamber by pipetting and discarding 5 milliliters of Krebs solution three times.

    - Talent adding 5 microliters of phenylephrine stock into the Krebs buffer-filled chamber.

# Results

- Results

  - The interlobar artery was successfully dissected from the renal medulla, showing a thick vascular wall and surrounding adipose tissue that required careful removal to preserve structural integrity [1]. The arcuate artery was isolated at the corticomedullary junction, displaying a thinner vascular wall and an arched trajectory [2].

    - LAB MEDIA: Figure 1C. Video editor: Highlight the thick white artery running vertically, labelled as the interlobar artery

    - LAB MEDIA: Figure 1C. Video editor: Highlight the arch-shaped vessel branching horizontally, labelled as the arcuate artery

  - The interlobular artery was identified running linearly through the renal cortex with a very thin wall and tightly integrated with cortical tissue [1].

    - LAB MEDIA: Figure 1C. Video editor: Highlight the thin vertical artery labelled as the interlobular artery

  - Histological analysis revealed that the interlobar artery had the thickest vascular wall with a distinct adventitia [1], while arcuate and interlobular arteries exhibited progressively thinner walls and fewer smooth muscle layers [2].

    - LAB MEDIA: Figure 2. Video editor: Highlight the largest pink-stained circular structure labeled “Interlobar artery” at the left.

    - LAB MEDIA: Figure 2. Video editor: Highlight the images labelled as “Arcuate artery” and “Interlobular artery”.

  - Arterial normalization involved repeated mechanical stretching and potassium-induced stimulation, establishing stable baseline tension conditions across samples [1].

    - LAB MEDIA: Figure 5.

  - Cumulative addition of phenylephrine from 10-9 to 10-4 molar concentrations induced progressively stronger contractions in arterial rings in a dose-dependent manner [1]. In phenylephrine-precontracted arteries, increasing concentrations of acetylcholine from 10-8 to 3×10-5 molar produced concentration-dependent vasodilation [2].

    - LAB MEDIA: Figure 6. Video editor: Trace the rising trend of the line graph as each concentration (from left to right) is added.

    - LAB MEDIA: Figure 7. Video editor: Highlight the declining slope of the graph as concentration of acetylcholine increases, beginning at the leftmost arrow.