# Submission ID #: 68580

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=20916948

# Title: Femoral Vascular Graft Implantation in a Swine Model to Test Small-Diameter Vascular Grafts

# Authors and Affiliations:

# Georgina Iraola-Picornell1,2*, Esther Jorge1,3*, Albert Teis1,4,5, Oriol Rodríguez-Leor1,4,5, Daina Martínez-Falguera1,5, Maria Kalil1,2, Borja Montejo1, Antoni Bayes-Genis1,2,4,5, Elisabet Berastegui1,4, Christian Muñoz-Guijosa1,4,6, Carolina Gálvez-Montón1,4,5

# 1ICREC Research Program, Germans Trias i Pujol Health Research Institute (IGTP)

# 2Department of Medicine, Autonomous University of Barcelona

# 3Department of Basic Sciences, Faculty of Health Sciences at Manresa, University of Vic - Central University of Catalonia (UVic-UCC)

# 4Heart Institute (iCOR), Germans Trias i Pujol University Hospital

# 5CIBER Cardiovascular, Instituto de Salud Carlos III

# 6Cardiac Surgery Department, 12 de Octubre University Hospital

*These authors contributed equally

# Corresponding Authors:

# Carolina Gálvez-Montón 		cgalvez@igtp.cat

# Email Addresses for All Authors:

Georgina Iraola-Picornell		giraola@igtp.cat

Esther Jorge				ejorge@igtp.cat

Albert Teis				ateis.germanstrias@gencat.cat

Oriol Rodríguez-Leor			oriolrodriguez@gmail.com

Daina Martínez-Falguera		mdmartinez@igtp.cat

Maria Kalil				mkalil@igtp.cat

Borja Montejo				bmontejo@igtp.cat

Antoni Bayes-Genis			abayesgenis@gmail.com

Elisabet Berastegui			elisabetberasteguigarcia@gmail.com

# Christian Muñoz-Guijosa		chrmunozg@gmail.com

# Carolina Gálvez-Montón 		cgalvez@igtp.cat

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  22

Number of Shots:  35

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

  - Georgina Iraola-Picornell: We present a standardized surgical protocol to evaluate tissue-engineered vascular grafts in a porcine model of an end-to-end femoral artery transplantation, bridging preclinical testing and clinical application, especially for vascular grafts less than 6 mm of diameter [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.3.1

What technologies are currently used to advance research in your field?

  - Christian Muñoz-Guijosa: In our field, several advanced technologies play a key role in driving research forward. Reproducibly implanting small-caliber vascular grafts in the femoral position of pigs requires refined microsurgical techniques, precise perioperative management protocols and also, high-resolution ultrasound and angiography [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.3.1

What research gap are you addressing with your protocol?

  - Georgina Iraola-Picornell: Our protocol aims to standardize large-animal models for testing small-diameter grafts in arterial positions like the femoral artery to evaluate new tissue-engineering vascular grafts [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.8.1

What research questions will your laboratory focus on in the future?

  - Christian Muñoz-Guijosa: We aim to evaluate short-term patency and host response to a bioengineered vascular graft, named VasCraft, in a preclinical model. This graft comprises in a decellularized human saphenous vein and reendothelialized with umbilical cord blood-derived endothelial cells [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 4.2.1

Videographer: Obtain headshots for all authors available at the filming location.

# Ethics Title Card

This research has been approved by the Animal Experimentation Unit Ethical Committee at the Germans Trias i Pujol Health Research Institute (IGTP) and Government Authorities of the Generalitat de Catalunya

# Protocol

Videographer’s NOTE: Some shots without a clapboard were B-roll and are not necessary.

- Hemodynamic Monitoring and Preparation of the Surgical Area

Demonstrator: Georgina Iraola-Picornell

  - To begin, place the anesthetized animal in a ventral recumbent position on the operating table [1-TXT]. Using tape or bandages, secure each limb to the table to prevent movement during the procedure [2-TXT].

    - WIDE: Talent placing the animal on the operating table in a ventral recumbent position. TXT: Anesthesia (Induction): Dexmedetomidine (0.03 mg/kg); midazolam (0.3 mg/kg); Ketamine (3 mg/kg)

    - Talent taping or bandaging the limbs securely to the table. TXT: Maintain anesthesia with sevoflurane (1 - 3%)

  - Place surface electrodes on the animal's limbs to monitor electrocardiogram changes and heart rate throughout the surgical procedure [1].

    - Talent attaching electrodes to each limb of the animal.

  - Next, position a pulse oximeter on the tongue or the corner of the lip to monitor oxygen saturation continuously [1].

    - Talent positioning the pulse oximeter sensor gently on the animal's tongue or lip.

  - Wrap a non-invasive blood pressure cuff around the animal’s forelimb [1] and insert a temperature probe into the esophagus to measure body temperature [2].

    - Talent fitting a blood pressure cuff snugly around the forelimb of the animal.

    - Talent inserting the probe carefully into the esophagus of the animal.

  - Now, using surgical clippers, shave both the right and left femoral areas [1]. Wash the shaved areas with surgical soap to remove any debris [2] and disinfect the cleaned areas with three alternating rounds of 0.7 percent iodine and 70 percent ethanol under sterile conditions [3].

    - Talent shaving the right and left femoral regions using surgical clippers.

    - Talent scrubbing the shaved areas with surgical soap.

    - Talent disinfecting the areas by wiping the area with an iodine-soaked cotton.

  - Perform a surgical handwash thoroughly [1], then wear a sterile gown and gloves [2].

    - Talent scrubbing hands and forearms following surgical protocol.

    - Talent donning a sterile gown and gloves.

  - Drape the animal with a sterile surgical sheet to maintain aseptic conditions [1] and monitor the depth of anesthesia every 10 minutes throughout the surgery [2].

    - Talent covering the animal completely with a sterile drape.

    - Talent performing toe pinch to check anesthesia.

- Vascular Graft Implantation

Demonstrator: Christian Muñoz-García

  - For intraoperative analgesia, administer a fentanyl bolus if the animal shows signs of pain such as an increase in heart rate [1]. Author’s NOTE: Step removed

    - Talent injecting fentanyl into the animal’s catheter upon observing elevated heart rate.

  - Use an ultrasound probe to locate the right femoral artery and assess the blood flow [1-TXT].

    - Shot of the Ultrasound screen showing the right femoral artery and its blood flow. TXT: For intraoperative analgesia, administer a fentanyl bolus NOTE: The deleted information from step 3.1 is placed as a text overlay here

  - Using an electric scalpel, make a 7 to 10-centimeter vertical incision perpendicular to the linea inguinalis [1].

    - Talent making a vertical incision with an electric scalpel over the marked surgical site. Videographer’s NOTE: 3.3.1, 3.4.1, and 3.4.2 are combined.

  - Spear the subcutaneous tissues to access the underlying muscle layers [1] and electrocoagulate the small surrounding blood vessels to achieve hemostasis [2].

    - Talent spreading apart the subcutaneous tissues.

    - Talent applying electrocoagulation tool to small vessels until bleeding stops.

  - Use two Weitlaner retractors to gently separate the Sartorius, Rectus Femoris, Gracilis, and Pectineus muscles for better exposure to the superficial femoral artery [1].

    - Talent inserting and adjusting two Weitlaner retractors to separate the Sartorius, Rectus Femoris, Gracilis, and Pectineus muscles.

  - Administer 300 international units per kilogram of heparin intravenously to prevent intraoperative thrombosis prior to femoral artery clamping [1]. Author’s NOTE: Step removed

    - Talent injecting heparin intravenously using a syringe.

  - Now, apply vascular clamps proximally and distally on the right superficial femoral artery to achieve complete occlusion [1-TXT], ensuring the distance between clamps is approximately 1 centimeter longer than the segment to be resected [2].

    - Talent placing vascular clamp above the intended resection site. TXT: Administer 300 IU/kg heparin IV to prevent thrombosis before femoral artery clamping NOTE: The deleted information from step 3.6 is placed as a text overlay here
Videographer’s NOTE: 3.7.1 and 3.7.2 are combined

    - Shot showing both the clamps in place.

  - Confirm that the replacement graft is anatomically compatible in both length and diameter with the arterial segment to be resected [1-TXT].

    - Talent holding the graft alongside the clamped arterial segment. TXT: Ensure clamps are correctly positioned to control blood flow near the resection site  NOTE: The deleted information from step 3.9 is placed as a text overlay here

  - Ensure the clamps are properly positioned to control blood flow around the area of resection [1]. Author’s NOTE: Step removed

    - Talent pointing to the clamped area.

  - Next, excise a 5-centimeter segment of the right superficial femoral artery in a slightly oblique manner to facilitate posterior termino-terminal anastomosis near the clamps [1].

    - Shot of cutting the artery at an oblique angle using surgical scissors or a scalpel.

  - Then, cut the proximal and distal ends of the graft to match the resected segment [1]. Author’s NOTE: Step removed

    - Talent trimming the end of the graft using micro scissors.

  - Using a continuous 7-0 (7-oh) Prolene suture in a running stitch pattern, suture the proximal and distal anastomoses of the experimental graft to the clamped artery [1] from posterior to anterior and medial to lateral [2].

    - Talent performing termino-terminal anastomosis with running suture.

    - Shot of fully stitched segment.

  - Now, unclamp the proximal and distal clamps to restore vascular flow [1] and verify the absence of blood leakage from the anastomoses [2].

    - Talent carefully releasing clamps.

    - Talent pointing to the are showing no blood leakage.

  - Then, close the muscle and subcutaneous tissue using a resorbable size 1 continuous suture [1], and close the skin with a resorbable size 0 intradermal continuous suture [2].

    - Talent suturing muscle and subcutaneous tissue using a resorbable size 1 continuous suture.

    - Talent closing the skin with a resorbable size 0 intradermal continuous suture.

  - Finally, repeat the same surgical procedure on the left superficial femoral artery using the autograft as a control of the graft technique, if needed, and apply a sterile transparent dressing over the surgical site [1]. NOTE: VO is modified to get covered with the filmed shot 3.15.1.

    - Talent placing and pressing down a transparent dressing to cover the incision.

    - Talent positioning tools and graft materials.

# Results

- Results

  - The successful incorporation of the autograft into the femoral artery and its patency were verified non-invasively using Doppler ultrasound. On day 7, the ultrasound confirmed 100% graft patency with no signs of acute rejection or infection [1].

    - LAB MEDIA: Figure 2A. Video editor: Highlight the area labeled as “proximal suture”.

  - On day 30, Doppler ultrasound confirmed persistent graft patency and consistent diameter, with no visible defects [1].

    - LAB MEDIA: Figure 2C.

  - Angiography on day 30 further validated patency with no structural abnormalities in the arterial graft [1].

    - LAB MEDIA: Figure 2D.

  - Histological staining of the extracted graft at day 30 showed intact vascular wall structure, including clearly defined tunica adventitia [1], media [2], and intima layers [3].

    - LAB MEDIA: Figure 3C. Video editor: Highlight the part labeled “tunica adventitia”.

    - LAB MEDIA: Figure 3C. Video editor: Highlight the part “tunica media”.

    - LAB MEDIA: Figure 3C. Video editor: Highlight the part “tunica intima”.

  - Immunohistofluorescence staining revealed proper localization of endothelial and smooth muscle markers across all vascular layers of the autograft, supporting functional tissue integration [1].

    - LAB MEDIA: Figure 3D

Pronunciation Guide:

dexmedetomidine

Pronunciation link: https://www.howtopronounce.com/dexmedetomidine YouTubeCollins Dictionary+10How To Pronounce+10Definitions+10

IPA (AmE): /dɛksˌmɛdəˈtoʊmɪˌdin/

Phonetic: deks-med-eh-TOH-mi-deen

midazolam

No confirmed link found (not available in Merriam-Webster)

IPA (AmE): /ˌmɪdəˈzoʊləm/

Phonetic: mid-uh-ZOH-luhm

sevoflurane

No confirmed link found

IPA (AmE): /ˌsɛvoʊˈflʊərin/

Phonetic: sevo-FLOOR-een

linea inguinalis

linea link: https://www.merriam-webster.com/medical/linea How To PronounceMerriam-Webster+6Merriam-Webster+6Merriam-Webster+6

IPA: /ˈlɪniə/

Phonetic: LEE-nee-uh

inguinal link: https://www.merriam-webster.com/dictionary/inguinal Merriam-Webster+3Merriam-Webster+3Definitions+3

IPA: /ˈɪŋgwənəl/

Phonetic: ING-gwuh-nuhl

anastomosis

Link: https://www.merriam-webster.com/dictionary/anastomosis Merriam-Webster+13Merriam-Webster+13Oxford English Dictionary+13

IPA: /əˌnæs.təˈmoʊ.sɪs/

Phonetic: uh-NAS-tuh-MOH-sis

esophagus

No confirmed link found (not looked up, but standard)

IPA: /ɪˈsɒf.əɡəs/

Phonetic: ih-SOF-uh-guhs

Weitlaner

No confirmed link found (proper noun, surgical retractor)

IPA (estim.): /ˈvaɪtlənər/

Phonetic: VYTL-uh-nuhr

electrocoagulate

No confirmed link found

IPA: /ɪˌlɛk.troʊˈkoʊ.əɡjʊ.leɪt/

Phonetic: ih-LEK-troh-KOH-uh-gyu-layt

anastomoses (plural)

Same as anastomosis but plural

IPA: /əˌnæs.təˈmoʊˌsiːz/

Phonetic: uh-NAS-tuh-MOH-seez

fentanyl

No confirmed link found

IPA: /ˈfɛntənəl/

Phonetic: FEN-tuh-nul

Sartorius, Gracilis, Pectineus, Rectus Femoris (muscles)

No confirmed links found; classical Latin-derived terms

Sartorius: /sɑrˈtɔːriəs/ → sar-TAWR-ee-us

Gracilis: /ˈɡreɪsɪlɪs/ → GRAY-sih-lis

Pectineus: /pɛkˈtɪniəs/ → pek-TIN-ee-us

Rectus: /ˈrɛktəs/ → REK-tus

Femoris: /fɪˈmɔːrɪs/ → fih-MOR-is

adventitia, intima

No confirmed links found

adventitia: /əˌdvɛnˈtɪʃə/ → uh-dven-TISH-uh

intima: /ˈɪntɪmə/ → IN-tih-muh