# Submission ID #: 68595

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=20921523

# Title: Expansion Microscopy: High-Resolution Fluorescent Imaging with a Conventional Microscope

# Authors and Affiliations:

# Perrine Hervé1*, Léo Krüttli2*, Mélanie Bonhivers3, Loïc Rivière1, Bénédicte Durand2, Eloïse Bertiaux3, Marine H. Laporte2

# 1University of Bordeaux, CNRS, MFP, iMet UMR 5234

# 2Université Claude Bernard Lyon 1, MeLiS—UCBL—CNRS UMR 5284-INSERM U1314

# 3University of Bordeaux, CNRS, MFP, ProParaCyto UMR 5234

# *These authors contributed equally

# Corresponding Authors:

# Eloïse Bertiaux 	eloise.bertiaux@u-bordeaux.fr

# Marine H. Laporte 	marine.laporte2@univ-lyon1.fr

# Email Addresses for All Authors:

Perrine Hervé 		perrine.herve@u-bordeaux.fr

Léo Krüttli 		leo.kruttli@etu.univ-lyon1.fr

Mélanie Bonhivers 	melanie.bonhivers@u-bordeaux.fr

Loïc Rivière		loic.riviere@u-bordeaux.fr

# Bénédicte Durand 	benedicte.durand@univ-lyon1.fr

# Eloïse Bertiaux 	eloise.bertiaux@u-bordeaux.fr

# Marine H. Laporte 	marine.laporte2@univ-lyon1.fr

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  NO

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  NO

3. Filming location: Will the filming need to take place in multiple locations?   NO

Current Protocol Length

Number of Steps:  23

Number of Shots:  53

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

  - Eloïse Bertiaux: We are cell biologists investigating the role of microtubules and associated structures in different model systems. We aim at uncovering their functions in cell organization, division, ciliogenesis and morphogenesis.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.2.1

What significant findings have you established in your field?

  - Marine Laporte:  Over the past five years, we have contributed to improving the understanding of centriole molecular composition through the use of expansion microscopy, revealing key features of their assembly and function.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.2.1

What research gap are you addressing with your protocol?

  - Eloise Bertiaux: By combining expansion microscopy with cryo-fixation, our protocol enables high-resolution imaging while preserving near-native cellular ultrastructure, overcoming key limitations of classical super-resolution microscopy.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 6.3.1

What advantage does your protocol offer compared to other techniques?

  - Marine Laporte: Expansion microscopy is a low-cost and easy-to-implement method that facilitates access to super-resolution microscopy in all research laboratories.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 5.2.1

# Protocol

- Cryo-Fixation of Coverslips for Electron Microscopy Using a Cryo-Plunger System

Demonstrator: Marine Laporte

  - To begin, use a syringe and a needle to fill each 5-milliliter tube with 1 milliliter of extra-dry acetone under a chemical hood [1]. Place the tubes upright in metal rack and put them in liquid nitrogen for freezing [2].

    - WIDE: Talent using a syringe to fill 5-milliliter tubes with extra-dry acetone under a chemical hood.

    - Talent placing the acetone-filled tubes upright into a rack and submerging it in liquid nitrogen.

  - Fill half of a 30 by 30 centimeters, thick-walled polystyrene box with dry ice [1] and then fill the Vitrobot-type dewar with liquid nitrogen until evaporation stops [2].

    - Talent filling a large polystyrene box with dry ice to the halfway mark.

    - Talent pouring liquid nitrogen into a Vitrobot-type dewar and checking for cessation of evaporation. TXT: After 10 to 15 min, refill the dewar with liquid nitrogen if necessary

  - Now, remove the spider from the plunging apparatus [1] and fill the metallic plunging chamber with liquid ethane [2]. Wait for 10 minutes to allow the ethane to reach equilibrium [3].

    - Talent removing the spider from the plunging setup.

    - Talent carefully filling the metal chamber with liquid ethane.

    - Close-up of the metal chamber stabilizing undisturbed.

  - Then, grab a 12-millimeter coverslip with thin tweezers [1] and dab it on a tissue paper to soak up any excess medium from the coverslip [2]. Hold the coverslip halfway using tweezers fitted with a clamping ring compatible with the cryo-plunger [3].

    - Talent picking up a 12-millimeter coverslip using thin tweezers.

    - Talent dabbing the coverslip with tissue paper to absorb excess medium.

    - Close-up of talent positioning the coverslip in the tweezers with a clamping ring.

  - Place the tweezers holding the coverslip into the cryo-plunger holder [1] and use Whatman paper to blot away any remaining medium [2]. Activate the cryo-plunger to plunge the coverslip into the ethane solution contained in the metal chamber [3].

    - Talent securing the tweezers into the holder of the cryo-plunger.

    - Talent using Whatman paper to remove the remaining liquid from the coverslip.

    - Talent pressing the activation mechanism on the cryo-plunger to plunge the coverslip.
AUTHOR’S NOTE: Merge shots 2.5.2 and 2.5.3 – The unique take called 2.5.2 was done

  - Then, quickly transfer the coverslip into the tube containing frozen acetone [1].

    - Talent immediately placing the freshly plunged coverslip into a tube pre-filled with frozen acetone.

- Freeze Substitution and Rehydration

Demonstrator: Eloïse Bertiaux

  - Incubate the tubes containing the coverslips in dry ice at an angle of approximately 45 degrees [1]. Place the closed container on an orbital shaker set to 4 degrees Celsius and agitate overnight to let the temperature rise gradually rise [2].

    - Talent arranging the tubes at a 45-degree tilt in a dry ice bed.

    - Talent placing the closed container on an orbital shaker.

  - Next, remove most of the dry ice from the box [1]. Continue agitation on the orbital shaker for an additional 45 minutes to let the temperature rise from minus 80 degrees Celsius to minus 20 degrees Celsius [2]. Briefly open and close each tube to release internal pressure [3].
NOTE: 3.2.2 was moved after 3.2.3 at author’s request. Shot numbers have been edited accordingly

    - Talent scooping out dry ice from the container.
AUTHOR’S NOTE: Shots 3.2.1 and 3.2.3 were merged under 1 unique take called “3.2.1

    - Talent quickly opening and closing the cap of a tube to release pressure.
AUTHOR’S NOTE: Move 3.2.2 after 3.2.3

    - Talent placing the tubes back on the shaker and starting the agitation.

  - Then, transfer each coverslip into a 12-well plate or a suitable container pre-filled with pre-chilled 100 percent ethanol solution [1]. After a 5-minute incubation, rehydrate the coverslips through a graded ethanol series [2-TXT].

    - Talent using tweezers to place frozen coverslips into a 12-well plate with chilled ethanol.

    - Talent transferring coverslip to the next well. TXT: EtOH 100% (5 min); EtOH 95% (3 min 2x); EtOH 70% (3 min); EtOH 50% (3 min); EtOH 25% (3 min); H2O

  - After transferring the coverslips into PBS, place them under a microscope [1]. Use fine tweezers to gently scratch the surface and orient all coverslips with the correct side facing up [2].

    - Talent placing coverslips in PBS.

    - Close-up of scratching the surface of a coverslip with fine tweezers to adjust its orientation.

- Protein Anchoring, Gelation and Denaturation

Demonstrators: Léo Krüttli, Marine Laporte & Eloïse Bertiaux

  - Place the coverslips into a 4-well plate filled with 0.5 to 1 milliliter of acrylamide and formaldehyde solution [1]. Then, incubate the coverslips in a solution containing 1.4 percent formaldehyde and 2 percent acrylamide in 1x PBS for without agitation [2-TXT].

    - Talent transferring coverslips into wells containing the AA/FA mixture.

    - Talent placing the plate in a 37-degree Celsius incubator and closing the door. TXT: 3 - 5 h; 37 °C

  - Next, thaw 10% APS and the gelation solution on ice for 10 minutes before gelation [1]. Prepare a humid chamber using a thin layer of wet tissue and parafilm [2], then store it at 4 degrees Celsius [3]. After 10 minutes, place the humid chamber on a cold block for use during gelation [4].

    - Talent placing labeled tubes of APS, and gelation solution on ice to thaw.

    - Talent lining a chamber with wet tissue and parafilm.

    - Talent keeping the chamber at 4 degrees Celsius.

    - Talent removing and placing the prepared humid chamber on a cold block.

  - Now, remove the coverslips from the protein anchoring solution [1] and blot away excess liquid using tissue paper in two successive passes [2]. Add TEMED and APS to the gelation solution to reach a final concentration of 0.5 percent [3] and vortex for 2 to 3 seconds [4].

    - Talent picking up coverslip from anchoring solution.

    - Talent placing the coverslip on folded tissue paper.
AUTHOR’S NOTE: 4.3.1 and 4.3.2 shots were merged under 1 unique take called “4.3.1”

    - Talent adding APS to the gelation solution.

    - Talent vortexing the gelation mixture briefly.

  - Then, pipette two 35-microliter drops onto the parafilm in the humid chamber [1] and gently place each coverslip over a drop with the cells facing downward into the gelation solution [2].

    - Talent pipetting droplets onto the parafilm surface.

    - Close-up of talent inverting and placing coverslips over the gelation drops.
AUTHOR’S NOTE: 4.4.1 and 4.4.2 shots were merged under 1 unique take called “4.4.1”

  - Incubate the setup on ice for 5 minutes to facilitate gel penetration [1]. Then Transfer the humid chamber to a 37 degrees Celsius incubator for 30 to 60 minutes [2].

    - Talent placing the chamber on ice.
NOTE:Shot deleted by authors

    - Talent transferring the chamber to an incubator.

  - For denaturation, use a biopsy punch tool with a 0.4-centimeter diameter to extract gel pieces [1] and place them into a 6-well plate filled with 1 milliliter of denaturation buffer [2].

    - Talent using a biopsy punch and taking out gel discs.

    - Talent placing the denaturation buffer containing 6-well plate.

  - Agitate the plate for 10 to 15 minutes until the gels detach from the coverslips [1] and transfer the detached gel pieces into 1.5-milliliter microcentrifuge tubes filled with fresh denaturation buffer [2].

    - Talent placing the 6-well plate in a shaker.

    - Talent transferring detached gels into labeled microcentrifuge tubes.

  - Incubate the gels for 90 minutes at 95 degrees Celsius [1]. After incubation, transfer the gels into double-distilled water for 10 minutes for washing [2-TXT]. Measure the diameter of the gels using millimeter paper to evaluate the gel expansion factor [3].

    - Talent placing microcentrifuge tubes in a heating block or water bath at 95 degrees Celsius.

    - Talent washing the gels in double-distilled water across three cycles. TXT: Wash the gel in ddH2O 3x

    - Talent placing the gel on a millimeter paper or measuring the gel with a caliper.

- Mounting and Imaging the Gel

Demonstrator: Léo Krüttli

  - Coat clean coverslips with approximately 200 milliliters of poly-lysine solution [1]. Incubate the coverslips for 1 hour at room temperature [2], then wash them three times with double-distilled water to remove any excess poly-lysine [3]. After the coverslips are dry, store them at 4 degrees Celsius until further use [4].

    - Talent pipetting poly-lysine solution over clean coverslips.

    - Talent placing the coverslips on the bench for incubation at room temperature.

    - Talent placing the coverslips in a dish filled with double-distilled water.

    - Talent placing the dried coverslips in a refrigerator.

  - Next, place the expanded gels onto non-coated coverslips situated in the imaging chamber [1. Perform fluorescence microscopy to check and confirm the orientation of the samples [2].
NOTE:Step deleted by authors

    - Talent carefully transferring gel samples onto plain coverslips inside the imaging chamber.

    - Shot of talent operating the fluorescence microscope.

  - Place the gel properly orientated with the cells facing up onto lint-free paper and allow them to dry to eliminate excess water [1].

    - Talent placing oriented gels onto lint-free paper and patting them dry.

  - Finally, mount the dried gels with the cells facing down onto poly-D-lysine-coated coverslips [1]. and observe the mounted samples using an inverted microscope [2].

    - Talent carefully mounting the dry gel onto poly-D-lysine-coated coverslip.

    - Talent placing the sample in an inverted microscope.
NOTE:Shot deleted by authors

# Results

- Results

  - Cryo-fixation preserved the mitochondrial network in RPE1 cells more effectively than PFA fixation, as shown by NHS-ester and ATP5a staining [1], and allowed resolution of mitochondrial cristae, which were not visible with PFA fixation [2].

    - LAB MEDIA: Figure 3A. Video editor: Highlight the NHS-ester and ATP5a Images.

    - LAB MEDIA: Figure 3B. Video editor: Zoom in on the red-arrowed mitochondrial cristae in 3B".

  - Cryo-fixation resulted in better preservation of dynamic microtubules, including cytoplasmic and astral microtubules in mitotic RPE1 cells [1], compared to PFA fixation [2].

    - LAB MEDIA: Figure 3D, 3E. Video editor: Highlight the area pointed by red-arrows in D.

    - LAB MEDIA: Figure 3D, 3E. Video editor: Highlight the area pointed by red-arrows in E.

  - In Trypanosoma brucei, cryo-fixation better preserved the architecture of the mitochondrion and general cellular structure [1] compared to PFA fixation, as visualized with TDH and NHS-ester staining [2]. Additionally, cryo-fixation preserved the endoplasmic reticulum in them better than PFA fixation, based on BiP (B-I-P) staining [3].

    - LAB MEDIA: Figure 4A, 4B, 4C, 4D. Video editor: Highlight yellow TDH image and NHS ester in A

    - LAB MEDIA: Figure 4A, 4B, 4C, 4D. Video editor: Highlight yellow TDH image and NHS ester in C

    - LAB MEDIA: Figure 4E, 4F. Video editor: Highlight the BiP image in E

    - LAB MEDIA: Figure 4E, 4F. Video editor: Highlight the BiP image in F

  - Cracks were observed in cryo-fixed RPE1 cells, but these did not disrupt the ultrastructure of organelles such as mitochondria or microtubules [1].

    - LAB MEDIA: Figure 5A. Video editor: Show the cracks (indicated by WHITE arrowheads).

  - Poor cryo-fixation was indicated by bubble-like structures and wavy microtubules, accompanied by loss of membranous organelle integrity [1].

    - LAB MEDIA: Figure 5B. Video editor: Highlight the bubble-like formations indicated by white arrowheads.

  - Cryo-fixation quality was dependent on the clarity of sodium acrylate solutions, with usable solutions appearing colourless or slightly yellow and translucent [1], while unusable ones appeared cloudy and orange [2].

    - LAB MEDIA: Figure 5C. Video editor: highlight the tubes 1 and 2

    - LAB MEDIA: Figure 5C. Video editor: highlight the tube 3

Pronunciation Guide:

1. microtubules

Pronunciation link: https://www.merriam-webster.com/dictionary/microtubules YouTube+10How To Pronounce+10How To Pronounce+10
IPA: /ˌmaɪ.kroʊˈtuː.bjuːlz/
Phonetic Spelling: my-kroh-TOO-byools

2. tubulin

Pronunciation link: https://www.merriam-webster.com/dictionary/tubulin Merriam-Webster
IPA: /ˈtuː.bə.lɪn/
Phonetic Spelling: TOO-buh-lin

3. Trypanosoma brucei

Pronunciation link: https://www.definitions.net/pronounce/trypanosoma%20brucei Definitions
IPA: /ˌtraɪpənoʊˈsoʊmə ˈbruːsaɪ/
Phonetic Spelling: try-puh-noh-SOH-muh BROO-sigh

4. ciliogenesis

Pronunciation link: No confirmed link found
IPA: /ˌsɪ.li.oʊˈdʒɛn.ə.sɪs/
Phonetic Spelling: sih-lee-oh-JEN-uh-sis

5. morphogenesis

Pronunciation link: No confirmed link found
IPA: /ˌmɔːr.foʊ.dʒəˈnɛs.ɪs/
Phonetic Spelling: mor-foh-jen-ESS-is

6. centriole

Pronunciation link: No confirmed link found
IPA: /ˈsɛn.tri.oʊl/
Phonetic Spelling: SEN-tree-ohl

7. ultrastructure

Pronunciation link: No confirmed link found
IPA: /ˈʌl.trəˌstrʌk.tʃər/
Phonetic Spelling: UL-truh-STRUK-chur

8. super-resolution

Pronunciation link: No confirmed link found
IPA: /ˈsuː.pər ˌrɛzəˈluː.ʃən/
Phonetic Spelling: SOO-per rez-uh-LOO-shun

9. cryo-fixation

Pronunciation link: No confirmed link found
IPA: /ˈkraɪ.oʊ fɪkˈseɪʃən/
Phonetic Spelling: KRY-oh fik-SAY-shun

10. Vitrobot

Pronunciation link: No confirmed link found
IPA: /ˈvaɪ.troʊ.bɑːt/
Phonetic Spelling: VY-troh-bot

11. ethane

Pronunciation link: No confirmed link found
IPA: /ˈiːθeɪn/
Phonetic Spelling: EE-thayn

12. acrylamide

Pronunciation link: No confirmed link found
IPA: /əˈkrɪl.əˌmaɪd/
Phonetic Spelling: uh-KRIL-uh-myd

13. formaldehyde

Pronunciation link: No confirmed link found
IPA: /ˌfɔːr.məˈlɛ.haɪd/
Phonetic Spelling: for-muh-LE-hyd

14. TEMED

Pronunciation link: No confirmed link found
IPA: /ˈtɛmɛd/
Phonetic Spelling: TEM-ed

15. microcentrifuge

Pronunciation link: No confirmed link found
IPA: /ˌmaɪ.kroʊˈsɛn.trəˌfjuːdʒ/
Phonetic Spelling: my-kroh-SEN-truh-fyoohj

16. poly-lysine / poly-D-lysine

Pronunciation link: No confirmed link found
IPA: /ˌpɑː.liˈlaɪ.saɪn/
Phonetic Spelling: PAH-lee-LY-syne

17. RPE1

Pronunciation link: No confirmed link found
IPA: /ɑːr piː iː wʌn/
Phonetic Spelling: R-P-E-one

18. BiP

Pronunciation link: No confirmed link found
IPA: /bɪp/
Phonetic Spelling: bip

19. sodium acrylate

Pronunciation link: No confirmed link found
IPA: /ˈsoʊ.di.əm əˈkrɪl.eɪt/
Phonetic Spelling: SOH-dee-um uh-KRIL-ayt