# Submission ID #: 68620

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=20929148

# Title: Accessing the Subdural Space of the Rodent Spinal Cord for Treatment Delivery

# Authors and Affiliations:

# Salvador Lopez1, Chien Kow2, Manju Ganesh1, Darren Svirskis1, Bruce Harland1

# 1School of Pharmacy, University of Auckland

# 2 Neurosurgical Department, Auckland City Hospital

# Corresponding Authors:

# Salvador Lopez	salvador.lopez@auckland.ac.nz

# Email Addresses for All Authors:

# Chien Kow		chienyew@gmail.com

# Darren Svirskis		d.svirskis@auckland.ac.nz

# Manju Ganesh		manju.ganesh@auckland.ac.nz

# Bruce Harland		bruce.harland@auckland.ac.nz

# Salvador Lopez	salvador.lopez@auckland.ac.nz

*Note for JoVE: We have included timestamps for each of the pre-op wide shots (2.1-2.5) that were uploaded by the JoVE videographer. These timestamps indicate the section of each video that we want included for each corresponding step. Timestamps and associated video file names are written at the end of each step in the protocol.

*Final script version uploaded to project page: 68620_Lopez_Shotlist and Script_Final.docx

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  Yes

If Yes, can you record movies/images using your own microscope camera?

SCOPE shots: 3.1.1b, 3.1.2, 3.2.1, 3.2.2, 3.3.1, 3.3.2, 3.3.3, 3.4.1, 3.4.2, 3.5.1, 3.6.1, 3.6.2, 4.1.1, 4.1.2, 4.1.3,

4.2.1, 4.2.2, 4.3.1, 4.4.1, 4.5.1, 4.6.1, 5.1.1.1, 5.2.1, 5.2.2, 5.3.1, 5.3.2, 5.5.1, 5.6.1, 5.6.2, 5.7.1

Authors, please use your microscope camera to film the SCOPE shots and upload the files to your project page along with timestamp indications as soon as possible: https://review.jove.com/account/file-uploader?src=20929148

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  25

Number of Shots:  46 (30 Scope)

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

Authors: I have chosen the 5 best answers. Please let me know if you would like to swap any questions with the deleted ones.
Please make a note of who presented which answer and let us know after the shoot

REQUIRED: What is the scope of your research? What questions are you trying to answer?

  - Salvador Lopez: We’re exploring whether delivering treatment directly onto injured spinal tissue, via subdural administration, can improve recovery and function after spinal cord injury.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.3.1

What are the current experimental challenges?

  - Manju Ganesh: Current challenges for our research revolve around broadening the scope of our treatment modalities. We want to create a reproducible and reliable treatment paradigm that can be applied in multiple scenarios.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll:3.2.1

What significant findings have you established in your field?

  - Bruce Harland: Recently, our group has shown improvements in motor and sensory function in spinal cord injured rodents following electric field stimulation treatment.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll:4.3.1

What advantage does your protocol offer compared to other techniques?

  - Manju Ganesh : Our protocol delivers treatments into the intrathecal space, bypassing the blood-spinal cord barrier, offering potentially greater efficacy than standard epidural delivery methods.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll:5.2.1

What research questions will your laboratory focus on in the future?

  - Salvador Lopez: Moving forward, our group plans to utilize this method in larger animal models as well as examine its effects on bladder function following SCI.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll:6.3.1

Videographer: Obtain headshots for all authors available at the filming location.

# Testimonial Questions (OPTIONAL):

# Videographer: Please capture all testimonial shots in a wide-angle format with sufficient headspace, as the final videos will be rendered in a 1:1 aspect ratio. Testimonial statements will be presented live by the authors, sharing their spontaneous perspectives.

How do you think publishing with JoVE will enhance the visibility and impact of your research?

  - Salvador Lopez, Research Fellow/Did Not answer: (authors will present their testimonial statements live)

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll:6.3.1

Can you share a specific success story or benefit you’ve experienced—or expect to experience—after using or publishing with JoVE? (This could include increased collaborations, citations, funding opportunities, streamlined lab procedures, reduced training time, cost savings in the lab, or improved lab productivity.)

  - Bruce Harland, Senior Research Fellow/Did Not answer: (authors will present their testimonial statements live)

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll:5.2.1

Ethics Title Card

This research has been approved by the University of Auckland’s Institution Animal Ethics Committee (AEC) and the University Animal Welfare Officers (AWO)

# Protocol

- Preparation of the Rat for the Surgical Procedure

Demonstrator: Manju Ganesh

  - To begin, position the anesthetized rat on a clean table [1] and shave the fur along the back of the animal, starting at the base of the neck and ending at the top of the hips [2]. After shaving, administer preoperative subcutaneous injections [3].

    - WIDE: Talent placing the anesthetized animal on a clean workbench.TXT: Anesthesia: 3% Isoflurane. Timestamp: 0:05-0:35/ Video: P1003919

    - Talent shaving the back of the rat from the base of the neck to the hips using electric clippers. Timestamp: start 0:25-0:50/ Video:  P1003920

    - Added Shot: Talent administers preoperative subcutaneous injections of Temgesic, Baytril, Metacam, and saline (described in protocol). Timestamp: 0:16-0:41/ Video: P1003921

  - Using chlorhexidine scrub, disinfect the shaved area in a circular motion moving outward from the center [1]. Then apply chlorhexidine tincture in the same manner. Administer subcutaneous Marcaine and repeat previous steps for disinfection [2] Then apply ophthalmic lubricant to the eyes to prevent drying out [3].

    - Talent scrubbing the center of the shaved area with chlorhexidine scrub and tincture using a circular motion. Timestamp: 0:10-25 & 0:30-0:45/ Video: P1003923

    - B-roll captured and labelled as: 2.2.1b

    - Talent applying chlorhexidine tincture using a circular motion. Timestamp: 0:30-0:45

    - B-roll captured and labelled as: 2.2.2b

    - 2.2.2.5	Added Shot: Talent administering subcutaneous Marcaine in several locations along the site of surgical incision. Timestamp: 0:15-0:40/ Video: P1003924

2.2.1b Added Shot: Talent applying second round of chlorhexidine scrub and tincture using a circular motion. Timestamp: 0:10-25 & 0:30-0:40/ Video: P1003925

    - 2.2.2b Added Shot: Talent applying second round of tincture using a circular motion. Combine 2.2.2b with 2.2.1b

    - Talent applying lubricant to both eyes of the rat. Timestamp: 0:15-0:35/ Video: P1003926

  - Now, transfer the rat onto a heating pad placed under the microscope and organize all sterile tools [1]. Drape a press-and-seal film over the shaved area [2]. Cut an opening just large enough to expose the surgical field without revealing unshaved fur [3].

    - Talent placing the rat on a heating pad under the surgical microscope. Timestamp: 0:20-0:32/ P1003928

    - 2.3.1.1 (Moved 2.5.2 to here and renamed): Talent being handed sterile surgical tools to place in sterile field. Timestamp: 0:24-0:36 & 1:13-1:23/ Video: P1003929

    - Talent laying a press-and-seal drape over the rat’s back and cutting circular window to expose the surgical region. Timestamp: 0:06-0:13/ Video: P1003930

    - Talent cutting a circular window in the drape to expose only the surgical region. Timestamp: 0:13-0:29/ Video: P1003930

  - Insert a rectal temperature probe to monitor the rat’s temperature [1]. Adjust the heating pad as needed to maintain a temperature of approximately 37 degrees Celsius [2] and perform animal welfare checks every 5 to 10 minutes [3].

    - Talent inserting the rectal temperature probe into the rat. Timestamp: 0:06-0:26/ Video: P1003932

    - Talent adjusting heating pad settings while monitoring temperature. Timestamp: 0:18-0:26/ Video: P1003933

    - Talent checking the rat’s respiration. Timestamp: 0:10-0:20/ Video: P1003934

  - Next, administer a pedal reflex test to confirm the animal has reached a surgical plane of anaesthesia. Once confirmed, begin the surgical procedure [1].

    - Talent pinching the hind paw of the rat with fingers to assess pedal reflex. Timestamp: 0:09-0:16/ Video: P1003935

    - Please move shot 2.5.2 (talent being handed sterile surgical tools to place in sterile field) directly after 2.3.1

- Tissue Dissection to Access the Spine

Demonstrator: Salvador Lopez

  - Using a scalpel blade, make a 6 to 8-centimeter long linear incision in a rostral to caudal direction, beginning just below the base of the neck and extending over the thoracic hump to expose vertebrae T9 to T13 [1]. Then, with Graefe forceps and micro spring scissors, gently retract the connective tissue beneath the incision to expose the spinal muscle tissue [2].

    - Used WIDE and SCOPE shot for this step: making a straight incision from the base of the neck to the thoracic hump using a scalpel blade. Timestamp: 0:13-0:22/ Video: P1003936

    - 3.1.1b	Added shot: SCOPE: same as 3.1.1 but through the microscope, can use a combination of the shots

    - SCOPE: using micro spring scissors to separate the connective tissue under the incision.

  - Now, identify the T13 and T12 spinous processes by locating the two white V-shaped tendons in the caudal half of the incision. The spinous process just rostral to this second V, moving head to tail, is T13 [1].

    - SCOPE:  Close-up of the two white V-shaped tendons within the incision area, pointing to the spinous process just in front of the V-shaped tendons.

  - Starting between T13 and T12, moving toward T9, use micro scissors to make small incisions in the connective tissue between each vertebral process to separate them [1]. Then, cut a parallel channel along the side of the spinous processes to expose the lamina of each segment [2]. Repeat this dissection on both sides of the spine [3].

    - SCOPE:  making small cuts between spinous processes from T13 to T9.

    - SCOPE: cutting longitudinal channels in muscle parallel to the spinous processes.

    - SCOPE:  repeating the muscle cuts on the opposite side of the spine.

  - Next, attach serrefine clamps to the muscle walls to hold the surgical site open [1]. Place one pair at T13, another at the T11-T12 border, and a third at the T11-T10 border [2].

    - SCOPE: Talent placing serrefine clamps to each side of the exposed muscle wall starting with T12.

    - SCOPE: Talent positioning final two clamps at T10, having already placed clamps at T11.

  - Once the channels are defined, use micro scissors or rongeurs to remove the connective tissue between each spinous process [1].

    - SCOPE: excising the inter-process connective tissue with micro scissors.

  - Continue clearing until all connective tissue and muscle is dissected from the lamina between rostral T13 and caudal T9 [1]. Ensure the laminae are clearly exposed on both sides for the laminectomy.

    - SCOPE: Talent using fine instruments to clean tissue off and remove ligaments from the laminae along the vertebrae.

- Laminectomy Procedure

Demonstrator: Salvador Lopez

  - Locate the caudal portion of the T12 lamina that overlaps the rostral portion of T13 [1]. Position the tip of the rongeurs underneath this overlapping area [2] and gently remove small pieces of the lamina [3].

    - SCOPE:  pointing to the caudal T12 lamina above the rostral T13.

    - SCOPE:  inserting the rongeur tip under the lamina.

    - SCOPE:  extracting bone fragments.

  - Alternate from one side to the other, continuing to remove portions of the T12 lamina [1]. To enlarge the opening, extend the cut into the midline of the process and expose the spinal cord, forming a hole approximately 5 millimeters across [2].

    - SCOPE:  trimming away bone from the T12 lamina from side to side using rongeurs.

    - SCOPE:  widening the hole toward the midline until the spinal cord is visible.

  - Then, position the rongeurs under the lamina on both sides and carefully create a uniform channel extending toward the cranial end of T12 [1].

    - SCOPE: inserting rongeurs beneath both sides of the lamina and clearing a clean channel forward along T12.

  - Repeat the same procedure to remove the lamina from T11 [1] and T10 [2].

    - SCOPE:  of lamina being removed at T11 using rongeurs.

    - SCOPE:  of lamina being removed at T10 using rongeurs.

  - Once the laminectomy exposes an adequate area from T10 to T12, gently flush the thecal sac with saline [1].

    - SCOPE: Talent using a syringe to gently irrigate the thecal sac with saline.

- Durotomy for Treatment Application

  - Perform the durotomy using the dissection microscope to access the subdural space of the spinal cord [1]. Confirm that the chosen site for the durotomy and the region directly in front of it are free of branching blood vessels from the midline vessel [2].

    - Shot wide: Talent gathering instruments for durotomy. Timestamp: 0:05-0:17/ P1003937

    - SCOPE: “Microscope view showing a clear area around the midline vessel at the entry site.”

  - Bend a 27-gauge needle to a 90-degree angle with the bevel facing upward [1].

    - SCOPE: Talent holding the 27-gauge needle with forceps and bending it to a right angle with the bevel oriented upward.

  - At the desired entry point, pierce the dura mater gently with the tip of the bent needle. Once the dura is punctured, carefully lift the needle dorsally to tear a circular hole in the dura, no more than 2 millimeters on either side of the midline blood vessel [1]. Look for a slight leakage of cerebrospinal fluid to confirm successful durotomy [2].

    - SCOPE:  Shot of the needle tip piercing the dura mater with delicate pressure at the target location then lifting the needle dorsally to create a small round opening in the dura mater.

    - SCOPE: kimwipe clearing area of CSF seeping from the dura; a sign of successful durotomy.

  - Use the holes made in the dura to deliver treatment or insert a device into the subdural space [1]. For this protocol, insert intrathecal catheters either to guide a stimulation device or to inject hydrogel.

    - SCOPE: using fine tools to insert an intrathecal catheter through the dural hole.

  - Confirm that the catheter tips are visible under the dura mater before proceeding. A successful placement is evident if the catheters glide beneath the dural membrane and over the small spinal blood vessels [1]. Once in place, use the catheters to deliver the desired treatment [2] / [3].

    - SCOPE:  View of the catheter tips beneath the transparent dura, smoothly sliding over spinal vessels beneath dura.

    - SCOPE: Close-up of the catheter guiding the device or hydrogel into the subdural space

    - Added shot: SCOPE:  Device being pulled through subdural space

  - After delivering the treatment, insert a piece of sterile surgical gel-foam into the laminectomy cavity to promote haemostasis, aid healing, and maintain spinal structure [1].

    - SCOPE: Talent positioning a rectangular piece of gelfoam into the exposed spinal cavity.

  - Finally, use 4-0 (4-oh) polydioxanone absorbable suture to close the muscle [1] and skin [2] layers above the spine.

    - WIDE SHOT: Talent stitching the muscle layers closed using absorbable sutures. Timestamp (muscle): 0:18-0:34/ P1003941

    - Added shot: WIDE SHOT: Talent stitching the skin layer closed using absorbable suture. Timestamp (skin): 0:20-0:40/ P1003943

# Results

- Results

  - A bioelectronic device was implanted [1] or a hydrogel was delivered into 6-to-8-week-old Sprague-Dawley rats [2].

    - LAB MEDIA: Figure 4A

    - LAB MEDIA: Figure 4B

  - Following each procedure, all animals maintained normal motor function with no difference in BBB score between groups 7 days post-procedure, indicating the safe implementation of the described protocol [1].

    - LAB MEDIA: Figure 4C Video editor: Highlight the black, blue and yellowish dot for D7

NOTE to the Authors:

1. It's better if you upload the video files for the scope shots (tagged as SCOPE) after naming each file according to the shot number (3-digit bullet, e.g, 2.2.1, 2.1.3, etc).

Please note that every shot (3-digit step) should have the best corresponding video clip of only 20 to 25 seconds and not more than that, so that the voice narration can match the duration

2. But if you prefer to record all the steps together in a continuous video and upload a single file, that is okay.

In this case, you need to indicate the corresponding timestamp against each shot.

For example, if the action described in shot 2.1.2 is present in the file xxx.mp4 and occurs between timestamps 00:30 and 00:45 , then write the following in the script after each shot description.

Example:

Shot 2.1.2 SCOPE: performing ........... action.   xxx.mp4 00:30-00:45.

2.2.2 SCOPE: cutting the.................part... xxx.mp4 01:00-01:25

Again, only 20 to 25 seconds per shot

If a particular shot takes minutes to complete, please choose timestamps at the beginning and end of the process so that the total time is again within the 25-second limit.

Example: 3.1.1 SCOPE: inserting a 7-gauge intravenous needle into the aorta and clamping it securely.  xxx.mp4 00:00-00:10 and 02:30-02:40