# Submission ID #: 68644

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=20936468

# Title: High-Plex Imaging Using Spectral Confocal Microscopy to Minimize Non-Specific Tissue Fluorescence

# Authors and Affiliations:

# Saven Denha*1,2, Vitoria M. Olyntho*1,2, Jake Colautti1,2, Dima Traboulsi1,2, Dan Stevens3, Susan Waserman2, Doron D. Sommer4, Manel Jordana1,2, Joshua F.E. Koenig1,2

# 1McMaster Immunology Research Centre, Department of Medicine, Faculty of Health Sciences, McMaster University

# 2Schroeder Allergy and Immunology Research Institute, Faculty of Health Sciences, McMaster University

# 3Carl Zeiss Canada MicroImaging

# 4Division of Otolaryngology, Department of Surgery, McMaster University Medical Centre, McMaster University

*These authors contributed equally

# Corresponding Authors:

# Joshua F.E. Koenig		Koenigjf@mcmaster.ca

# Email Addresses for All Authors:

Saven Denha			denhas@mcmaster.ca

Vitoria M. Olyntho		Vitoria.MurakamiOlyntho@rockefeller.edu

Jake Colautti			colauttj@mcmaster.ca

Dima Traboulsi		trabould@mcmaster.ca

Dan Stevens			dan.stevens@zeiss.com

Susan Waserman		waserman@mcmaster.ca

Doron D. Sommer		sommerd@mcmaster.ca

# Manel Jordana		jordanam@mcmaster.ca

# Joshua F.E. Koenig		Koenigjf@mcmaster.ca

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes

If Yes, we will need you to record using screen recording software.

We recommend using the screen capture program OBS. JoVE’s tutorial for using OBS Studio is provided at this link: https://review.jove.com/v/5848/screen-capture-instructions-for-authors?status=a7854k

As these files are necessary for finalizing your script, please upload all screen-captured video files to your project page as soon as possible.

3. Filming location: Will the filming need to take place in multiple locations?   Yes

If Yes, how far apart are the locations? 500 meters

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. Yes

Current Protocol Length

Number of Steps: 25

Number of Shots: 57 (32 SC)

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

INTRODUCTION:

What is the scope of your research? What questions are you trying to answer?

  - Saven Denha: We optimized an IBEX workflow combining heparin blocking and spectral detection to suppress background in eosinophil-rich human tissues.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.3.1

What are the current experimental challenges?

  - Dima Traboulsi: Autofluorescence and eosinophil granule binding obscure true signal; we target both to enable clean, high-plex maps

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.5.1

CONCLUSION:

What significant findings have you established in your field?

  - Saven Denha: Heparin treatment eliminated granule binding without reducing true signal, maintaining strong background suppression across IBEX cycles.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.3.1

What advantage does your protocol offer compared to other techniques?

  - Saven Denha: Spectral unmixing minimized bleed-through and autofluorescence, increasing signal-to-background and significantly speeding acquisition versus multi-track confocal.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 4.3.1

How will your findings advance research in your field?

  - Dima Traboulsi: This enables sensitive, robust spatial proteomics in challenging tissues, supporting accurate cell-state mapping and neighborhood-level analysis.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 5.1.2

Videographer: Obtain headshots for all authors available at the filming location.

# Testimonial Questions (OPTIONAL):

# Videographer: Please capture all testimonial shots in a wide-angle format with sufficient headspace, as the final videos will be rendered in a 1:1 aspect ratio. Testimonial statements will be presented live by the authors, sharing their spontaneous perspectives.

- Testimonial statements will not appear in the video but may be featured in the journal’s promotional materials.

- Provide the full name and position (e.g., Director of [Institute Name], Senior Researcher [University Name], etc.) of the author delivering the testimonial.

- Please answer the testimonial question live during the shoot, speaking naturally and in your own words in complete sentences.

How do you think publishing with JoVE will enhance the visibility and impact of your research?

  - Saven Denha, PhD candidate: (authors will present their testimonial statements live)

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.1.1

Can you share a specific success story or benefit you’ve experienced—or expect to experience—after using or publishing with JoVE? (This could include increased collaborations, citations, funding opportunities, streamlined lab procedures, reduced training time, cost savings in the lab, or improved lab productivity.)

  - Saven Denha, PhD candidate: (authors will present their testimonial statements live)

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 5.3.1

Ethics Title Card

The use of nasal polyps was approved by the Hamilton Integrated Research Ethics Board

# Protocol

- Blocking the Mounted Sample and Antibody Immunolabeling

Demonstrator: Dima Traboulsi

  - To begin, draw a circle on the glass slide around the mounted tissue using a PAP pen to create a hydrophobic barrier [1-TXT].

    - WIDE: Talent drawing a circle around the tissue section on a glass slide using a PAP pen. TXT: Avoid drawing over the optimal cutting temperature compound area

  - Add approximately 100 microliters of PBS to each tissue to rehydrate them [1] and incubate the slide for about 2 minutes at room temperature to wash off the optimal cutting temperature compound [2].

    - Talent pipetting 100 microliters of PBS onto the tissue within the PAP pen boundary. Authors: I am omitting the information regarding the chambered cover glasses at every step as we are already over the limit of 55 shots. Please use normal cover glasses for filming as the volumes of buffers stated here are as per those

    - Talent placing the slide aside.

  - Aspirate the PBS from around the tissue using a pipette [1]. Then, add 100 microliters of blocking buffer to each tissue [2] and incubate at room temperature in a covered humidity chamber for 1 hour [3].

    - Talent aspirating PBS from the glass slide.

    - Talent pipetting 100 microliters of blocking buffer into the hydrophobic circle.

    - Talent placing the slide into a covered humidity chamber at room temperature.

  - Now, aspirate the blocking buffer using a pipette [1]. Add 100 microliters of staining mix to each tissue [2] and incubate the slide in a covered humidity chamber inside a refrigerator at 4 degrees Celsius overnight [3].

    - Talent aspirating blocking buffer from the glass slide.

    - Talent pipetting 100 microliters of staining mix onto the tissue within the PAP pen boundary.

    - Talent placing the covered humidity chamber inside a refrigerator set at 4 degrees Celsius.

  - Prepare sections stained with only one fluorophore and a negative control without any stain to define the fluorophore spectrum and tissue autofluorescence [1].

    - Talent labeling additional slides for single fluorophore staining and negative control.

  - After aspirating the staining mix, wash the tissue three times with PBS [1-TXT]. Aspirate the liquid after each wash [2]. Mount the sample on the microscope for imaging [3-TXT].

    - Talent pipetting PBS onto the tissue. TXT: Add 100 µL PBS; Incubate for 2 min between washes

    - Talent aspirating liquid between washes.

    - Talent placing the sample on the microscope stage. TXT: Do not use the DAPI filter when focusing the sample using the eyepieces

- Spectral Confocal Microscopy

Demonstrator: Saven Denha

  - Focus on the sample using brightfield or differential interference contrast mode [1]. Then switch to fluorescence mode and configure the spectral acquisition parameters according to the experimental requirements [2].

    - SCREEN: adjusting the focus to achieve a sharp image.

    - SCREEN: Show the switch from brightfield to fluorescence mode and configuration of the spectral acquisition settings.

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible:  https://review.jove.com/account/file-uploader?src=20936468

  - Assign the appropriate lasers for the fluorophores used in the experiment [1].

    - SCREEN: User assigning specific laser lines to each fluorophore on the control panel.

  - Select a beam splitter that corresponds to the chosen laser lines [1]. Enable all detectors to capture the full emission spectrum [2] and adjust the pinhole size to 1 Airy Unit for optimal optical sectioning [3].

    - SCREEN: User selecting an appropriate beam splitter for the laser configuration.

    - SCREEN: All detectors being activated in the acquisition software.

    - SCREEN: Adjustment of pinhole settings to 1 Airy Unit for optimal imaging.

  - Optimize the laser settings before acquisition. Using a section stained with all markers, start Continuous scanning mode at low laser power [1]. Gradually increase laser intensity until all markers are identifiable with a good signal-to-background ratio [2].

    - SCREEN: User starting Continuous scanning mode at low laser intensity.

    - SCREEN: Incremental laser intensity adjustments being made for clear visualization of all markers.

  - Fine-tune laser intensities and iteratively alternate between single-stain and multi-stain sections until all fluorophore signals are visible without oversaturation [1].

    - SCREEN: User switching between single-stained and multi-stained sections for comparison.

  - Enable the Tiles feature in the acquisition software [1] and define the tile acquisition region around the tissue to ensure full image coverage [2].

    - SCREEN: Show the user enabling the Tiles feature in the software interface.

    - SCREEN: Highlight the defined tile acquisition region encompassing the tissue on the software display.

  - Optimize the focus for the tile acquisition region at the center of the tissue [1]. Identify a unique structural feature, such as a distinct set of cells in a recognizable arrangement, to serve as a reference for alignment in later imaging rounds [2].

    - SCREEN: Show adjustment of the focus controls to achieve a sharp image at the center of the tissue.

    - SCREEN: Display the distinct cell cluster used as a visual reference point for alignment.

  - Now, enable the Z-stack feature and navigate to the Z-stack tab [1]. Under the Center tab, set the number of slices to at least 6, with an interval of no less than 1 micrometer between slices [2]. Click the Center button to set the current focus point, determined in the previous step, as the middle slice of the stack [3].

    - SCREEN: Show the user enabling the Z-stack feature and selecting the Z-stack tab.

    - SCREEN: User entering “6 slices” and “1 micrometer interval” in the Center tab.

    - SCREEN: Cursor clicking the Center button to confirm the focus point as the middle slice.

  - Click the Start experiment button to initiate image acquisition [1]. Once acquisition is complete, right-click on the image thumbnail, select Save As, choose a file location, and name the image [2]. Backup the image data to secure storage [3].

    - SCREEN: Show the cursor clicking the Start experiment button to begin acquisition.

    - SCREEN: Display the Save As dialog box as the user assigns a file name and selects a directory.

    - SCREEN: User saving a copy of the image data to a backup drive.

  - Unmix the image using the same optimized settings applied during tiled region acquisition [1]. Capture a Snap from each single-stained control to obtain the true signal spectrum and from the unstained control to capture background or autofluorescence [2].

    - SCREEN: Show the Unmixing interface being opened and optimized settings being loaded.

    - SCREEN: Show snapshots being captured from single-stained and unstained control samples.

  - Use these images exclusively for spectral unmixing. Confirm that the selected region represents a true signal by checking that the intensity and morphology are absent or equivalent to background levels in the unstained control [1], and that the marker’s localization matches its expected cellular pattern, such as membrane, nuclear, cytoplasmic, or secreted distribution [2].

    - SCREEN: Compare fluorescence intensity in the stained sample versus the unstained control side by side.

    - SCREEN: Cursor hovering over localization pattern of the marker consistent with its expected cellular location.

  - Open the snap image in the software [1]. Click on the Unmixing tab and select one of the region selection tools under the Unmix tools tab [2]. Draw a region of interest (ROI) around the positive signal to generate the spectrum of that region [3-TXT].

    - SCREEN: User opening the snap image .

    - SCREEN: navigating to the Unmixing tab, Cursor selecting a region selection tool from the Unmix tools tab.

    - SCREEN: Drawing an ROI around a bright fluorescent signal; a spectral plot appears. TXT: Repeat this process for each fluorophore to create reference spectra

  - Compare each generated spectrum to the theoretical emission spectrum of the corresponding fluorophore [1]. Ensure that the region accurately represents the true signal, name each spectrum according to the fluorophore identity, and save it to the spectral database [2].

    - SCREEN: Display side-by-side comparison of measured and theoretical fluorophore spectra.

    - SCREEN: User entering fluorophore names. saving spectra to the database.

  - Next, load the tiled image and open the Unmixing tool [1]. Use the + (plus) button to add the reference spectra for each fluorophore to the unmixing list [2]. Click the Linear unmixing button to separate the spectral signals and save the unmixed image [3].

    - SCREEN: User loading the tiled image and navigating to the Unmixing tool.

    - SCREEN: Cursor clicking the + button to add reference spectra to the unmixing list.

    - SCREEN: User clicking Linear unmixing to process the image and saving the image.

- Fluorophore Bleaching and Cycling

  - Prepare the bleaching solution in a fume hood [1]. Using a micro spatula, weigh 10 milligrams of lithium borohydride directly into a dry beaker [2]. Carefully add 10 milliliters of ultrapure water into the beaker and mix until the compound is fully dissolved [3-TXT].

    - Talent placing the containers in the fume hood, wearing proper protective equipment.

    - Talent measuring 10 milligrams of lithium borohydride into a dry beaker using a micro spatula.

    - Talent adding 10 milliliters of ultrapure water into the beaker and stirring until dissolved. TXT: Exercise extreme caution as LiBH4 is highly flammable

  - Add approximately 100 microliters of the freshly prepared bleaching solution to each tissue section using a glass Pasteur pipette [1] and incubate the slides at room temperature for 15 minutes inside the fume hood [2].

    - Talent pipetting bleaching solution onto tissue sections within the fume hood.

    - Talent placing the slide on the fume hood’s work surface.

  - If bleaching Brilliant Violet dyes such as BV-421 or BV-510, carefully transfer the slide to an epifluorescent microscope [1] and select the DAPI filter, focus the sample, and illuminate it at full power [2].

    - Talent transferring the slide from the fume hood to the microscope stage.

    - SCREEN: Selection of the DAPI filter and activation of full-power illumination.

  - Now, use a 5X objective to achieve a larger field of illumination [1] and lower the stage completely to prevent any contact between the slide and the objective or housing [2].

    - Show the 5× objective being selected.

    - Talent lowering the stage and ensuring safe clearance between the sample and the objective.

  - Finally, decant the lithium borohydride solution from each well using a glass Pasteur pipette and return it to the original beaker of bleaching solution [1]. Wash the slides three times with dilution buffer [2] and incubate for approximately 3 minutes during each wash [3]. Add the antibody mix and stain the sample as demonstrated earlier [4].

    - Talent aspirating the bleaching solution from the slide and transferring it back into the beaker.

    - Talent pipetting dilution buffer onto the slide for the first wash.

    - Talent placing the slide aside for incubation.

    - Talent adding the antibody mix to the sample.

# Results

- Results

  - In untreated nasal polyp tissue, staining with CD3, CD4, and CD20 showed intense, non-specific granular fluorescence in eosinophil-rich regions [1], while true membrane-localized signal was present at lower intensity [2].

    - LAB MEDIA: Figure 1A. Video editor: Highlight the yellow arrows pointing to the cells with bright, granular fluorescence in the CD3, CD4, and CD20 panels.

    - LAB MEDIA: Figure 1A. Video editor: Highlight the blue arrows pointing to the dimmer, membrane-localized signals for CD3 and CD4.

  - Pretreatment with 5% skim milk failed to block non-specific fluorescence and also reduced signal intensity for fluorophores such as AF532 [1].

    - LAB MEDIA: Figure 1B. Video editor: Highlight the CD45 (AF532) panel showing reduced signal intensity compared to untreated tissue.

  - DAB pretreatment reduced non-specific fluorescence from CD20, CD3, and CD4 [1-TXT], but also decreased true signal intensity across multiple channels including DAPI [2].

    - LAB MEDIA: Figure 1C. Video editor: Highlight the CD20, CD3, and CD4 panels showing reduced granular signal. TXT: DAB: 3,3'-Diaminobenzidine

    - LAB MEDIA: Figure 1C. Video editor: Highlight the DAPI panel showing visibly reduced nuclear staining.

  - Pretreatment with heparin effectively blocked non-specific binding while maintaining high signal intensity for CD3, CD4, CD20, and DAPI [1].

    - LAB MEDIA: Figure 1D. Video editor: Highlight all panels sequentially.

  - Heparin blocked non-specific signal in a concentration-dependent manner, with complete suppression achieved at 10 units [1], and no detrimental effects observed at higher concentrations [2].

    - LAB MEDIA: Figure 1E. Video editor: Highlight the 10 U panel showing strong signal and minimal background.

    - LAB MEDIA: Figure 1E. Video editor: Highlight the 20 U panel to show that signal and tissue integrity remain unaffected.

  - A 3-track imaging setup reduced acquisition time but introduced crosstalk, including bleed-through of AF594 into the PE channel [1], and enhanced tissue autofluorescence due to multiple laser excitations [2].

    - LAB MEDIA: Figure 2A. Video editor: Highlight the area inside small white box

    - LAB MEDIA: Figure 2A. Video editor: Highlight the yellow arrows pointing to autofluorescent regions.

  - Spectral imaging eliminated nearly all background and crosstalk, while also reducing acquisition time and improving signal-to-background ratio [1].

    - LAB MEDIA: Figure 2B. Video editor: Highlight the area inside small white box.

  - Spectral IBEX (I-bex) imaging of a human nasal polyp revealed epithelial structures marked by EpCAM [1], immune cells such as CD3-positive and CD20-positive lymphocytes [2], and vessels expressing CD31 and CD34 [3].

    - LAB MEDIA: Figure 3. Video editor: Highlight the image with EpCAM to show epithelial structures.

    - LAB MEDIA: Figure 3. Video editor: Highlight the images showing CD4 CD3 and CD31 CD20 staining.

    - LAB MEDIA: Figure 3. Video editor: Highlight the images CD31 CD20 and HLA-DR CD34.