TITLE

Exploring Protein‒Glycan Interactions: Advances in Nuclear Magnetic Resonance

AUTHORS AND AFFILIATIONS

Fabio C. L. Almeida1,2, Francisco Felipe Bezerra1,3, Ariana A. Vasconcelos1,2

1Institute of Medical Biochemistry Leopoldo de Meis (IBqM), Federal University of Rio de Janeiro (UFRJ), Brazil.

2National Center for Structural Biology and Bioimaging (CENABIO), Federal University of Rio de Janeiro (UFRJ), Brazil.

3Laboratory of Connective Tissue, Clementino Fraga Filho University Hospital.

Email address of the corresponding author:

Ariana A. Vasconcelos                    (ariana.vasconcelos@bioqmed.ufrj.br)

Email addresses of the co-authors:

Fabio C. L. Almeida                         (falmeida@bioqmed.ufrj.br)

Francisco Felipe Bezerra                (francisco.bezerra@bioqmed.ufrj.br)

SUMMARY

This NMR-based protocol investigates weak protein-glycan interactions using cyanovirin-N and D-mannose. Combining ligand- and protein-detected methods, it maps binding sites, detects allosteric effects, and identifies encounter complexes. The approach outlines sample preparation and data analysis, offering structural and dynamic insights valuable for glycan-specific diagnostics and recognition mechanisms.

ABSTRACT

Protein–glycan interactions are central to many biological processes and are increasingly recognized as promising targets for diagnostic strategies in infectious, inflammatory, and neoplastic diseases. However, characterizing these interactions, especially when they are weak and transient, remains technically challenging. Nuclear magnetic resonance (NMR) spectroscopy offers unique advantages for studying such interactions under near-physiological conditions, providing atomic-resolution insights into both structural and dynamic aspects. In this study, we present an integrated NMR protocol designed to investigate low-affinity protein–glycan interactions using the mannose-specific lectin cyanovirin-N and the monosaccharide D-mannose as a model system. The protocol combines ligand- and protein-based NMR approaches to map binding sites comprehensively and detect subtle binding events, including potential allosteric effects or encounter complexes. The critical steps of the protocol include careful sample preparation, precise control of concentrations, and spectral standardization to ensure data reproducibility and reliability. NMR in solution enables the detection of transient interactions that are often inaccessible by other techniques. Although weak interactions are essential for life, there is a bias in the literature toward higher-affinity complexes, even though low-affinity complexes are relevant in many important biological functions, such as signal transduction and cell–cell communication. This technique provides a powerful platform for investigating protein–glycan recognition and may serve as a valuable tool for identifying novel diagnostic markers on the basis of glycan-specific interactions.

INTRODUCTION

Protein‒glycan interactions are fundamental to a wide range of biological processes, including cell‒cell recognition, immune response modulation, and pathogen‒host interactions1,2. These molecular interactions are highly specific and dynamic and play crucial roles in both physiological and pathological mechanisms3–5. A classic example of such interactions occurs with lectins, proteins that specifically recognize and bind to glycans. Cyanovirin-N (CVN), for example, has been widely studied as an experimental model because of its ability to interact in high-affinity with high mannose oligosaccharides5–7.

In addition to lectins, many glycosylated proteins, such as integrins, also play essential roles in recognition and cellular signaling. Integrins are transmembrane receptors that frequently undergo glycosylation of their subunits, influencing their stability and affinity for ligands, including glycans5,8–10. However, the structural characterization of these interactions remains challenging owing to the intrinsic heterogeneity and flexibility of glycans, which complicates traditional structural biology approaches. In this context, the development of advanced methodologies capable of capturing these interactions in solution is of great value to this field, particularly for glycobiology11,12.

Nuclear magnetic resonance (NMR) spectroscopy has emerged as a powerful tool for investigating protein‒ligand interactions at the atomic level. Unlike other techniques, such as X-ray crystallography and cryo-electron microscopy, NMR allows the study of biomolecular interactions under near-physiological conditions, providing insights into both structure and dynamics. This flexibility enables researchers to modulate environmental parameters such as pH (typically between 4.0–8.0), ionic strength (e.g., 0–500 mM NaCl), and temperature (generally 273–330 K), thereby tailoring conditions to the specificities of protein‒glycan complexes. Additionally, NMR is particularly advantageous for analyzing transient and weak interactions, which are often characteristic of protein‒glycan recognition events13,14.

Several NMR techniques have been employed to investigate protein‒glycan interactions, utilizing both protein‒ and ligand‒based approaches. From the protein perspective, heteronuclear single quantum coherence (HSQC) titration is widely used to map binding sites by monitoring chemical shift perturbations upon ligand addition. Relaxation dispersion experiments enable the detection of conformational and chemical exchange processes occurring on the micro- to millisecond timescale, providing insights into the dynamic aspects of glycan recognition15,16. These protein-based techniques typically require sample concentrations in the range of 50 µM to 2 mM, depending on protein stability and labeling efficiency17,18.

Ligand-based NMR techniques offer complementary information by focusing on glycan changes during binding. Saturation transfer difference (STD-NMR) is particularly useful for identifying glycan epitopes involved in recognition, as it selectively saturates the NMR signals of ligand regions in close contact with the protein19,20. Water-Ligands Observed via Gradient Spectroscopy (WaterLOGSY)21,22 and Transferred Nuclear Overhauser Effect Spectroscopy (Tr-NOESY) provide additional means to assess ligand binding and conformational changes during interactions23. Furthermore, Carr–Purcell–Meiboom–Gill (CPMG) relaxation experiments aid in identifying weak and transient interactions that are difficult to detect with conventional methods24. Ligand-based experiments are often performed at protein concentrations of 10–50 µM and may require optimization of mixing times and saturation parameters25. Notably, these methods can be limited by the low solubility of glycans or the poor signal-to-noise ratio when working with small ligands.

Together, these NMR methods provide a comprehensive framework for elucidating the structural and dynamic properties of protein‒glycan interactions. With ongoing advancements in sensitivity and isotopic labeling strategies, this technique is becoming increasingly essential in glycobiology, offering new perspectives on molecular recognition mechanisms with potential applications in drug development and biomarker discovery. Nonetheless, the success of these approaches depends on factors such as sample homogeneity, glycan complexity, and the presence of flexible or disordered regions that may broaden NMR signals or hinder interpretation26. NMR is a powerful method for studying transient events, which is a major challenge in structural biology.

CVN recognizes α(1,2)-linked mannosyl residues present in high-mannose oligosaccharides with nanomolar affinities27–29. However, it poorly recognizes the monosaccharide D-mannose. In the present work, we studied the interaction of CVN with D-mannose as a way to illustrate how NMR is powerful in understanding transient interactions.

PROTOCOL

NOTE: Perform all steps involving bacterial cultures and buffer preparations in accordance with institutional biosafety protocols. Use personal protective equipment (PPE) and, when handling open cultures or chemicals, work in a biosafety cabinet or chemical fume hood if required.

1. Expression of Cyanovirin-N

  - Transform E. coli BL21 cells with a plasmid encoding the CVN gene inserted into the pET26a vector.

  - Cultivate the cells in 1 L of Super Broth (32 g of tryptone, 20 g of yeast extract, 5 g of NaCl per L), supplemented with kanamycin (50 µg/mL), 0.5% (w/v) glucose, and 1.6 mM MgSO₄. Incubate under continuous agitation at ~250 rpm at 37 °C.

  - For production of ¹⁵N-labeled CVN, grow the cells in modified M9 minimal medium using ¹⁵NH₄Cl as the sole nitrogen source.

    - Prepare 1 L of M9 minimal medium by mixing 200 mL of sterile 5x M9 salts solution containing 64 g/L of Na2HPO4, 15 g/L of KH2PO4, 2.5 g/L of NaCl, 5 g/L of NH4Cl.

    - Add 2 mL of 1 M MgSO4 (0.22 µm filter-sterilized), 0.1 mL of 1 M CaCl2 (0.22 µm filter-sterilized), and 4 g of glucose (final concentration 0.4%, dissolved in sterile distilled water and 0.22 µm filter-sterilized).

    - Bring the final volume to 1 L with sterile distilled water.

NOTE: Supplements such as amino acids or vitamins can be added as needed.

  - Monitor the culture until it reaches an optical density (OD₆₀₀) of approximately 1.2. Then, add isopropyl-beta-D-thiogalactopyranoside (IPTG) to a final concentration of 1.0 mM to induce protein expression.

  - Incubate for 20 h post-induction at 37 °C.

  - Harvest the cells by centrifugation at 7,000 × g for 10 min at 4 °C.

  - Isolate the periplasmic fraction as follows:

    - Resuspend the cell pellets in 0.4 volumes of 30 mM Tris-HCl buffer (pH 8.0) containing 20% sucrose and 1 mM EDTA and incubate at room temperature with shaking for 10 min.

    - Centrifuge at 10,000 × g for 10 min at 4 °C.

    - Resuspend the resulting pellets in 0.4 volumes of ice-cold distilled water and incubate on ice with shaking for 10 min.

    - Centrifuge again at 10,000 × g for 10 min at 4 °C.

    - Collect the supernatant, which corresponds to the periplasmic fraction, for protein extraction.

NOTE: All buffers must contain a protease inhibitor cocktail to prevent protein degradation.

- NMR experiments

NOTE: All experiments were acquired at 14.1 T using a 1H/15N/13C triple resonance inverse-detection probe (see Table of Materials) at 298K.

  - Ligand-based experiments

    - Sample preparation

      - Prepare two samples: one containing CVN and one without CVN (ligand-only control). Each sample contains 600 µL of 80 µM CVN in sodium phosphate buffer (pH 7.4) containing 50 mM NaCl.

      - Add D-mannose to a final concentration of 4 mM.

      - Add 5% (v/v) D₂O to each sample.

      - Transfer the sample to a 5 mm NMR tube.

      - Use a 50-fold molar excess of the ligand relative to the protein.

NOTE: A ligand excess in the range of 10-fold to 1000-fold is typically recommended, with lower excesses preferred for small proteins.

    - Spectral acquisition for D-mannose assignment

      - Acquire 2D TOCSY and 2D ¹H–¹³C HMBC spectra using standard pulse sequences:

2D TOCSY: Phase-sensitive total correlation spectroscopy (TOCSY) with excitation sculpting for water suppression (DIPSI2ESGPPH).

2D HMBC: ¹H–¹³C Heteronuclear Multiple-Bond Correlation with gradient selection and non-decoupled acquisition (HMBCGPNDQF).

NOTE: These experiments were performed for the purpose of D-mannose assignment but are not shown, as chemical shift assignment is not within the scope of this study.

    - Mapping ligand binding through chemical shift perturbation (CSP) of the ligand

      - For the sample preparation and acquisition of ¹H-¹³C HSQC spectra, prepare a 4 mM solution of D-mannose in sodium phosphate buffer (pH 7.4), with 50 mM NaCl and 5% D₂O. Divide the solution into two samples: one with 80 µM CVN and one without CVN (ligand-only reference).

NOTE: These spectra will be used to determine the CSP of D-mannose upon interaction with CVN.

      - For the setup of ¹H-¹³C HSQC acquisition parameters, use the standard pulse sequence HSQCETGPSI (gradient selection and sensitivity-enhanced ¹H-¹³C HSQC). Set the following parameters for acquisition:

Time Domain (TD): 1024 × 128 complex points (¹H × ¹³C dimensions)

Spectral Width (SW): 10.0171 ppm (6009.615 Hz) for ¹H, 80 ppm (12069.106 Hz) for ¹³C

Carrier Frequencies: 4.7 ppm for ¹H, 75 ppm for ¹³C

- Number of Scans: 448.

      - Calculate the CSP using the following equation:

CSP= ∆δH2+(∆δC4)2          (equation 1)

where ∆δH and ∆δC represent the chemical shift differences between D-mannose in the presence of CVN (Figure 1A, labeled in red) and free D-mannose (Figure 1B).

[Figure 1 here]

    - Mapping ligand binding through the saturation transfer difference (STD)

      - Create a new dataset and configure parameters for STD-NMR experiments. Two experimental strategies may be applied: (i) screening of different saturation frequencies to optimize signal-to-noise ratio; (ii) fixing the saturation frequency and varying the saturation time to evaluate build-up.

      - Set up 1D ¹H NMR experiments using the zgpr pulse sequence. Tune the spectrometer for ¹H, perform shimming, and measure a hard 90° pulse (e.g., ~10 µs at –10.6 dB attenuation, equivalent to ~11.482 W).

      - Center the ¹H carrier frequency at approximately 4.7 ppm, corresponding to the water signal.

      - Select the STDDIFFESGP.3 pulse sequence and configure the acquisition parameters as follows:

Set the spectral width according to sample requirements.

Set the interscan delay (d1) to 4 s.

Define the saturation time (d20) based on the desired experiment.

Load the FQ2LIST containing the off-resonance saturation frequency (–40 ppm) and the on-resonance frequencies to saturate the protein signal only.

NOTE: On-resonance frequencies must correspond to protein protons and must be at least 1000 Hz away from any ligand signal to avoid direct saturation.

      - Test the following on-resonance saturation frequencies to determine optimal conditions: –0.59 ppm, 0.73 ppm, and 8.1 ppm (see Figure 2A).

NOTE: 0.73 ppm was selected as an on-resonance saturation frequency, as it provided the best signal-to-noise ratio. Using this frequency, the STD amplification factor (ASTD) was quantified as a function of saturation time. To this end, STD spectra were acquired at different saturation times (0.5, 1, 1.5, 2, 2.5, 3, and 4 s) to construct the STD build-up curve (Figure 2B). The corresponding ASTD values were then plotted as a function of saturation time (Figure 2C), allowing the evaluation of the efficiency of the saturation transfer, which is proportional to the time and proximity of the ligand hydrogen to the protein. ASTD factor is defined as:

ASTD=ISTDIo[L]T[P]          (equation 2)

ISTD is the difference in signal intensity between the on- and off-resonance spectra, I₀ is the signal intensity in the off-resonance spectrum, [L]T is the total ligand concentration, and [P] is the protein concentration. This normalization accounts for concentration effects and allows for comparison across different experimental conditions. STD relies on spin diffusion and slow molecular tumbling to efficiently transfer saturation from the protein to the bound ligand. Larger proteins (slow tumbling) allow more efficient saturation and spread of magnetization within the protein19, 30–32.

      - Set the number of scans (ns) to 64 and average the experiment l4 (loop 4) times.

      - Calculate the total number of scans as: Total scans = ns × l4. Use 32,768 complex points in the direct dimension (TD) with a spectral width (SW) of 10.0171 ppm (6,009.615 Hz).

      - Set the interscan delay (d1) to 4 s and acquisition time (AQ) to 2.7262976 s. Total relaxation delay equals d1 + AQ.

      - Configure the saturation pulse as follows: duration 50 ms; shape Gaussian (p42); controlled via shaped program 9 (SP9).

      - Use an STD pulse sequence variant that includes a T₁ρ filter to suppress residual protein signals. Set the spin-lock time (d29) based on protein molecular weight. For larger proteins, use smaller spin-lock times (~20 ms) and for smaller proteins, larger (> 40 ms).

NOTE: All STD spectra were acquired at 599.93 MHz (BF1). Adjustment of d29 is critical to minimize protein background while preserving ligand signal integrity. Optimize this parameter empirically if necessary.

[Place Figure 2 here]

    - Mapping of ligand binding via 1H-R2

      - Select the CPMG_ESGP2D pulse program from the Bruker standard library. This sequence is based on the Carr–Purcell–Meiboom–Gill (CPMG) experiment, incorporating excitation sculpting for water suppression33.

      - Set the experiment as a 2D acquisition.

NOTE: This is a pseudo-2D experiment in which only the direct dimension is used for chemical shift information.

      - Configure the acquisition parameters as follows:

Time domain (TD): 32,768 complex points in the direct ¹H dimension.

(TD1): 2 points.

Spectral width (SW): 7.9932 ppm (4,795.396 Hz).

Interscan delay (d1): 4 s.

Carrier frequency (o1): 4.7 ppm (centered on water signal).

Number of scans (ns): 8.

Number of dummy scans (ds): 4.

CPMG delay (d20): ≤1 ms (i.e., shorter than 1⁄3 J_HH).

Number of averages (TDav): 200.

      - Run the experiment for 200 cycles of 8 scans each, resulting in a total accumulation of 1,600 scans. Adjust the variable counter list (vclist) according to the desired total time of the CPMG cycle (TCPMG). Here, 2 cycles were chosen for a TCPMG=8 ms and 200 cycles were chosen for a TCPMG=800 ms.

      - Run two 1H-CPMG experiments, one for the sample containing the protein (4 mM D-mannose and 80 µM CVN) and the other for the free D-mannose (4 mM D-mannose).

      - Plot the 1H spectra for each TCPMG via the command efp (exponential multiplication followed by Fourier transform and phase correction) or sinm (sine multiplication shifted by 90°, SSB=2) followed by fp (Fourier transform and phase correction). Adjust the window function according to the best processing strategy.

NOTE: In this experiment sinm and fp were used. The 1st serial will be the spectra with a TCPMG of 8 ms (first in the vclist), and the 2nd serial will be 800 ms (second in the vclist). The processed 1H-spectra are plotted in Figure 3.

      - Calculate the CPMG quotient (Q) via equation 3.

Q= I800msBoundI8msBoundI800msfreeI8msfree          (equation 3)

[Place Figure 3 here].

  - Protein-based experiments

    - Sample preparation

      - Prepare 600 µL of a sample containing 436 µM uniformly 15N-labeled CVN in sodium phosphate buffer (pH 7.4) supplemented with 50 mM NaCl and 5% deuterium oxide (D₂O). Titrate D-mannose into the sample to a final concentration of 60 mM. For the titration, use a 5 mm NMR tube.

    - Resonance Assignment (Optional Validation Step)

      - Confirm the assignment of free CVN in solution using previously reported chemical shift values34.Acquire the following triple resonance spectra: HNCO, HNCA, HNCACB, and CBCACONH35–37.

NOTE: These experiments serve only to validate the assignment and are not shown, as chemical shift assignment is not the primary objective of this protocol.

    - Mapping the binding of D-mannose on CVN by chemical shift perturbation of the protein

      - Acquire 1H–15N HSQC spectra of 15N-labeled CVN at 298 K. Perform a titration by adding D-mannose to reach the following final concentrations: 0, 1, 2, 5, 10, 20, 40, and 60 mM.

      - Set up the 1H–15N Fast-HSQC experiment using the phase-sensitive FHSQCF3GPPH pulse sequence from the Bruker standard library. Use the following parameters:

Time domain (TD): 1024 × 140 complex points in the 1H and 15N dimensions.

Spectral width (SW): 16.0274 ppm (9615.385 Hz) in the 1H dimension and 34 ppm (2067.119 Hz) in the 15N dimension.

Carrier frequency: 4.7 ppm (1H) and 119 ppm (15N).

Number of scans: 128.

      - Calculate the chemical shift perturbation (Δδ) using the following equation:

CSP= ∆δH2+(∆δN10)2          (equation 4)

where ∆δH and ∆δN are the chemical shift differences between the CVN in the presence and absence of D-mannose (Figure 4A).

NOTE: The factor 10 accounts for the difference in spectral widths between the proton and nitrogen dimensions.

      - Calculate the dissociation constant (KD) by plotting the CSP values for each residue as a function of D-mannose concentration. Fit the resulting data to a single-site binding isotherm using the following equation (5).

CSP={(CSPmax-CSPmin2PT)PT+ LT+ KD- PT+ LT+ KD2- 4PTLT+CSPmin

equation 5

where [PT] is the protein concentration used in the titration (CVN), LT is the ligand concentration (D-mannose), and CSPmax is the CSP at the saturation concentration of the ligand and CSPmin in the absence of the ligand.

[Place Figure 4 here]

    - Mapping the binding of D-mannose to CVN via the 15N‒R2 position of the protein.

      - Measure the 15N-R2 values of the individual residues of CVN in the absence and presence of 60 mM D-mannose. Perform all experiments at 298 K.

      - Set up the 15N-R₂ experiment using the phase-sensitive CPMG-based pulse sequence HSQCT2ETF3GPSI3D from the Bruker standard library. Use the following parameters:

Time domain (TD): 1024 × 128 complex points in the 1H and 15N dimensions, with 8 points in the pseudo dimension.

Spectral width (SW): 16.0274 ppm (1H; 9615.385 Hz) and 34 ppm (15N; 2067.119 Hz).

Carrier frequency: 4.7 ppm (1H) and 119 ppm (15N).

Number of scans: 32. (v) Variable counter list: 1, 8, 2, 6, 3, 5, 4, and 7, corresponding to relaxation delays (Trelax) of 16.96, 135.68, 33.92, 101.76, 50.88, 84.96, 67.84, and 118.72 ms, respectively.

      - Process the pseudo3D spectra as a series of HSQC-like 1H/15N correlation relaxation spectra using NMRPipe38, following the software tutorial (see Table of Materials for a link to the tutorial).

      - Import the processed spectra into a suitable NMR analysis platform (e.g., CCPNMR 39). Select all spectra and apply the "Follow Intensity Changes" tool. Plot the intensity of each cross-peak as a function of Trelax and fit the decay to a mono-exponential function to extract 15N-R₂ values for each residue.

      - Calculate the experimental error from the signal-to-noise ratio of the HSQC-like spectra at 67.84 ms. Process a region of the spectrum containing only noise and convert it to a text file using the following command:

“pipe2txt.tcl ./PROCs/ft/R2_67_84.ft2 > noise67_84.txt“

      - Determine the standard deviation of the noise region using statistical software (e.g., Origin(Pro), Version 2021). Define this value as the noise intensity Inoise.

      - Calculate the experimental error for each residue using the following equation:

R2error= 1TrelaxInoiseIeach_residue          (equation 6)

NOTE: It is important to use the experimental error rather than the fitting error. The experimental error defines the interpretation limits when comparing the 15N-R2 in the absence and presence of D-mannose.

[Place Figure 5 here]

REPRESENTATIVE RESULTS

In the present work, we presented NMR experiments to investigate protein–glycan interactions. Protein–glycan molecular recognition underlies numerous biological processes involving the extracellular matrix (ECM), including the regulation of cell adhesion, migration, proliferation, and various other forms of cellular communication.

Here, we chose to work with the monosaccharide D-mannose, which is described as a weak binder. The goal was to describe methods for measuring transient interactions and to highlight their importance in elucidating binding mechanisms. Three different ligand-based NMR approaches were employed to characterize the interaction sites of the ligand.

The first method involved evaluating the chemical shift perturbations (CSPs) of D-mannose in the presence of CVN. For this purpose, we recorded ¹H–¹³C-HSQC spectra of a sample containing a 50-fold excess of D-mannose relative to CVN (Figure 1A) and of free D-mannose under identical buffer conditions (Figure 1B). Chemical shift changes were observed for both anomeric configurations (Figure 1C). Notably, greater perturbations were detected for β-D-mannose, suggesting preferential binding to CVN.

For most D-mannose hydrogens, two distinct peaks were observed in the spectra in the presence of CVN, corresponding to the free (blue) and bound (green) states (Figure 1C). This behavior is characteristic of a slow exchange regime (Figure 3B), indicating that the chemical shift difference between the free and bound states for each proton (Δω, in Hz) exceeds the exchange rate (kₑₓ = kₒₙ + koff). Only one hydrogen (H5β) exhibited a fast exchange regime. Additionally, no CSP was observed for the anomeric proton of α-D-mannose. In contrast, the cross peak corresponding to the anomeric proton of β-D-mannose was not detected, which may reflect intermediate exchange due to binding. The hydrogens involved in the binding of both anomeric forms are summarized in Figure 1D.

The second ligand-based experiment we conducted was saturation transfer difference (STD) NMR, which is well suited for ligands that exchange between the bound and free states on the microsecond timescale. This exchange regime allows experiments to be performed with a large excess of ligand relative to the protein. In our case, we used 4 mM D-mannose and 80 µM CVN, corresponding to a 50-fold molar excess.

The STD results complemented the CSP-mannose data. The hydrogen nuclei exhibiting the highest ASTD values were associated with β-D-mannose (Figure 2C), in agreement with the CSP analysis, which showed pronounced perturbations in H2β, H3β, H4β, and H6β (Figure 1D). ASTD reflects the efficiency of magnetization transfer from the saturated protein (CVN) to the ligand (D-mannose), mediated by dipolar interactions between the ¹H nuclear spins of both molecules. Since this transfer depends on spatial proximity, higher ASTD values are expected for D-mannose protons in close contact with the CVN binding site.

Another way to map the interaction sites on a ligand is by evaluating the transverse relaxation rates (¹H-R₂) of its hydrogen atoms (Figure 3B). ¹H-R₂ relaxation occurs primarily through dipolar interactions with neighboring ¹H nuclei via multiple relaxation pathways. Upon ligand binding to a protein, new relaxation pathways are introduced owing to the proximity of protein hydrogens to those of the ligand. These additional pathways increase relaxation efficiency, resulting in increased ¹H-R₂ values. An additional factor contributing to higher ¹H-R₂ values is the increase in the apparent rotational correlation time (τc) upon binding. When a small, fast-tumbling ligand interacts with a larger, slower-tumbling protein, its effective τc increases (equation 7), which also promotes more efficient transverse relaxation, leading to higher observed ¹H-R₂ values.

τcapp (ligand)=fboundτcprotein+ligand+ ffreeτcligand           (equation 7)

where τcapp, (ligand) is the apparent rotational correlation time and where τcprotein+ligand and τcligand correspond to the correlation time of the complex and ligand, respectively. Typically, τcprotein+ligand≫ τcligand, resulting in τcapp(ligand), such that the ligand behaves as a larger molecule with elevated ¹H-R₂ values. The apparent correlation time is influenced by the bound fbound and ffree molar fractions of the ligand. As fbound increases, τcapp(ligand) increases, leading to enhanced ¹H-R₂ relaxation.

Another contributing factor to the increase in ¹H-R₂ upon binding is the contribution from chemical exchange between the free and bound states (Rex), which further elevates the apparent transverse relaxation rate, R2app(ligand) (equation 8).

R2app(ligand)= R2DD+ Rex          (equation 8)

Rex depends on the on/off chemical exchange regime, as follows (equations 9, 10, and 11):

Rexslow exchange~pBkex          (equation 9)

Rexintermediate exchange~pFpB(∆ω)2kexkex2+ (∆ω)2          (equation 10)

Rexfast exchange~pFpB(∆ω)2kex          (equation 11)

The exchange rate is kex= kon+koff, where ∆ω is the chemical shift difference between the bound and free state of the ligand (rad.s-1), and pF and pB are the populations of the free and bound states of the ligand, respectively (Figure 3B).

In the ¹H-R₂ mapping results (Figure 3C), we did not observe significant differences among the ligand protons. Most of the peaks remained unchanged, with R₂ quotient (Q) close to 1, except for Hβ4. A possible explanation for Q ~1 is the low binding affinity (i.e., small pB), combined with the fact that the ligand protons are in the slow exchange regime (Figure 1A, Figure 3A). Under these conditions, the contribution of chemical exchange Rex to R2app(ligand) is minimal, as Rex in the slow exchange regime does not depend on a second-order term, such as Δω² (equation 9).

The combination of these three NMR methods is valuable for mapping ligand–protein interactions, as it provides complementary information through distinct physical mechanisms. CSP detects changes in the local chemical environment, reflected in chemical shift perturbations. CPMG-based ¹H-R₂ measurements report the impact of the rotational freedom of the ligand and exchange between the free and bound states on transverse relaxation. The STD provides information on direct dipolar interactions between the ligand and the protein, resulting in magnetization transfer from the saturated protein to the ligand. The integrated interpretation of these three approaches offers a more comprehensive understanding of the nature of the interaction.

Thus far, we mapped the binding behavior of the ligand (D-mannose). Next, protein-based NMR experiments were conducted to map the binding site of D-mannose on CVN. The first highly informative approach was the acquisition of a series of ¹H–¹⁵N HSQC spectra with various concentrations of D-mannose. The 1H and 15N chemical shift perturbations (CSPs, equation (4)) induced by the addition of a ligand (D-mannose) are highly sensitive to subtle changes in the chemical environment. The greatest perturbation occurred at concentrations of D-mannose up to 10 mM (Figure 4A, top); however, certain residues, especially Gly2, showed a significant change in CSP up to 60 mM D-mannose (Figure 4B, bottom).

To understand the binding behavior of D-mannose on CVN, we analyzed the most significant CSPs in the structure of CVN complexed with the disaccharide Manα1-2Man40 (Figure 4B). It was observed that the β-strand comprising residues I40, E41, N42, V43, D44, and G45 corresponds to the previously described high-affinity Manα1–2Manα binding site. Significant CSPs, including residues L1, G2, E10, and R24, were also detected near the low-affinity binding site. Interestingly, none of the known Manα1–2Manα binding sites were fully defined by the observed CSPs. One possible interpretation is that D-mannose, which is smaller than the disaccharide Manα1–2Manα or a high-mannose oligosaccharide, is not sufficiently large to occupy the full extent of the binding site. As a result, its binding occurs with lower affinity.

This interpretation suggests that D-mannose binds preferentially to residues in the β-strand (40–44) within the high-affinity binding site and to residues L1, G2, E10, and R24 within the low-affinity site, albeit only transiently. This transient interaction also perturbs residues such as H90 and W49, which are located at the interface between the low-affinity (residues 1–39) and high-affinity (residues 40–90) domains of CVN. These observations suggest that D-mannose binding may induce perturbations in residues distant from the well-defined binding sites, either by promoting allosteric structural changes or by forming transient encounter complexes.

The latter possibility highlights a key advantage of studying interactions with low-affinity ligands. NMR spectroscopy is particularly well-suited to detect such transient, low-affinity interactions, as it can capture subtle chemical shift perturbations and saturation transfer effects even when the bound-state population is low. These features enable the detection of encounter complexes and weak binding events that may not be accessible through techniques requiring tight or stable complex formation. In this context, NMR provides valuable structural and dynamic insights that extend beyond the immediate binding site41,42.

We also analyzed the binding isotherms by plotting the CSPs of individual residues as a function of D-mannose concentration (Figure 4C). Notably, residues within the β-strand (40–44), which form the high-affinity binding site, exhibited an average KD of approximately 7 mM. Among them, residue D44 showed a particularly higher affinity, with a KD of 1.1 ± 0.35 mM, indicating its significant role in ligand binding. The binding affinity observed for D-mannose is approximately 5000-fold lower than that reported for the disaccharide Manα1–2Manα, which has a KD of 139 nM at the high-affinity site and 1.47 µM at the low-affinity site40.

Another approach used to map the D-mannose binding site on CVN was the measurement of ¹⁵N-R₂ relaxation rates in the absence (Figure 5A) and presence (Figure 5B) of the ligand. To highlight the effects of binding, we calculated ΔR₂ values (Figure 5C), defined as  ∆R2= R2bound-R2free, and considered the propagation of experimental error from both conditions. This analysis underscores the importance of estimating measurement uncertainty when interpreting dynamic NMR data.

Changes in ¹⁵N-R₂ upon ligand addition can occur for several reasons. An increase in R₂ may result from either the higher τc of the complex or from conformational and chemical exchange contributions (Rex) arising from the on–off equilibrium with the ligand. Conversely, decreases in R₂ can also be observed. In cases where ligand binding follows a conformational selection mechanism, Rex often decreases upon ligand addition, reflecting reduced exchange dynamics.

Therefore, both positive and negative ΔR₂ values that exceed the propagated experimental error can be used to identify regions involved in ligand binding.

Ligand concentration is another critical factor in interpreting these data. At saturating concentrations, binding tends to suppress conformational and chemical exchange contributions, leading to a reduction in Rex and, consequently, smaller R₂ values. Under these conditions, CSPs also typically reach their maximum values. In contrast, at sub-saturating concentrations, there is a substantial ligand on-off equilibrium, resulting in elevated Rex and reaching intermediate values of CSPs. These concentration-dependent effects must be considered when analyzing both ΔR₂ and CSP data.

To further understand the binding behavior of D-mannose to CVN, we analyzed the residues showing the most significant ΔR₂ changes in the context of the CVN–Manα1-2Manα disaccharide complex structure (Figure 5D). Residues C58, R59, K74, and R76 exhibited pronounced ΔR₂ values and correspond to the high-affinity binding site for Manα1-2Manα. In addition, residues F4, C8, R24, G27, G28, A92, and G96 also displayed significant ΔR₂ and are part of the low-affinity binding site.

Interestingly, the ΔR₂ analysis provided complementary insights to those obtained from CSPs, offering independent evidence of D-mannose interactions with both binding sites.

We also observed ΔR₂ changes in other residues located away from the known binding sites. These residues are primarily found near the central region of the protein, at the interface between the high- and low-affinity domains (Figure 5C). A plausible interpretation is that D-mannose binding induces subtle allosteric structural rearrangements. Alternatively, the observed effects may reflect transient interactions that form part of an initial encounter complex during ligand recognition.

FIGURE AND TABLE LEGENDS

Figure 1: Chemical shift perturbation of D-mannose in the presence of CVN. (A) 1H-13C-HSQC spectrum of D-mannose in the presence of CVN. (B) 1H-13C-HSQC spectrum of free D-mannose. The labels in the spectra show each of the chemical shift assignments and the respective anomeric configuration ( or β). The labels in red are those assigned to the bound conformation, and those in black are those assigned to the free conformation. (C) Superposition of the 1H-13C-HSQC spectra in the presence (green) and absence (blue) of CVN. (D) Chemical structure of - and β-D-mannose highlighting the observed CSP, which was calculated according to equation (1). The CSP values are depicted below the green circles. The red circle represents the β-anomeric hydrogen that vanished in the presence of CVN.

Figure 2: Saturation transfer difference (STD) spectra of D-mannose in the presence of CVN. (A) STDs acquired at different saturation frequencies: -0.59, 0.73, and 8.1 ppm. (B) STD acquired at different saturation times: 0.5, 1, 1.5, 2, 2.5, 3, and 4 s. (C) STD amplification factor (ASTD) calculated according to equation 2 as a function of the saturation time.

Figure 3: 1H-R2 of D-mannose in the presence and absence of CVN. (A) Representation of the expected D-mannose 1H-CPMG spectra for a binder and a nonbinder acquired with R2 relaxation times of 8 and 800 ms in the absence (bottom) and presence (top) of CVN. Q was calculated according to equation 3. In the absence of interaction (Q ≈ 1), the signal intensities remain similar at both relaxation times of the ligand with or without the protein. Upon binding (Q < 1), the signal intensities decrease with increasing relaxation for the sample containing the protein. (B) Schematic representation of the chemical exchange between free and bound states of D-mannose. Depending on the relative values of the chemical shift difference (Δω) and the exchange rate constant (kex = kon + koff), the system falls into slow, intermediate, or fast exchange regimes. (C) 1H NMR spectra of free D-mannose and D-mannose in the presence of CVN, showing differences in signal intensities at 8 ms and 800 ms relaxation times. The calculated Q factors for specific hydrogens (H6β, H3β, H5α, H4β) are shown, with lower Q values indicating stronger interactions with CVN and identifying the regions of the sugar involved in binding.

Figure 4: Chemical shift perturbation (CSP) analysis revealing CVN residues involved in D-mannose binding. (A) Bar plots showing the backbone amide (15N-1H) CSPs of CVN upon titration with D-mannose at 10 mM (top) and 60 mM (bottom) concentrations. The horizontal lines indicate the average CSP plus one or two standard deviations (av + sd, av + 2 sd), which were used as thresholds to identify significantly perturbed residues. (B) Mapping of significantly perturbed residues onto the structure of CVN complexed with the disaccharide Manα1-2Manα (PDB ID: 1IIY40). Residues with CSPs above the av + 2 sd threshold are highlighted in red, and those between av + sd and av + 2 sd are shown in blue. The disaccharide Manα1-2Manα molecules are displayed in orange to indicate their binding location. (C) Selected binding curves representing CSPs as a function of D-mannose concentration. The data were fitted according to equation 5 as a single-site binding model to estimate KD, revealing a range of affinities, with D44 showing the strongest interaction (KD = 1.1 ± 0.35 mM) and E41 the weakest (KD = 35 ± 29 mM).

Figure 5: Analysis of 15N-R2 for CVN free and bound to D-mannose. (A, B) Transverse relaxation rates (15N-R₂) measured for each residue of CVN in its (A) free and (B) D-mannose-bound states. (C) Difference in R₂ values (ΔR₂ = R₂bound − R₂free) plotted per residue. The blue and red lines represent one and two standard deviations from the average (av ± sd and av ± 2 sd), respectively. Residues with significant ΔR₂ values are labeled. (D) Mapping of the ΔR₂ values for the structure of CVN complexed with the disaccharide Manα1-2Manα (PDB ID: 1IIY40). Residues with ΔR₂ above or below av± 2 sd are shown in red; those between av± sd and av± 2 sd are shown in blue. D-mannose molecules are displayed in orange to indicate their binding location.

DISCUSSION

NMR spectroscopy represents a versatile approach for investigating protein‒glycan interactions, particularly because of its ability to detect weak and transient interactions, such as those observed between CVN and D-mannose. The protocol described here combines ligand- and protein-based NMR methods, allowing for a more comprehensive characterization of the interaction interface.

The critical steps of the protocol include careful sample preparation with precise control of protein and ligand concentrations, standardization of spectral parameters, and appropriate selection of NMR experiments. The reproducibility of the data strongly depends on sample stability, especially in sensitive experiments such as STD and WaterLOGSY21,43, which are influenced by solvent saturation.

The use of complementary NMR approaches in this work, from both the ligand and protein perspectives, enabled the validation of binding sites and the detection of potential allosteric effects or the formation of encounter complexes, phenomena that would be difficult to observe via isolated methods.

In practice, the protocol may require adjustment depending on the system under investigation. For proteins with low solubility or low expression yields, selective isotope labeling (e.g., only methionine or leucine residues) may be used to reduce the cost and complexity of full ¹⁵N-labeling. For ligands with low affinity or poor solubility, increasing their concentration or using longer mixing/saturation times in STD or WaterLOGSY experiments may help enhance signal detection. Moreover, acquisition parameters such as the number of scans and recycle delays can be optimized to improve sensitivity in low-signal systems.

Common troubleshooting strategies include verifying sample purity and pH stability, as minor shifts can affect both spectral quality and binding behavior. For STD-NMR, ensuring appropriate saturation power and avoiding excessive irradiation times is critical to avoid false positives or protein damage.

Among the limitations of this method is the requirement for relatively large amounts of isotopically labeled and purified protein, which may represent a challenge when working with low-yield targets or membrane proteins. Nevertheless, compared to techniques such as surface plasmon resonance (SPR) or isothermal titration calorimetry (ITC), NMR offers important advantages. Since NMR experiments are performed in solution and do not require immobilization, they preserve the native dynamics and conformational plasticity of the biomolecules, avoiding potential artifacts associated with surface attachment. Furthermore, NMR can detect weak and transient interactions, including encounter complexes41, through chemical shift perturbations and saturation transfer experiments, even in cases where only a small fraction of the ligand is bound. This level of sensitivity and resolution makes NMR particularly valuable for characterizing dynamic or partially disordered binding interfaces, commonly referred to as fuzzy complexes44.

These unique features make NMR spectroscopy a particularly powerful tool for elucidating the molecular details of low-affinity interactions. As demonstrated by Luna et al.45, NMR enables the detection and characterization of low- and ultralow-affinity protein interactions, which are often invisible to other biophysical methods but play critical roles in regulating cellular processes. The ability to monitor dynamic interactions in real time and at atomic resolution allows for the identification of transient binding events that do not result in stable complexes yet are biologically relevant. For example, chemical shift perturbation assays provide a rapid and effective means to map protein binding interfaces, even in weakly bound systems46. Despite a historical bias toward high-affinity complexes in structural biology, it is now increasingly clear that transient and low-affinity interactions underlie key biological phenomena, including signal transduction, molecular recognition, and cell–cell communication46–48.

This type of protocol has direct implications for studies of molecular recognition, particularly those involving weak and transient interactions such as protein–glycan binding. Accurate characterization of these interactions, as demonstrated here, is crucial for identifying biologically relevant targets involved in infectious, inflammatory, and tumor-related processes. The combined application of NMR-based approaches allows for the detailed investigation of both structural and dynamic aspects of binding, including low-affinity events that are often overlooked by other techniques. By providing atomic-level insights into transient recognition mechanisms, NMR contributes not only to a deeper understanding of biomolecular interactions but also to the rational development of diagnostic strategies based on specific glycan ligands.

ACKNOWLEDGMENTS

We thank the Centro Nacional de Ressonância Magnética Nuclear Jiri Jonas (CNRMN) for the use of the NMR spectrometers. We are grateful to Dr. Carole Bewley for sharing with us the chemical shift assignments of CVN in complex with Manα1-2Man. We also thank the funding agencies FAPERJ and CNPq.

DISCLOSURES

The authors declare that they have no competing interests.

REFERENCES

- Varki, A. Biological roles of glycans. Glycobiology. 27 (1), 3–49 (2017).

- van Kooyk, Y., Rabinovich, G. A. Protein-glycan interactions in the control of innate and adaptive immune responses. Nat. Immunol. 9 (6), 593–601 (2008).

- Gagneux, P., Hennet, T., Varki, A. Biological functions of glycans. Cold Spring Harbor Laboratory Press, Cold Spring Harbor, NY (2022).

- Varki, A. Biological roles of glycans. Glycobiology. 27 (1), 3–49 (2017).

- He, M., Zhou, X., Wang, X. Glycosylation: Mechanisms, biological functions and clinical implications. Signal Transduct. Target. Ther. 9 (1), 194 (2024).

- Maier, I., Kontaxis, G., Zimmermann, C., Steininger, C. Cyanovirin-N binding to N-acetyl-d-glucosamine requires carbohydrate-binding sites on two different protomers. Biochemistry. 63 (10), 1270–1277 (2024).

- Muñoz-Basagoiti, J. et al. Cyanovirin-N binds to select SARS-CoV-2 spike oligosaccharides outside of the receptor binding domain and blocks infection by SARS-CoV-2. Proc. Natl. Acad. Sci. U. S. A. 120 (10), e2214561120 (2023).

- Margraf-Schönfeld, S., Böhm, C., Watzl, C. Glycosylation affects ligand binding and function of the activating natural killer cell receptor 2B4 (CD244) protein. J. Biol. Chem. 286 (27), 24142–24149 (2011).

- Thomas, D., Rathinavel, A. K., Radhakrishnan, P. Altered glycosylation in cancer: A promising target for biomarkers and therapeutics. Biochim. Biophys. Acta Rev. Cancer. 1875 (1), 188464 (2021).

- Vasconcelos, A. A., Estrada, J. C., David, V., Wermelinger, L. S., Almeida, F. C. L., Zingali, R. B. Structure-function relationship of the disintegrin family: Sequence signature and integrin interaction. Front. Mol. Biosci. 8 (December), 1–20 (2021).

- Gimeno, A., Valverde, P., Ardá, A., Jiménez-Barbero, J. Glycan structures and their interactions with proteins. A NMR view. Curr. Opin. Struct. Biol. 62, 22–30 (2020).

- Haslam, S. M. et al. Essentials of glycobiology. Cold Spring Harbor Laboratory Press, Cold Spring Harbor, NY (2022).

- Angulo, J. et al. NMR investigations of glycan conformation, dynamics, and interactions. Prog. Nucl. Magn. Reson. Spectrosc. 144–145, 97–152 (2024).

- Bewley, C. A., Shahzad-Ul-Hussan, S. Characterizing carbohydrate-protein interactions by nuclear magnetic resonance spectroscopy. Biopolymers. 99 (10), 796–806 (2013).

- Quintana, J. I., Atxabal, U., Unione, L., Ardá, A., Jiménez-Barbero, J. Exploring multivalent carbohydrate-protein interactions by NMR. Chem. Soc. Rev. 52 (5), 1591–1613 (2023).

- Pomin, V. H., Wang, X. Glycosaminoglycan-protein interactions by nuclear magnetic resonance (NMR) spectroscopy. Molecules. 23 (9), 2314 (2018).

- Walinda, E., Morimoto, D., Sugase, K. Overview of relaxation dispersion NMR spectroscopy to study protein dynamics and protein-ligand interactions. Curr. Protoc. Protein Sci. 92 (1), e57 (2018).

- Altincekic, N. et al. Large-scale recombinant production of the SARS-CoV-2 proteome for high-throughput and structural biology applications. Front. Mol. Biosci. 8, 89 (2021).

- Mayer, M., Meyer, B. Characterization of ligand binding by saturation transfer difference NMR spectroscopy. Angew. Chem. Int. Ed. 38 (12), 1784–1788 (1999).

- Haselhorst, T., Lamerz, A. C., von Itzstein, M. Saturation transfer difference NMR spectroscopy as a technique to investigate protein-carbohydrate interactions in solution. Methods Mol. Biol. 534, 375–386 (2009).

- Dalvit, C., Fogliatto, G. P., Stewart, A., Veronesi, M., Stockman, B. WaterLOGSY as a method for primary NMR screening: Practical aspects and range of applicability. J. Biomol. NMR. 21 (4), 349–359 (2001).

- Dalvit, C., Fasolini, M., Flocco, M., Knapp, S., Pevarello, P., Veronesi, M. Spectroscopy experiments: Detection of high-affinity ligands. J. Med. Chem. 45, 2610–2614 (2002).

- Lucas, L. H., Yan, J., Larive, C. K., Zartler, E. R., Shapiro, M. J. Transferred nuclear Overhauser effect in nuclear magnetic resonance diffusion measurements of ligand-protein binding. Anal. Chem. 75 (3), 627–634 (2003).

- Loria, J. P., Rance, M., Palmer, A. G. A relaxation-compensated Carr−Purcell−Meiboom−Gill sequence for characterizing chemical exchange by NMR spectroscopy. J. Am. Chem. Soc. 121 (10), 2331–2332 (1999).

- Di Carluccio, C. et al. Investigation of protein-ligand complexes by ligand-based NMR methods. Carbohydr. Res. 503, 108313 (2021).

- Mulloy, B., Hart, G. W., Stanley, P. Structural analysis of glycans. Essentials of glycobiology. Cold Spring Harbor Laboratory Press, Cold Spring Harbor, NY (2009).

- Driessen, N. N. et al. Cyanovirin-N inhibits mannose-dependent Mycobacterium–C-type lectin interactions but does not protect against murine tuberculosis. J. Immunol. 189 (7), 3585–3592 (2012).

- Botos, I. et al. Structures of the complexes of a potent anti-HIV protein cyanovirin-N and high mannose oligosaccharides. J. Biol. Chem. 277 (37), 34336–34342 (2002).

- Shenoy, S. R. et al. Multisite and multivalent binding between cyanovirin-N and branched oligomannosides: Calorimetric and NMR characterization. Chem. Biol. 9 (10), 1109–1118 (2002).

- Mayer, M., Meyer, B. Group epitope mapping by saturation transfer difference NMR to identify segments of a ligand in direct contact with a protein receptor. J. Am. Chem. Soc. 123 (25), 6108–6117 (2001).

- Meyer, B., Peters, T. NMR spectroscopy techniques for screening and identifying ligand binding to protein receptors. Angew. Chem. Int. Ed. 42 (8), 864–890 (2003).

- Marchetti, R. et al. “Rules of engagement” of protein-glycoconjugate interactions: A molecular view achievable by using NMR spectroscopy and molecular modeling. Chem. Open. 5 (4), 274–296 (2016).

- Carr, H. Y., Purcell, E. M. Effects of diffusion on free precession in nuclear magnetic resonance experiments. Phys. Rev. 94 (3), 630–638 (1954).

- Bewley, C. A. Solution structure of a cyanovirin-N:Manα1-2Manα complex: Structural basis for high-affinity carbohydrate-mediated binding to gp120. Structure. 9 (10), 931–940 (2001).

- Weisemann, R., Rüterjans, H., Bermel, W. 3D triple-resonance NMR techniques for the sequential assignment of NH and 15N resonances in 15N- and 13C-labelled proteins. J. Biomol. NMR. 3 (1), 113–120 (1993).

- Muhandiram, D. R., Kay, L. E. Gradient-enhanced triple-resonance three-dimensional NMR experiments with improved sensitivity. J. Magn. Reson. B. 103 (3), 203–216 (1994).

- Whitehead, B., Craven, C. J., Waltho, J. P. Double and triple resonance NMR methods for protein assignment. Methods Mol. Biol. 60, 29–52 (1997).

- Delaglio, F., Grzesiek, S., Vuister, G. W., Zhu, G., Pfeifer, J., Bax, A. NMRPipe: A multidimensional spectral processing system based on UNIX pipes. J. Biomol. NMR. 6 (3), 277–293 (1995).

- Skinner, S. P. et al. Structure calculation, refinement and validation using CcpNmr Analysis. Acta Crystallogr. D Biol. Crystallogr. 71 (Pt 1), 154–161 (2015).

- Bewley, C. A. Solution structure of a cyanovirin-N:Manα1-2Manα complex. Structure. 9 (10), 931–940 (2001).

- Sanches, K., Caruso, I. P., Almeida, F. C. L., Melo, F. A. The dynamics of free and phosphopeptide-bound Grb2-SH2 reveals two dynamically independent subdomains and an encounter complex with fuzzy interactions. Sci. Rep. 10 (1), 1–13 (2020).

- Almeida, F. C. L., Sanches, K., Caruso, I. P., Melo, F. A. NMR relaxation dispersion experiments to study phosphopeptide recognition by SH2 domains:the Grb2-SH2–phosphopeptide encounter complex. SH2 Domains. 2705, 135–151 (2023).

- Huang, R., Bonnichon, A., Claridge, T. D. W., Leung, I. K. H. Protein-ligand binding affinity determination by the WaterLOGSY method: An optimised approach considering ligand rebinding. Sci. Rep. 7 (March), 1–6 (2017).

- Caruso, I. P. et al. Insights into the specificity for the interaction of the promiscuous SARS-CoV-2 nucleocapsid protein N-terminal domain with deoxyribonucleic acids. Int. J. Biol. Macromol. 203 (January), 466–480 (2022).

- Luna, R. E., Akabayov, S. R., Ziarek, J. J., Wagner, G. Examining weak protein–protein interactions in start codon recognition via NMR spectroscopy. FEBS J. 281 (8), 1965–1973 (2014).

- Qin, J., Gronenborn, A. M. Weak protein complexes: Challenging to study but essential for life. FEBS J. 281 (8), 1948–1949 (2014).

- Perkins, J. R., Diboun, I., Dessailly, B. H., Lees, J. G., Orengo, C. Transient protein-protein interactions: Structural, functional, and network properties. Structure. 18 (10), 1233–1243 (2010).

- Ross, J. L. The dark matter of biology. Biophys. J. 111 (5), 909–916 (2016).