# Submission ID #: 68674

# Scriptwriter Name: Sulakshana Karkala

# Project Page Link: https://review.jove.com/account/file-uploader?src=20945618

# Title: Exploring Protein‒Glycan Interactions: Advances in Nuclear Magnetic Resonance

# Authors and Affiliations:

# Fabio C. L. Almeida1,2, Francisco Felipe Bezerra1,3, Ariana A. Vasconcelos1,2

# 1Institute of Medical Biochemistry Leopoldo de Meis (IBqM), Federal University of Rio de Janeiro (UFRJ)

# 2National Center for Structural Biology and Bioimaging (CENABIO), Federal University of Rio de Janeiro (UFRJ)

# 3Laboratory of Connective Tissue, Clementino Fraga Filho University Hospital

# Corresponding Authors:

Ariana A. Vasconcelos                    (ariana.vasconcelos@bioqmed.ufrj.br)

# Email Addresses for All Authors:

Fabio C. L. Almeida                         (falmeida@bioqmed.ufrj.br)

# Francisco Felipe Bezerra                (francisco.bezerra@bioqmed.ufrj.br)

Ariana A. Vasconcelos                    (ariana.vasconcelos@bioqmed.ufrj.br)

## Author Questionnaire

1. We have marked your project as author-provided footage, meaning you film the video yourself and provide JoVE with the footage to edit. JoVE will not send the videographer. Please confirm that this is correct.

√ Correct

2. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

3. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes, all done

4. Proposed filming date: To help JoVE process and publish your video in a timely manner, please indicate the proposed date that your group will film here: 10/09/2025

When you are ready to submit your video files, please contact our Content Manager, Utkarsh Khare.

Current Protocol Length

Number of Steps:  24

Number of Shots:  49

# Introduction

  - Ariana Azevedo Vasconcelos: We study protein–protein and protein–ligand interactions, focusing on integrin–disintegrin complexes. Our model explores structural dynamics and highlights the role of surface forces in mediating complex formation and interaction stability.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:5.2

What significant findings have you established in your field?

  - Fabio Ceneviva Lacerda de Almeida: Our group is dedicated to the study of the role of surface forces in molecular recognition and the evolution of binding sites by studying surface hydrophobic clusters present in proteins.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:5.6

What advantage does your protocol offer compared to other techniques?

  - Ariana Azevedo Vasconcelos: NMR in solution enables the detection of transient interactions. Although weak interactions are essential for life, there is a bias toward higher-affinity complexes, even though they are present in many important biological functions.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

What research questions will your laboratory focus on in the future?

  - Fabio Ceneviva Lacerda de Almeida: Our group is focusing on using integrative structural biology approaches to study large protein complex. We are using NMR as the main tool to study dynamics and functional aspects.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

# Protocol

- Mapping Ligand–Protein Interactions Using Chemical Shift Perturbation, Saturation Transfer Difference, and Relaxation NMR Techniques

Demonstrator: Ariana Azevedo Vasconcelos

  - To begin, prepare a four millimolar solution of D-mannose in sodium phosphate buffer at pH 7.4 with fifty millimolar sodium chloride and five percent deuterium oxide [1]. Divide the solution into two samples, one containing eighty micromolar CVN (C-V-N) and the second, without [2-TXT].

    - WIDE: Talent measuring and mixing sodium phosphate buffer, sodium chloride, deuterium oxide, and D-mannose to produce a four millimolar solution.

    - Talent aliquoting the prepared solution into two separate tubes. TXT: CVN: Cyanovirin-N

  - For mapping the ligand binding through chemical shift perturbation, on the NMR spectrometer, set up the HSQCETGPSI (H-S-Q-C-E-T-G-P-S-I) pulse sequence for ¹H-¹³C (H-One-C-Thirteen) HSQC (H-S-Q-C) acquisition [1]. Configure the time domain, the spectral width, carrier frequency and number of scans [2].

    - SCREEN: 68674_screenshot_1 		00:00-00:30

    - SCREEN: 68674_screenshot_2.		00:05-00:44
Video Editor: please speed up the video
AND
TEXT ON PLAIN BACKGROUND:
Time Domain (TD): 1024 × 128 complex points (¹H × ¹³C dimensions)
Spectral Width (SW): 10.0171 ppm (6009.615 Hz) for ¹H, 80 ppm (12069.106 Hz) for ¹³C
Carrier Frequencies: 4.7 ppm for ¹H, 75 ppm for ¹³C
Number of Scans: 448.
Video Editor: Please play both shot side by side

  - Calculate chemical shift perturbation using the equation provided [1].

    - SCREEN: 68674_screenshot_3.	00:21-00:44

  - To map ligand binding through saturation transfer difference, first create a new dataset [1]. Configure the parameters for STD-NMR experiments by either screening different saturation frequencies [2] or fixing the frequency while varying saturation times to evaluate buildup [3].

    - SCREEN: 68674_screenshot_4. 		00:00-00:16

    - SCREEN: 68674_screenshot_5.		00:02-00:20

    - SCREEN: 68674_screenshot_6. 		00:04-00:20

  - Set up 1D ¹H NMR experiments using the zgpr (Z-G-P-R) pulse sequence [1]. Tune the spectrometer for ¹H [2], perform shimming, and measure a hard 90 degree pulse [3].

    - SCREEN: 68674_screenshot_7.	00:02-00:18

    - SCREEN: 68674_screenshot_8.	00:19-00:25

    - SCREEN: 68674_screenshot_9.1.	00:08-00:20

  - Center the carrier frequency approximately at 4.7 parts per million corresponding to the water signal [1]. Then select the STDDIFFESGP.3 (S-T-D-D-I-F-F-E-S-G-P-POINT-Three) pulse sequence and set the acquisition parameters [2].

    - SCREEN: 68674_screenshot_10. 	00:00-00:11

    - SCREEN: 68674_screenshot_11. 	00:07-00:23

  - Set the spectral width as required, interscan delay d1 (d-One) to four seconds, and define saturation time d20 (d-Twenty) as per experiment goals [1]. Load the FQ2LIST  (F-Q-Two-List) with off-resonance frequency at minus 40 parts per million and on-resonance frequencies to saturate protein only [2].

    - SCREEN: 68674_screenshot_12. 	00:10-00:30

    - SCREEN: 68674_screenshot_13. 	00:05-00:22

  - Test the on-resonance frequencies of minus 0.59, 0.73, and 8.1 parts per million, to determine optimal conditions [1].

    - SCREEN: 68674_screenshot_14. 	00:04-00:28

  - Use the optimal frequency to acquire STD spectra at saturation times of 0.5, 1, 1.5, 2, 2.5, 3, and 4 seconds [1]. Plot the corresponding ASTD (A-S-T-D) values as a function of saturation time [2].

    - SCREEN: 68674_screenshot_15. 	00:44-00:58

    - SCREEN: 68674_screenshot_16. 	00:04-00:18,00:50-00:54,01:06-01:09

  - Now set number of scans to 64 and average the experiment loop four times [1]. Then calculate the total number of scans [2-TXT].

    - SCREEN: 68674_screenshot_17. 	00:03-00:11

    - SCREEN: 68674_screenshot_17.	00:12-00:20
TXT: Use 32,768 complex points in the direct dimension (TD), Spectral width: 10.0171 ppm (6,009.615 Hz)
AND
TEXT ON PLAIN BACKGROUND:
Total scans = ns x l4
Video Editor: Please play both shot side by side

  - Set interscan delay to 4 seconds and acquisition time to 2.7262976 seconds [1]. Configure saturation pulse by setting the duration to 50 milliseconds and control of the Gaussian shape via the shaped program nine SP9 [2].

    - SCREEN: 68674_screenshot_18. 		00:00-00:08

    - SCREEN: 68674_screenshot_19. 		00:00-00:13

  - Apply a T1ρ (T-One-rho) filter in STD-NMR variant then set the spin-lock time d29 (d-Twenty-nine)  based on protein size [1-TXT].

    - SCREEN: 68674_screenshot_20. 	00:00-00:11
TXT: Use smaller times for larger proteins and vice versa

  - To map the ligand binding 1H-R2, first select the CPMG_ESGP2D (C-P-M-G-E-S-G-P-2-D)  pulse program from the Bruker standard library [1].  Set the experiment as a 2D acquisition [2].

    - SCREEN: 68674_screenshot_21.		00:00-00:22

    - SCREEN: 68674_screenshot_21. 		00:23-00:32

  - Configure the acquisition parameters as shown [1]. Then run the experiment for 200 cycles of 8 scans each [2]. Adjust the variable counter list according to the desired total time of the CPMG cycle [3].

    - TEXT ON PLAIN BACKGROUND:
Time domain (TD): 32,768 complex points in the direct ¹H dimension
(TD1): 2 points
Spectral width (SW): 7.9932 ppm (4,795.396 Hz)
Interscan delay (d1): 4 s
Carrier frequency (o1): 4.7 ppm (centered on water signal)
Number of scans (ns): 8
Number of dummy scans (ds): 4
CPMG delay (d20): ≤1 ms (i.e., shorter than 1⁄3 J_HH)
Number of averages (TDav): 200
Video Editor: Please play both shot side by side

    - SCREEN: 68674_screenshot_22. 		00:00-00:05

    - SCREEN: 68674_screenshot_23. 		00:00-00:11

  - Run two experiments, one for the sample with the protein and one for the ligand-only sample [1].

    - SCREEN: 68674_screenshot_24.		00:03-00:26

  - Plot the 1H spectra for each TCPMG (T-C-P-M-G) using the command efp (E-F-P) or sinm (SINE-M) followed by fp (f-p). Then adjust the window function according to the best processing strategy [1]. Calculate the CPMG quotient Q using the equation [3].

    - SCREEN: 68674_screenshot_25. 		00:00-00:35

    - SCREEN: 68674_screenshot_26.		00:02-00:19

- Mapping the binding of D-mannose on CVN by chemical shift perturbation of the protein

Demonstrator: Fabio C. L. Almeida

  - Acquire 1H–15N (H-One-N-Fifteen) HSQC spectra of 15N-labeled CVN at 298 Kelvin [1]. Titrate with D-mannose to reach the final concentrations  of 0, 1, 2, 5, 10, 20, 40, and 60 millimoles [2].

    - SCREEN: 68674_screenshot_27.		00:02-00:16

    - Talent performing titrating with D-Mannose.

  - Use phase-sensitive FHSQCF3GPPH (F-H-S-Q-C-F-3-G-P-P-H) Fast-HSQC pulse sequence from the Bruker standard library [1]. Then calculate the chemical shift perturbation using the formula [2].

    - SCREEN: 68674_screenshot_28. 		00:10-end
AND
TEXT ON PLAIN BACKGROUND:
Use parameters:
Time domain (TD): 1024 × 140 complex points in the 1H and 15N dimensions
Spectral width (SW): 16.0274 ppm (9615.385 Hz) in the 1H dimension and 34 ppm (2067.119 Hz) in the 15N dimension
Carrier frequency: 4.7 ppm (1H) and 119 ppm (15N)
Number of scans: 128
Video Editor: Please play both shot side by side

    - SCREEN: 68674_screenshot_29. 		00:03-00:15

  - Plot CSP (C-S-P) values for each residue as a function of D-mannose concentration to determine dissociation constant KD (K-D) using equation five [1]. Then fit the resulting data to a single-binding isotherm using the formula [2].

    - SCREEN: 68674_screenshot_30. 		00:06-00:32

    - SCREEN: 68674_screenshot_31. 		00:04-00:20, 00:46-00:48
AND
TEXT ON PLAIN BACKGROUND:
CSP={(CSPmax-CSPmin2PT)PT+ LT+ KD- PT+ LT+ KD2- 4PTLT+CSPmin

[PT] : protein concentration used in the titration (CVN)
LT : ligand concentration (D-mannose)
CSPmax : CSP at the saturation concentration of the ligand

CSPmin: CSP in the absence of the ligand
Video Editor: Please play both shot side by side

- Mapping the binding of D-mannose on CVN via the 15N‒R2 position of the protein

Demonstrator: Ariana Azevedo Vasconcelos

  - Use phase-sensitive HSQCT2ETF3GPSI3D (H-S-Q-C-T-2-E-T-F-3-G-P-S-I-3-D) pulse sequence and configure parameters as shown to measure the 15N-R2 values of the individual residues of CVN residues in absence and presence of 60 millimolar D-mannose at 298 Kelvin [1]. 
NOTE: Shots merged and VO edited accordingly

    - SCREEN: 68674_screenshot_32.		00:00-end

AND
TEXT ON PLAIN BACKGROUND: 
Time domain (TD): 1024 × 128 complex points in the 1H and 15N dimensions, with 8 points in the pseudo dimension

Spectral width (SW): 16.0274 ppm (1H; 9615.385 Hz) and 34 ppm (15N; 2067.119 Hz).

Carrier frequency: 4.7 ppm (1H) and 119 ppm (15N).

Number of scans: 32. (v) Variable counter list: 1, 8, 2, 6, 3, 5, 4, and 7, corresponding to relaxation delays (Trelax) of 16.96, 135.68, 33.92, 101.76, 50.88, 84.96, 67.84, and 118.72 ms, respectively
Video Editor: Please play both shot side by side

  - Process pseudo-three-dimensional spectra using NMRPipe (N-M-R-Pipe) to generate HSQC-like spectra for each relaxation delay [1]. Import the processed spectra into an analysis platform [2].

    - SCREEN: 68674_screenshot_33. 		00:05-00:20,01:52-01:56

    - SCREEN: 68674_screenshot_34. 		00:12-00:21, 00:35-00:40

  - Then select all spectra and apply the Follow Intensity Changes tool [1]. Plot the intensity of each cross-peak as a function of Trelax (T-Relax) and fit the decay to a mono-exponential function to extract 15N-R₂ values for each residue [2].

    - SCREEN: 68674_screenshot_35. 	00:14-00:24

    - SCREEN: 68674_screenshot_35. 	00:25-00:31

  - Calculate the experimental error from the signal-to-noise ratio of the HSQC-like spectra at 67.84 milliseconds [1]. Process a region of the spectrum containing only noise and convert it to a text file using the given command [2-TXT].

    - SCREEN: 68674_screenshot_36. 	00:04-00:28

    - SCREEN: 68674_screenshot_36. 	00:47-00:55, 01:14-01:16	
TXT: Command: pipe2txt.tcl ./PROCs/ft/R2_67_84.ft2 > noise67_84.txt

  - Then determine the standard deviation of the noise region or noise intensity, using statistical software [1]. Compute the experimental error using the given equation [2].

    - SCREEN: 68674_screenshot_37. 		00:04-00:08, 00:13-00:23

    - SCREEN: 68674_screenshot_37. 		00:28-00:43
AND
TEXT ON PLAIN BACKGROUND:
R2error= 1TrelaxInoiseIeach_residue          
Video Editor: Please play both shot side by side

# Results

- Results

  - Chemical shift perturbations were observed for both anomeric forms of D-mannose in the presence of CVN (C-V-N), with greater shifts seen in β (beta)-D-mannose, indicating preferential binding [1]. Two distinct peaks were observed for most D-mannose hydrogens in the presence of CVN, corresponding to free and bound states [2].

    - LAB MEDIA: Figure 1C.

    - LAB MEDIA: Figure 1C. Video editor: Sequentially highlight blue and green points

  - The hydrogen nuclei exhibiting the highest ASTD (A-S-T-D) values were associated with β-D-mannose, consistent with strong dipolar interactions with CVN [1].

    - LAB MEDIA: Figure 2C	 Video editor: Sequentially highlight the four plots with red hydrogen labels

  - In the transverse relaxation rate analysis, Hβ4 (H-Beta-Four) was the only proton that exhibited a notable increase in relaxation upon binding to CVN [1].

    - LAB MEDIA: Figure 3C. Video editor: Highlight the position labeled Hβ4 in the “D-mannose + CVN” red spectrum

  - The 1H and 15N chemical shift perturbations, induced by the addition of a ligand was highest at ligand concentration up to 10 millimoles [1].

    - LAB MEDIA: Figure 4A	Video editor: Highlight the top graph labelled “D-mannose=10 mM”

  - Residues I40(I-Forty), E41 (E-Forty-One), N42 (N-Forty-Two), V43 (V-Forty-three), D44 (D-Forty-Four), and G45 (G-Forty-Five) within the β-strand showed significant chemical shift perturbations, identifying them as the high-affinity binding site for D-mannose [1]. Binding isotherms showed residue D44 had the highest affinity among the β-strand residues, with a dissociation constant of approximately 1 millimolar [2].

    - LAB MEDIA: Figure 4A and 4B. Video editor: Highlight the columns labelled I40, E41, N42, V43, D44, and G45 in 4A

    - LAB MEDIA: Figure 4C. Video editor: Highlight the D44 plot

  - Measurement of ¹⁵N-R₂ (N-Fifteen-R-Two) relaxation rates showed both increases and decreases in ΔR₂ (Delta-R-Two) values upon D-mannose binding, suggesting a mix of conformational stabilization and exchange processes [1].

    - LAB MEDIA: Figure 5C. Video editor: Highlight the red and blue lines

  - Residues C58( C-Fifty-Eight), R59 (R-Fifty-Nine), K74 (K-Seventy-Four), and R76 (R-Seventy-Six) demonstrated increased ΔR₂ values, confirming their involvement in the high-affinity binding site [1], while residues F4 (F-Four), C8 (C-Eight), R24 (R-Twenty-Four), and G27(G-Twenty-Seven) represented the low-affinity region [2].

    - LAB MEDIA: Figure 5D. Video editor: Highlight the data points corresponding to C58, R59, K74, and R76

    - LAB MEDIA: Figure 5D. Video editor: Highlight the data points labeled F4, C8, R24, and G27

Pronunciation Guide:

1. Glycan

Pronunciation link:
https://www.merriam-webster.com/dictionary/glycan
IPA: /ˈɡlaɪˌkæn/
Phonetic Spelling: gly-kan

2. Integrin

Pronunciation link:
https://www.merriam-webster.com/dictionary/integrin
IPA: /ˈɪntəɡrɪn/
Phonetic Spelling: in-tuh-grin

3. Disintegrin

Pronunciation link:
https://www.merriam-webster.com/dictionary/disintegrin
IPA: /dɪsˈɪntəɡrɪn/
Phonetic Spelling: dis-in-tuh-grin

4. Hydrophobic

Pronunciation link:
https://www.merriam-webster.com/dictionary/hydrophobic
IPA: /ˌhaɪdrəˈfoʊbɪk/
Phonetic Spelling: hy-droh-foh-bik

5. Ligand

Pronunciation link:
https://www.merriam-webster.com/dictionary/ligand
IPA: /ˈlɪɡənd/ or /ˈlaɪɡænd/
Phonetic Spelling: lig-and (common) or lie-gand

6. Mannose

Pronunciation link:
https://www.merriam-webster.com/dictionary/mannose
IPA: /ˈmænoʊs/
Phonetic Spelling: man-ohs

7. Cyanovirin

Pronunciation link:
https://www.howtopronounce.com/cyanovirin
IPA: /saɪˌænoʊˈvɪrɪn/
Phonetic Spelling: sigh-an-oh-veer-in

8. NMR (Nuclear Magnetic Resonance)

Pronunciation link:
https://www.merriam-webster.com/dictionary/nuclear%20magnetic%20resonance
IPA: /ˈnuːkliər mæɡˈnɛtɪk ˈrɛzənəns/
Phonetic Spelling: new-klee-er mag-net-ik reh-zuh-nuhns

9. HSQC (Heteronuclear Single Quantum Coherence)

Pronunciation link:
No confirmed link found
IPA: /ˌhɛtərəˈnuːkliər ˈsɪŋɡəl ˈkwɑntəm koʊˈhiːrəns/
Phonetic Spelling: het-er-oh-new-klee-er sin-guhl kwahn-tum koh-heer-ens

10. CPMG (Carr–Purcell–Meiboom–Gill)

Pronunciation link:
No confirmed link found
IPA: /ˈkær pɜrsəl ˈmaɪbuːm ɡɪl/
Phonetic Spelling: care per-suhl my-boom gill

11. Isotherm

Pronunciation link:
https://www.merriam-webster.com/dictionary/isotherm
IPA: /ˈaɪsəˌθɝm/
Phonetic Spelling: eye-so-therm

12. Resonance

Pronunciation link:
https://www.merriam-webster.com/dictionary/resonance
IPA: /ˈrɛzənəns/
Phonetic Spelling: reh-zuh-nuhns

13. Conformational

Pronunciation link:
https://www.merriam-webster.com/dictionary/conformation
IPA: /ˌkɑnfɔrˈmeɪʃənəl/
Phonetic Spelling: con-for-may-shuh-nuhl

14. Anomeric

Pronunciation link:
https://www.merriam-webster.com/dictionary/anomeric
IPA: /ˌænəˈmɛrɪk/
Phonetic Spelling: an-uh-mer-ik

15. Dipolar

Pronunciation link:
https://www.merriam-webster.com/dictionary/dipolar
IPA: /daɪˈpoʊlər/
Phonetic Spelling: dye-poh-lur