# Submission ID #: 68697

# Scriptwriter Name: Sulakshana Karkala

# Project Page Link: https://review.jove.com/account/file-uploader?src=20952633

# Title: Creating Rapid Oxygen Oscillations in Microbial Single-Cell Growth Analysis Using a Microfluidic Double-Layer Device

# Authors and Affiliations:

# Keitaro Kasahara1,2, Dietrich Kohlheyer1

# 1IBG-1: Biotechnology, Institute of Bio- and Geosciences, Forschungszentrum Jülich GmbH

# 2Computational Systems Biotechnology (AVT.CSB), RWTH Aachen University

# Corresponding Authors:

Keitaro Kasahara		(k.kasahara@fz-juelich.de)

# Email Addresses for All Authors:

Keitaro Kasahara		(k.kasahara@fz-juelich.de)

Dietrich Kohlheyer		(d.kohlheyer@fz-juelich.de)

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes,all done

3. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  25

Number of Shots:  55

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

  - Keitaro Kasahara: We have developed a double-layer microfluidic chip that creates rapid oxygen oscillations, allowing us to observe microbial growth behavior under rapidly oscillating oxygen conditions mimicking natural and industrial environments.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:5.3

What are the current experimental challenges?

  - Keitaro Kasahara: Traditional experimental setups lack the precision and speed needed to mimic rapid oxygen fluctuations and do not support continuous, high-resolution monitoring of microbes.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What advantage does your protocol offer compared to other techniques?

  - Keitaro Kasahara: The double-layer PDMS microfluidic chip enables microbial single-cell growth analysis under controlled oxygen conditions, including rapidly oscillating oxygen conditions.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:5.4

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

- Fabrication and Bonding of a Multilayer PDMS Microfluidic Device

Demonstrator: Keitaro Kasahara

  - To begin, thoroughly mix the PDMS (P-D-M-S) base and curing agent in a 10 to 1 ratio to prepare the pre-cure solution [1]. Place the mixture in a desiccator and degas for 1 hour [2].

    - WIDE: Talent adding PDMS base and curing agent into a beaker and mixing thoroughly with a spatula.

    - Talent placing the beaker with PDMS mixture into a desiccator and closing the lid.

  - Clean the top layer mold with isopropanol [1] and dry it using compressed air [2].

    - Talent rinsing the top layer mold with isopropanol.

    - Talent using compressed air to dry the mold completely.

  - Now, pour the PDMS mixture into the cleaned top layer mold [1]. Then thermally cure the mold at 80 degrees Celsius for 20 minutes to partially cure the PDMS [2]. Check the PDMS stickiness by gently pressing it with tweezers or by hand [3-TXT].

    - Talent pouring the PDMS mixture into the top layer mold.

    - Talent placing the filled mold into an oven set at 80 degrees Celsius.

    - Talent checking PDMS surface stickiness using tweezers. TXT: If the PDMS is still sticky, adjust heating time

  - Once cured, peel the PDMS out of the mold [1] and cut it into 20 by 18-millimeter single pieces [2].

    - Talent peeling the cured PDMS from the mold.

    - Talent using a blade to cut the PDMS into rectangular pieces.  AUTHOR’S NOTE: Please don't use the last seconds

  - Punch 0.75-millimeter diameter holes for gas inlets into each piece [1]. Then rinse the bottom surface with isopropanol [2]. After drying with compressed air, attach adhesive tape [3].

    - Talent punching holes into each PDMS piece using a hole punch.

    - Talent rinsing the bottom surface of the PDMS with isopropanol.

    - Talent applying adhesive tape to the bottom surface of each piece.
Note: 2.5.3 removed by the authors

  - Next, clean the bottom layer mold with isopropanol and dry it [1]. Fix the mold on a spin coater [2] then pour the PDMS mixture at the center, close to the wafer, to minimize bubble formation [3-TXT].

    - Talent cleaning the bottom mold with isopropanol.

    - Shot of the mold being fixed on a spin coater.

    - Talent pouring PDMS onto the center of the mold. TXT: Use tweezers/needles to move visible air bubbles to the outer edge

  - Spin coat the PDMS at 1000 revolutions per minute for 60 seconds [1]. Thermally cure the coated PDMS at 80 degrees Celsius for 10 minutes [2-TXT].

    - Talent setting the spin coater and starting the spin coating process.

    - Talent placing the mold in the oven for 10 minutes at 80 degrees Celsius. TXT: Check stickiness and adjust heating time, if necessary

  - Now place the top PDMS pieces onto the bottom PDMS layer on the mold [1] and press firmly to ensure surface attachment [2].

    - Talent placing the rectangular top PDMS pieces onto the bottom layer.

    - Talent using gloved fingers or a roller to press and attach both layers.

  - Thermally cure the assembled layers at 80 degrees Celsius for at least 1 hour to complete the bonding [1].

    - Talent placing the assembled chip back into the oven and setting the timer for 1 hour.

  - Then gently cut the bottom PDMS layer into individual pieces [1] and carefully peel off the entire PDMS chip from the mold [2].

    - Talent using a scalpel to cut the bottom layer into chip-sized pieces.

    - Talent gently peeling the PDMS chip from the mold by hand.  AUTHOR’s NOTE: 2.10.1 and 2.10.2 are in one clip

  - Punch 0.5-millimeter diameter holes for fluid inlets and outlets at both ends of the fluid channels [1]. Rinse the bottom surface of the chip with isopropanol [2].  and attach scotch tape to the dried chip [3].

    - Talent punching small holes at both ends of the PDMS fluid channels.

    - Talent rinsing the chip bottom surface with isopropanol.

    - Talent applying scotch tape to the bottom surface of the chip.
Note: 2.11.3 removed by the authors

  - Activate the PDMS surface by plasma oxidation for 25 seconds [1] and bond it to a 0.175-millimeter-thick cover glass [2].

    - Talent placing the chip and a cover glass in a plasma cleaner.

    - Talent aligning and pressing the PDMS chip onto the glass substrate.

  - After placing the chip on the glass, flush the top layer with pressurized air to ensure that the PDMS membrane is properly attached to the glass [1]. Heat the bonded chip at 80 degrees Celsius for 10 seconds to 1 minute to increase bonding stability [2].

    - Talent flushing the top layer with air. 
AUTHOR’s NOTE: 2.12.2 and 2.13.1 are in one clip

    - Talent placing the bonded chip into the oven and setting a short heating cycle.

- Microscopy Setup, Dye Perfusion, and Gas Control for FLIM-Based Oxygen Sensing

  - Select the appropriate objective lens on the microscope, such as 20X for oxygen sensing or 100X for microbial observation [1-TXT]. Then fix the double-layer chip on the chip holder using adhesives [2].

    - Talent rotating the microscope turret to select the objective lens. TXT: Add immersion oil if necessary

    - Talent fixing the double layer chip onto the chip holder with adhesives

  - Next prepare an oxygen-sensitive dye solution at the appropriate concentration [1-TXT].

    - Shot of prepared dye solution in a labeled microtube. TXT: 3 mM tris(2,2’-bipyridyl)dichlororuthenium(II)hexahydrate, RTDP is being used here

  - Calibrate the FLIM (Flim) camera using a reference slide with a known lifetime, such as 3.75 nanoseconds [1]. Measure the signal intensity with the FLIM camera [2-TXT].

    - Talent placing a calibration slide under the FLIM camera.

    - SCREEN: 3.3.2.mp4		00:06-00:15
TXT: Adjust exposure time to reach signal intensity between 0.68 - 0.72

  - Fix the chip holder on the microscope stage and connect the appropriate tubing to the fluid inlet and outlet [1]. Then start perfusing the oxygen-sensitive dye solution at a constant flow rate of 100 nanoliters per minute using a syringe pump [2].

    - Talent securing the chip holder onto the microscope stage and attaching inlet and outlet tubing to the chip.  AUTHOR’s NOTE: 3.4.1 is in two clips. Second clip is from “ connecting the tubing”

    - Talent programming the syringe pump to initiate dye perfusion.

  - Next, connect the gas inlet and mass flow controllers with suitable tubing [1]. Begin flushing gas with controlled oxygen concentrations [2-TXT].

    - Talent connecting gas tubing to the chip and mass flow controllers.

    - Talent setting the flow rates on the gas controller and starting the gas flow. TXT: 0% and 21% oxygen, total mass flow rate 600 mL/min

  - Measure the phase lifetime in the absence of oxygen and at another known oxygen concentration [1]. Calculate the quenching constant using the Stern-Volmer equation [2].

    - SCREEN: 3.6.1.mp4		00:00-00:17

    - SCREEN: 3.6.2.mp4		00:00-00:11
AND
TEXT ON PLAIN BACKGROUND:
O2= 1Kqτ0τ-1=1KqI0I-1

Video editor: Please play both shots side by side

- Cell Seeding, Cultivation, and Time-Lapse Imaging under Controlled Oxygen Conditions

  - Prepare a seeding solution by diluting the Escherichia coli culture to the desired optical density [1]. Transfer the solution into a 1-milliliter syringe for loading [2].

    - Talent measuring the E. coli culture’s optical density and diluting it in a tube.

    - Talent drawing the seeding solution into a 1 milliliter syringe.

  - Fix the chip holder on the microscope stage [1].  Allow the chip and holder to warm up in the incubator at 37 degrees Celsius for several hours to reduce defocusing during imaging [2].

    - Shot of the chip holder being fixed on a microscope stage.

    - Talent placing the chip holder with the mounted chip onto the microscope stage inside an incubator.
Note: 4.2.2 removed by the authors

  - Before starting cultivation, begin gassing with the desired initial oxygen concentration by adjusting the oxygen and nitrogen mixture while maintaining the total and carbon dioxide flow rates constant [1]. Then connect the gas inlet tubing to the chip and mass flow controllers [2].

    - Talent adjusting the gas controller settings to set the oxygen, nitrogen, and carbon dioxide mixture.

    - Talent connecting the gas inlet tubing securely between the chip and the flow controller ports.

  - Connect the seeding syringe to the fluid outlet and manually push the syringe to begin seeding the bacteria onto the chip [1]. Observe under the microscope to verify successful trapping of cells in the cultivation chambers [2].

    - Talent connecting the syringe with seeding solution to the fluid outlet.

    - Talent observing cells in the cultivation chambers on the screen.

  - After cell inoculation, disconnect the seeding syringe and attach a syringe filled with fresh medium [1]. Manually push the syringe to flush out any remaining cells from the medium supply channel [2].

    - Talent removing the seeding syringe and replacing it with a fresh medium syringe.

    - Talent manually flushing the channel with medium using the syringe.

  - Begin perfusing the medium at a constant rate of 100 nanoliters per minute using the syringe pump [1-TXT].

    - Talent starting the syringe pump and monitoring the medium flow. TXT: Ensure that flow rate is enough to prevent chamber drying

  - Start time-lapse image acquisition [1]. For cultivation under oscillating oxygen conditions, begin automatic control of mass flow controllers simultaneously with image capture [2-TXT].

    - SCREEN: 4.7.1.mp4	00:00-00:08

    - Talent starting time-lapse image acquisition and automatic gas control. TXT: If there is any delay, record the timing difference

# Results

- Results

  - The oxygen concentration in the fluid channel of the microfluidic chip decreased from 21% to 0% within 10 seconds [1] and increased back to 21% within a similar time frame upon reoxygenation [2].

    - LAB MEDIA: Figure 2B (left panel). Video editor: Highlight the red box in 2 B left curve.

    - LAB MEDIA: Figure 2B (right panel). Video editor: Highlight the steep rise in the oxygen concentration curve

  - After 3 hours of cultivation, E. coli colonies grown under 21% oxygen conditions appeared visibly larger than those under 0% oxygen conditions [1].

    - LAB MEDIA: Figure 3A-B. Video editor: Please highlight images of 3A

  - The normalized colony area increased more rapidly at higher oxygen concentrations, with a clear trend shown from 0% to 21% oxygen [1]. The exponential growth rate μ (mew) increased sharply from 0% to 0.5% oxygen and plateaued beyond 5% [2].

    - LAB MEDIA: Figure 3C. Video editor: Highlight the curves corresponding to 10 and 21% O2

    - LAB MEDIA: Figure 3D. Video editor: Highlight the steep portion of the plot from 0% to 0.5% oxygen.

  - Under oscillating oxygen with a 60-minute cycle, E. coli growth displayed three phases, a rapid response to deoxygenation [1], a recovery phase with gradual increase [2], and a final stabilization around the anaerobic growth rate [3].

    - LAB MEDIA: Figure 4 (T’=60 min). Video editor: Highlight the area pointed at by  “response”

    - LAB MEDIA: Figure 4 (T’=60 min). Video editor: Highlight the area pointed at by  “Recovery”

    - LAB MEDIA: Figure 4 (T’=60 min). Video editor: Highlight the area pointed at by  “Stabolization”

  - As the switching interval T’ (T-dash) decreased from 60 to 1 minute, the growth rate oscillations became more synchronized with gas phases, and adaptation effects diminished [1].

    - LAB MEDIA: Figure 4 (all panels). Video editor: Sequentially show the graphs from T’ = 60 min to T’=1 min

  - Single-cell area measurements showed faster growth during aerobic phases and clear drops during anaerobic phases across all tested switching intervals [1].

    - LAB MEDIA: Figure 5. Video editor: Please highlight all blue dots in the white portions of each graph

Pronunciation Guide:

1. Escherichia
Pronunciation link: https://www.merriam-webster.com/medical/Escherichia YouTube+8Merriam-Webster+8YouTube+8
IPA: /ˌɛʃəˈrɪkiə/
Phonetic Spelling: ESH-uh-RIK-ee-ə

2. Escherichia coli
Pronunciation link: https://www.collinsdictionary.com/us/dictionary/english/escherichia-coli YouGlish+1Merriam-Webster+1YouTube+4Collins Dictionary+4YouGlish+4
IPA: /ˌɛʃəˈrɪkiə ˈkoʊlaɪ/
Phonetic Spelling: ESH-uh-RIK-ee-ə COH-lye

3. E. coli
Pronunciation link: https://www.merriam-webster.com/dictionary/E.%20coli YouTube+4Collins Dictionary+4Definitions+4Merriam-Webster+9YouGlish+9YouTube+9Merriam-Webster+1YouTube+1
IPA: /ˌiː ˈkoʊ·laɪ/
Phonetic Spelling: ee COH-lye

4. microfluidic
Pronunciation link: https://www.howtopronounce.com/microfluidic YouTube+8How To Pronounce+8How To Pronounce+8 (via HowToPronounce domain, default resource)
IPA: /ˌmaɪkroʊfluˈɪdɪk/
Phonetic Spelling: my-kroh-flu-ID-ik

5. PDMS (polydimethylsiloxane)
Pronunciation link: https://www.howtopronounce.com/pdms Definitions+15How To Pronounce+15How To Pronounce+15Merriam-Webster+13YouGlish+13pronounce.voanews.com+13
IPA: /ˌpiː.diː.emˈɛs/
Phonetic Spelling: P-D-M-S (pee-dee-em-ess)

6. Stern–Volmer
Pronunciation link: https://www.synonyms.com/pronounce/stern-volmer YouGlish+7synonyms.com+7pronouncekiwi.com+7
IPA: /stɝnˈvɔlmər/
Phonetic Spelling: STERN-VOL-mer

7. FLIM (fluorescence lifetime imaging microscopy)
Pronunciation link: No confirmed link found (acronym commonly pronounced “F-L-I-M”, each letter spoken)
IPA: /ɛf-ɛl-aɪ-ɛm/
Phonetic Spelling: F-L-I-M

8. tris(2,2′-bipyridyl)dichlororuthenium(II) hexahydrate
Pronunciation link: No confirmed link found (too complex for standard entries; approximate)
IPA: /traɪs tuːˌbaɪpɪˈrɪdəl daɪklɔːroʊˈruːθiːniəm/
Phonetic Spelling: trys too by-pi-RID-il dye-KLOR-oh-ROO-thee-nee-əm

9. spin coater
Pronunciation link: No specific entry—standard terms: “spin” and “coater”
IPA: /spɪn ˈkoʊtər/
Phonetic Spelling: spin COH-ter

10. nanoliters
Pronunciation link: Merriam-Webster (implied from “nanoliter”)
IPA: /ˈnænəˌliːtərz/
Phonetic Spelling: NAN-oh-LEE-terz