# Submission ID #: 68701

# Scriptwriter Name: Sulakshana Karkala

# Project Page Link: https://review.jove.com/account/file-uploader?src=20953853

# Title: 3D Flipwell Engineering for Developing Asynchronous Systems for Toxicologic and Immunomodulatory Therapies in Bacterial, Gut, and Immune Cells

# Authors and Affiliations:

# Maria A. Beamer1, Saori Furuta2

# 1Division of Pediatric Rheumatology, Department of Pediatrics, University of Michigan

# 2MetroHealth Medical Center, Case Western Reserve University School of Medicine

# Corresponding Authors:

Maria A. Beamer	(mbeamer@med.umich.edu)

Saori Furuta		(sxf494@case.edu)

# Email Addresses for All Authors:

Maria A. Beamer	(mbeamer@med.umich.edu)

Saori Furuta		(sxf494@case.edu)

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  NO

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  34

Number of Shots:  60

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

REQUIRED: What is the scope of your research? What questions are you trying to answer?

  - Furuta Saori: We developed a co-culture system mimicking gut mucosa to study intercellular crosstalk and evaluate drug responses, enabling potential use in drug screening and understanding mucosal cellular interactions.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:4.4.2

What are the most recent developments in your field of research?

  - Furuta Lsaori: Different groups have developed several different culture systems to model the gut mucosal environment by using complex bioengineering techniques. We reported our new co-culture system which is easy to create using less expensive materials.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What significant findings have you established in your field?

  - Maria Beamer: Our co-culture system revealed that a pro-immunogenic drug triggers coordinated responses among gut bacteria, epithelia, and immune cells, activating host immunity, likely via bacterial metabolites mediating cross-species communication.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:6.2

What research questions will your laboratory focus on in the future?

  - Maria Beamer : We aim to study both aerobic and anaerobic bacterial strains and their metabolites to uncover immune-modulating effects, paving the way for novel immunotherapeutic strategies based on bacterial metabolites.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

- Assembly and Sterilization of the Flipwell Co-culture Insert Stack

Demonstrator: Maria Beamer

  - To begin, open a Petri dish cover and set it aside [1]. Place a scalpel blade on the edge of the Petri dish bottom [2].

    - WIDE: Talent opening the Petri dish and setting lid aside.

    - Talent removing a scalpel blade from its casing and placing it on the edge of the dish.

  - Take a set of sterile tweezers and gently grab the insert bottom up, to remove it from the packaging [1]. With the other hand take the sterile scalpel blade and bring it up to the insert [2].

    - Talent using tweezers to lift the insert out.

    - Talent positioning the blade near the insert.

  - With a C-shaped motion pierce the membrane at the edge and gently slide the scalpel blade around the insert bottom staying close to the plastic wall [1].

    - Talent performing the C-shaped cut along the membrane edge.

  - Cut the PET (P-E-T) membrane out and use the second set of tweezers to remove it [1]. Then use the scalpel blade to scrape any white membrane shavings to clean the edge and rim of the insert [2-TXT].

    - Talent removing the cut membrane with second tweezers.

    - Talent scraping shavings from the insert rim. TXT: Carefully set the newly prepared insert back into its original sterile pack

  - Next, gently spread a thin two to three millimeters bead of silicone all the way around the insert bottom being careful not to touch the membrane [1].

    - Talent applying silicone bead around insert bottom.

  - Pick up the second set of sterile forceps and carefully lift the first insert without the membrane out of its protective pack [1]. Bring the two inserts together bottom-to-bottom so that the bottom rims align [2-TXT].

    - Shot of the first insert being lifted out of its pack.

    - Talent aligning and stacking the two inserts bottom to bottom. TXT: Ensure final FCI stack has only 1 PET membrane in the middle

  - When the Flipwell stack is glued place it inside the original sterile pack or a deep Petri dish to dry for 72 hours [1]. Use the lid from the specified Petri dishes to cover the stack assembly [2].

    - Talent placing stack into pack or dish.

    - Talent covering with Petri dish lid.

  - To sterilize the stack assemblies, use sterile forceps to hang the Flipwell stack on the rim of the specified deep Petri dish rim [1]. Then close the glass sash of the biosafety cabinet and turn on the ultraviolet light [2-TXT].

    - Talent using forceps to hang the stack.

    - Talent closing cabinet sash and switching on UV light. TXT: Step away and sterilize each membrane side for 30 min - 1 h

- Leakage Testing and Collagen Coating of the Flipwell Membrane

  - To test for leakage prior to collagen coating, first add 500 microliters of sterile PBS or sterile deionized water [1-TXT]. The next day, aspirate the liquid used for testing before drying the stacks for one to two hours inside the biosafety cabinet [2].

    - Talent pipetting PBS or water into the Flipwell. TXT: Cover dish and let it rest overnight

    - Talent aspirating liquid.

  - To coat the membrane with collagen, open the Petri dish lid and let the Flipwell hang off the Petri dish rim [1]. Carefully add 200 microliters of collagen solution to one side of the insert stack [2].

    - Talent opening lid and positioning Flipwell hanging off rim.

    - Talent pipetting collagen solution into one side. TXT: Let sit for 1 h

  - Aspirate the collagen solution after 1 hour [1]. Then pipette 200 microliters of sterile PBS [2].

    - Talent pipetting out the collagen solution.

    - Talent adding 200 µL of PBS.

  - After aspirating the PBS, cover the Petri dish with lid and let the membrane dry for 60 minutes inside the cabinet [1]. When the membrane is dry, use sterile forceps or tweezers and flip the Flipwell to the opposite side and let it hang off the Petri dish rim [2].

    - Talent covering dish and letting dry in cabinet.

    - Talent flipping Flipwell with forceps and hanging it off rim.

  - To make the bacterial insert, place two sets of sterile forceps and the twenty-four-well inserts inside the biosafety cabinet [1-TXT]. With sterile tweezers lift the insert from its sterile pack [2]. With the second set of tweezers or forceps break off the small plastic feet at the bottom of the insert [3].

    - Talent arranging forceps and inserts in cabinet. TXT: Making the bacterial insert is an optional step

    - Talent lifting twenty-four-well insert.

    - Talent breaking off plastic feet.

  - Test the twenty-four-well insert by fitting it inside one of the sterile Flipwell co-culture insert stacks or the original twelve-well inserts [1-TXT].

    - Talent inserting test insert into stack. TXT: UV sterilize before use

- Epithelial and THP-1 Cell Seeding on the Flipwell Co-culture Insert

  - To begin seeding the Flipwells, seed 500 microliters of the epithelial cells lines on each side of the Flipwell’s apical side [1-TXT]. Cover the assemblies with a Petri dish lid then incubate at 37 degrees Celsius with 5 percent carbon dioxide overnight until the cells attach [2].

    - Talent pipetting cell suspension into apical side. TXT: 7.5 x 104 cells per cm2

    - Talent covering assemblies with a Petri dish lid and placing them in an incubator.

  - Open the Petri dish lid and aspirate the DMEM media from the apical side of the Flipwell stack [1]. With sterile forceps, pick up the Flipwell carefully by the hook and rotate it by 180 degrees [2].

    - Talent aspirating apical DMEM.

    - Talent lifting Flipwell by hook and rotating it by 180°.

  - With the second set of sterile forceps, grab the Flipwell stack by the other hook now facing upward [1]. Then lower the assembly back into the Petri dish and hang the hook over the Petri dish rim [2].

    - Talent re-grasping with second forceps after rotation.

    - Talent positioning assembly hanging on dish rim.

  - Add 500 microliters of THP-1 (T-H-P-One) cell suspension to the apical side of the co-culture insert stack [1]. Adjust the HT-29 (H-T-Twenty-Nine) media for the colon epithelial cells by adding media to the deep well Petri dish [2].

    - Talent adding THP-1 suspension to the apical side.

    - Talent topping up DMEM media for the gut epithelial cells in deep well dish.

  - To remove any air bubbles, hold the Flipwell stack by the arm with sterile forceps [1]. Carefully lower a gavage needle in and under the insert assembly and very gently place the soft tip up to the air bubble [2].

    - Talent grasping Flipwell arm with forceps.

    - Talent lowering needle and positioning near air bubble.

  - Carefully and very slowly pull the syringe plunger up and watch the air bubble disappear slowly [1-TXT]. Then carefully remove the needle and syringe from the Flipwell [2].

    - Talent pulling plunger gradually. TXT: Stop when bubble is 2 mm in diameter

    - Talent pausing when bubble is two millimeters.

    - Talent removing needle and syringe.

  - After the air bubbles have been removed with a gavage needle, culture both cell types in the cell culture incubator until THP-1 cells attach and proliferate for one to two days [1-TXT].

    - Talent placing in incubator and monitoring proliferation. TXT: 37°C, 5% CO2, 100%, overnight

- Bacterial Co-culture, Fixation, and Immunostaining Preparation of the Flipwell System

  - With forceps, set the insert inside a sterile Petri dish and cover with the lid [1]. Transfer the Petri dish with the Flipwells from the incubator into the biosafety cabinet and set aside on the opposite side of the dish with bacterial inserts [2].

    - Talent placing insert in dish and covering.

    - Talent placing dish opposite bacterial insert side.

  - Pipette out 300 microliters of the DMEM media from the apical side of the Flipwell to allow for the bacterial insert, then cover with the petri dish lid [1 -TXT].

    - Talent aspirating DMEM leaving two hundred microliters and covering assembly with a Petri dish lid. TXT: Leave about 200 µL in the stack’s top side

    - Talent covering assemblies with lid.

  - Now add 50 to 100 microliters of Bacillus subtilis culture to the 24-well bacterial inserts and insert it into the Flipwell top side [1].

    - Talent pipetting B. subtilis culture into bacterial inserts and transferring them into the flipwell.

  - Rotate the arm and hang it from the rim of the Flipwell co-culture insert stack [1]. Hold the stack with the second set of sterile forceps [2]. After a 3 hour-incubation, use sterile forceps to remove the bacterial insert from the Flipwell [3-TXT].

    - Shot of the arm being rotated and hung from the Flipwell rim.

    - Talent stabilizing stack with second forceps.

    - Talent removing bacterial insert carefully. TXT: Label with a solvent-proof marker

  - Place the Flipwell inside a 50-milliliter conical tube without disassembly [1-TXT].

    - Talent placing assembly into conical tube. TXT: Submerge in 4% PFA overnight or for longer storage, use 0.4% PFA

  - To disassemble the Flipwell for electron microscopy, hold it with both hands and twist off each glued part of the stack [1]. Rotate the insert with the membrane and set it down with the membrane facing up [2].

    - Talent twisting apart the glued stack.

    - Talent placing the membrane insert membrane-side up.

  - Hold the insert and carefully cut out the membrane with a scalpel blade then place it inside a 1.7 milliliter tube containing paraformaldehyde solution [1-TXT].

    - Talent cutting membrane of insert and placing the cut membrane into fixation vessel. TXT: 500 µL - 1 mL of 0.4 - 4% PFA

  - For confocal microscopy, add 300 microliters of sterile PBS to the apical side after aspirating the DMEM [1]. Carefully remove the stack from the Petri dish after pipetting out the PBS [2].

    - Talent adding three hundred microliters PBS, then aspirating PBS

    - Talent removing stack from dish.

  - Now flip the Flipwell and add 300 microliters of PBS to the now facing upward RPMI (R-P-M-I) side of the Flipwell co-culture insert stack then aspirate the phosphate-buffered saline [1].

    - Talent flipping stack and adding and removing three hundred microliters PBS to RPMI side.

  - Using a 200-microliter pipette, create a large droplet of PBS [1]. Twist off the insert assembly to separate into the inserts [2].

    - Talent pipetting a two-hundred microliter large drop of PBS.

    - Talent twisting assembly to separate inserts.

  - Then carefully position the Flipwell stack with THP-1 cells on top of the 200-microliter drop of PBS [1]. Add 200 microliters of permeabilization buffer to the top side with colon epithelial cells and leave for ten minutes [2-TXT].

    - Talent positioning THP-1 insert over PBS drop.

    - Talent adding two hundred microliters permeabilization buffer. TXT: Aspirate and  add 200 µL PBS; Repeat wash 2 x

  - For immunofluorescent staining, pipette 300 microliters of primary antibody to the top of the insert [1]. Add 300 microliters of primary antibody to the well of a twelve-well plate then place the insert inside the well on top of the droplet [2-TXT].

    - Talent pipetting antibody onto insert top.

    - Talent pipetting 300 µL primary antibody being pipetted to the well on 12-well plate then placing the insert inside the well, on top of the droplet. TXT: Cover and incubate at 4 °C overnight

# Results

- Results

  - Staining showed dramatic increases of mucous secretion in the gut epithelial compartment after Sepiapterin treatment, as indicated by the increases of the epithelial marker protein CK20 and mucosal protein MUC2 [1].

    - LAB MEDIA: Figure 1. Video editor: Highlight the SEP row in the left image and the CK20-SEP and MUC2 SEP columns in the right graph

  - In THP-1 monocultures, Sepiapterin treatment significantly increased the expression of the M1 macrophage marker CD80 [1], while downmodulating M2 macrophage marker CD163, indicating M1 polarization [2].

    - LAB MEDIA: Figure 2A. Video editor: Highlight the green CD80 panel in the left image and the CD80 SEP column in the graph on the right

    - LAB MEDIA: Figure 2B. Video editor: Highlight the red CD163 panel in the left image and the CD163 SEP column in the graph on the right

  - Scanning electron microscopy revealed increased mucus secretion from epithelial cells in SEP-treated co-cultures [1]. Sepiapterin treatment induced macrophage morphology resembling M1 phenotype in both mono-cultures and co-cultures [2].

    - LAB MEDIA: Figure 3B. Video editor: Highlight the mucus-covered regions in panels O and P

    - LAB MEDIA: Figure 3. Video editor: Highlight panel E,S and T

  - Bacterial cells in Sepiapterin-treated mono-cultures displayed wrinkled surfaces characteristic of biofilm formation [1].

    - LAB MEDIA: Figure 3A. Video editor: Highlight the Panel L