TITLE:

Navigating the Mass Spectrometry-Based Proteomic Data Using Free Computational Tools

AUTHORS AND AFFILIATIONS:

Shijie He1, Fangfei Zhang2

1School of Life Sciences, Westlake University, Hangzhou, China

2Molemuse Biotech Studio, Xihu District, Hangzhou, China

Email address of co-author:

Shijie He		(heshijie@westlake.edu.cn)

Corresponding author:

Fangfei Zhang		(pushomix@molemuse.ac.cn)

SUMMARY:

Mass spectrometry-based proteomic data is available in open databases and accessible using free tools. Given the complexity of database searches and descriptions, many biologists lack the knowledge to utilize these datasets. Here, we provide a guide on using free tools for basic proteomic data searches.

ABSTRACT:

Mass spectrometry (MS)-based proteomics data, including Data-Dependent Acquisition (DDA) and Data-Independent Acquisition (DIA), are widely used in biological research. However, the application of these datasets in validation studies is still limited due to the lack of clear demonstrations on how to effectively search and analyze proteomic data. To fill this gap, we selected one DDA and one DIA dataset deposited in the PRoteomics IDEntifications Database (PRIDE) data repository to better illustrate the proteomic data analysis workflow and downstream post-processing of protein search results. For demonstration purposes, we used two free computational tools: FragPipe (v22.0) for DDA datasets and DIA-NN (2.1.0) for DIA datasets. Post-processing steps, such as generating volcano plots and lists of dysregulated proteins, were demonstrated using R code. This study provides basic protocols for searching and analyzing proteomic data, serving as an essential beginner's guide to effectively handle proteomic datasets. Through this work, we aim to empower researchers with the knowledge necessary to leverage proteomic data in their biological investigations.

INTRODUCTION:

The field of proteomics has been revolutionized by the widespread adoption of open-access data repositories and free software tools1, which have been instrumental in advancing research capabilities and promoting a culture of data sharing. The establishment of centralized resources, such as PRIDE2, and the development of standardized data formats by the Human Proteome Organization (HUPO) Proteomics Standards Initiative (PSI)3 have created an unprecedented opportunity for data reuse and integration. However, the analysis and interpretation of proteomic data still present significant technical barriers, particularly for biologists and physician scientists who wish to perform orthogonal validation of their own experimental results4. This manuscript directly addresses this challenge by presenting a vendor-agnostic computational workflow to provide an accessible framework for researchers to independently query, analyze, and integrate public proteomics data with their own findings, thereby facilitating robust orthogonal validation without the need for additional wet-lab experiments. This work is particularly appropriate for researchers seeking to orthogonally validate the differential expression of specific proteins identified in their own experiments and generate novel hypotheses by exploring the expression patterns of their proteins of interest across a wide range of published studies. By providing a step-by-step guide, we aim to empower a wider range of scientists to harness the full potential of public proteomics data, ultimately enhancing the robustness and impact of their research.

In mass spectrometry (MS)-based proteomics, MS1 and MS2 scans are fundamental to data acquisition. MS1 scans, or survey scans, detect and measure the mass-to-charge (m/z) ratios of all ions in a sample, providing a comprehensive overview of the ion population5. MS2 scans, also called fragmentation scans, involve isolating and fragmenting specific precursor ions chosen from the MS1 scan, generating fragment ion spectra for peptide identification6.

Data-dependent acquisition (DDA) selects the most abundant ions from MS1 scans for MS2 fragmentation, producing simpler spectra that streamline analysis. However, DDA can lead to stochastic sampling and missing values in complex samples, limiting reproducibility7. In contrast, data-independent acquisition (DIA) systematically fragments all ions within predefined m/z windows, ensuring comprehensive coverage and minimizing missing values. This enhances quantitative accuracy and reproducibility8, though DIA demands advanced computational tools to deconvolve its highly complex MS2 spectra9.

PRIDE (https://www.ebi.ac.uk/pride)2 is a leading repository within the ProteomeXchange Consortium (https://www.proteomexchange.org)10, a global platform for sharing proteomics data. Users can efficiently navigate this vast resource using keyword searches to locate datasets of interest.

DIA-NN11, a software suite leveraging deep neural networks, is a gold-standard tool for analyzing data-independent acquisition (DIA) proteomics data. It excels in identification and quantification accuracy, making it particularly effective for processing complex DIA datasets12. For data-dependent acquisition (DDA) workflows, MSFragger13, integrated into the FragPipe pipeline, offers ultrafast peptide identification. Its innovative fragment-ion indexing approach enables rapid spectral matching, outperforming traditional tools by over 100-fold in speed.

Equipped with these comprehensive databases and advanced tools, researchers can readily reuse proteomic data for cross-study validation and novel biological discoveries. This accessibility has driven the identification of numerous protein biomarkers across diverse diseases14,15, improved prognosis prediction16, and enabled molecular subtype classification based on proteomic profiles17.

PROTOCOL:

NOTE: See Figure 1 for an overview of the workflow and Table of Materials for detailed descriptions of the software used.

[Place Figure 1 here]

- Example file and software setup

NOTE: All applications are free for individual research use and can be run on a personal computer (Windows and Linux are supported).

  - Create a new directory for a project.

  - Download JMU_LM2_8526_55.raw, JMU_LM2_8568_51.raw, JMU_LM2_8696_37.raw, JMU_LM2_2112_87.raw, JMU_LM2_2195_91.raw, JMU_LM2_2377_73.raw, 20150127_liver_XH03_T_01.raw, 20150127_liver_XH03_T_02.raw, 20150127_liver_XH03_T_03.raw, 20150127_liver_XH03_P_01.raw, 20150127_liver_XH03_P_02.raw, 20150127_liver_XH03_P_03.raw from PRIDE.

NOTE: The two groups of files can be downloaded from https://www.ebi.ac.uk/pride/archive/projects/PXD039273 and https://www.ebi.ac.uk/pride/archive/projects/PXD006512 separately (Figure 2).

  - Download and install the required software tools.

1.3.1 Download the latest version of DIA-NN as a .msi file. DIA-NN is freely available as a .msi file for Windows or .zip file for Linux from https://github.com/vdemichev/DiaNN/releases/tag/2.0

1.3.2 Download the latest version of MSFileReader as an .exe file. MSFileReader is freely available as an .exe file from https://github.com/thermofisherlsms/MSFileReader/blob/main/MSFileReader_x64.exe, which is required to process raw formatted Thermo Fisher DIA data.

1.3.3 Download the latest version of .NET SDK as an .exe file. .NET SDK is freely available as an .exe file from https://dotnet.microsoft.com/en-us/download, which is required in Windows to execute MSFileReader.

1.3.4 Download the latest version of FragPipe as a .zip file. FragPipe is freely available as a .zip file from https://github.com/Nesvilab/FragPipe/releases. The zip file with jre is recommended as it contains a Java runtime for Windows, which is needed to execute FragPipe. MSFragger is embedded in the FragPipe.

1.4 Download the latest version of R and Rstudio and R script files from Supplementary File 1 and Supplementary File 2. R and RStudio can be downloaded freely from https://cran.rstudio.com/bin/windows/base/R-4.5.0-win.exe and https://posit.co/download/rstudio-desktop/.

1.5 Unzip the compressed files and click on the .exe or .msi Files to install the applications.

- DIA data analysis using DIA-NN

2.1 Download the reference proteome fasta file from UniProt. To download the Homo sapiens reference proteome (isoform information not included) go to https://www.uniprot.org/proteomes/UP000005640 (Figure 3A).

2.2 Click on the Add FASTA button to load the reference proteome file into DIA-NN.

2.3 Select the two options FASTA digest for library-free search /library generation and Deep learning-based spectra, RTs and lMs prediction under Precursor ion generation part. Click on the Run button to generate a predicted spectral library (Figure 3B).

2.3.1 Once the predicted spectral library file was generated, reuse that for other experiments within the same experimental acquisition settings through clicking on Spectral button to add the predicted library.

2.4 Unselect the two options under the Precursor ion generation part and click on the Type button that correlates with the file format under the Input part to load the DIA data.

2.5 Set the Mass Accuracy and MS1 Accuracy under Algorithm to 0.0 ppm.

2.6 Adjust the precursor and fragment mass range settings under Precursor ion generation part based on the experimental settings.

2.7 Keep the other default settings.

2.7.1	DIA-NN's precursor generation uses Trypsin/P (cleaves after K/R unless followed by proline) with 1 missed cleavage to model partial digestion; disable N-term M excision for non-canonical peptides (e.g., immunopeptidomics) or increase missed cleavages to 2 for longer peptides.

2.7.2	Explicitly enable Variable modifications (disabled by default) for PTM studies (e.g., Ox (M) for oxidation, K-GG for ubiquitination), and expand peptide filters (length: 7-30 aa; charge: 1-4+; precursor m/z: 300-1800) according to the experimental settings (e.g., length:5-50 aa, charge=1-6) for large PTMs or non-tryptic digests.

2.7.3	Algorithm settings leverage auto-calibration (mass accuracy=0.0 ppm) for precise mass alignment—override with manual tolerances (e.g., 10 ppm) for low-resolution instruments—and Match Between Runs (MBR) transfers IDs across runs (disable for heterogeneous samples).

2.7.4	Neural network scoring boosts identification confidence (retain unless poor data quality), while Quant UMS enables high-precision label-free quantification; switch to MS2 quantification for isobaric tags (e.g., TMT). Use library-free search only with high-resolution DIA data to bypass spectral library requirements.

2.8 Click on the Run button. Wait for results until Finished is shown in the operation interface indicating the run is finished (Figure 3C).

- DDA data analysis using FragPipe

3.1 Click on Fragpipe icon in the ~\FragPipe\fragpipe\bin folder after installation. All the dependent settings can be seen in the Config tab. Check whether the three modules (MSFragger, IonQuant, diaTracer, DIA-NN and Python) exist on your computer. If not, download them by clicking on Download/Update or Download (Figure 4A).

3.2 Switch to next tab, Workflow. Select Default for the workflow and click on Load workflow. Click on Add files to add file paths. Assign experiment name and bioreplicate number under Assign files or just leave it (Figure 4B).

3.3 Now, click on Database and turn to that tab. Load a fasta file or download any fasta files according to the species of the samples. While downloading, choose Reviewed sequences only, Add decoys and Add	common contaminants options for a simple run (Figure 4C).

3.4 Then click on MSFragger to change the tab. Choose the Closed Search default config and click on Load.

NOTE: The closed search is basically a search for the modifications that you have set while the open search will explore all the possible modifications from the data, which is more time-and memory-consuming.

3.5 For Peak matching, keep the default settings except that choosing None for Calibration and Optimization is recommended, as it will save time.

3.6	For protein digestion, adjust the parameters according to the experimental conditions. Keep the following default settings (Figure 5A).

NOTE: The precursor and fragment mass tolerances (± 20 ppm) define the maximum allowed mass error for matching experimental spectra to theoretical peptides, with tighter values recommended for high-resolution instruments. Enzyme rules (strict trypsin) specify cleavage specificity (after K/R, unless followed by P), with missed cleavages (2) permitting partially digested peptides. Peptide length (5-50 residues) and mass (500-5000 Da) filters exclude unlikely candidates. Variable modifications (e.g., +15.9949 Da on methionine for oxidation, +79.96633 Da on S/T/Y for phosphorylation) allow detection of chemical alterations, while Max combinations (5) limit simultaneous modifications per peptide to manage search space. Max variable mods per site (e.g., 3 on M) controls modification frequency. Split database improves efficiency for large proteomes, and isotope error (0/1/2) accounts for misassigned isotopic peaks. These settings collectively balance sensitivity, specificity, and computational load in peptide identification.

3.7 Change to Validation tab. Unselect Predict RT and Predict spectra, which is for DIA data (Figure 5B).

3.8	Click on Quant (MS1) tab. Select Run MS1 quant and click on Load Quant defaults. Select IonQuant and keep the default settings (Figure 6A). Disable MBR for unstable retention times or disable Intensity normalization if global scaling biases result; enable Keep index on disk to conserve RAM with large datasets.

NOTE: Quant (MS1) defaults to optimize label-free quantification using MaxLPQ (MaxLFQ-based Protein Quantification, an algorithm extending the MaxLFQ method to protein groups), requiring ≥1 unique ion per protein (increase to ≥2 for higher stringency at the cost of coverage). Match Between Runs (MBR) transfers identifications across runs using retention time (RT) tolerance (default 0.4 min; tighten to 0.2 min for stable chromatography or loosen for gradient variability) and ion mobility (IM) tolerance (default 0.05 1/k0 [inverse reduced mobility]; adjust based on IM precision), filtered by ≤1% FDR (False Discovery Rate; default MBR ion FDR=0.01; lower to 0.005 for rigor). Protein inference uses uniquer+rascor (razor peptide assignment for protein grouping). Feature detection employs 10 ppm mass tolerance (increase to 20 ppm for low-resolution MS or decrease to 5 ppm for high-resolution).

3.9 Now, click on the Run tab. Select Output directory. Click on RUN to start analyzing the data (Figure 6B).

NOTE: For label-free proteomics data not focusing on modifications or using label-based quantification, PTMs, Glyco, and Quant (Isobaric) tabs are not used.

- DIA-NN/FragPipe downstream analysis

NOTE: Detailed codes used in this study are included as Supplementary File 1 and Supplementary File 2. The used package versions in this study can be found in Supplementary File 3.

4.1 Open RStudio and read the protein expression data generated by DIA-NN or FragPipe. Protein group matrix data ending with .pg_matrix.tsv from DIA-NN and combined protein data ending with .tsv from FragPipe are normally used for analysis.

4.2 Remove the missing values without filling in NA and filter out the protein groups identified and quantified with less than two unique peptides.

NOTE: The DIA figures in this study were generated by filtered protein groups, while the DDA figures were from unfiltered protein groups.

4.3 Normalize protein intensities using median scaling or quantile normalization across samples. DIA data in this study was normalized by median normalization while DDA data by quantile normalization to better demonstrate the two methods.

NOTE: Median normalization is robust against outliers and suitable for scenarios where the majority of proteins remain unchanged across samples, making the median protein abundance a stable reference point. However, it may fail in scenarios with highly differentially expressed proteins18. Quantile normalization ensures a uniform distribution of protein abundances across samples, which is helpful when comparing samples with disparate distributions. It has been shown to reduce statistical power and may over-normalize data, potentially masking true biological differences18. In proteomics, median normalization is widely used in label-free studies to remove systematic biases related to MS instrument performance. Quantile normalization is preferred for large-scale datasets, differential expression analysis, and cluster analysis19,20.

4.4 Calculate log2 fold changes (e.g., PDAC Serum versus Normal) and t-test p-values. Calculate corrected p-values using the Benjamini-Hochberg method. Define significance thresholds (e.g., |log2FC| > 1, p_adj (FDR) < 0.05).

NOTE: The cutoff p-values utilized in this study's figures represent raw values, as employing adjusted p-values (p_adj) would eliminate all detectable proteins within the statistical power of these limited datasets.

4.5 Check the existence of missing/NA values and exclude them. Generate a volcano plot to visualize significant hits.

4.6 Select significantly dysregulated proteins. Apply Z-score normalization to protein expression values. Plot a clustered heatmap with sample group annotations.

4.7 Map proteins to Entrez IDs using org.Hs.eg.db. Perform GO enrichment (Biological Process) and KEGG pathway analysis.

NOTE: Biological Process (BP) categories are commonly selected in Gene Ontology (GO) analysis because they describe the larger processes or biological programs accomplished by multiple molecular activities20. These terms focus on cellular functions that go awry in diseases like cancer, including essential processes such as cell cycle regulation, metabolic processes, etc. In addition to BP, GO analysis also includes Molecular Function (MF) and Cellular Component (CC) categories. MF terms outline the biochemical activities of gene products, such as ion transport or DNA-binding capabilities. CC terms define the cellular locations where gene products carry out their functions, such as the nucleus, mitochondrial membrane, or cytoskeleton.

4.8 Query STRINGdb for interactions (confidence score ≥ 400). Generate an interactive network using visNetwork.

RESULTS:

To better illustrate the search for proteomic datasets, we selected two types of datasets for this analysis. Aiming to demonstrate dysregulated proteins in clinical contexts, we searched in PRIDE using keywords such as clinic. Specifically, we selected a dataset suitable for DIA analysis from the Clinical Proteomic Tumor Analysis Consortium (CPTAC)21, and another suitable for DDA analysis from the Chinese Human Proteome Project (CNHPP)22. These selections facilitated a comprehensive evaluation and demonstration of the software capabilities in analyzing both healthy and disease-related proteomic data.

The volcano plots presented provide a comprehensive analysis of the differential expression of serum proteins between normal individuals and those afflicted with Pancreatic Ductal Adenocarcinoma (PDAC), as well as between hepatocellular carcinoma (HCC) tumor samples and their paired tissues (Figure 7A). These plots utilize the Log2 fold change (Log2FC) and p-values to pinpoint statistically significant alterations in protein expression. Proteins are color-coded to denote their significance. In the context of PDAC, red indicates proteins with significant changes. Proteins such as Serpin Family A Member 5 (SERPINA5) and Heparanase (HPSE) exhibit reduced expression in patients' serum, whereas Secreted Protein Acidic and Rich in Cysteine (SPARC), Cysteine-Rich Secretory Protein 3 (CRISP3), and Fibrinogen Beta Chain (FGB) display increased expression levels. In Figure 7B, a nuanced color strategy is implemented: green signifies proteins surpassing a specific Log2FC threshold, blue marks surpassing a p-value threshold, and red denotes proteins meeting both criteria. Notably, proteins like Pleckstrin 3 (PLS3), Enolase 3 (ENO3), Methylthiotransferase Alpha 1 (MTAP), Serpin Family B Member 9 (SERPINB9), and Inositol-1,4,5-Trisphosphate Receptor, Type 2 (ITPR2) demonstrate decreased expression in HCC tumor samples, while Malic Enzyme 1 (ME1), Cytochrome P450 Family 27, Subfamily A, Member 1 (CYP27A1), Ribosomal Protein S16 (RPS16), and ATP Synthase, H+ Transporting, Mitochondrial Fo Complex, Subunit F (ATP5PF) exhibit increased expression. This analysis is instrumental in identifying potential biomarkers for the diagnosis and treatment of PDAC and HCC, offering valuable insights into the potential molecular mechanisms underlying these diseases.

The heatmaps effectively visualize the expression levels of significant proteins in the serum of both normal individuals and those with PDAC, as well as in paired tissue and HCC tumor samples (Figure 8A-B). Employing a color gradient that transitions from blue to red, the heatmaps reveal distinct expression patterns between these groups. Certain proteins are consistently expressed at higher levels in the serum of PDAC patients and in HCC tumors. The dendrogram on the left side of the heatmaps indicates the clustering of related proteins, suggesting similarities in their expression profiles. Meanwhile, the arrangement at the top of the heatmaps highlights the variability within each group and the potential for subgroup identification.

Two distinct heatmap demonstration strategies are applied. For PDAC serum proteins, they successfully distinguish between PDAC serum and normal serum, and for HCC tumors, differentially expressed proteins show less classification power from paired tissues. This observation underscores the tumor heterogeneity and the potentially close relationship between the tumor and adjacent tissues. Visualization is instrumental for understanding protein heterogeneity in PDAC serum and HCC tumors, and for evaluating potential subgroup classification strategies.

Gene Ontology (GO) biological process enrichment analyses were performed for significantly upregulated genes in the serum of patients with PDAC and in HCC tumor samples, respectively (Figure 8C-D). The GeneRatio on the x-axis indicates the proportion of upregulated genes within each process relative to all annotated genes, while the color gradient represents the adjusted p-value, with red indicating higher significance. Based on the GO enrichment analysis, the molecular mechanisms in PDAC and HCC appear to be driven by distinct biological processes. In PDAC serum, there is a significant enrichment of upregulated genes involved in the regulation of blood coagulation and hemostasis, indicating that the body's coagulation and wound healing systems are key aspects of the disease's pathogenesis. In stark contrast, HCC tumor samples show a different biological signature, with significant enrichment in processes like purine nucleotide metabolic process and nucleotide metabolic process. This suggests that HCC is characterized by a profound metabolic reprogramming within the cancer cells, likely to support their rapid growth and proliferation.

The scatter plots present a comprehensive Kyoto Encyclopedia of Genes and Genomes (KEGG) pathway enrichment analysis of significantly upregulated genes in the serum of patients with PDAC (Figure 8E) and the tumor sample of HCC patients (Figure 8F). The x-axis denotes the GeneRatio, which is the ratio of the number of upregulated genes related to a specific pathway to the total number of genes annotated to that pathway23. Each point on the plot corresponds to a specific KEGG pathway, with the size of the point indicating the count of genes involved in the pathway and the color representing the p-value, where red indicates a more significant enrichment (lower p-value) and blue indicates a less significant enrichment (higher p-value). The KEGG pathway enrichment analyses for PDAC serum and HCC tumors reveal distinct biological signatures. For PDAC, the most significant result is the high enrichment of the complement and coagulation cascades pathway, with glycosaminoglycan degradation also being identified. This finding points to a systemic hypercoagulable and inflammatory state, a known clinical feature of the disease24. In contrast, the analysis of HCC tumors highlights a different set of processes, showing enrichment in pathways such as the PPAR signaling pathway, carbon metabolism, and several neurodegenerative disease pathways like Parkinson's disease and Alzheimer's disease. This suggests that HCC is characterized by a broad reprogramming of cellular and energy metabolism to support tumor growth, although the statistical significance of these pathways is lower than what is observed in the PDAC analysis.

Figure 8G presents a protein-protein interaction network of significantly upregulated proteins in PDAC serum. The network is generated by querying the STRING database for interactions with a confidence score of at least 400 and is visualized using the visNetwork library. Each node represents a specific protein, labeled with its UniProt identifier, and the connecting lines indicate interactions between these proteins. This protein-protein interaction network, identified from PDAC patient serum, reveals a core functional cluster alongside several isolated proteins relevant to cancer biology. A central interacting group is formed by Coagulation factor XI (P03951), Fibrinogen beta chain (P02675), and Plasma serine protease inhibitor (P05154), the core interaction group is composed of Coagulation factor XI (P03951), Fibrinogen beta chain (P02675), and Plasma serine protease inhibitor (P05154). Their robust connections are crucial as they play a role in coagulation and inflammation pathways, which are known to be dysregulated in the tumor microenvironment of PDAC25. This primary cluster also includes a connection to Immunoglobulin heavy variable 3-15 (A0A0B4J1V0). Additionally, the network displays several proteins as isolated nodes: Heparanase (Q9Y251), CD5 antigen-like (O43866), and Cysteine-rich secretory protein 3 (P54108). These individual proteins are noteworthy for their respective roles in tumor invasion and metastasis (Heparanase), immune regulation (CD5 antigen-like), and inflammation (Cysteine-rich secretory protein 3)26, indicating their potential importance in PDAC even without direct interactions in this visualized network.

FIGURE AND TABLE LEGENDS:

Figure 1: An example of keyword search in PRIDE.

Figure 2: Overview of study design.

Figure 3: Data analysis using DIA-NN. (A) An example of reference proteome (Homo sapiens, version 2024/12/12) in UniProt. (B)An example of DIA-NN settings while preparing for predicted spectral libraries. (C) An example of DIA-NN settings while running for raw formatted data.

Figure 4: FragPipe setup. (A) An example of FragPipe Config tab settings before analysis. (Make sure the main tools are all downloaded.) (B) An example of the FragPipe Workflow tab's regular settings. (C) An example of the FragPipe Database tab's regular settings. (The reference proteome can be easily downloaded here.)

Figure 5: FragPipe settings. (A) An example of the FragPipe MSFragger tab regular settings. (B) An example of the FragPipe Validation tab regular settings.

Figure 6: FragPipe runtime settings. (A) An example of FragPipe Quant (MS1) tab regular settings. (B) An example of FragPipe RUN tab regular settings.

Figure 7: Volcano plots. (A) Volcano plot of PDAC patient serum protein versus normal serum protein. (B) Volcano plot of HCC tumor protein versus paired tissue protein.

Figure 8: Heatmaps, GO enrichment, and KEGG pathway for protein-protein analysis. (A) Heatmap of normal serum protein versus PDAC patient serum. (B) Heatmap of paired tissue protein versus HCC tumor protein. (C) GO enrichment of significantly upregulated genes in PDAC serum. (D) GO enrichment of significantly upregulated genes in HCC tumors. (E) KEGG pathway enrichment of significant upregulated genes in PDAC serum. (F) KEGG pathway enrichment of significantly upregulated genes in HCC tumors. (G) Protein-protein interaction network of significantly upregulated proteins in PDAC serum

Supplementary File 1: DIA-NN output data analysis pipeline.

Supplementary File 2: FragPipe output data analysis pipeline.

Supplementary File 3: Environment requirements for data analysis in R

DISCUSSION:

The presented protocol illustrates proteomic data analysis by integrating free tools like DIA-NN and FragPipe. Success hinges on critical steps such as accurate precursor ion selection and mass accuracy settings for spectral library generation in DIA-NN (step 2.3)27, which are instrument-dependent and crucial for complete peptide identification. Similarly, meticulous database setup in FragPipe (step 3.4), including decoy generation and contaminant filtering, is vital for controlling FDR.

Modifications and troubleshooting often involve verifying software dependencies (e.g., .NET for MSFileReader, Java for FragPipe) and managing computational resources, especially for large datasets where increased memory for MSFragger or use of high-performance computing (HPC) for DIA-NN may be necessary. Community forums (e.g., GitHub) provide valuable, evolving troubleshooting support.

Key limitations include the protocol's restriction to label-free DIA and DDA workflows, excluding comprehensive PTM profiling28 or label-based quantification, although MSFragger has open search capabilities13. The reliance on R scripting for downstream analysis (step 4) can be a barrier for non-programmers, though tools like DIAgui29 and FragPipe-Analyst30 are emerging to address this. Furthermore, the discovery of entirely novel peptides is constrained by reliance on sequence databases, even in library-free DIA approaches10,28. Small sample sizes, as potentially indicated in representative results, also limit statistical power for robust biomarker discovery.

The significance with respect to existing methods lies in the enhanced speed, sensitivity, and data completeness offered by DIA-NN and FragPipe. MSFragger's fragment-ion indexing significantly accelerates peptide matching. Innovations like MSFragger-DDA+ improve the identification of co-fragmented peptides31. This, combined with DIA-NN's neural network-based processing, leads to deeper proteome coverage and more consistent quantification compared to many traditional DDA approaches, particularly for complex datasets and advanced acquisition methods like dia-PASEF32.

Future applications are promising, particularly in clinical proteomics for biomarker discovery in large cohorts31,33 and drug screening34, leveraging the high throughput and consistent quantification. The workflow's modularity allows adaptation for multi-omics integration35 and specialized areas like single-cell proteomics36 and PTM analysis37. Continued development of user-friendly GUIs and cloud-based pipelines38 will further enhance accessibility and manage computational demands39. This evolution supports a more open, reproducible scientific environment, accelerating basic and translational research.

ACKNOWLEDGMENTS:

This project is supported by Molemuse Biotech Studio.

DISCLOSURES:

F.Z. is the founder of Molemuse Biotech Studio.

REFERENCES:

1.	Pedrioli, P. G. A. et al. A common open representation of mass spectrometry data and its application to proteomics research. Nat Biotechnol. 22 (11), 1459–1466 (2004).

2.	Perez-Riverol, Y. et al. The PRIDE database at 20 years: 2025 update. Nucl Acids Res. 53 (D1), D543–D553 (2025).

3.	Deutsch, E. W. et al. Proteomics Standards Initiative at Twenty Years: Current Activities and Future Work. J Proteome Res. 22 (2), 287–301 (2023).

4.	Schubert, O. T., Röst, H. L., Collins, B. C., Rosenberger, G., Aebersold, R. Quantitative proteomics: challenges and opportunities in basic and applied research. Nat Protoc. 12 (7), 1289–1294 (2017).

5.	Wang, X., Shen, S., Rasam, S. S., Qu, J. MS1 ion current‐based quantitative proteomics: A promising solution for reliable analysis of large biological cohorts. Mass Spectro Rev. 38 (6), 461–482 (2019).

6.	Vaudel, M. MS2-Based Quantitation. Proteome Inform. 155–177 (2016).

7.	Meyer, J. G. Qualitative and Quantitative Shotgun Proteomics Data Analysis from Data-Dependent Acquisition Mass Spectrometry. Shotgun Proteomics. 2259, 297–308 (2021).

8.	Fröhlich, K. et al. Data-Independent Acquisition: A Milestone and Prospect in Clinical Mass Spectrometry–Based Proteomics. Mol Cell Proteomics. 23 (8), 100800 (2024).

9.	Zhang, F., Ge, W., Ruan, G., Cai, X., Guo, T. Data‐Independent Acquisition Mass Spectrometry‐Based Proteomics and Software Tools: A Glimpse in 2020. Proteomics. 20 (17–18), 1900276 (2020).

10.	Deutsch, E. W. et al. The ProteomeXchange consortium at 10 years: 2023 update. Nucl Acids Res. 51 (D1), D1539–D1548 (2023).

11.	Demichev, V., Messner, C. B., Vernardis, S. I., Lilley, K. S., Ralser, M. DIA-NN: neural networks and interference correction enable deep proteome coverage in high throughput. Nat Meth. 17 (1), 41–44 (2020).

12.	Zhang, F. et al. A Comparative Analysis of Data Analysis Tools for Data-Independent Acquisition Mass Spectrometry. Mol Cell Proteomics. 22 (9), 100623 (2023).

13.	Kong, A. T., Leprevost, F. V., Avtonomov, D. M., Mellacheruvu, D., Nesvizhskii, A. I. MSFragger: ultrafast and comprehensive peptide identification in mass spectrometry–based proteomics. Nat Meth. 14 (5), 513–520 (2017).

14.	Anderson, N. L., Anderson, N. G. The Human Plasma Proteome. Mol Cell Proteomics. 1 (11), 845–867 (2002).

15.	Kowal, J. et al. Proteomic comparison defines novel markers to characterize heterogeneous populations of extracellular vesicle subtypes. Proc Natl Acad Sci. 113 (8), E968-977 (2016).

16.	Cao, L. et al. Proteogenomic characterization of pancreatic ductal adenocarcinoma. Cell. 184 (19), 5031-5052.e26 (2021).

17.	Dong, L. et al. Proteogenomic characterization identifies clinically relevant subgroups of intrahepatic cholangiocarcinoma. Cancer Cell. 40 (1), 70-87.e15 (2022).

18.	Dubois, E., Galindo, A. N., Dayon, L., Cominetti, O. Comparison of normalization methods in clinical research applications of mass spectrometry-based proteomics. 2020 IEEE Conf Computat Intelligence Bioinfo Computat Biol. 1–10 (2020).

19.	Dressler, F. F., Brägelmann, J., Reischl, M., Perner, S. Normics: Proteomic Normalization by Variance and Data-Inherent Correlation Structure. Mol Cell Proteomics. 21 (9), 100269 (2022).

20.	Dimmer, E. C. et al. The Gene Ontology — Providing a Functional Role in Proteomic Studies. Proteomics. 8 (23–24), pmic.200800002 (2008).

21.	Lih, T. M. et al. Detection of Pancreatic Ductal Adenocarcinoma-Associated Proteins in Serum. Mol Cell Proteomics. 23 (1), 100687 (2024).

22.	Chinese Human Proteome Project (CNHPP) Consortium et al. Proteomics identifies new therapeutic targets of early-stage hepatocellular carcinoma. Nature. 567 (7747), 257–261 (2019).

23.	Kanehisa, M., Furumichi, M., Sato, Y., Matsuura, Y., Ishiguro-Watanabe, M. KEGG: biological systems database as a model of the real world. Nucl Acids Res. 53 (D1), D672–D677 (2025).

24.	Campello, E., Ilich, A., Simioni, P., Key, N. S. The relationship between pancreatic cancer and hypercoagulability: a comprehensive review on epidemiological and biological issues. Br J Cancer. 121 (5), 359–371 (2019).

25.	Fang, L., Xu, Q., Qian, J., Zhou, J. Y. Aberrant Factors of Fibrinolysis and Coagulation in Pancreatic Cancer. OncoTargets Ther. 14, 53–65 (2021).

26.	Vlodavsky, I., Elkin, M., Ilan, N. Impact of heparanase and the tumor microenvironment on cancer metastasis and angiogenesis: basic aspects and clinical applications. Rambam Maimonides Med J. 2 (1), rmmj.10019 (2011).

27.	Fröhlich, K. et al. Benchmarking of analysis strategies for data-independent acquisition proteomics using a large-scale dataset comprising inter-patient heterogeneity. Nat Comm. 13 (1), 2622 (2022).

28.	Polasky, D. A. et al. MSFragger-Labile: A Flexible Method to Improve Labile PTM Analysis in Proteomics. Mol Cell Proteomics. 22 (5), 100538 (2023).

29.	Gerault, M. A., Camoin, L., Granjeaud, S. DIAgui: a Shiny application to process the output from DIA-NN. Bioinfo Adv. 4 (1), vbae001 (2024).

30.	Hsiao, Y. et al. Analysis and Visualization of Quantitative Proteomics Data Using FragPipe-Analyst. J Proteome Res. 23 (10), 4303–4315 (2024).

31.	Yu, F., Deng, Y., Nesvizhskii, A.I. MSFragger-DDA+ enhances peptide identification sensitivity with full isolation window search. Nat Comm. 16 (1), 3329 (2025).

32.	Li, K., Teo, G. C., Yang, K. L., Yu, F., Nesvizhskii, A.I. diaTracer enables spectrum-centric analysis of diaPASEF proteomics data. Nat Comm. 16 (1), 95 (2025).

33.	Qiao, R., Li, H., Bian, H., Xin, L., Shan, B. De Novo sequencing-assisted homology search for DIA data analysis enables low abundance peptide variants discovery. Biorvix. 10.1101/2025.05.30.657054 (2025).

34.	Meissner, F., Geddes-McAlister, J., Mann, M., Bantscheff, M. The emerging role of mass spectrometry-based proteomics in drug discovery. Nat Rev Drug Disc. 21 (9), 637–654 (2022).

35.	Zheng, Y. et al. Multi-omics data integration using ratio-based quantitative profiling with Quartet reference materials. Nat Biotechnol. 42 (7), 1133–1149 (2024).

36.	Bubis, J. A. et al. Challenging the Astral mass analyzer to quantify up to 5,300 proteins per single cell at unseen accuracy to uncover cellular heterogeneity. Nat Meth. 22 (3), 510–519 (2025).

37.	Leutert, M., Entwisle, S. W., Villén, J. Decoding Post-Translational Modification Crosstalk With Proteomics. Mol Cell Proteomics. 20, 100129 (2021).

38.	Schneider, M. et al. A Scalable, Web-Based Platform for Proteomics Data Processing, Result Storage and Analysis.  Proteome Res. 24 (3), 1241–1249 (2025).

39.	Jalili, V. et al. The Galaxy platform for accessible, reproducible and collaborative biomedical analyses: 2020 update. Nucl Acids Res. 48 (W1), W395–W402 (2020).