# Submission ID #: 68708

# Scriptwriter Name: Sulakshana Karkala

# Project Page Link: https://review.jove.com/account/file-uploader?src=20955988

# Title: Microfluidics-Based High-Throughput Circulating Tumor Cell Sorting and Single-Cell Sequencing Technology

# Authors and Affiliations:

# Zheyu Liu1*, Ye Tian3*, Zian Zhang1, Lingling Wu2, Liang Dong1, Wei Xue1

# 1Department of Urology, Ren Ji Hospital, Shanghai Jiao Tong University School of Medicine

# 2Institute of Molecular Medicine, Ren Ji Hospital, Shanghai Jiao Tong University School of Medicine

# 3Shanghai Jiao Tong University School of Medicine

# *These authors contributed equally

# Corresponding Authors:

Liang Dong                                   drdongliang@126.com

Wei Xue                                        xuewei@renji.com

# Email Addresses for All Authors:

Zheyu Liu                                     lzy02626@126.com

Ye Tian                                         late_fall@sjtu.edu.cn

Zian Zhang                                  zhang-zian@outlook.com

Lingling Wu                                 llwu@shsmu.edu.cn

Liang Dong                                   drdongliang@126.com

Wei Xue                                        xuewei@renji.com

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No.

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No.

3. Filming location: Will the filming need to take place in multiple locations?   No.

Current Protocol Length

Number of Steps:  26

Number of Shots:  56

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

REQUIRED: What is the scope of your research? What questions are you trying to answer?

  - Zheyu Liu: We focus on developing high-performance CTC isolation and analysis methods to advance precision oncology through liquid biopsy.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:2.6

What are the current experimental challenges?

  - Zheyu Liu: Current commercial CTC isolation methods have low throughput and efficiency. They are also incompatible with downstream sequencing platforms, which limits biological information recovery.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What research gap are you addressing with your protocol?

  - Zheyu Liu: This study uses a microfluidic CTC isolation platform to greatly improve the detection rates. We also apply a rare-cell single-cell sequencing chip for deeper liquid biopsy analysis.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:3.8

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

- Preparation of Immunomagnetic Beads and Initial HB-Chip Operation for Tumor Cell Isolation

Demonstrator: Zheyu Liu

  - To begin, pipette 25 microliters of washed and resuspended streptavidin-modified magnetic beads [1] into a tube containing 1 microgram of biotinylated EpCAM (Epp-Cam) antibody [2-TXT]. Incubate the mixture at room temperature while rotating at 20 revolutions per minute for 40 minutes to prepare the immunomagnetic beads [3].

    - WIDE: Talent pipetting streptavidin-modified magnetic beads into a microcentrifuge tube.

    - Talent pipetting EpCAM antibody into the tube. TXT: EpCAM: Epithelial Cell Adhesion Molecule

    - Talent placing the tube on a rotator set to 20 revolutions per minute.

  - Using a magnetic rack, separate the beads from the supernatant [1]. After removing the supernatant, resuspend the beads in 25 microliters of isolation buffer [2-TXT].

    - Talent placing the tube on a magnetic rack and allowing beads to collect at the side.

    - Talent adding 25 µL isolation buffer to the bead pellet and gently resuspending it. TXT: Isolation Buffer: 1% BSA, 2 mM EDTA, D-PBS

  - Pipette 20 microliters of the magnetic bead suspension within 1 to 2 seconds, ensuring no air bubbles are introduced [1]. Immediately place the chip vertically on a magnet and allow the beads to settle for 5 minutes without disturbance [2].

    - Talent drawing 20 microliters of bead suspension into a pipette quickly, ensuring no bubbles form.

    - Talent positioning the chip vertically against a magnet.

  - Collect 4 milliliters of the blood sample into a syringe, ensuring air bubbles are removed [1]. Seal the chip’s inlet and outlet with liquid to eliminate air bubbles [2]. Then insert the sample inlet and outlet tubes into the chip [3].

    - Talent drawing blood into a syringe while avoiding air bubbles.

    - Talent sealing the chip’s inlet and outlet with liquid.

    - Talent connecting inlet and outlet tubing to the chip.

  - Using a syringe pump, inject the sample at a flow rate of 1.5 milliliters per hour [1]. After loading the sample, inject 60 microliters of DPBS into the HB(H-B) -chip at a flow rate of 0.2 milliliters per hour to wash off unbound cells [2].

    - Talent setting the syringe pump and starting the injection.

    - Talent operating syringe pump to inject Dulbecco’s phosphate-buffered saline at the set flow rate.

  - Remove the magnet [1] and manually inject 1.5 milliliters of 5 percent BSA to wash the chip and release captured tumor cells [2].

    - Talent removing magnet from the chip.

    - Talent manually injecting bovine serum albumin solution into the chip.

- Microfluidic-based Circulating Tumor Cells (CTC) purification

  - Prepare a 10% MPTS (M-P-T-S) solution in ethanol [1-TXT]. Immediately introduce 20 microliters of the solution into the HB-chip and incubate at room temperature for 1 hour [2].

    - Talent measuring and preparing (3-Mercaptopropyl) trimethoxysilane solution in ethanol. TXT: MPTS: (3-Mercaptopropyl) trimethoxysilane

    - Talent pipetting 20 microliters of the solution into the HB-chip inlet.

  - Rinse the HB-chip once with anhydrous ethanol [1]. Then dry it at 100 degrees Celsius for 1 hour [2].

    - Talent using a pipette to rinse the chip with anhydrous ethanol.

    - Talent placing the chip in a drying oven set to 100 degrees Celsius.

  - Next, prepare GMBS (G-M-B-S) solution in ethanol at a concentration of 0.5 milligram per milliliter [1-TXT]. After cooling the chip to 37 degrees Celsius, introduce the GMBS solution and incubate [2-TXT].

    - Talent preparing GMBS solution in a microcentrifuge tube. TXT: GMBS: N-𝛾-maleimidobutyryl-oxysuccinimide

    - Talent pipetting GMBS solution into the HB-chip inlet. TXT: Incubation: RT, 30 min

  - Rinse the HB-chip two times with double-distilled water, followed by two rinses with DPBS [1]. Immediately add 15 micrograms per milliliter of streptavidin into the HB-chip and incubate at room temperature for 1 hour or overnight at 4 degrees Celsius [2].

    - Talent the chip twice with double-distilled water and DPBS.
AUTHORS: Please perform one representative action

    - Talent pipetting streptavidin solution into the HB-chip inlet.

  - After incubation, rinse the HB-chip two times with DPBS saline [1].

    - Talent performing two Dulbecco’s phosphate-buffered saline rinses on the chip.

  - Now, prepare the CD45 (C-D-Forty-Five) antibody buffer by adding 0.2 percent BSA (B-S-A) and 20 micrograms per milliliter biotinylated CD45 antibody to DPBS, and adjust to the final volume [1]. Inject 20 microliters of the CD45 antibody buffer into the HB-chip for negative selection of white blood cells [2]. After a 1-hour incubation at room temperature, rinse the chip with DPBS [3].

    - Talent preparing CD45 antibody buffer in a microcentrifuge tube.

    - Talent pipetting CD45 antibody buffer into the HB-chip inlet.

    - Talent rinsing the HB-chip with Dulbecco’s phosphate-buffered saline.

  - Add a blocking solution containing 3 percent BSA and 0.05 percent Tween-20 into the HB-chip [1-TXT].

    - Talent pipetting blocking solution into HB-chip inlet. TXT: Incubate and rinse with DPBS before sample loading

  - Aspirate the prepared sample into a syringe, ensuring no air bubbles are present [1]. Then seal the chip’s inlet and outlet with liquid [2], and connect inlet and outlet tubes to the chip [3].

    - Talent aspirating sample into syringe while avoiding air bubbles.

    - Talent sealing chip inlet and outlet with liquid.

    - Talent attaching inlet and outlet tubes to chip.

  - Using a syringe pump, inject the sample at a flow rate of 0.6 milliliters per hour [1]. Collect purified tumor cells from the chip outlet for counting and single-cell sequencing [2].

    - Talent setting and starting syringe pump for injection.

    - Talent collecting tumor cells from chip outlet into a microcentrifuge tube.

- Circulating Tumor Cell Capture, Barcoded Bead Loading, and mRNA Preparation Using the HB-Chip

  - Pipette 200 microliters of 0.5 percent F-68 (F-Sixty-Eight) solution prepared in DPBS, into the chip inlet [1]. Perform water bath sonication while holding the chip [2]. When bubbles are visible across the microporous region, continue sonication for 30 seconds to remove bubbles from the dual wells [3].

    - Talent pipetting F-68 solution in Dulbecco’s phosphate-buffered saline into chip inlet.

    - Talent submerging chip in water bath sonicator.

    - Close-up of bubbles across microporous region during sonication.

  - Next, inject 200 microliters of tumor cell suspension into the chip containing 60,000 dual wells [1-TXT].

    - Talent pipetting tumor cell suspension into chip inlet. TXT: Shake at 10 rpm/min, 5 min

  - Gently pipette the cell suspension in the chip up and down twice [1], then place on a decolorizing shaker again at for 5 minutes to resuspend uncaptured cells and allow them to settle again [2]. Now add 200 microliters of DPBS and 0.5 percent F-68 solution through inlet and aspirate from outlet [3-TXT].

    - Talent gently pipetting suspension inside chip.

    - Talent placing chip back on decolorizing shaker.

    - Talent adding wash solution through inlet and aspirating from outlet. TXT: Repeat for a total of 3 washes

  - After resuspending the barcoded bead suspension. Immediately inject 200 microliters of suspension into the chip inlet [1] and shake at 10 revolutions per minute for 20 seconds [2].

    - Talent injecting resuspended beads into chip inlet.

    - Talent placing the chip on a shaker set to 10 revolutions per minute.

  - Gently pipette bead suspension twice before placing it back on the shaker [1-TXT]

    - Talent gently pipetting bead suspension in chip. TXT: Repeat once

  - Now withdraw barcoded bead suspension from outlet [1]. Then pipette 200 microliters of 20x Tris-EDTA (Tris-E-D-T-A) and 50 millimolar dithiothreitol solution [2]. Withdraw liquid from outlet [3-TXT].

    - Talent withdrawing bead suspension from outlet.

    - Talent adding Tris-EDTA and dithiothreitol solution into chip.

    - Talent withdrawing solution from outlet. TXT: Repeat wash for a total of 4 washes

  - For cell lysis and mRNA (M-R-N-A) capture, slowly add 200 microliters of cell lysis buffer into the chip inlet [1]. Immediately add 200 microliters of mineral oil to seal the dual wells [2].

    - Talent pipetting lysis buffer into chip inlet.

    - Talent adding mineral oil immediately after lysis buffer.

  - Remove solution flowing from chip outlet into waste reservoir [1]. Then place the chip horizontally and let stand at room temperature for 5 minutes [2].

    - Talent removing waste solution from outlet.

    - Close-up of chip resting horizontally on bench.

  - Slowly add 200 microliters of 6x saline-sodium citrate solution into inlet [1]. Remove the waste liquid and aspirate the remaining solution from chip outlet [2].

    - Talent adding saline-sodium citrate solution into chip inlet.

    - Talent removing waste solution from outlet and pipetting out the remaining solution from chip outlet.

  - Slowly add 200 microliters of 6x saline-sodium citrate to fill chip [1]. Hold a magnet near chip surface and move it slowly from inlet to outlet to gather barcoded beads [2]. Quickly aspirate solution and beads into a centrifuge tube containing 6x saline-sodium citrate [3].

    - Talent filling chip with saline-sodium citrate.

    - Close-up of magnet being moved along chip surface.

    - Talent aspirating bead-containing solution into centrifuge tube.

  - Wash the barcoded beads three times with 200 microliters of 6x saline-sodium citrate [1], followed by once with reverse transcription buffer [2].

    - Talent washing beads three times with saline-sodium citrate.

    - Talent washing beads once with reverse transcription buffer.

# Results

- Results

  - A uniform distribution of immunomagnetic beads was observed under a magnetic field [1], while minimal residual beads after magnetic removal confirmed successful release [2].

    - LAB MEDIA: Figure 2C. Video editor: please highlight the top image

    - LAB MEDIA: Figure 2C. Video editor: please highlight the bottom image.

  - The CTC (C-T-C) isolation chip efficiently captured tumor cells from both 1 milliliter and 10 milliliter samples [1-TXT]. The addition of the purification chip further increased tumor cell purity [2].

    - LAB MEDIA: Figure 2D. 
TXT: CTC: Circulating Tumor Cell
Video editor: Please highlight the 1mL and 10 mL bars

    - LAB MEDIA: Figure 2E. Video editor: Highlight the bars in the “Both” group

  - In peripheral blood samples spiked with minimal LNCaP (Lin-Cap) cells [1], the CTC sorting system maintained high capture efficiency and purity [2].

    - LAB MEDIA: Figure 2F. Video editor: Highlight the green fluorescent cells in the “LNCaP-Calcein AM” panel.

    - LAB MEDIA: Figure 2G. Video editor: Highlight both the red bars and blue dots in the “1 mL” and “10 mL” groups.

  - The single-cell barcoding chip achieved an 85.6% cell [1] and 95.7% barcoded bead occupancy ratio [2], resulting in a pairing rate of 81.9% [3].

    - LAB MEDIA: Figure 3D (left). Video editor: Please highlight the red bar labeled “Cell”

    - LAB MEDIA: Figure 3D (left). Video editor: Highlight the blue bar labeled “Bead”

    - LAB MEDIA: Figure 3D (left). Video editor: Highlight the green bar labeled “Pairing”

  - Increasing the number of loaded cells improved the microwell occupancy ratio [1] without reducing capture efficiency [2].

    - LAB MEDIA: Figure 3C. Video editor: Highlight the blue line increasing toward the right side of the graph.

    - LAB MEDIA: Figure 3C. Video editor: Highlight the flat red line across the graph.

  - The integrated CTC isolation and single-cell sequencing workflow produced highly pure tumor cells [1] and accurately retained the original tumor cell ratios [2]. t-SNE (T-S-N-E) analysis distinctly separated PC3(P-C-Three), LNCaP, and Jurkat cells into three clusters, each characterized by unique marker expression [3].

    - LAB MEDIA: Figure 4B. Video editor: Highlight the large yellow and blue sections in the “Purified” bar.

    - LAB MEDIA: Figure 4B. Video editor: Highlight the yellow and blue in “Purified” and “Output” bars.

    - LAB MEDIA: Figure 4A. Video editor: Highlight the three distinct color clusters.

  - The Jurkat cell cluster was identified by strong expression of TRBC1(T-R-B-C-One), IGLL1 (Eye-G-L-L-One), CD1E(C-D-One-E), and CD3D (C-D-Three-D0 [1], confirmed by pathway enrichment for T cell activation [2].

    - LAB MEDIA: Figure 4D. Video editor: Highlight the TRBC1, CD3D and CD1E marker plots

    - LAB MEDIA: Figure 4E. Video editor: Highlight the “alpha-beta T cell activation” and “T cell differentiation” boxes.

  - PC3 and LNCaP clusters were distinguished by differentially expressed genes, with PC3 showing high microseminoprotein expression [1] and LNCaP expressing NEDD4 (Ned-Four) [2].

    - LAB MEDIA: Figure 4F–G. Video editor: Highlight the the MSMP plot in 4F

    - LAB MEDIA: Figure 4F–G. Video editor: Highlight the NEDD4 plot in 4F