# Submission ID #: 68745

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=20967273

# Title: Collecting and Processing Drone-Based Remotely Sensed Data for Use in Forest Recovery Monitoring

# Authors and Affiliations:

# Dmytro Movchan, Charumitha Selvaraj, Zhouxin Xi, Angeline Van Dongen, Dani Degenhardt

# Natural Resources Canada, Canadian Forest Service, Northern Forestry Centre

# Corresponding Authors:

# Dani Degenhardt					dani.degenhardt@nrcan-rncan.gc.ca

# Email Addresses for All Authors:

Dmytro Movchan					dmytro.movchan@nrcan-rncan.gc.ca

Charumitha Selvaraj					charumitha.selvaraj@nrcan-rncan.gc.ca

Zhouxin Xi						zhouxin.xi@alumni.uleth.ca

Angeline Van Dongen					angeline.vandongen@nrcan-rncan.gc.ca

# Dani Degenhardt					dani.degenhardt@nrcan-rncan.gc.ca

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes, all done

3. Filming location: Will the filming need to take place in multiple locations?   Yes

If Yes, how far apart are the locations? Drone flight outside the building and data processing/interview inside the building. Distance between locations is ~100 m

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. No

Current Protocol Length

Number of Steps: 20

Number of Shots: 55 (50 SC)

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

Videographer’s NOTE: Camera: Sony fs5 – 1920x1080,29.97 fps, s-log 2

Audio (interviews) – Channel 1 – On board shot gun mic (back-up)

Channel 2 – Lav Mic on Authors

Clip0001 – Grey Card/White Card for interview space

*The interview room has a noticeable hum from HVAC system, will want to remove with post audio editing tools.

Clip0002 – WS of 1.1.1, 1.3.1, 1.4.1

Clip0003 – MS of 1.1.1, 1.3.1, 1.4.1

Clip0004 – Head shot of Angeline Van Dongen

Clip0005 – WS 1.2.1, 1.5.1

Clip0006 – MS 1.2.1, 1.5.1

Clip0007 – Head shot of Dmytro Movchan

INTRODUCTION:

  - Angeline Van Dongen: Our research aims to facilitate the use of drone-based LiDAR in forest recovery assessment through the development of a user-friendly protocol.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll:2.2.1

  - Dmytro Movchan: Recent developments that make this work possible include advancement in LiDAR sensor technology and integration with deep learning models.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.3.1

CONCLUSION:

  - Angeline Van Dongen: This protocol aims to address the research gap of individual tree detection and segmentation for young trees in complex forested environments.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.6.1

  - Angeline Van Dongen: This protocol offers a practical, effective, user-friendly, and versatile method for extracting individual tree metrics from LiDAR data.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 4.2.1

  - Dmytro Movchan: Our protocol will advance forest recovery monitoring and is particularly useful for assessing reclaimed wellsites, reducing time and costs of doing plot surveys.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 5.1.1

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

- Data Collection Using RPAS and Processing

Demonstrator: Dmytro Movchan

  - To begin, check the RPAS (R-P-A-S) equipment and update the firmware if necessary [1-TXT]. Set up the RTK Base Station in a clear area that is away from obstacles and the tree line. Wait till the base station is completely powered up before starting the drone flight [2]. Create a flight file for each mission and upload them to the remote controller [3-TXT].  Videographer’s NOTE: VO added for the extra shot

    - WIDE: Talent inspecting the equipment. TXT: RPAS: Remotely Piloted Aircraft Systems Videographer’s NOTE: Clip0011 – 2.1.1 – Take 2 – CU Shot, an option for edit

Added shot: Clip0012 – 2.1.1a – Shot of the RTK Base Station Power fully powered up

    - Show the remote controller, with the Upload Flight File option being selected. TXT: Prepare a safety plan and emergency procedure document

  - Now, conduct the flight missions to collect LiDAR (lie-daar) and multispectral data over the area of interest [1-TXT].

    - Drone flying over the field area while collecting LiDAR and multispectral data. TXT: LiDAR: Light Detection and Ranging

  - Set up a GNSS base station over the RPAS’s base station location [1] and run the base station to obtain accurate coordinates for Precise Point Positioning correction [2].

    - Talent pointing to the GNSS base station at the designated site. Shot of GNSS base station powered on and recording location data.

  - Use a card reader to transfer the collected LiDAR and multispectral data from the sensors to a workstation [1].

    - Talent inserting the memory card into a card reader and transferring files to a computer workstation. Videographer’s NOTE: Clip0009 – 2.4.1 Take 2 – CU shots options for edit

  - Use photogrammetry software to preprocess the multispectral data [1] and perform geometric and radiometric corrections [2]. Generate a multispectral orthomosaic [3].

    - SCREEN:  2.5.1.mp4 00:01-00:03 and 00:45-00:49 and 02:22-2:25 and 02:58-3:10 Video editor: Please try to include all the timestamps mentioned and speed up if required

    - SCREEN: 2.5.2.1.mp4 03:35-03:40 and 2.5.2.2.mp4 02:04-2:06 Video editor: Please try to include all the timestamps mentioned and speed up if required

    - SCREEN: . 2.5.3.mp4 0:42-00:48 and 01:16-01:19 and 01:34-01:38 Video editor: Please try to include all the timestamps mentioned and speed up if required

  - Use appropriate software to reconstruct the raw LiDAR data into a point cloud file [1].

    - SCREEN:  2.6.1.mp4 0:01-00:04 and 00:30-00:44

  - Use a geospatial tool to align the LiDAR point cloud with the multispectral orthomosaic [1]. Run a Python script to merge the aligned LiDAR point cloud with the multispectral orthomosaic to generate a multispectral point cloud [2].

    - SCREEN:  2.7.1.mp4 00:06-00:09 and 00:16-00:19 and 00:26-00:29 and 00:56-01:09  Video editor: Please try to include all the timestamps mentioned and speed up if required

    - SCREEN: 2.7.2.mp4 00:20-00:25 and  01:04-01:07 and  01:30-01:34 Video editor: Please try to include all the timestamps mentioned and speed up if required

- Using the TreeAIBox Plugin for Individual Tree Segmentation

  - Download CloudCompare (cloud compare), the three-dimensional point cloud processing software, and install it [1]. Then download the TreeAIBox_Plugin_Installer_v1.0.exe (tree-A-I-box plugin-installer-version 1) from GitHub [2], run the installer, and follow the on-screen prompts [3].

    - SCREEN:  3.1.1.mp4

    - SCREEN:  3.1.2.mp4 00:00 – 00:08

    - SCREEN:  3.1.3.mp4 00:09-00:12 and 00:59- 01:06

  - Open CloudCompare from the desktop icon or by selecting Start followed by All Programs and CloudCompare [1]. Load the point cloud file by using Open one or several files [2-TXT] and click Apply [3].

    - SCREEN: 3.2.1.mp4

    - SCREEN:  3.2.2.mp4  TXT: Point cloud file: .las or .laz

    - SCREEN: 3.2.3.mp4

  - If the point coordinates are large, accept the prompt to apply a global shift or scale [1]. Select Input, which reads metadata from the file [2], and click Yes so the point cloud appears in the canvas [3].

    - SCREEN:  3.3.1.mp4 00:00-00:01

    - SCREEN: 3.3.1.mp4 00:01-00:03

    - SCREEN: 3.3.1.mp4 00:03-00:06

  - Open the Python plugin toolbar [1]. Expand the Script Register dropdown [2] and click TreeAIBox to open the plugin graphical user interface [3].

    - SCREEN:  3.4.1.mp4 00:00-00:03

    - SCREEN:  3.4.1.mp4 00:03-00:04

    - SCREEN:  3.4.1.mp4 00:04-00:10

  - Ensure the Use GPU (CUDA) (use G-P-U) checkbox is selected if a Compute Unified Device Architecture supported graphics processing unit is available [1]. From the top panel, select TreeFiltering [2] and choose ALS if tree stems are not visible in the RPAS data [3].

    - SCREEN:  3.5.1.mp4 00:03-00:09

    - SCREEN:  3.5.1.mp4 00:09-00:12

    - SCREEN:  3.5.1.mp4 00:13-00:18

  - Now, clear the Tile size checkbox [1]. From the Predefined Models dropdown, select “treefiltering_als_esegformer3D_128_15cm(GPU3GB)” (tree filtering A-L-S -E-seg-former) [2]. If using this model for the first time, click Download and confirm the popup showing the local path [3].

    - SCREEN:  3.6.1.mp4 00:03:-0:04

    - SCREEN:  3.6.1.mp4 00:05:-0:07

    - SCREEN: . 3.6.1.mp4 00:07-00:11

  - Select the point cloud in the canvas so it is highlighted with a bounding box [1]. In the TreeFilter panel, click Apply [2]. Confirm a new scalar field named treefilter is created with value 2 for tree points in red and value 1 for other points in blue before proceeding [3].

    - SCREEN:  3.7.1.mp4 00:00-00:05

    - SCREEN: . 3.7.1.mp4 00:05-00:10

    - SCREEN:  3.7.3.mp4

- Detecting Treetops and Segmenting Tree Crowns

  - From the TreeAIBox (tree-A-I-box) top panel, select TreeisoNet (Tree-iso-net) [1]. Enable Reclamation, ALS (stem implicit) (A-L-S stem implicit), and TreeLoc (tree loke) [2]. From the dropdown, choose the required pretrained model [3]. Ensure the point cloud in the canvas is selected, then click Apply [4].

    - SCREEN:  4.1.1.mp4 00:00-00:04

    - SCREEN: 4.1.1.mp4 00:04-00:19

    - SCREEN: 4.1.1.mp4 00:21-00:26

    - SCREEN:  4.1.1.mp4 00:27-00:35

  - After processing, confirm that a new item named treetops appears under the original point cloud in the DB Tree window [1]. Select this item and increase the point size, for example to 16, for improved visibility [2] and verify that treetop positions appear as white dots in the canvas [3].

    - SCREEN: . 4.2.1mp4 00:00-00:04

    - SCREEN: . 4.2.1mp4 00:04-00:11

    - SCREEN:  4.2.1mp4 00:11-00:15

  - For segmenting tree crowns, re-select the tree point cloud item [1]. From the TreeAIBox top panel, select TreeisoNet and enable TreeOff (tree off) [2]. Download the required pretrained model [3] and then click Apply to run the model [4].

    - SCREEN: 4.3.1.mp4 00:00-00:03

    - SCREEN:  4.3.1.mp4 00:15-00:20

    - SCREEN: 4.3.1.mp4 00:20-00:26

    - SCREEN: . 4.3.1.mp4 00:26-00:32

  - Next, confirm that a new scalar field named treeoff is created [1]. Verify that each tree is assigned a unique identifier, with points from the same tree sharing the same identifier [2]. Optionally, reset the treetop point size to default to reduce visual clutter [3].

    - SCREEN:  4.4.1.mp4 00:00-00:08

    - SCREEN:  4.4.1.mp4 00:11-00:18

    - SCREEN: 4.4.1.mp4 00:22-00:31

  - To improve visual contrast and randomize tree colors by identifier, clone the original point cloud to preserve the data by selecting Edit and Clone [1]. Then, go to Edit followed by Scalar Fields and Convert to Random RGB [2]. Enter a large value to ensure enough discrete colors and click OK [3]. View the point cloud displaying trees in random colors [4].

    - SCREEN: 4.5.1.mp4 00:00-00:07

    - SCREEN: . 4.5.1.mp4 00:08-00:16

    - SCREEN: 4.5.1.mp4 00:17-00:19

    - SCREEN:  4.5.1.mp4 00:20-00:24

  - Finally, from the TreeAIBox top panel, select TreeisoNet and click Export stats to export segmentation results [1]. Then click Open Output Path to view the exported file in the results folder [2]. Confirm the output is a comma-separated values file containing tree identifiers, coordinates, tree height, and crown area [3].

    - SCREEN:  4.6.1.mp4

    - SCREEN:  4.6.2.mp4

    - SCREEN:  4.6.3.mp4

# Results

- Results

  - The TreeAIBox plugin successfully filtered tree points and delineated individual trees across all three sites and a total of 2,755 individual trees were detected [1].

    - LAB MEDIA: Figure 17.

  - The model identified 1,706 trees on Site 1 [1], 882 trees on Site 2 [2], and 167 trees on Site 3 [3].

    - LAB MEDIA: Figure 17. Video editor: Highlight A.

    - LAB MEDIA: Figure 17. Video editor: Highlight “B.”

    - LAB MEDIA: Figure 17. Video editor: Highlight  “C”

  - The detection rate for reference trees varied among the sites, with 100% detected for Site 2 [1], 95% for Site 1 [2], and 21% for Site 3 [3].

    - LAB MEDIA: Table 1. Video editor: Highlight the cell or row showing “100%” for Site 2.

    - LAB MEDIA: Table 1. Video editor: Highlight the cell or row showing “95%” for Site 1.

    - LAB MEDIA: Table 1. Video editor: Highlight the cell or row showing “21%” for Site 3.

  - The model achieved a 100% detection rate for all reference trees taller than 1 meter [1].

    - LAB MEDIA: Table 2. Video editor: Highlight the bottom three rows “1 -1.5 m”, “1.5-2m” and “>2m” showing a 100% detection rat.

  - Detection performance decreased for shorter trees, with only 45% of trees between 0.5 to 1 meter detected [1], and none of the trees below 0.5 meter identified [2].

    - LAB MEDIA: Table 2. Video editor: Highlight the row “0.5-1m”.

    - LAB MEDIA: Table 2. Video editor: Highlight the row “0-0.5m”.

  RPAS (Remotely Piloted Aircraft Systems)
Pronunciation link: https://www.icao.int/frequently-used-terms — search for “remotely piloted aircraft system (RPAS)”. ICAO+1
IPA: /ˌɑːr-pi-ˈeɪ-ɛs/
Phonetic Spelling: AR-pee-AY-ess

  RTK Base Station
Pronunciation link: No confirmed link found for “RTK (Real Time Kinematic)” as a standalone term in this context.
IPA: /ˌɑr-ti-ˈkeɪ beɪs ˈsteɪʃən/
Phonetic Spelling: AR-tee-KAY base STAY-shun

  LiDAR (Light Detection and Ranging)
Pronunciation link: https://www.howtopronounce.com/lidar — typical pronunciation given.
IPA: /ˈlaɪdɑːr/
Phonetic Spelling: LY-dar

  multispectral
Pronunciation link: https://www.merriam-webster.com/dictionary/multispectral
IPA: /ˌmʌltiˈspɛktrəl/
Phonetic Spelling: MUL-tee-SPEK-truhl

  GNSS (Global Navigation Satellite System)
Pronunciation link: No confirmed link found for the acronym “GNSS” specifically.
IPA: /ˌdʒiː-ɛn-ˈɛs-ɛs/
Phonetic Spelling: GEE-EN-ESS-ESS

  photogrammetry
Pronunciation link: https://www.merriam-webster.com/dictionary/photogrammetry
IPA: /ˌfoʊtoʊɡrəˈmɛtri/
Phonetic Spelling: FOH-toh-gruh-MEH-tree

  orthomosaic
Pronunciation link: No confirmed link found for “orthomosaic” in Merriam-Webster.
IPA: /ˌɔːrθoʊməˈzeɪɪk/
Phonetic Spelling: OR-tho-moh-ZAY-ick

  point cloud
Pronunciation link: No specific entry; use standard pronunciation for “point” and “cloud”.
IPA: /pɔɪnt klaʊd/
Phonetic Spelling: POINT cloud

  segmentation
Pronunciation link: https://www.merriam-webster.com/dictionary/segmentation
IPA: /ˌsɛɡmɛnˈteɪʃən/
Phonetic Spelling: seg-men-TAY-shun

  plugin
Pronunciation link: https://www.merriam-webster.com/dictionary/plugin
IPA: /ˈplʌɡˌɪn/
Phonetic Spelling: PLUG-in