# Submission ID #: 68748

# Scriptwriter Name: Sulakshana Karkala

# Project Page Link: https://review.jove.com/account/file-uploader?src=20968188

# Title: Use of Cerebral Open-Flow Microperfusion for the Longitudinal Collection of Interstitial Fluid in an Animal Model of Glioblastoma

# Authors and Affiliations:

# Ian E. Olson, Rafal Chojak, Umme H. Faisal, Jillyn Turunen, Irene Lazanyi, Jason Miska, Atique U. Ahmed

# Department of Neurological Surgery, Feinberg School of Medicine, Northwestern University

# Corresponding Authors:

Atique U. Ahmed		(atique.ahmed@northwestern.edu)

# Email Addresses for All Authors:

Ian E. Olson			(ian.olson1@northwestern.edu)

Rafal Chojak			(rafal.chojak@northwestern.edu)

Umme H. Faisal		(faisal.ummeh@northwestern.edu)

Jillyn Turunen			(jillyn.turunen@northwestern.edu)

Irene Lazanyi			(irenelazanyi2027@u.northwestern.edu)

Jason Miska			(jason.miska@northwestern.edu)

Atique U. Ahmed		(atique.ahmed@northwestern.edu)

## Author Questionnaire

- Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  Yes.

If your protocol involves microscopy but you are not able to record movies/images with your microscope camera, JoVE will need to use our scope kit.

If your microscope does not have a camera port, the scope kit will be attached to one of the eyepieces and you will have to perform the procedure using one eye.

SCOPE: 2.3., 2.4., 2.5., 2.6., 2.7., 2.9.

Videographer: Please film the SCOPE steps with a SCOPE kit

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Filming location: Will the filming need to take place in multiple locations?   No

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. No

Current Protocol Length

Number of Steps: 24

Number of Shots: 54

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

INTRODUCTION:

What is the scope of your research? What questions are you trying to answer?

  - Umme Faisal: We are characterizing soluble signaling components within the brain tumor interstitial fluid that contribute to disease progression and treatment response.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What are the current experimental challenges?

  - Ian E. Olson: Evaluating absolute solute concentrations within the brain tumor interstitium remains an investigational challenge, requiring the use of sophisticated experimental methods.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

CONCLUSION:

What research gap are you addressing with your protocol?

  - Jason Miska: Relative nutrient availability within the brain tumor microenvironment remains poorly understood. This protocol is being used to address this gap.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

How will your findings advance research in your field?

  - Jason Miska: Characterizing nutrient availability and signaling within the tumor microenvironment can elucidate mechanisms of glioma progression and adaptive resistance to treatment.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:5.2

Videographer: Obtain headshots for all authors available at the filming location.

Ethics Title Card

This research has been approved by the Institutional Animal Care and Use Committee (IACUC) at Northwestern University

# Protocol

- Stereotaxic Cerebral Open-Flow Microperfusion (cOFM) Guide Installation and Intracranial Cell Infusion Procedure

Demonstrator: Ian E. Olson

  - To begin, with a sterile scalpel, make a sagittal midline incision along the surface of the skull, starting between the eyes and extending caudally for 8 to 10 millimeters [1]. Then use sterile forcepsto laterally retract the skin from the midline incision [2]. Ensure both Bregma and Lambda are visible, and extend the incision if necessary [3].

    - WIDE: Talent making a midline scalp incision using a sterile scalpel.

    - Talent retracting the skin with sterile forceps and cotton-tipped applicators.

    - Close-up of the exposed skull with Bregma and Lambda becoming visible.

  - Next, with a sterile scalpel,incise the periosteum and retract the tissue laterally with forceps [1]. Apply 1% to 3% hydrogen peroxide with a sterile cotton-tipped applicator to clean the skull surface [2].

    - Talent making a periosteum incision and retracting it with forceps.

    - Talent cleaning the skull surface using a hydrogen peroxide-soaked cotton applicator.

  - Now use a 0.3 to 1.0 milliliter insulin syringe to apply 20 to 40 microliters of self-etching dental adhesive evenly onto the skull surface [1]. Then expose it to  a a dental curing light to etch the adhesive [2]. Once cured, wipe away any excess adhesive with sterile cotton-tipped applicators and saline [3].
Videographer: please film all shots labelled SCOPE with a SCOPE kit

    - SCOPE: Talent applying dental adhesive using an insulin syringe.

    - SCOPE: Talent holding a curing light over the adhesive area.

    - SCOPE: Talent wiping excess adhesive with cotton-tipped applicators and saline.

  - Reposition the drill bit to the cOFM (C-O-F-M) guide coordinate, set the drill to 15,000 revolutions per minute, and drill a burr hole using the DV micromanipulator dial [1]. Frequently monitor skull thickness during drilling and flush with physiological saline to remove debris and reduce heat [2].

    - SCOPE: Talent drilling burr hole with micromanipulator dial. TXT: cOFM: cerebral Open-Flow Microperfusion

    - SCOPE: Talent flushing the skull with saline while monitoring skull thickness.

  - Repeat the drilling procedure at each anchor screw coordinate [1-TXT].

    - SCOPE: Talent repositioning the drill and creating additional burr holes for anchor screws. TXT: Drill co-ordinates: ML = 2.5, AP = -2.5, and ML = -2.5, AP = -2.5

  - Now, install anchor screws using sterile forceps and a sterile screwdriver, until inserted 1 to 2 thread lengths deep [1]. Then mount the cOFM guide and dummy assembly into the guide holder and attach the holder to the stereotactic frame [2]. Position the guide tip above the burr hole and lower it slowly to the predetermined depth at 1 millimeter per minute [3].

    - Talent positioning and screwing in an anchor screw with forceps and screwdriver.

    - Talent installing cOFM guide into holder and frame.

    - Talent adjusting the DV knob to slowly lower the guide to target depth.

  - Apply the cementing agent to the skull in thin, uniform layers, covering areas under and around the cOFM guide and screws [1]. Use a dental curing light to harden each layer before applying the next one [2]. Ensure the cement covers the entire area between components while avoiding the locking wedge [3].

    - SCOPE: Talent applying cementing agent in a thin layer around components.

    - SCOPE: Talent curing the cement with a dental light.

    - SCOPE: Shot of cured cement surrounding the guide and screws.

  - Detach the guide holder from the cOFM guide and slowly raise it using the DV  (D-V) knob of the micromanipulator [1-TXT]. Then, remove the micromanipulator from the stereotactic frame [2].

    - Talent detaching and lifting the guide holder. TXT: DV: Dorsoventral

    - Talent removing the micromanipulator.

  - Using forceps, remove the locking wedge from the cOFM guide and dummy assembly [1]. Gently lift out the dummy from the guide [2].

    - SCOPE: Talent removing locking wedge using forceps.

    - SCOPE: Talent lifting out dummy from the guide.

  - Use the ML (M-L) and AP (A-P) micromanipulator knobs to align the syringe-infusion insert assembly over the cOFM guide [1]. Lower the infusion insert using the DV knob  [2] and secure it in position by firmly reinserting the locking wedge with forceps [3].

    - Talent aligning infusion insert above the guide. TXT: ML: Mediolateral; AP: Anteroposterior

    - Talent lowering infusion insert.

    - Talent reinserting wedge with forceps.

  - Now, inject 3 microliters of cell suspension from the syringe at 1 microliter per minute [1]. Monitor the area around the cOFM guide to ensure that no cell suspension effluxes out of the assembly [2-TXT].

    - Talent injecting cell suspension using syringe.

    - Close-up of infusion site for any leakage. TXT: Wait 1 min post injection to equilibrate intracranial pressure

  - Use forceps to remove the locking wedge from the guide and infusion insert assembly [1]. Then carefully lift the infusion insert from the guide using the DV adjustment knob [2].

    - Talent removing locking wedge.

    - Talent lifting infusion insert using micromanipulator knob.

- Microperfusion Tubing Preparation and System Flush for cOFM Setup

  - Prefill the push and pull tubing lines with artificial cerebrospinal fluid using gastight glass syringes [1]. Install the peristaltic section of the tubing into the pump head of the microperfusion pump [2].

    - Talent filling push and pull tubing lines using syringes.

    - Talent installing tubing into pump head.

  - Connect the perfusate bag to the push tubing using a Luer lock connector [1]. Then connect the sampling insert to the tubing using flanged connectors [2]. Attach the longer inlet shaft to the output end of the push tubing and the shorter outlet shaft to the input end of the pull tubing [3]. Place the connected sampling insert into a sterile microcentrifuge tube containing 750 microliters of artificial cerebrospinal fluid [4].

    - Talent connecting perfusate bag to tubing with Luer lock.

    - Talent connecting sampling insert inlet to push tubing.

    - Talent connecting sampling insert outlet to pull tubing.

    - Talent placing the insert into a microcentrifuge tube containing 750 µL.

  - Connect the fraction collector needle to the pull tube output line using a flanged tubing connector [1]. Then perform a tubing flush for 20 minutes at 10 microliters per minute to eliminate air bubbles [2].

    - Talent attaching fraction collector needle to tubing.

    - Shot of the display of tubing system flushing with digital timer at 10 microliters/minute.

  - Next, prefill the pull tubing line with artificial cerebrospinal fluid using gastight syringes [1]. Reinstall the peristaltic tubing into the pump head of the microperfusion pump [2].

    - Talent filling pull tubing line with a syringe.

    - Talent reinstalling tubing into pump head.

  - Connect the sampling insert again using flanged tubing connectors [1]. Attach the inlet to the output end of the pull tubing and the outlet to the input end of the pull tubing [2]. Place the insert into a sterile microcentrifuge tube with 750 microliters of artificial cerebrospinal fluid [3].

    - Talent reconnecting insert to pull tubing.

    - Talent securing outlet and input connections.

    - Talent placing insert into microcentrifuge tube.

  - Perform another 20-minute tubing system flush at 10 microliters per minute to clear any residual air bubbles [1].

    - Shot of second tubing flush in progress with active flow rate indicator.

- Animal Jacketing, cOFM Sampling, and Post‑Sampling Handling Procedure

AUTHORS: Please note that as per JoVE’s guidelines, anesthesia and euthanasia of animals cannot be demonstrated

  - Place an anesthetized animal in an appropriately sized jacket [1-TXT]. Draw the forelimbs completely through the arm holes and fasten the jacket securely along the animal’s back [2].

    - Talent selecting and placing the jacket on the animal. TXT: Anesthesia : Isoflurane inhalation (3%)

    - Shot of forelimbs positioned and fasteners being secured.

  - Use a pair of forceps to remove the locking wedge from the guide and dummy assembly [1]. Then carefully lift the dummy insert out of the guide [2]. Gently install the sampling insert into the guide and reinsert the locking wedge [3].

    - Talent using forceps to remove the locking wedge.

    - Talent lifting the dummy insert from the guide.

    - Talent inserting the sampling insert and reinserting the wedge.

  - Remove the animal from anesthesia and place it into the rotating cage system [1]. Attach the cage tether to the animal jacket and activate the rotating cage system [2].

    - Talent placing the animal into the rotating cage.

    - Talent attaching tether and switching on the rotating cage.

  - After cOFM sampling, deactivate the rotating cage system and detach the cage tether from the animal jacket [1-TXT].

    - Talent stopping the rotating cage and removing the tether. TXT: Transfer the animal to the anesthesia induction chamber

  - Now, use forceps to remove the locking wedge from the guide and sampling insert assembly [1]. Carefully lift the sampling insert out of the guide [2]. Then gently insert the dummy insert back into the guide and reinsert the locking wedge [3].

    - Talent using forceps to remove the locking wedge.

    - Talent lifting out the sampling insert.

    - Talent inserting the dummy and reinserting the wedge.

  - Detach the fasteners on the back of the animal jacket and gently remove the jacket from the animal [1].

    - Talent undoing jacket fasteners and removing it from the animal.

# Results

- Results

  - Bioluminescent imaging confirmed the presence of tumors in animals with cOFM guide-mediated tumor cell engraftment [1]. Proteomic principal component analysis of cOFM-sampled interstitial fluid revealed distinct separation between temozolomide-treated and dimethyl sulfoxide-treated animal groups [2].

    - LAB MEDIA: Figure 4A. Video editor: Highlight the bright signal regions within the red boxes sequentially from left to right

    - LAB MEDIA: Figure 4B. Video editor: Highlight the orange cluster labeled “TMZ” and then the blue cluster labeled “DMSO”

  - Global metabolomic signal distributions overlapped almost perfectly between temozolomide and dimethyl sulfoxide samples [1]. Despite signal overlap, metabolomic volcano analysis revealed a distinct temozolomide-driven shift [2].

    - LAB MEDIA: Figure 4C (left).

    - LAB MEDIA: Figure 4C (right).

  - Quantitative comparisons of metabolomic values from the same animals showed that recirculated configurations improved analyte concentrations while preserving relative metabolite proportions [1].

    - LAB MEDIA: Figure 4D. Video editor: Highlight all four scatter plots sequentially