# Submission ID #: 68775

# Scriptwriter Name: Pallavi Sharma

# Project Page Link: https://review.jove.com/account/file-uploader?src=20976423

Title: Echocardiography-Guided Injection for Targeted and Reliable Intramyocardial Stem Cell Delivery in a Rat Model of Myocardial Infarction

# Authors and Affiliations:

Ellen Heeren, Vincent Vandenboer, Lotte Vastmans, Dorien Deluyker, Marc Hendrikx, Virginie Bito

UHasselt, Cardio & Organ Systems (COST), Biomedical Research Institute

# Corresponding Authors:

Virginie Bito 			Virginie.bito@uhasselt.be

Ellen Heeren 			Ellen.heeren@uhasselt.be

# Email Addresses for All Authors:

Vincent.vandenboer@student.uhasselt.be

Lotte.Vastmans@uhasselt.be

Dorien.deluyker@uhasselt.be

Marc.hendrikx@uhasselt.be

# Virginie.bito@uhasselt.be

Ellen.heeren@uhasselt.be

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  NO

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  All done

3. Filming location: Will the filming need to take place in multiple locations?   NO

Current Protocol Length

Number of Steps:  17

Number of Shots:  41

Ethics Title Card

This research has been approved by the Local Ethical Committee of UHasselt

# Protocol

Videographer: There is no interview section to film

- Animal Preparation for Echocardiography-Guided Intramyocardial Injections

Demonstrators: Ellen Heeren, Vincent Vandenboer

  - To begin, shave the anesthetized rat’s chest [1] and apply depilatory cream to remove any remaining hair [2-TXT]. After complete hair removal, disinfect the injection site by alternating an appropriate surgical scrub agent with 70 percent ethanol [3-TXT]. Then, using a sterile applicator, apply ophthalmic gel to both eyes to prevent dryness [4].

    - WIDE: Talent shaving the rat’s chest. TXT: Anesthesia: 2.5% isoflurane supplemented with O2 (Flow rate: 2 L/min)

    - Talent removing hair with depilatory cream.

    - Talent wipes the skin with 70 percent ethanol to disinfect the surgical area. TXT: Repeat 3x

    - Talent applying ophthalmic gel to the eyes of the rat.

  - Transfer the rat from the heating pad onto the animal platform of the ultrasound imaging system [1]. Apply a small amount of electrode gel to attach the rat’s paws to the platform electrodes, enhancing signal quality [2]. Then, insert the rectal temperature probe to monitor body temperature continuously [3].

    - Talent moving the anesthetized rat onto the imaging platform.  TXT: Anesthesia Maintenance:  1–3% isoflurane in 1–1.5 L/min O₂

    - Talent applying electrode gel and attaching each paw to the platform electrodes.

    - Talent gently inserting the rectal temperature probe into the rat.

  - Now, acquire the desired baseline or pre-injection images, including parasternal long axis [1] and short axis views in both B-mode and M-mode [2], as well as four-chamber images for anatomical and functional assessment [3].

    - SCREEN: 68775_screenshot_1.mp4: 00:02-00:04

    - SCREEN: 68775_screenshot_2.mp4: 00:02-00:04 and 68775_screenshot_3.mp4: 00:02-00:04 Video editor: Use split screen and show both the videos together

    - SCREEN: 68775_screenshot_4.mp4

- Echocardiography-Guided Intramyocardial Injection Procedure

  - To prepare the injection system, attach a 22-gauge guide needle to a 1-milliliter syringe [1]. Mount the syringe with the guide needle onto the injection clamp and secure it in place [2].

    - Talent attaching a 22-gauge needle to a 1 milliliter syringe.

    - Talent placing the syringe into the injection mount and tightening the clamp
.

  - Next, align the ultrasound transducer with the injection mount to ensure the guide needle is visible in the imaging field [1]. Adjust the transducer position using the transducer mount and holding clamp [2-TXT].

    - Talent aligning the ultrasound transducer with the injection mount.

    - Talent adjusting the transducer using the mount and holding clamp. TXT: Rotate the injection mount to achieve alignment if required

  - Without moving the rat, rotate the animal platform until the notch on the transducer points toward the rat’s right shoulder [1-TXT].

    - Talent rotating the animal platform to orient the transducer notch toward the rat’s right shoulder. TXT: Use the 3D motor to realign the transducer if misaligned

  - To fine-tune the imaging, adjust the micromanipulator screws on the animal platform rail [1]. Keep the transducer steady to maintain alignment with the needle [2-TXT].

    - Talent using the micromanipulator screws on the rail to fine-tune imaging alignment.

    - Shot of the talent with the transducer. TXT: Rotate or translate the animal platform further to optimize image clarity

  - Now, using parasternal long axis B-mode and M-mode imaging, visualize the infarcted area [1]. Evaluate the extent of infarction, regional wall motion, and areas of wall thinning to determine injection feasibility [2-TXT].

    - SCREEN: 68775_screenshot_5.mp4, 68775_screenshot_6.mp4  Video editor: Please use markedscreenrecordings.pptx to annotate 68775_screenshot_5.mp4

    - SCREEN: 68775_screenshot_7.TIFF  TXT: Use 16-segment scoring to detect akinetic/severely hypokinetic regions

  - Identify the peri-infarct zone and choose a hypokinetic area next to the infarct core that has an end-diastolic wall thickness greater than 1 millimeter [1].

    - Display side-by-side images of infarct core and adjacent hypokinetic zone with measured wall thickness.

  - Next, using the rail system, advance the injection mount towards the animal platform [1]. Then, adjust the injection mount’s micromanipulator screws to fine-align the needle tip with the exact center of the transducer field [2].

    - Talent sliding the injection mount along the rail toward the rat.

    - Talent adjusting micromanipulator screws to center the needle tip in the ultrasound field.

  - Now, to slowly advance the guide needle and puncture the skin, turn the inject micromanipulator screw on the injection mount [1] and confirm the needle tip is visible in the ultrasound field [2].

    - Talent turning the inject screw to advance the guide needle toward the skin.

    - SCREEN: 68775_screenshot_8.mp4 Video editor: Please use markedscreenrecordings.pptx to annotate 68775_screenshot_8.mp4

  - Then, activate the needle guide feature in the ultrasound software to confirm the planned trajectory of the guide needle [1]. Advance the guide needle toward the selected injection site, stopping with the bevel 1 to 2 millimeters from the left ventricular anterior wall [2].

    - SCREEN: 68775_screenshot_9.mp4

    - Display real-time advancement of the guide needle toward the LVAW, with the bevel stopping 1 to 2 millimeters from the wall.

  - Once the needle is in position, instruct one operator to stabilize the guide needle at its base to maintain constant ultrasound visualization [1]. Ask another operator to loosen the syringe clamp, carefully remove the syringe [2], and replace it with the syringe containing the injectate [3-TXT].

    - Talent holding the guide needle firmly in place under ultrasound guidance.

    - Talent loosening the clamp, removing the empty syringe.

    - Talent attaching the injectate-loaded syringe. TXT: Ensure to keep the guide needle stationary

  - Next, attach an 88 millimeter long 29-gauge needle to the syringe containing the injectate.[1]. Secure the syringe onto the injection mount [2]. Manually insert the needle through the stationary guide needle [3], making minor adjustments with the micromanipulator screws along the x-axis, if required [4]. Continue to advance the needle manually until the bevel becomes visible in the thoracic cavity on ultrasound [5].

    - Talent attaching the 29 gauge by 88 millimeter needle to the filled syringe.

    - Talent mounting the syringe onto the injection holder.

    - Talent making minor adjustments along the x-axis.

    - Talent beginning manual insertion through the guide needle.

    - Show ultrasound image with the 29 gauge needle bevel appearing inside the thoracic cavity.

  - Once the 29-gauge needle is visible on ultrasound, advance it into the myocardium of the left ventricular anterior wall using the micromanipulator screws for precise control [1]. Ensure the full length of the needle bevel is embedded in the myocardium [2].

    - Talent turning the micromanipulator screws to guide the needle into the myocardium.

    - SCREEN: 68775_screenshot_10.mp4 Video editor: Please use markedscreenrecordings.pptx for annotation

  - Slowly inject the injectate into the myocardium [1]. Confirm a successful injection by observing a bright, dense echogenic spot at the injection site that moves with the wall motion of the left ventricular anterior wall [2-TXT].

    - Talent pressing the syringe plunger slowly to administer the injectate.

    - SCREEN: 68775_screenshot_11mp4. TXT: Wait for 10 s post-injection to prevent backflow Video editor: Please use markedscreenrecordings.pptx for annotation

  - Finally, remove the syringe and needles from the injection mount to prevent accidental needle sticks [1]. Carefully lift the rat off the animal platform and place it on a heating pad for recovery [2-TXT].

    - Talent unfastening the syringe from the mount and removing it with the needle.

    - Talent gently lifting the rat from the platform and placing it on a heating pad. TXT: Monitor rat’s vital signs until complete recovery

# Results

- Results

  - Bioluminescence imaging revealed partial leakage of the injected stem cells due to premature needle withdrawal, indicated by diffuse luminescent signal in the thoracic cavity [1]. A successful intramyocardial injection was confirmed by a concentrated luminescent signal at the left ventricular mid-apex region [2].

    - LAB MEDIA: Figure 3. Video editor: Highlight panel 3A and show the small blue-green fluorescence toward the left side!

    - LAB MEDIA: Figure 3. Video editor: Highlight panel 3B and show the circular, bright luminescent area in the center.

  - Ex vivo imaging of sectioned hearts showed the strongest bioluminescent signal in the mid-apex slice [1], with weaker signals in the apex [2] and mid-ventricle sections [3].

    - LAB MEDIA: Figure 3. Video editor: Highlight panel 3C and emphasize the middle circular section labeled “mid-apex” with the most intense glow.

    - LAB MEDIA: Figure 3C. Video editor: Highlight panel 3C and emphasize the right section labeled “apex” with the dimmer glow.

    - LAB MEDIA: Figure 3C. Video editor: Highlight panel 3C and emphasize the left section labeled “mid” with the faint luminescence.

  - Post-mortem imaging of the heart showed a visible purple hydrogel adjacent to the infarct zone, confirming targeted delivery [1]. Fluorescence imaging of heart tissue showed that the red tracer was confined to the peri-infarct zone, confirming localized delivery [2].

    - LAB MEDIA: Figure 4A. Video editor: Highlight the region marked with white arrows.

    - LAB MEDIA: Figure 4B. Video editor: Highlight the bright red cluster in the lower-left region of the blue-stained tissue.

  - Sirius Red staining confirmed that the tracer was delivered to a region of intermediate wall thickness, between the infarct core and healthy myocardium [1].

    - LAB MEDIA: Figure 4C. Video editor: Highlight the red-stained fibrotic areas surrounding the open central cavity of the heart slice.

  - Color Doppler imaging confirmed that the needle was correctly placed inside the myocardium with no visible blood flow before injection [1]. During injection, the anterior wall remained intact with no external blood flow detected [2]. After needle withdrawal, the retained injectate was visible within the myocardium, and no hemorrhage was observed [3].

    - LAB MEDIA: Figure 5A. Video editor: Highlight the area with red and yellow arrows

    - LAB MEDIA: Figure 5B.  Video editor: Highlight the area with yellow arrows and circle

    - LAB MEDIA: Figure 5C.  Video editor: Highlight the area with yellow arrows and circle