# Submission ID #: 68834

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=20994418

# Title: Field-Deployable Lens-Free Imaging Platform for Rapid Label-Free Analysis of Natural Killer Cell Activation

# Authors and Affiliations:

# Samir Kumar1*, Sanghoon Shin1*, Hojin Cheon1, Inha Lee2,3, Young-Sun Lee4, Myung-Hyun Nam5, Sunmi Han1,3, Hyun Sik Jun2,3, Sungkyu Seo1,3

# 1Department of Electronics and Information Engineering, Korea University

# 2Department of Biotechnology and Bioinformatics, Korea University

# 3Metaimmunetech Inc.

# 4Department of Gastroenterology and Hepatology, Guro Hospital, Korea University College of Medicine

# 5Department of Laboratory Medicine, Anam Hospital, Korea University College of Medicine

*These authors contributed equally

# Corresponding Authors:

# Sungkyu Seo 		sseo@korea.ac.kr

# Email Addresses for All Authors:

# Samir Kumar 		skumar@korea.ac.kr

# Sanghoon Shin 	ghost10s@korea.ac.kr

# Hojin Cheon 		cheon_hj@korea.ac.kr

# Inha Lee 		dlsgk1017@korea.ac.kr

# Young-Sun Lee 	lys810@korea.ac.kr

# Myung-Hyun Nam 	yuret@korea.ac.kr

# Sunmi Han 		sunmi.han@metaimmunetech.kr

# Hyun Sik Jun 		toddjun@korea.ac.kr

# Sungkyu Seo 		sseo@korea.ac.kr

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes, all done

3. Filming location: Will the filming need to take place in multiple locations?   Yes

If Yes, how far apart are the locations? less than 100 m

Current Protocol Length

Number of Steps:  25

Number of Shots:  55 (15 SC)

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

  - Samir Kumar: Our goal is to rapidly assess immune function by quantifying NK cell activation in real time using label-free, lens-free imaging to improve immune monitoring.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.3.1

What significant findings have you established in your field?

  - Samir Kumar: We have shown that the innate immunity index or I³ reliably distinguishes between the NK activity of healthy and immunocompromised donors and correlates strongly with cytokine and flow cytometry markers.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.5.1

What research gap are you addressing with your protocol?

  - Samir Kumar: The existing NK cell assays require labels, bulky instruments, and long incubation times. Our LSIT platform fills this gap by enabling fast, affordable, label‑free profiling using simple optoelectronics.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.4.1

What advantage does your protocol offer compared to other techniques?

  - Samir Kumar: Our label‑free LSIT platform differentiates NK cell activation at the single cell level within thirty seconds using shadow parameters, eliminating the need for staining and costly flow cytometry.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 4.2.1

What research questions will your laboratory focus on in the future?

  - Samir Kumar: We will extend LSIT to monitor T and B cell activation and integrate deep learning algorithms for high-throughput classification of different immune cell states.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 5.3.1

Videographer: Obtain headshots for all authors available at the filming location.

# Ethics Title Card

This research has been approved by the Institutional Review Board of Korea University Anam Hospital

# Protocol

- Natural Killer Cell Isolation and Activation

Demonstrator: Inha Lee

  - To begin, remove the kits for natural killer or NK cell isolation from the refrigerator [1] and allow them to come to room temperature between 15 and 28 degrees Celsius [2].

    - WIDE: Talent opening a refrigerator and taking out the kits.

    - Talent placing the kits on a bench.

  - Prepare the required equipment by placing a magnetic separator, pipettes with sterile tips, [1] and set an incubator or heating block to 37 degrees Celsius [2].

    - Talent arranging the magnetic separator, pipettes with sterile tips on the workbench.

    - Talent setting the temperature to 37 degrees Celius on the heatblock or incubator.

  - Using a pipette, add 0.5 milliliters of whole blood to the green-capped reaction tube containing the antibody cocktail [1]. Gently mix the contents by pipetting up and down the tube 5 to 6 times [2], then incubate the tube for 5 minutes [3].

    - Talent pipetting whole blood into a green-capped tube.

    - Talent gently pipetting the green-capped tube.

    - Talent placing the green-capped tube in a rack and starting a 5-minute timer.

  - Transfer the entire contents of the green-capped tube into the red-capped separation tube 1 using a pipette [1] and gently mix by pipetting the tube 5 to 6 times [2].

    - Talent transferring the contents into a red-capped tube.

    - Talent pipetting the red-capped tube.

  - Place the red-capped tube on the magnetic separator and incubate at room temperature for 10 minutes [1].

    - Talent placing the red-capped tube on the magnetic separator and starting a 10-minute timer.

  - While keeping the magnet in place, use a pipette to transfer approximately 1.5 milliliters of the supernatant into the purple-capped separation tube 2 [1]. Gently mix the contents by pipetting the tube 5 to 6 times [2].

    - Talent transferring the supernatant into a purple-capped tube while the red-capped tube remains on the magnet.

    - Talent gently pipetting the purple-capped tube.

  - Now, place the purple-capped tube on the magnetic separator and incubate for 10 minutes [1].

    - Talent placing the purple-capped tube on the magnetic separator and starting a 10-minute timer.

  - While the magnet is still in place, transfer approximately 1 milliliter of the supernatant into a recovery tube with a gray lid using a pipette [1] and carefully mix the contents by pipetting the tube [2].

    - Talent transferring the supernatant into a gray-lidded tube while keeping the purple-capped tube on the magnet.

    - Talent gently pipetting the gray-lidded tube.

  - Next, add 100 microliters of the isolated natural killer cell suspension into the Vehicle tube [1] and another 100 microliters into the activation stimulator cocktail or ASC tube [2].

    - Talent pipetting 100 microliters of NK cell suspension into the labelled “Vehicle tube”.

    - Talent pipetting 100 microliters of NK cell suspension into the labelled “ASC tube”.

  - Gently mix the contents of both tubes using a vortex mixer to ensure even distribution [1]. Incubate the tubes in the heating block or incubator set at 37 degrees Celsius for 1 hour [2].

    - Talent gently mixing both the Vehicle and ASC tubes by vortex mixing.

    - Talent placing the tubes into a 37-degree Celsius incubator and setting a 1-hour timer.

- Imaging the Cells with the LSIT Platform for NK Cell Activity Assessment

Demonstrator: Hojin Cheon

  - Switch on the LSIT (L-S-I-T) platform [1-TXT].

    - Talent reaching to power on the LSIT platform. TXT: LSIT: Lens-free Shadow Imaging Technology

  - Launch the LSIT Capture software [1], log in by entering your ID and password [2]. Then, select Calibrate to initiate calibration mode [3].

    - SCREEN: 68834-Screenshot.mp4 00:00–00:10

    - SCREEN: 68834-Screenshot.mp4 00:11–00:20 and 00:23-00:24

    - SCREEN: 68834-Screenshot.mp4 00:25–00:30

  - Press the Open button on the touchscreen or use the physical sliding door button to open the drawer [1]. When the drawer opens, remove the calibration slide and store it [2].

    - Talent pressing Open on the screen or using physical button; drawer slides open.

    - Talent removing the calibration slide and placing it in a storage box.

  - Now, click Set background to calibrate the optical intensity [1].

    - SCREEN: 68834-Screenshot.mp4 00:35–00:40

  - Replace the calibration slide in the drawer [1] and close the drawer [2].

    - Talent placing the calibration slide back into the drawer.

    - Talent pressing Close or sliding the door shut.

  - Then, click Start calibration to complete the process [1].

    - SCREEN: 68834-Screenshot.mp4 02:32–02:35 and 02:59-03:00

  - Remove an assay slide from its pouch, label it with the sample data before placing it on a clean, flat surface [1]. Pipette 10 microliters of the vehicle sample into channels A and B [2] and then pipette 10 microliters of the ASC-stimulated sample into channels C and D [3].

    - Talent unsealing an assay slide pouch, labeling the slide with a marker.

    - Talent pipetting the vehicle sample into channels A and B of the assay slide.

    - Talent pipetting the ASC-stimulated sample into channels C and D of the assay slide.

  - Now, press Open to eject the drawer [1], remove the calibration slide [2], and place it in a storage box [3].

    - Talent pressing Open on the touchscreen or using physical button.

    - Talent lifting the calibration slide from the drawer.

    - Talent placing the slide in the storage box.

  - Insert the prepared assay slide into the drawer [1] and press Close to secure it [2].

    - Talent inserting the assay slide into the drawer.

    - Talent pressing Close or sliding the drawer shut.

  - Go back to the Main Menu and select the NK Cell Activity and Test from the main screen [1]. Enter the sample ID or scan the barcode to load the sample profile [2]. Then, press Capture to begin image recording [3].

    - SCREEN: 68834-Screenshot.mp4 03:05–03:19

    - SCREEN: 68834-Screenshot.mp4 03:19–03:29

    - SCREEN: 68834-Screenshot.mp4 03:48–04:00

  - After capturing the image, click on Analyze to begin processing [1]. Review the NK cell activity percentage and cell count displayed on the screen [2]. Download the results to a USB drive or print them using the touchscreen menu [3].

    - SCREEN: 68834-Screenshot.mp4 04:00–04:08

    - SCREEN: 68834-Screenshot.mp4 04:49–05:00

    - SCREEN: 68834-Screenshot.mp4 05:25–05:30

- Cell Counting and Post-Analysis Clean-Up

Demonstrator: Hojin Cheon

  - Select Cell counting and Test on the LSIT main interface [1]. Load 10 microliters of the new sample onto a fresh assay slide channel using a pipette [2]. Insert the slide into the drawer, close the drawer using the touchscreen or button [3], and enter the sample ID on the system [4].

    - SCREEN: 68834-Screenshot.mp4 05:45–05:53

    - Talent pipetting 10 microliters of the sample into a new channel on a clean assay slide.

    - Talent inserting the assay slide into the drawer, pressing Close.

    - Talent entering the sample ID.

  - Enter the dilution factor on the touchscreen if applicable [1] and press Counting to begin cell quantification [2]. View the results displayed as cells per microliter on the screen [3].

    - Show the dilution factor input field and talent entering the value.

    - SCREEN: 68834-Screenshot.mp4 06:02–06:05 and 06:28-06:30

    - SCREEN: 68834-Screenshot.mp4 06:40–06:47

  - After analysis, open the drawer by pressing the Open button or toggling the physical switch [1]. Remove the used slide from the drawer using gloved hands [2].

    - Talent pressing Open or using the switch; drawer slides open.

    - Talent carefully lifting out the used assay slide and discarding it in the designated waste container.

  - Finally, place the calibration slide back into the drawer to protect the sensor [1] and close the drawer using the touchscreen or the physical toggle switch [2].

    - Talent retrieving the calibration slide from the storage box and placing it in the drawer.

    - Talent pressing Close or sliding the drawer shut.

# Results

- Results

  - The LSIT platform, with integrated software, quantitatively assessed NK cell activation by comparing the CSP values of activated and unstimulated control cells [1]. It provided the immunity report with a summarized statistical table [2].

    - LAB MEDIA: Figure 5A and B

    - LAB MEDIA: Figure 5C

  - ASC stimulation induced clear morphological changes in NK cells, including increased size and cytoplasmic complexity, as seen in Hema-3–stained cytospins after 30 minutes and 2 hours [1].

    - LAB MEDIA: Figure 2A. Video editor: Highlight the “ASC”-stimulated cells.

  - Shadow images showed ASC-stimulated cells had visibly more complex diffraction patterns compared to vehicle controls [1], with extracted parameters indicating increased peak-to-peak distance, cytoplasmic granularity and combined shadow parameter [2].

    - LAB MEDIA: Figure 2B. Video editor: Focus on the “ASC”-stimulated shadow images in the lower row. These have a blue background.

    - LAB MEDIA: Figure 2C. Video editor: Highlight the bars for “ASC” in all 3 graphs of 2C.

  - Combined shadow parameter values allowed clear classification between Healthy donors [1] and Cancer patients [2], with minimal overlap in single-cell distributions of activated and non-activated NK cells [3].

    - LAB MEDIA: Figure 2D. Video editor: Highlight the box plot for “ HD”.

    - LAB MEDIA: Figure 2D. Video editor: Highlight the box plot for “ CP”.

    - LAB MEDIA: Figure 2E. Video editor: Highlight red points (activated) .

  - Healthy donors showed significantly higher Innate Immunity Index values [1] than cancer patients following ASC stimulation [2].

    - LAB MEDIA: Figure 3A. Video editor: Highlight the scatter plot for HD group.

    - LAB MEDIA: Figure 3A. Video editor: Highlight the scatter plot for CP group.

Pronunciation Guide:
1. Immunocompromised

Pronunciation link:
https://www.merriam-webster.com/dictionary/immunocompromised
IPA: /ˌɪm.jə.noʊˈkɑːm.prəˌmaɪzd/
Phonetic Spelling: im-yuh-noh-KOM-pruh-myzd

2. Cytokine

Pronunciation link:
https://www.merriam-webster.com/dictionary/cytokine
IPA: /ˈsaɪ.t̬oʊˌkaɪn/
Phonetic Spelling: SYE-toh-kine

3. Flow Cytometry

Pronunciation link:
https://www.merriam-webster.com/dictionary/cytometry
IPA: /floʊ saɪˈtɑː.mə.tri/
Phonetic Spelling: floh sy-TAH-muh-tree

4. Optoelectronics

Pronunciation link:
https://www.merriam-webster.com/dictionary/optoelectronics
IPA: /ˌɑːp.toʊ.ɪˌlɛkˈtrɑː.nɪks/
Phonetic Spelling: op-toh-ee-lek-TRAH-niks

5. Incubator

Pronunciation link:
https://www.merriam-webster.com/dictionary/incubator
IPA: /ˈɪŋ.kjəˌbeɪ.tɚ/
Phonetic Spelling: IN-kyuh-bay-ter

6. Microliter

Pronunciation link:
https://www.merriam-webster.com/dictionary/microliter
IPA: /ˈmaɪ.krəˌliː.t̬ɚ/
Phonetic Spelling: MY-kroh-lee-ter

7. Vehicle (scientific usage: “control vehicle”)

Pronunciation link:
https://www.merriam-webster.com/dictionary/vehicle
IPA: /ˈviː.ɪ.kəl/
Phonetic Spelling: VEE-ih-kul

8. Vortex (as in vortex mixer)

Pronunciation link:
https://www.merriam-webster.com/dictionary/vortex
IPA: /ˈvɔːrˌtɛks/
Phonetic Spelling: VOR-teks

9. Assay

Pronunciation link:
https://www.merriam-webster.com/dictionary/assay
IPA: /ˈæˌseɪ/
Phonetic Spelling: A-say

10. Cytoplasmic

Pronunciation link:
https://www.merriam-webster.com/dictionary/cytoplasmic
IPA: /ˌsaɪ.toʊˈplæz.mɪk/
Phonetic Spelling: SYE-toh-plaz-mik

11. Morphological

Pronunciation link:
https://www.merriam-webster.com/dictionary/morphological
IPA: /ˌmɔːr.fəˈlɑː.dʒɪ.kəl/
Phonetic Spelling: mor-fuh-LAH-jih-kul

12. Cytospin

Pronunciation link:
https://www.howtopronounce.com/cytospin
IPA: /ˈsaɪ.toʊˌspɪn/
Phonetic Spelling: SYE-toh-spin

13. Granularity

Pronunciation link:
https://www.merriam-webster.com/dictionary/granularity
IPA: /ˌɡrænjʊˈlɛrəti/
Phonetic Spelling: gran-yoo-LAIR-uh-tee

14. Anam (as in Korea University Anam Hospital)

Pronunciation link:
https://www.howtopronounce.com/anam
IPA: /ˈɑː.nɑːm/
Phonetic Spelling: AH-nahm

15. Metaimmunetech

Pronunciation link:
No confirmed link found
IPA (constructed): /ˌmɛt.ə.ɪˈmjuːn.tɛk/
Phonetic Spelling: MET-uh-ih-MYUN-tek

16. Gastroenterology

Pronunciation link:
https://www.merriam-webster.com/dictionary/gastroenterology
IPA: /ˌɡæs.troʊˌɛn.təˈrɑː.lə.dʒi/
Phonetic Spelling: gas-troh-en-tuh-RAH-luh-jee

17. Hepatology

Pronunciation link:
https://www.merriam-webster.com/dictionary/hepatology
IPA: /ˌhɛp.əˈtɑː.lə.dʒi/
Phonetic Spelling: hep-uh-TAH-luh-jee

18. Immunity

Pronunciation link:
https://www.merriam-webster.com/dictionary/immunity
IPA: /ɪˈmjuː.nə.t̬i/
Phonetic Spelling: ih-MYOO-nuh-tee

19. Diffraction

Pronunciation link:
https://www.merriam-webster.com/dictionary/diffraction
IPA: /dɪˈfræk.ʃən/
Phonetic Spelling: dih-FRAK-shun

20. Calibration

Pronunciation link:
https://www.merriam-webster.com/dictionary/calibration
IPA: /ˌkæl.əˈbreɪ.ʃən/
Phonetic Spelling: kal-uh-BRAY-shun