# Submission ID #: 68941

# Scriptwriter Name: Sulakshana Karkala

# Project Page Link: https://review.jove.com/account/file-uploader?src=21027053

# Title: Functional Cardiac Imaging in Zebrafish Embryos Using Standard Microscopy and Video Analysis: Applications in Environmental and Biomedical Research

# Authors and Affiliations:

# Nikola Mitovic1, Sanjin Kovacevic1, Jelena Nesovic Ostojic1, Darko Puflovic2, Marija S. Stankovic1

# 1Department of Pathophysiology, Medical Faculty, University of Belgrade

# 2Faculty of Electronic Engineering, University of Nis

# Corresponding Authors:

# Nikola Mitovic			(nikola.mitovic@med.bg.ac.rs) ; (nikolamitovic@gmail.com)

# Email Addresses for All Authors:

# Nikola Mitovic			(nikola.mitovic@med.bg.ac.rs) ; (nikolamitovic@gmail.com)

# Sanjin Kovacevic		(sanjin.kovacevic@med.bg.ac.rs)

# Jelena Nesovic Ostojic		(jelena.nesovic-ostojic@med.bg.ac.rs)

# Darko Puflovic			(darko.puflovic@elfak.ni.ac.rs)

# Marija S. Stankovic		(marija.stankovic.med@gmail.com)

## Author Questionnaire

1. We have marked your project as author-provided footage, meaning you film the video yourself and provide JoVE with the footage to edit. JoVE will not send the videographer. Please confirm that this is correct.

√ Correct

2. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

3. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

4. Proposed filming date: To help JoVE process and publish your video in a timely manner, please indicate the proposed date that your group will film here:  MM/DD/YYYY

When you are ready to submit your video files, please contact our Content Manager, Utkarsh Khare.

Current Protocol Length

Number of Steps:  07

Number of Shots:  11

# Introduction

INTRODUCTION:

What technologies are currently used to advance research in your field?

  - Nikola Mitovic: High-resolution microscopy, fluorescent reporters, and digital image analysis enable detailed assessment of embryonic cardiac function in zebrafish.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:2.1

What are the current experimental challenges?

  - Nikola Mitovic: Maintaining consistent imaging conditions and minimizing embryo movement remain major challenges for accurate cardiac function measurements.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

CONCLUSION:

What research gap are you addressing with your protocol?

  - Nikola Mitovic: We address the lack of standardized, accessible methods for quantifying zebrafish embryonic cardiac function with reproducible workflows.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What advantage does your protocol offer compared to other techniques?

  - Nikola Mitovic: This protocol uses standard microscopy and open-source analysis, making high-quality cardiac assessment feasible without specialized equipment.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:2.3

How will your findings advance research in your field?

  - Nikola Mitovic: Our findings provide a reliable protocol that improves comparability between studies and enables more precise evaluation of cardiac phenotypes.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:3.2

Ethics Title Card

The study was approved by the Animal Care and Use Committee of the Faculty of Medicine, University of Belgrade

# Protocol

NOTE: Scripted from author provided footage

- Assessment of Zebrafish Embryo Heart Dynamics via Image-Based Motion Analysis

Demonstrator: Nikola Mitovic

  - To begin, record videos of zebrafish embryos at 96 hours post in uncompressed formats to allow accurate analysis of cardiac motion dynamics [1-TXT].

    - LAB MEDIA: 1st.mp4 	00:00-00:16 
TXT: Frame rate: 30 fps

  - Sequentially click on File,  Open  then choose the AVI (A-V-I)  file in ImageJ (Image-J)  to import the videos [1-TXT]. Use Analyze then click on Set Scale, input the known field diameter and enable the Global option to apply the scale across all measurements [2].

    - LAB MEDIA:  2nd.mp4 	00:00-00:14
TXT: If needed, convert to image stack

    - LAB MEDIA:  2nd.mp4 00:23-00:35

  - Use the ROI (R-O-I) tools to mark the long and short ventricular axes during end-systole and end-diastole [1].

    - LAB MEDIA:  2nd.mp4 00:45-00:51, 00:58-01:02, 01:13-01:16

  - Then determine the ventricular surface area using the ellipse approximation formula [1]. Estimate the ventricular volume at end-diastole and end-systole using the ellipsoid volume formula [2].

    - LAB MEDIA:  3rd.mp4 00:00-00:15
Video Editor: Please  Highlight the top formula VA = ((D_L - D_S)/2) &#215; π and the values of VAS and VAD

    - LAB MEDIA:  3rd.mp4 00:00-00:15
Video Editor: Please  Highlight the middle formula VV = (1/6) &#215; π &#215; D_L &#215; D_S² and the values of VVS and VVD

  - Now, calculate the fractional area change and fractional shortening [1]. Then determine the stroke volume SV (S-V) and Ejection Fraction EF (E-F) [2].

    - LAB MEDIA:  3rd.mp4 00:00-00:15
Video Editor: Please  Highlight the formulae 
FPP (%) = ((VAD - VAS) / VAD) &#215; 100 and 
FS (%) = ((D_L - D_S) / D_L) &#215; 100

    - LAB MEDIA:  3rd.mp4 00:00-00:15
Video Editor: Please  Highlight the formulae 
SV= VVD=VVS= 367039 µL and 
EF (%) = (SV / VVD) &#215; 100

  - For automated analysis, launch the  ZembryoAnalyzer (Z-Embryo-Analyzer) software and click on File then Open Video to import the desired video [1]. Click on the Detect Heart button to allow the software to automatically locate the region of interest based on brightness fluctuations across video frames [2-TXT].

    - LAB MEDIA:  4th.mp4 	00:00-00:02

    - LAB MEDIA:  4th.mp4 	00:02-00:30
TXT: Adjust region manually if needed

  - Let the software analyze brightness changes within the detected region of interest to estimate heart rate then calculate cardiac output [1-TXT].

    - LAB MEDIA:  4th.mp4 00:31-00:55 TXT: Export heart rate and corresponding intensity plots

# Results

- Results

  - The average ventricular area in zebrafish embryos at 96 hours post fertilization was significantly higher at end-diastole compared to end-systole [1]. The average ventricular volume in zebrafish embryos at 96 hours post fertilization was significantly higher at end-diastole compared to end-systole [2].

    - LAB MEDIA: Figure 2A and C. Video editor: Highlight the red violin plot labelled "EDA" in 2A

    - LAB MEDIA: Figure 2B and B . Video editor: Highlight the red violin plot labelled "EDA" in 2 B

  - In healthy zebrafish embryos, the average stroke volume was 0.213 nanoliters [1], cardiac output was 27.8 nanoliters per minute [2], and ejection fraction was 41.99 percent [3].

    - LAB MEDIA: Figure 3 Video Editor: Please highlight 3D

    - LAB MEDIA: Figure 3 Video Editor: Please highlight 3A

    - LAB MEDIA: Figure 3 Video Editor: Please highlight 3C

  - In healthy zebrafish embryos, the average fractional area change was 61.2 percent [1], and fractional shortening was 44.5 percent [2].

    - LAB MEDIA: Figure 4 	Video Editor: Please highlight A

    - LAB MEDIA: Figure 4	Video Editor: Please highlight B

  - Cadmium-exposed embryos exhibited abnormal cardiac morphology with enlarged ventricles and pericardial edema [1].

    - LAB MEDIA: Figure 5 	Video Editor: Please highlight A

  - In cadmium-exposed embryos, stroke volume was significantly lower than in controls [1], cardiac output was diminished [2], and heart rate was reduced [3].

    - LAB MEDIA: Figure 5B. Video editor: Highlight the yellow violin plot labelled "Cd²⁺"

    - LAB MEDIA: Figure 5C. Video editor: Highlight the yellow violin plot labelled "Cd²⁺"

    - LAB MEDIA: Figure 5D. Video editor: Highlight the yellow violin plot labelled "Cd²⁺"

Top of Form

Bottom of Form