JoVE-SCREEN_69084_ step 2.26.2_Start ventilation:

26.2.1 (In flexiware software) 00:01

26.2.2 (Double click on ventilation EVLP) 00:04

JoVE-SCREEN_69084_ step 2.27.1_setting ventilation parameters :

set ventilation at 7 stroke/min, 3 mL/kg and PEEP at 3 cmH2O

27.1.1 (In flexiware software, on the right corner, under tasks, click on the 3 dots next to ventilation EVLP) 00:03

27.1.2 (Click on ventilation) 00: 06

27.1.3 (Set respiratory rate at 7/min) 00: 07- 00:10

27.1.4 (Set tidal volume at 3 mL/kg) 00:12-00:14

27.1.5 (Set PEEP level at 3 cm H2O) 00:17- 00:20

27.1.6 (Click ok) 00:21 STOP VIDEO HERE

JoVE-SCREEN_69084_ step 2.26.3_setting ventilation parameters

set ventilation at 15 stroke/min and 6 mL/kg

26.3.1 (In flexiware software, on the right corner, under tasks, click on the 3 dots next to ventilation EVLP) 00:00

26.3.2 (Click on ventilation) 00:02

26.3.3 (Set respiratory rate at 15/min) 00:03-00:06

26.3.4. (Set tidal volume at 6 mL/kg) 00:07-00:09

26.3.5 (Click ok) 00:10

26.3.6 (Click ok) 00:11

JoVE-SCREEN_69084_ step 2.28.1_ Recruitment maneuver

28.1.1 (In flexiware software, on the right corner, under tasks, click on the 3 dots next to deep inflation) 00:01

28.1.2 (Click on waveform properties) 00: 03

28.1.3 (Set pressure amplitude at 15 cm H2O) 00:05- 00:10

28.1.4 (Click ok) 00:11

28.1.5 (Double click on deep inflation to start the recruitment maneuver) 00:13

JoVE-SCREEN_69084_ step 2.29.1_Compliance

29.1.1 (In flexiware software, on the right corner, under tasks, click on the 3 dots next to PV-V) 00:01

29.1.2 (Click on waveform properties) 00: 03

29.1.3 (Set amplitude relative to weight at 10 mL/kg) 00:06- 00:09

29.1.4 (Click ok) 00:10

29.1.5 (Double click on PV-V to start the compliance measurment) 00:11

29.1.6 (Read on the left side the static pulmonary compliance (Cst) and peak inspiratory pressure (Pmax) values) 00:30