# Submission ID #: 69161

# Scriptwriter Name: Sulakshana Karkala

# Project Page Link: https://review.jove.com/account/file-uploader?src=21094153

# Title: CT-Guided Preoperative Localization of Pulmonary Nodules Using a Glucose Test and Tissue Adhesive

# Authors and Affiliations:

# Zhuoyang Fan123*, Linnan Zhang123*, Xiaohua Gu123*, Xudong Qu123, Juncheng Feng4, Ming Li5

# 1Department of Interventional Radiology, Zhongshan Hospital, Fudan University

# 2Shanghai Institute of Medical Imaging

# 3National Clinical Research Center for Interventional Medicine, Zhongshan Hospital, Fudan University

# 4Department of Cardiothoracic Surgery, Yixing People's Hospital

# 5Department of Thoracic Surgery, Zhongshan Hospital, Fudan University

# *These authors contributed equally

# Corresponding Authors:

Xudong Qu		zs071215@163.com

Juncheng Feng		Staff1141@yxph.com

Ming Li			Dr_liming@163.com

# Email Addresses for All Authors:

Zhuoyang Fan		fan.zhuoyang@zs-hospital.sh.cn

Linnan Zhang		179818159@qq.com

Xiaohua Gu		gu.xiaohua@zs-hospital.sh.cn

Xudong Qu		zs071215@163.com

Juncheng Feng		Staff1141@yxph.com

Ming Li			Dr_liming@163.com

## Author Questionnaire

1. We have marked your project as author-provided footage, meaning you film the video yourself and provide JoVE with the footage to edit. JoVE will not send the videographer. Please confirm that this is correct.

√ Correct

2. Interview statements: Which interview statement filming option is the most appropriate for your group? Please select one.

NOTE: Authors do not want any Introduction section

3. Proposed interview filming date: Please indicate the proposed date that your group will self-film interviews:

When you are ready to submit your video files, please contact our China Location Producer, Yuan Yue.

Current Protocol Length

Number of Steps:  08

Number of Shots:  16

# Introduction

NOTE: Authors do not want any Introduction section

Ethics Title Card

This research has been approved by the Institutional Review Board (IRB) at the Zhongshan Hospital, Fudan University

# Protocol

- CT-Guided Localization of Pulmonary Nodules Using Alpha-Cyanoacrylate Adhesive

Demonstrator: Zhuoyang Fan

NOTE: Scripted from footage

  - To begin, position the patient in a prone position for nodule identification [1]. Use the biopsy grid and a surgical marker to identify and mark the optimal skin entry point [2-TXT].

    - LAB MEDIA: 2.1.MOV 	00:00 – 00:05

    - LAB MEDIA: 2.1.MOV 	00:06-00:18
TXT: Plan needle trajectory and depth based on baseline CT scan

  - After disinfecting the operative field with iodophor, apply sterile drapes to the surgical field [1].

    - LAB MEDIA: 2.2-1.MOV 00:00 – 00:15

  - Infiltrate local anesthesia using 2 percent lidocaine from the skin to the parietal pleura [1]. With a sterile vascular clamp, create a central perforation in a disposable dressing bowl [2].

    - LAB MEDIA: 2.3-1.MOV	00:00 – 00:15

    - LAB MEDIA: 2.2-3.MOV	00:13 – 00:20, 00:23-00:26

  - Invert and position the bowl over the operative site [1], then secure it to the drapes with a vascular clamp [2]. Align the puncture needle through the bowl perforation so that it lightly contacts the marked entry point [3].

    - LAB MEDIA: 2.3-1.MOV	00:16-00:20

    - LAB MEDIA: 2.3-1.MOV	00:21-00:27

    - LAB MEDIA: 2.3-1.MOV	00:29-00:43

  - Next, advance the 22 gauge Chiba needle along the planned trajectory [1]. Shift the entire secured system, including the drapes and bowl to adjust the needle angle or direction [2-TXT].

    - LAB MEDIA: 2.3-3.MOV	00:00 – 00:13

    - LAB MEDIA: 2.3-1.MOV	00:44-00:54
TXT: If pleural re-entry is required, complete it swiftly to reduce risk of pneumothorax

  - Perform intermittent CT scans 2 centimeters from the target after each 1 to 2 centimeter advancement [1-TXT].

    - LAB MEDIA: 2.3-2.MOV 	00:00 – 00:10
TXT: Stop advancement at 5 - 10 mm from pleural surface; readjust needle position if necessary

  - Inject 0.3 milliliters of 5 percent glucose solution slowly at a rate of 0.05 milliliters per second [1]. Then inject 0.3 to 0.5 milliliters of alpha cyanoacrylate medical adhesive [2]. After 3 seconds, withdraw the needle [3-TXT].

    - LAB MEDIA: 2.4&2.5.	MOV 00:00 – 00:11

    - LAB MEDIA: 2.4&2.5.MOV 		00:12 – 00:17

    - LAB MEDIA: 2.4&2.5.MOV		00:18-00:22
TXT: Assess patient for pain, headache and cough

  - Perform a confirmatory CT scan immediately after injection [1]. Verify the presence of a spherical or flake shaped hyperdense marker adjacent to the nodule [2-TXT].

    - LAB MEDIA: 2.6.MOV	 00:00 – 00:07

    - LAB MEDIA: 2.6.MOV	00:08-00:15
TXT: If pneumothorax is seen, repeat scanning after 5 min to assess progression

# Results

- Results

  - Localization of a peripherally located pulmonary lesion by targeting its medial aspect resulted in close spatial correspondence between the tumor nodule [1] and the deposited medical adhesive [2].

    - LAB MEDIA: Figure 1. Video editor: Highlight the area indicated by the red arrow

    - LAB MEDIA: Figure 1 and 2. Video editor: Highlight the area indicated by the blue  arrow in Figure 1 and then show Figure 2

  - Localization of a peripherally located pulmonary lesion by targeting its lateral aspect demonstrated adhesive deposition adjacent to the tumor margin [1] with clear separation between the tumor tissue [2].

    - LAB MEDIA: Figure 3. Video editor: Highlight the region outlined by the red circle

    - LAB MEDIA: Figure 3 and 4 . Video editor: Highlight the region outlined by the blue circle in Figure 3 and then show Figure 4