JoVE69182 – Test plan and experimental procedures

Effect of Change of Direction (COD) Movement on Plantar Pressure and Foot Balance in Bilateral Limbs

Objective

Quantify bilateral plantar pressure/force distribution, Foot Balance Index (FBI), and foot axis angle during a standardized barefoot 45° side-step COD task using a Footscan 2-m pressure plate with controlled approach speed.

Key equipment & software

Footscan 2-m pressure plate (2.0 m × 0.40 m × 0.02 m; 16,384 sensors; 480 Hz) embedded flush with runway using protective mats.

Infrared timing gates (two beams; 2.0 m spacing; beam height 80 cm) to monitor mean speed over final 2 m before plate contact.

10 m laboratory runway; plate located ~6 m from start line; ~2 m post-contact for deceleration.

Footscan software for segmentation/export (analysis software typically off-camera).

Testing workflow

1. Laboratory preparation and system check

Install the pressure plate flush with the runway; confirm no step-height artifact.

Zero and weight-calibrate the system; verify plate serial number and calibration file.

Place timing gates along the approach path: two beams 2.0 m apart (80 cm height), downstream gate aligned to the proximal edge of the active area.

Set required approach speed: 3.5 m·s⁻¹ (±5%).

2. Participant preparation

Confirm eligibility: healthy adults, no lower-limb injury/foot deformity within past 6 months; right-leg dominance (kicking-leg test; ≥2/3 trials).

Obtain written informed consent; demonstrate the COD task.

Clothing: tight-fitting athletic pants; barefoot testing.

Record anthropometrics: height, body mass, foot length, shoe size, arch type.

Warm-up: 10 min slow jog.

3. Static measurement (baseline/segmentation check)

3.1 Posture and setup

Bipedal stance, feet ~shoulder-width, knees extended but not locked, arms relaxed, gaze fixed on a wall mark ~3 m ahead.

Align feet with the plate anterior–posterior axis; avoid underfoot visual targets.

3.2 Recording

After 3–5 s quiet-standing stabilization, record one static snapshot.

Collect 2 valid repetitions; ≥30 s seated rest between trials.

3.3 Validity criteria

Full plantar contact of both feet within active area.

No obvious sway or step adjustment during stabilization window.

No footprint overlap or truncation at plate margins; repeat invalid trials.

4. Dynamic measurement: barefoot 45° side-step COD trials

4.1 Familiarization and start mark

Allow multiple practice runs to establish a natural approach without visual targeting on the plate.

Set an individual fixed start mark to achieve a repeatable step sequence with full turning-foot contact.

4.2 Trial execution (two directions)

Rightward-cut: left foot is the turning/plant foot contacting the plate (right-then-left step sequence).

Leftward-cut: mirror the setup so the right turning/plant foot contacts the plate.

4.3 Speed control and acceptance

Accept a trial only if mean speed over the final 2 m is 3.5 m·s⁻¹ (±5%) and the turning foot achieves full contact.

Discard and repeat trials with partial contact, visible targeting/stride adjustment, or out-of-range speed.

4.4 Repetitions and recording

Record ≥3 valid trials per condition (direction).

Footscan system records pressure/force data automatically at 480 Hz.

5. Data export and processing

Screen trials for missing data/disrupted contact; confirm left/right identification using plantar pressure image and center-of-pressure trajectory.

Apply Footscan 10-region segmentation: T1, T2–T5, M1–M5, MH, LH, MF; manually adjust boundaries if needed.

Export region-wise pressure/force, foot balance, and foot axis angle.

Time-normalize stance to 101 points; normalize regional forces to average vertical force over stance (Zavg).

Compute FBI and foot axis angle from exported data (definitions below).

Outcome variables (definitions)

Regional plantar pressure (kPa) and force/normalized force trajectories across 10 plantar regions (101-point stance).

Foot Balance Index (FBI): medial–lateral loading index during stance (positive = greater medial loading; negative = greater lateral loading).

Foot axis angle: rotation indicator defined by heel midpoint to midpoint between 2nd and 3rd metatarsal (negative = internal, positive = external).

Quality control (quick)

Calibration verified before testing; plate remains flush and stable.

No underfoot visual targets; rely on familiarization + start mark to reduce targeting while ensuring full contact.

Every accepted trial meets speed criterion (3.5 m·s⁻¹ ±5%) and full plantar contact within active area.

≥3 valid trials per condition/direction; archive all raw exports.