TITLE

Exploring the Application of Surface-enhanced Raman Scattering-based Biosensing of Individual sEVs in Disease Diagnosis and Therapeutics

AUTHORS AND AFFILIATIONS

Jun Liu1,*, Siddharth Srivastava1,*, Ya-Hong Xie1

1Department of Materials Science and Engineering, University of California Los Angeles, Los Angeles, CA 90095, USA

*These authors contributed equally

Email address of the corresponding author:

Ya-Hong Xie                                         yhx@ucla.edu

Email addresses of co-authors:

Jun Liu                                                 ljun@g.ucla.edu

Siddharth Srivastava                         sidsri@g.ucla.edu

SUMMARY

Here, we present a protocol to analyze individual small extracellular vesicles (sEVs) using label-free surface-enhanced Raman spectroscopy (SERS), enabling minimally invasive disease diagnostics and assessment of sEVs as therapeutic delivery vehicles.

ABSTRACT

This protocol outlines a comprehensive, label-free platform that integrates surface-enhanced Raman spectroscopy (SERS) with machine learning (ML) to detect and molecularly profile individual small extracellular vesicles (sEVs) for diagnostic and therapeutic applications. The method begins with sEV isolation using either size exclusion chromatography or ultracentrifugation. Isolated vesicles are then analyzed on engineered plasmonic gold nanopyramid 2D array substrates capable of single-vesicle sensitivity. By leveraging intrinsic Raman biochemical fingerprints, the protocol enables high-specificity detection without external labels. Following spectral acquisition, data undergo preprocessing and are analyzed using trained machine learning algorithms (e.g., LDA, SVC) to classify disease states, successfully distinguishing gastric cancer from healthy controls using sEVs from tissue, plasma, and saliva with respective classification accuracies of 90.1%, 70.9%, and 60.7%. Additionally, its therapeutic application is shown by quantifying doxorubicin loading in single sEVs, a measurement enhanced by using graphene-coated substrates as an internal standard. This approach allows for high-throughput analysis that captures population heterogeneity essential for early disease detection and understanding drug loading efficiency at the single-vesicle level.

INTRODUCTION

Small extracellular vesicles (sEVs), typically ranging from 30 to 200 nm in diameter, are naturally secreted by cells into biological fluids and carry proteins, lipids, and nucleic acids indicative of their cell of origin1-3. As non-invasive biomarkers present in saliva, urine, and other body fluids4, they hold promise for early disease diagnosis, prognosis, and therapeutic monitoring, especially in oncology5-7. However, their clinical translation is hindered by their high heterogeneity and low abundance in complex biological matrices8. To overcome such challenges, this protocol leverages surface-enhanced Raman spectroscopy (SERS), a powerful analytical method capable of amplifying Raman scattering by up to 10¹⁰–10¹¹ times via plasmonic substrates9-11. SERS captures intrinsic molecular “fingerprint” spectra of individual sEVs within seconds, enabling ultra-sensitive single-vesicle analysis while avoiding long assay times.

Our method synergistically combines label-free based SERS to achieve rapid, ultrasensitive characterization of individual sEVs12-16. Label-free SERS captures intrinsic biochemical signatures and, when coupled with machine-learning models like LDA, SVC, has been shown to discriminate sEVs derived from healthy versus diseased cells with high specificity13. To further enhance reproducibility and quantitative capability of our platform, our method is fully compatible with graphene-covered Raman substrates16. Single layer graphene offers a chemically uniform surface that stabilizes plasmonic hot spots and enhances molecular adsorption17,18, and more importantly, single layer graphene serves as the “built-in” gauge of SERS signals12,19, which enables the quantification ability of molecules detected. Despite the Raman laser beam diameter (~1 µm) being larger than the typical sEV, our platform achieves effective single-vesicle resolution due to the unique interplay of physical and statistical mechanisms. The quasi-periodic gold nanopyramid substrate generates highly localized plasmonic “hot spots” of sizes smaller than 100 nm that amplify Raman signals in a super linear fashion12. As a result, the signal from the sEV stochastically closest to the most intense hot spot within the beam dominates the spectrum. Additionally, SEM imaging confirms that sEVs are well spaced (> 1 µm apart), minimizing spectral overlap, while Raman intensity maps reveal signal localization that matches the size of individual vesicles13,14. Together, these features ensure that each collected spectrum corresponds predominantly to a single sEV, enabling statistically robust, label-free single-vesicle analysis. Compared to conventional approaches, such as western blot, mass spectrometry, or regular Raman scattering that require large sample volumes and cannot resolve single vesicles20, our single-vesicle SERS platform offers superior sensitivity, throughput, and molecular resolution. This workflow is appropriate for researchers aiming to develop minimally invasive, high-resolution diagnostic tools or to explore sEV heterogeneity and drug-loading efficiency in individualized therapeutic studies. In this protocol, we demonstrate how our SERS platform utilizes single sEVs to enable disease detection and therapeutic applications, particularly in cancer, including the whole workflow of vesicle isolation, plasmonic substrate preparation, Raman acquisition, and data analysis with machine learning.

PROTOCOL

The collection of human specimens was conducted in accordance with the Declaration of Helsinki and was approved by the Institutional Review Board (IRB) of UCLA. All donors provided written informed consent prior to their participation in the study.

NOTE: A total of 4 cell lines, AGS (CRL-1739), A549 (CCL-185), NCI-N87 (CRL-5822), Hs 738.St/Int (CRL-7869), all purchased from a commercial vendor, were used in this study. These cell lines were cultured and passaged strictly according to the vendor’s instructions and reagents. The conditioned media were collected and stored at -20 °C before being processed. At Samsung Medical Center in Korea, donors voluntarily contributed tissue, plasma, and saliva specimens. Gastric cancer tissue was obtained during surgical resection, while biopsies from non-cancer controls were collected via endoscopy. Plasma was gathered in EDTA tubes according to standard clinical protocols. About 5 mL of unstimulated whole saliva was collected from each subject, then centrifuged at 2,600 × g for 15 min at 4 °C. To preserve extracellular RNA, the supernatant was supplemented with SUPERase-In RNase inhibitor at 20 U/mL. All specimens were kept at -80 °C until further processing. sEVs were isolated from conditioned media and clinical specimens following established protocols21.

- sEV isolation and characterization

  - Prepare sEV samples

    - Thaw the cleared cell culture supernatant completely at room temperature.

    - Pass the supernatant through a sterile 0.22 µm syringe filter to remove debris and larger contaminants.

    - Equilibrate a 35 nm size-exclusion chromatography (SEC) column with 1× PBS solution.

    - Apply 500 µL of the filtered sample to the column.

    - Elute the sample with 1x PBS according to the manufacturer's instructions.

    - Collect the EV‑enriched fractions and store them in clean vials at −20 °C.

  - Determine particle concentration and size

    - Dilute sEVs samples in sterile DPBS to a final particle concentration of 10⁷–10⁹ particles/mL.

    - Using a nanoparticle tracking analysis instrument, record five 60 s videos per sample. Use a camera level of 14 and a constant flow rate setting of 25.

    - Analyze the videos with the instrument's software, applying the dilution factor to determine the final particle concentration (particles/mL) and mean diameter (nm).

  - Characterize tetraspanin expression

    - Pre-scan human tetraspanin chips (1 cm × 1 cm, pre-coated with anti-CD9, anti-CD63, and anti-CD81) to establish a baseline signal.

    - Load diluted sEV samples onto the chips in a sealed 24-well plate and incubate overnight at room temperature.

    - Add 35 µL of detection antibodies (anti-CD9 CF488, anti-CD63 CF647, anti-CD81 CF555) and incubate for 1 h at room temperature.

    - Wash the chips according to the manufacturer's protocol.

    - Measure vesicular tetraspanin expression using a biomarker characterization reader and its associated scanning software.

  - Confirm sEV morphology with TEM

    - Fix ~100 µL sEVs with ~100 µL of 4% paraformaldehyde for 30 min.

    - Load approximately 100 µL of fixed sEVs onto 300-mesh copper grids and incubate for 5 min.

    - Rinse the grids with deionized water.

    - Stain the grids with a 1% solution of uranyl acetate (~ 20 µL) in distilled water.

    - Perform a final rinse with water and allow the grids to air-dry completely.

    - Image the samples via transmission electron microscopy (TEM) to confirm sEV size and morphology.

- SERS plasmonic substrate fabrication12,18,19

  - Create nanosphere monolayer

    - Spread 500 nm diameter polystyrene spheres on a water surface within a Langmuir–Blodgett trough.

    - Gently compress the spheres to form a close-packed hexagonal monolayer.

    - Transfer the sphere monolayer onto a 4 inch Si wafer (with ~50 nm thermal SiO₂) by dipping the wafer into the trough and pulling it upward slowly.

  - Etch nanosphere array

    - Reduce the sphere diameter from 500 nm to ~250 nm using a reactive-ion etching system with oxygen plasma (O2 gas flow at 100 sccm, 50 s, 200 W power). For more details of the fabrication process, please refer to the publication from our previous work12,18,19.

    - Deposit a 50 nm Cr layer over the shrunken sphere array via electron-beam evaporation at 1.5 Å per s.

    - Lift off the polystyrene spheres using acetone to leave a perforated Cr mask on the SiO₂ surface.

    - Etch the exposed SiO₂ layer in the Cr openings using a mixture of 25 sccm Ar, 25 sccm CHF3, with RIE power at 200W for 2 min.

  - Fabricate inverted pyramids

    - Immerse the wafer in 60 wt% KOH at 60 °C for 2 min to etch inverted pyramidal pits into the silicon.

    - Remove the Cr mask using a Cr etchant for 5 min.

    - Remove the SiO₂ layer in 6:1 buffered oxide etchant until fully cleared.

    - Deposit a 200 nm Au film conformally over the inverted pyramids using electron-beam evaporation.

  - Finalize plasmonic substrate

    - Apply fast-curing epoxy (~1 mL) to a separate SiO2/Si substrate. Press the gold-coated wafer onto the epoxy to form a sandwich structure with the gold layer in the middle.

    - Peel the epoxy away from the silicon mold to form a free-standing Au plasmonic substrate after the epoxy has fully cured.

- Graphene transfer to plasmonic substrate

  - Prepare graphene film

    - Prepare a 1 M FeCl₃ solution (A 1 M stock solution of anhydrous ferric chloride was prepared by dissolving 81 g of FeCl₃ in deionized water and diluting to a final volume of 500 mL) to etch the copper substrate from a PMMA-coated monolayer graphene sheet.

    - Place the graphene-coated copper sheet face-down on the surface of the FeCl₃ solution until the copper is completely dissolved.

  - Rinse graphene film

    - Using a clean glass slide, carefully transfer the floating graphene film to a deionized water bath to rinse off etchant residues.

    - Repeat this rinsing step three times.

  - Transfer graphene to substrate

    - Float the SERS plasmonic substrate beneath the graphene film in the final DI water bath.

    - Gently lift the substrate to settle the graphene evenly on its surface.

    - Air-dry the graphene-coated substrate at room temperature for 1 h.

    - Transfer it to a vacuum chamber and dry under reduced pressure at 180 °C for 1 h.

  - Clean final substrate

    - Immerse the dried substrate in an acetone bath at 50 °C for 1 h to dissolve the PMMA.

    - Rinse the substrate three times in isopropyl alcohol.

    - Dry the cleaned, graphene-transferred plasmonic substrate in a desiccator before use.

- SERS sample preparation and spectra acquisition

  - Prepare sample on substrate

    - Pipette approximately 5 µL of the sEV sample onto the appropriate plasmonic substrate.

NOTE: For quantification of drug-loaded sEVs, apply vesicles to the gold-graphene substrate. For all other experiments, use a plain gold plasmonic substrate.

    - Place the substrate in a desiccator and allow the droplet to dry completely (~15 min).

  - Calibrate Raman system

    - Transfer the substrate to a confocal Raman microscope.

    - Initiate data collection using the instrument's software (WiRE version 4.4).

    - Set the excitation laser to 785 nm at 5 mW.

    - Calibrate the system by aligning the silicon reference peak to 520 cm⁻¹.

  - Acquire spectra from gold substrate (diagnostics)

    - Perform a scouting scan over a 300 µm × 300 µm area to locate single vesicles using static mode with the following settings: 50% power, 0.1 s exposure, 10 µm step size.

    - After locating sEVs, conduct high-resolution mapping in static mode within a 5 × 5 grid using the following settings: 1 µm step size, 50% power, 0.2 s exposure per point.

  - Acquire spectra from graphene substrate (therapeutics)

    - Perform a scouting scan over a 300 µm × 300 µm area to locate individual vesicles using static mode with the following settings: 50% laser power, 0.1 s exposure, 10 µm step size.

    - Perform high-resolution mapping in extended mode with a 2 × 2 grid using the following settings: 1 µm step size, 1% laser power, 10 s exposure.

- Data preprocessing

  - Perform spectral quality control

    - Screen initial raw spectra to exclude highly noisy signals or spectra with abnormal shapes.

    - Remove isolated cosmic ray spikes using threshold-based detection or local deviation filtering.

  - Filter and normalize spectra

    - Apply a Savitzky-Golay filter to each spectrum to reduce fluorescence background and smooth the data.

    - Calculate the signal-to-noise ratio (SNR) of each spectrum, typically using either the peak-to-baseline ratio or standard-deviation method. To explain further, typically the SNR for each spectrum was calculated as the ratio of the peak intensity (after baseline subtraction) at a representative Raman band to the standard deviation of the noise. The noise was estimated from the residual spectrum obtained by subtracting a Savitzky-Golay-smoothed version of the spectrum (window size = 11, polynomial order = 3) from the original spectrum, thereby isolating high-frequency fluctuations. To empirically determine an appropriate SNR threshold, all spectra were ranked in ascending order of SNR, and the persistence of characteristic biochemical peaks - specifically the phenylalanine band (~1002 cm⁻¹, amino acid), the amide I band (~1650 cm⁻¹, protein), and the CH₂ deformation band (~1440 cm⁻¹, lipid) was evaluated across the ranked spectra. The threshold was set at the point where any one of these diagnostic peaks consistently disappeared into the baseline noise. For our gold-nanopyramid SERS substrate and acquisition parameters (785 nm excitation, 5 mW laser power, 0.2 s integration), this empirically corresponded to an SNR threshold of 28.

    - Exclude any spectra that fall below the SNR threshold as decided according to 5.2.2 above. In our workflow, spectra with a SNR below 28 were excluded from the training dataset, since setting the threshold at this level ensures that only high-quality spectra are retained for downstream analysis and the improvement of the model robustness.

    - Normalize each spectrum to either the maximum peak intensity or the total area under the curve.

- Spectral analysis with machine learning

  - Train the Model

    - Assign a label to each spectral measurement corresponding to its origin (e.g., gastric cancer patient vs. healthy control).

    - Input the spectral data into a Support Vector Classifier (SVC) with a linear kernel.

    - Use a stratified shuffle split for 5-fold cross-validation.

    - Average the accuracy metrics from each fold to assess model performance.

  - Reduce dimensionality and classify

    - Use Linear Discriminant Analysis (LDA) to visualize high-dimensional SERS data in reduced dimensions.

    - Use the LDA model for direct classification between sample groups.

  - Evaluate the Model

    - Evaluate the trained models on a holdout set of blinded patient samples to simulate a real-world diagnostic scenario. For more information, please refer to these publications from our group13,15,22.

NOTE: Perform all steps with clean tools in a dust-free environment. Monolayer graphene was grown or sourced on a copper substrate and coated with a PMMA layer to protect it during transfer.

REPRESENTATIVE RESULTS

Successful implementation of the single-vesicle SERS protocol yields distinct Raman spectra from individual sEVs, allowing downstream diagnostic classification and therapeutic drug loading monitoring. Figure 1 depicts the workflow of the diagnostic study for gastric cancer and the drug-loaded sEVs monitoring application using our protocol reported here, respectively.

[Place Figure 1 here]

In the diagnostic context, SERS spectra were acquired from tissue, plasma, and saliva-derived sEVs of gastric cancer patients and healthy controls. Following preprocessing, Linear Discriminant Analysis (LDA) and Support Vector Machine Classifier (SVC) revealed distinguishable molecular fingerprints between groups. Machine learning classifiers trained on these spectral datasets achieved classification accuracies of 90.1%, 70.9%, and 60.7% for tissue, plasma, and saliva samples, respectively. This demonstrates the diagnostic potential and fluid-specific variance in performance. Moreover, single-vesicle mapping uncovered patient-specific sEV subtypes traceable across bodily fluids, offering insights into vesicle biogenesis and inter-compartmental trafficking.

In therapeutic applications, the protocol enables quantification of drug loading at the single-vesicle level using doxorubicin (DOX) as the target cargo. SERS spectra from graphene covered substrates were used to monitor characteristic DOX peaks (e.g., at 442 cm⁻¹). The G peak from graphene served as an internal gauge to normalize local enhancement factors and deconvolve the influence of heterogeneous hotspot intensities. Drug loading efficiency was found to vary significantly across individual vesicles. Some vesicles exhibited disproportionately strong DOX signals, potentially representing subpopulations with higher loading capacity.

[Place Figure 2 here]

Figure 2 summarizes key findings for gastric cancer detection based on Raman spectral fingerprints acquired from single sEVs isolated from tissue, blood, and saliva samples. Panels (A-C) show the Raman spectra with the corresponding average spectra, revealing distinct molecular signatures across each sample type. Panel (D) demonstrates clear clustering of the EV data based on their sample source using Linear Discriminant Analysis (LDA), emphasizing significant differences between tissue, blood, and saliva-derived EVs. The binary classification performance using a Support Vector Machine (SVM) classifier (panel E) shows the highest classification accuracy for tissue samples (90.1%), followed by blood (70.9%) and saliva (60.7%) samples, indicating varying predictive capabilities depending on the EV source. Panels (F-H) illustrate the LDA-based classification split, visually demonstrating the predictive separation between healthy and unhealthy (gastric cancer) samples across each sample type, further highlighting the superior discriminative power of tissue-derived EV data compared to blood and saliva. These outcomes validate that this protocol enables reproducible, sensitive, and spatially resolved analysis of individual sEVs for both disease detection and therapeutic drug loading monitoring.

[Place Figure 3 here]

Figure 3 demonstrates that integrating graphene into the SERS platform enables reliable, quantitative monitoring of DOX uptake in sEVs. In panels (A)–(B), the distinct DOX Raman bands at approximately 1081, 1206, and 1440 cm⁻¹ are observable both with and without graphene. The inclusion of graphene, however, introduces stable D (∼1350 cm⁻¹) and G (∼1580 cm⁻¹) bands, which serve as internal intensity references for ratiometric quantification. Panel (C) highlights that the ratio of a representative DOX peak (442 cm⁻¹) to graphene G band increases progressively with higher DOX concentration and longer incubation, confirming that graphene incorporated SERS permits sensitive, quantitative tracking of DOX loading in sEVs.

FIGURE AND TABLE LEGENDS

Figure 1: Flow chart of the study. (A) Workflow for single-vesicle SERS-based diagnostics of gastric cancer. Clinical vesicles are isolated and deposited on plasmonic substrates, Raman-scanned individually, and analyzed using multivariate machine learning to achieve disease discrimination and subpopulation tracking. (B) Workflow for single-vesicle quantification of therapeutic loading. Drug-incubated vesicles are purified, deposited on graphene covered substrates, and analyzed via SERS. The graphene internal reference corrects for hotspot variability, enabling quantitation of drug uptake on a per-vesicle basis.

Figure 2: Single-sEV Raman spectra enable machine learning-based gastric cancer detection across bodily fluids. (A-C) Raman spectral fingerprints collected from single extracellular vesicles isolated from tissue, blood, and saliva samples. (D) LDA scatter plot demonstrates clear clustering of data by sample source. (E) Binary classification scores using Support Vector Machine Classifier, 5-fold cross-validation(F-H) Classification results using Linear Discriminant Analysis (LDA) for the samples from 3 sources.

Figure 3: Graphene-integrated SERS empowers single‑vesicle level, ratiometric quantification of DOX uptake in sEVs, correlating signal intensity with drug concentration and incubation time. (A–B) Overlaid Raman spectra of DOX with and without graphene on the SERS substrate show consistent DOX peaks at ~1081, 1206, and 1440 cm⁻¹ (blue circles). With graphene, additional D (~1350 cm⁻¹) and G (~1580 cm⁻¹) bands (purple circles) emerge, serving as internal standards for normalization. (C) The bar chart plots the ratio of a representative DOX peak (442 cm⁻¹) to the graphene G band across varying DOX concentrations (0.1, 0.5 mg/mL) and incubation times (0.5, 1, 2 h). The DOX-to-G ratio increases with both concentration and time, illustrating dose and time dependent loading of DOX into sEVs.

Figure S1: Single-vesicle resolution achieved through gold pyramidal array. (A) Schematics of FDTD simulation. (B) FDTD simulation result. (C-D) SEM image of SERS substrate without sEVs and with sEVs, respectively. (E) SERS spectra from the center and margin of a single sEV. (F) Raman intensity heatmap of a single sEV.

DISCUSSION

A pivotal aspect of this protocol is the substrate preparation and sEV purification. The performance of label-free SERS depends strongly on producing uniform, high conformal plasmonic substrates with strong enhancement “hot spot”. Likewise, high-purity sEV isolation via size-exclusion chromatography is essential to render reliable and reproducible SERS spectra. Impurities such as protein aggregates or lipoproteins not only weaken vesicle signal but can confound downstream machine-learning classification by introducing spectral noise.

A significant advantage of our platform is its validated single-vesicle resolution, a crucial factor for overcoming the challenge of sEV heterogeneity. While the laser spot is larger than a single vesicle, the signal is dominated by the sEV closest to a plasmonic hotspot due to the super-linear signal amplification from the gold nanopyramid geometry. This, combined with low vesicle density on the substrate, ensures each spectrum represents a molecular snapshot of an individual sEV. This capability allows for the identification of distinct sEV subpopulations that are otherwise obscured in bulk analysis methods like Western blot or mass spectrometry.

The clinical power of this protocol is demonstrated by its successful application across multiple biofluids from the same patient cohort—tissue, plasma, and saliva. Our findings show that while tissue-derived sEVs provide the highest diagnostic accuracy (90.1%), clinically accessible fluids like plasma (70.9%) and saliva still offer significant classification power. This multi-fluid approach provides a more comprehensive view of disease biology and opens avenues for developing minimally invasive screening tools that correlate with the primary disease site.

Throughput remains a key limitation in single-vesicle analysis compared to bulky analysis, and the intrinsic diversity of sEV populations necessitates large datasets for robust statistical confidence. Current systems still face low scanning efficiency, and future developments must focus on increasing areal density and scanning speed. To enhance throughput and automate detection, we incorporated a custom software module that autonomously scouts the Raman stage, identifies candidate vesicles, and triggers spectral measurements. This automated scouring replaces manual targeting, significantly accelerating sample acquisition and minimizing human bias, transitioning from tens to hundreds of vesicles per session.

Furthermore, the integration of graphene onto the SERS substrate is a key innovation for

quantitative therapeutic monitoring. As demonstrated in our previous work, the stable G-peak of graphene serves as an ideal internal standard to normalize the signal from therapeutic cargo, such as doxorubicin. This ratiometric approach corrects for substrate and system variations, enabling reliable, semi-quantitative assessment of drug loading at the single-vesicle level. This provides invaluable insights into loading efficiency and heterogeneity, which are critical for developing potent sEV-based drug delivery systems.

This protocol presents critical advantages over conventional methods. Unlike Western blot and mass spectrometry, which pool vesicle populations and obscure heterogeneity, single-vesicle SERS operates at the individual level. Our approach, through plasmonic enhancement and machine learning, offers label-free, biochemical content analysis capable of identifying subpopulations and rare vesicle signatures, avoiding the pitfalls of population-averaged methods.

ACKNOWLEDGMENTS

This work was supported by the National Center for Advancing Translational Sciences of the National Institutes of Health under award numbers 4UH3TR002978‑03 and 1U18TR003778‑01, and by a grant from the California Institute for Regenerative Medicine (CIRM) (Grant Number TRAN1-14649).

DISCLOSURES

The authors declare that they have no conflicts of interest related to this work.

REFERENCES

- Van Niel, G. et al. Challenges and directions in studying cell-cell communication by extracellular vesicles. Nat Rev Mol Cell Biol. 23 (5), 369-382 (2022).

- Herrmann, I. K., Wood, M. J. A., Fuhrmann, G. Extracellular vesicles as a next-generation drug delivery platform. Nat Nanotechnol. 16 (7), 748-759 (2021).

- Raposo, G., Stoorvogel, W. Extracellular vesicles: Exosomes, microvesicles, and friends. J Cell Biol. 200 (4), 373-383 (2013).

- Möller, A., Lobb, R. J. The evolving translational potential of small extracellular vesicles in cancer. Nat Rev Cancer. 20 (12), 697-709 (2020).

- Mathieu, M., Martin-Jaular, L., Lavieu, G., Thery, C. Specificities of secretion and uptake of exosomes and other extracellular vesicles for cell-to-cell communication. Nat Cell Biol. 21 (1), 9-17 (2019).

- Alvarez-Erviti, L. et al. Delivery of siRNA to the mouse brain by systemic injection of targeted exosomes. Nat Biotechnol. 29 (4), 341-347 (2011).

- Kahroba, H., Hejazi, M. S., Samadi, N. Exosomes: From carcinogenesis and metastasis to diagnosis and treatment of gastric cancer. Cell Mol Life Sci. 76 (9), 1747-1758 (2019).

- Ferguson, S. W., Nguyen, J. Exosomes as therapeutics: The implications of molecular composition and exosomal heterogeneity. J Control Release. 228, 179-190 (2016).

- Stiles, P. L., Dieringer, J. A., Shah, N. C., Van Duyne, R. R. Surface-enhanced Raman spectroscopy. Annu Rev Anal Chem. 1, 601-626 (2008).

- Jeon, T. Y., Kim, D. J., Park, S. G., Kim, S. H., Kim, D. H. Nanostructured plasmonic substrates for use as SERS sensors. Nano Converg. 3, 1-10 (2016).

- Nie, S., Emery, S. R. Probing single molecules and single nanoparticles by surface-enhanced Raman scattering. Science. 275 (5303), 1102-1106 (1997).

- Wang, P. et al. Ultra-sensitive graphene-plasmonic hybrid platform for label-free detection. Adv Mater. 25 (35), 4918-4924 (2013).

- Liu, Z. R. et al. Gold nanopyramid arrays for non-invasive surface-enhanced Raman spectroscopy-based gastric cancer detection via sEVs. ACS Appl Nano Mater. 5 (9), 12506-12517 (2022).

- Jonak, S. T. et al. Analyzing bronchoalveolar fluid derived small extracellular vesicles using single-vesicle SERS for non-small cell lung cancer detection. Sens Diagn. 2 (1), 90-99 (2023).

- Liu, Z. R. et al. Label-free single-vesicle based surface enhanced Raman spectroscopy: A robust approach for investigating the biomolecular composition of small extracellular vesicles. PLoS One. 19 (6), e0304287 (2024).

- Liu, J. et al. On the feasibility of SERS-based monitoring of drug loading efficiency in exosomes for targeted delivery. Biosensors (Basel). 15 (3), 198 (2025).

- Ling, X. et al. Can graphene be used as a substrate for Raman enhancement? Nano Lett. 10 (2), 553-561 (2010).

- Wang, P. et al. Giant optical response from graphene-plasmonic system. ACS Nano. 6 (7), 6244-6249 (2012).

- Wang, P. et al. Label-free SERS selective detection of dopamine and serotonin using graphene–Au nanopyramid heterostructure. Anal Chem. 87 (20), 10255-10261 (2015).

- Qiu, L. et al. Current advances in technologies for single extracellular vesicle analysis and its clinical applications in cancer diagnosis. Biosensors (Basel). 13 (1), 60 (2023).

- Akbar, N., Pinnick, K. E., Paget, D., Choudhury, R. P. Isolation and characterization of human adipocyte-derived extracellular vesicles using filtration and ultracentrifugation. J Vis Exp. 170, e64102 (2021).

- Li, T. et al. Label-free SARS-CoV-2 detection platform based on surface-enhanced Raman spectroscopy with support vector machine spectral pattern recognition. Eng Sci. 23, 862-870 (2023).