2.5.  Using LAS X (Las-Ex) software, define the region of interest where photobleaching will be performed [1]. Set lasers to 405 nanometers at 0.00, 488 nanometers at 0.20, and 561 nanometers at 0.20, and ensure the cells are in focus [2]. Then acquire a series of pre-bleach images at low laser intensity to establish baseline fluorescence intensity.

69628_screenshot_1

2.5.1.    SCREEN: ROI is being defined on the LAS-X software. [0:21-0:50]

69628_screenshot_1

2.5.2.    SCREEN: The laser intensity is being changed and the cells are being focused. [0:50-0.51]

2.6     Now apply a high-intensity laser pulse to bleach the region of interest by setting FRAP lasers to 405 nanometers at 4.00, 488 nanometers at 0.00, and 561 nanometers at 0.00 [1]. Set the pre-bleach to 5.140 seconds for 59 iterations and define the post-bleach timing [2].

69628_screenshot_1

2.6.1.    SCREEN: Laser settings showing high-intensity values configured for bleaching. [0:51-1.24]

2.7.     For post-bleach imaging, continue to acquire images at low laser intensity for 10 minutes using FRAP lasers at 405 nanometers at 0.00, 488 nanometers at 0.50, and 561 nanometers at 0.50 [1].

69628_Screenshot _2

    - 2.7.1.    SCREEN: The laser intensity is being changed for post-bleach imaging. [0:00-0:21]

  - To profile the merged green and red mitochondria, first install the RGB profiler plugin [1]. Open ImageJ software [2], then from the File menu, locate and open the image file to analyse [3].

69268_Screenshot_3

    - SCREEN: Fiji interface showing RGB profiler plugin being installed from the plugin menu. [0:00-0:16]

69628_Screenshot _4

    - SCREEN: Desktop shortcut of ImageJ being opened. [0:00-0:16]

69628_Screenshot _5

    - SCREEN: File browser in ImageJ selecting an image file. [0:11-0:26]

  - Select the color mode composite, check the Auto Scale option, and open the video or image stack to analyze [1]. Draw a rectangular area over the image [2] and right-click to select Duplicate [3].

69628 _Screenshot _5

    - SCREEN: ImageJ interface showing color mode set to composite and Auto Scale checkbox enabled then the video or image stack is being opened. [0:22-0:35]

69628 _Screenshot _5

    - SCREEN: Rectangular selection tool being used to highlight a section of mitochondria. [0:36-0:42]

69628_ Screenshot _5

    - SCREEN: Right-click context menu with Duplicate option selected. [0:42-0:46]

  - Select the duplicate hyper stack option to duplicate all the image stacks [1]. Select the image and choose RGB in the Image dropdown menu To convert the duplicated stack to RGB format [2].

69628_ Screenshot _5

    - SCREEN: Dialog box showing Duplicate Hyperstack option checked. [0:46-0:51]

69268_Screenshot_6]

    - SCREEN: Image > Type > RGB option selected in the ImageJ menu.[ [0:00-0:28]

  - Duplicate the RGB image and select only one frame [1]. If the mitochondria appear curved, draw a line along their length [2], then go to the Edit menu, choose Selection, and click Straighten [3].

  - 69628_figure5_Screenshot _7

    - SCREEN: RGB image being duplicated and one frame selected. [0:00-0:12]

69628_Screenshot _8

    - SCREEN: Curved mitochondrion are being traced. [0:02-0:04]

69628_ Screenshot _8

    - SCREEN: Edit > Selection > Straighten option selected. [0:04-0:09]

  - Now choose a title for the straightened image and set the line width to 20 pixels [1]. Draw a line along the straightened mitochondrion to align it properly [2] and open the RGB profiler plugin [3].

69628_Screenshot _8

    - SCREEN: Input field for image title and line width set to 20 pixels. [0:09-0:12]

69628_Screenshot _8

    - SCREEN: Line tool used to draw along the mitochondrion in the straightened image. [0:12-0:31]

69628_Screenshot _8

    - SCREEN: RGB profiler plugin interface being launched. [0:31-0:038]

  - Once activated, the plugin will generate a plot showing red and green fluorescence intensities along the drawn line [1]. The height of each peak will be directly proportional to the fluorescence intensity [2].

69628_Screenshot _8

    - SCREEN: Plot showing two lines corresponding to red and green signals across the selected path. [0:39-0.41]

69628_Screenshot _8

    - SCREEN: Peak heights visualized and labeled to indicate intensity values. [0:41-0.51]

  - Before mitochondrial fusion, confirm that red fluorescence is localized to one mitochondrion, and green fluorescence is localized to an adjacent one [1]. Verify that the corresponding intensity plot displays two distinct peaks with minimal overlap [2].

69628_Screenshot_8

    - SCOPE/SCREEN: Mitochondrial pair with clearly separated red and green fluorescence are being seen. [0:41-0.51]
AUTHORS: Please use the cursor to show the clearly differentiated colors

69628_Screenshot _8

    - SCREEN: Intensity plot showing distinct red and green peaks.
AUTHORS: Please use the cursor to hover over the 2 distinct peaks [0:41-0.51]

  - After fusion, confirm that the red fluorescence has spread into the adjacent green mitochondrion indicating content mixing [1]. Verify that the intensity plot now shows overlapping red and green peaks [2].

69628_Screenshot _9

    - SCOPE/SCREEN: Fused mitochondrion showing blended red and green signal. Peaks  [0:00-0.06]

69628_Screenshot _9

    - SCREEN: Updated plot showing overlapping peaks. [0:06-0.21]

  - To assess fusion, select healthy mitochondria and photobleached mitochondria for fusion in live cell imaging [1]. Monitor the change in red to green fluorescence ratio over time to indicate fusion events [2].

69628_Screenshot _10

    - SCOPE/SCREEN: Field showing selection of intact and bleached mitochondria. [0:00-0.39]

69628_Screenshot _11

    - SCREEN: Time-series graph displaying the ratio shift of red to green fluorescence over time. [0:00-0.54]