Submission ID #: 69373

Scriptwriter Name: Sulakshana Karkala

Project Page Link: https://review.jove.com/account/file-uploader?src=21158813

Title: Thermal Behavior and Power Efficiency Comparison of AC vs. DC Electrical Heating in a Distillation Column Using Infrared Thermography Analysis

Authors and Affiliations:

Gregorio Moreno-Sotelo, Adriana del Carmen Téllez-Anguiano, Mario Heras-Cervantes

Tecnológico Nacional de México, Instituto Tecnológico de Morelia Morelia

Corresponding Authors:

Adriana del C. Téllez-Anguiano                           adriana.ta@morelia.tecnm.mx

Email Addresses for All Authors:

Gregorio Moreno-Sotelo                                     gregorio.ms@morelia.tecnm.mx

Adriana del C. Téllez-Anguiano                           adriana.ta@morelia.tecnm.mx

Mario Heras-Cervantes                                        mario.hc@morelia.tecnm.mx

## Author Questionnaire

1. We have marked your project as author-provided footage, meaning you film the video yourself and provide JoVE with the footage to edit. JoVE will not send the videographer. Please confirm that this is correct.

√ Correct

2. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

3. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes, all done

4. Proposed filming date: To help JoVE process and publish your video in a timely manner, please indicate the proposed date that your group will film here: 01/17/26

Current Protocol Length

Number of Steps: 18

Number of Shots: 34

# Introduction

INTRODUCTION:

What is the scope of your research? What questions are you trying to answer?

  - Gregorio Moreno: The scope of my research is to demonstrate the advantage of using AC and DC power supplies in a heating element within a distillation column boiler.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

What are the most recent developments in your field of research?

  - Gregorio Moreno: Recent studies address thermographic analysis and AC/DC-powered distillation columns separately but no work combines both approaches.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

CONCLUSION:

What advantage does your protocol offer compared to other techniques?

  - Gregorio Moreno: Our protocol uses TIR analysis to provide a clearer view of thermal behavior in relation to power sources within the process.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

How will your findings advance research in your field?

  - Gregorio Moreno: Our findings show that selecting appropriate AC or DC supplies optimizes performance, reduces energy costs, and improves distillate quality.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

What questions will future research focus on?

  - Gregorio Moreno: Future research will investigate thermogram-based early fault detection methods and how thermal actuator fatigue influences process transient behavior.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

# Protocol

- Thermographic Data Acquisition and Image Processing for Boiler Plate Temperature Measurement

Demonstrator: Gregorio Moreno Sotelo

  - To begin, set up the thermal camera for acquisition [1].  Select the Iron color palette from the thermal camera menu and ensure it is correctly applied before proceeding [2].

    - WIDE: Talent standing in front of the experimental setup with the thermal camera and boiler plate visible.

    - SCREEN: 69373_screenshot_2.mp4	00:00-00:11

  - Set the emissivity correction value to 0.95 from the thermal camera menu [1].

    - SCREEN: 69373_screenshot_3.mp4 	00:00-00:10

  - Then position the thermal camera approximately 80 centimeters away from the boiler plate [1]. Holding the thermal camera approximately 60 centimeters above the floor, orient the camera toward the boiler plate in the distillation column [2].

    - Talent positioning the thermal camera at the specified distance from the boiler plate.

    - Talent holding the thermal camera at the specified height above the floor and aligning the camera toward the boiler plate in the distillation column

  - Press the save button to record the thermogram to the thermal camera memory [1].

    - Talent pressing the save button on the thermal camera.

  - Capture thermograms every 5 seconds during samples 1 to 1500 on the local interface [1]. Repeat the acquisition for 20, 60, and 100 volts under alternating current and direct current conditions [2].

    - SCREEN: 69373_screenshot_7.mp4 	00:00-00:20

    - SCREEN: 69373_screenshot_8c.mp4 	00:00-00:10
                69373_screenshot_8b.mp4 	00:20-00:25
                69373_screenshot_8a.mp4 	00:23-00:28
AND
TEXT ON PLAIN BACKGROUND:

For 100 V (AC and DC) : Capture every 10 s (samples 1500 – 2800)
                                                       Capture every 20 s ( samples 2800 until steady state)
For 20 V (AC and DC) : Capture every 10 s (samples 1500 to 5500)
                                                     Capture every 20 s ( samples 5500 until steady state)
For 60 V (AC and DC) : Capture every 15 s (samples 1500 to 4000)
                                                     Capture every 30 s ( samples 4000 until steady state)
Video Editor: Please play shots side by side in a split screen

  - Record the name of each captured thermogram and the corresponding boiler plate temperature sample displayed on the local interface [1].

    - SCREEN: 69373_screenshot_9.mp4	 00:00-00:06, 00:09-00:13

  - To begin acquisition of boiler plate temperature data, store the temporary files generated by the local process interface [1]. Rename and save the stored files into the current process folder [2].

    - SCREEN: 69373_screenshot_10.mp4	 	00:20-00:37

    - SCREEN: 69373_screenshot_11.mp4	 	00:00-00:15,00:20-00:23

  - Collect the .csv (C-S-V) files generated during the test and generate a single file containing all acquired process data [1]. Save the consolidated file for analysis [2].

    - SCREEN: 69373_screenshot_12.mp4	 	00:33-00:50

    - SCREEN: 69373_screenshot_13.mp4	 	00:00-00:20

  - To begin preparation of the data, extract thermograms from the thermal camera [1]. Transfer all image files to the analysis workstation [2]. Review the extracted thermograms and discard any unwanted images [3].

    - Talent connecting the thermal camera to the workstation.

    - SCREEN: 69373_screenshot_15.mp4	 	00:15-00:29

    - SCREEN: 69373_screenshot_16.mp4	 	00:00-00:12

  - Manually mark a reference point on each thermogram [1]. Then create a process temperature overview graph using the previously generated process data file [2].Relate the boiler temperature data from the local interface to the captured process thermograms [3].

    - SCREEN: 69373_screenshot_17.mp4	 	00:09-00:16

    - SCREEN: 69373_screenshot_18.mp4	 	00:00-00:13

    - SCREEN: 69373_screenshot_19.mp4	 	00:08-00:22

  - To develop an algorithm for thermogram processing, first load an image into a variable within the analysis environment [1]. Define the size of the vectors for the graph axes using the image variable [2]. Then define the area for detecting minimum and maximum temperature ranges based on the analysis image [3].

    - SCREEN: 69373_screenshot_20.mp4	 	00:00-00:16

    - SCREEN: 69373_screenshot_21.mp4	 	00:00-00:07

    - SCREEN: 69373_screenshot_22.mp4	 	00:21-00:42

  - Prepare the figure for graphing [1].

    - SCREEN: 69373_screenshot_23.mp4	 	00:00-00:10

  - Next, load the thermogram image into a variable for processing. Define a search area for the previously marked reference point [1].

    - SCREEN: 69373_screenshot_24.mp4	 	00:00-00:20

  - Isolate the reference point and obtain its coordinates [1]. Then use the coordinates to indicate the analysis area [2].

    - SCREEN: 69373_screenshot_25.mp4	 	00:00-00:12

    - SCREEN: 69373_screenshot_26.mp4	 	00:00-00:12

  - Now, convert the analysis area to grayscale [1]. Calculate the mean temperature value of the grayscale area [2].

    - SCREEN: 69373_screenshot_27.mp4	 	00:00-00:09

    - SCREEN: 69373_screenshot_28.mp4	 	00:08-00:22

  - Isolate the maximum and minimum temperature areas in the image [1]. Convert the graphical numbers to numeric text [2].

    - SCREEN: 69373_screenshot_29.mp4	 	00:00-00:15

    - SCREEN: 69373_screenshot_30.mp4	 	00:00-00:10

  - Convert the mean values to scaled maximum and minimum temperatures according to the image ranges [1], and store the total mean values and corresponding labels [2].

    - SCREEN: 69373_screenshot_31.mp4	 	00:00-00:19

    - SCREEN: 69373_screenshot_32.mp4	 	00:30-00:58

  - Lastly, plot the total mean results to visualize the processed thermogram data [1].

    - SCREEN: 69373_screenshot_33.mp4	 	00:00-00:15

# Results

- Results

  - The temperature measurements using 100-volt DC varied throughout the process, while the measurements using 100-volt AC remained more stable [1-TXT].

    - LAB MEDIA: Figure 5A. 
TXT: DC: Direct Current; AC: Alternating Current
Video editor: Please sequentially highlight the orange (VDC) and blue (VAC) lines

  - At 100-volt DC, thermal variation across the boiler section was higher than with alternating current [1]. DC presented more controlled areas due to smoother progression and increasing but not abrupt dispersion [2].

    - LAB MEDIA: Figure 6 A, B and C,D . Video editor: Please highlight 6C and D

    - LAB MEDIA: Figure 6E. Video editor: Please highlight the blue line

  - Under 60-volt conditions, temperature measurements were more stable with AC [1].

    - LAB MEDIA: Figure 5B. Video editor: Emphasize the blue smoother VAC curve

  - Thermal evolution using 60-volt AC showed a gradual temperature increase during start-up [1], whereas with 60 volt DC, the heating was more pronounced and rapid [2].

    - LAB MEDIA: Figure 9B. Video editor: Please sequentially show each image. Emphasize the box with the temperature on the top left corner of each image

    - LAB MEDIA: Figure 10B. Video editor: Please sequentially show each image. Emphasize the box with the temperature on the top left corner of each image

  - The graphical temperature trend at 20 volts showed noticeable variation DC [1], while AC resulted in a more gradual and stable profile [2].

    - LAB MEDIA: Figure 5C. Video editor: Highlight the orange VDC line with visible fluctuations.

    - LAB MEDIA: Figure 11A. Video editor: Highlight the smooth upward trend of the VAC line.

  - Thermograms under 20-volt DC showed temperatures ranging from 22.8 degrees Celsius to 26.3 degrees Celsius [1].

    - LAB MEDIA: Figure 11B. Video editor: Highlight each thermogram from IR_3068 to IR_3178, Emphasize the box with the temperature on the top left corner of each image

Pronunciation Guide:

  Thermography
Pronunciation link: https://www.merriam-webster.com/dictionary/thermography
IPA: /θərˈmɑːɡrəfi/
Phonetic Spelling: thur·maa·gruh·fee

  Thermographic
Pronunciation link: https://www.merriam-webster.com/dictionary/thermographic
IPA: /ˌθərməˈɡræfɪk/
Phonetic Spelling: thur·muh·graf·ik

  Distillation
Pronunciation link: https://www.merriam-webster.com/dictionary/distillation
IPA: /ˌdɪstəˈleɪʃən/
Phonetic Spelling: dis·tuh·lay·shuhn

  Infrared
Pronunciation link: https://www.merriam-webster.com/dictionary/infrared
IPA: /ˌɪnfrəˈrɛd/
Phonetic Spelling: in·fruh·red

  Emissivity
Pronunciation link: https://www.merriam-webster.com/dictionary/emissivity
IPA: /ˌiːmɪˈsɪvɪti/
Phonetic Spelling: ee·mi·si·vuh·tee

  Thermogram
Pronunciation link: https://www.merriam-webster.com/dictionary/thermogram
IPA: /ˈθɜːrməˌɡræm/
Phonetic Spelling: thur·muh·gram

  Alternating Current
Pronunciation link: https://www.merriam-webster.com/dictionary/alternating%20current
IPA: /ˈɔːltərˌneɪtɪŋ ˈkɜːrənt/
Phonetic Spelling: awl·ter·nay·ting kur·uhnt

  Direct Current
Pronunciation link: https://www.merriam-webster.com/dictionary/direct%20current
IPA: /dəˈrɛkt ˈkɜːrənt/
Phonetic Spelling: duh·rekt kur·uhnt

  Thermogram-Based
Pronunciation link: No confirmed link found
IPA: /ˈθɜːrməˌɡræm beɪst/
Phonetic Spelling: thur·muh·gram bayst

  Actuator
Pronunciation link: https://www.merriam-webster.com/dictionary/actuator
IPA: /ˈæk.tʃuˌeɪtər/
Phonetic Spelling: ak·choo·ay·ter

  Thermographic Analysis
Pronunciation link: No confirmed link found
IPA: /ˌθərməˈɡræfɪk əˈnæləsɪs/
Phonetic Spelling: thur·muh·graf·ik uh·nal·uh·sis

  Emissivity Correction
Pronunciation link: No confirmed link found
IPA: /ˌiːmɪˈsɪvɪti kəˈrɛkʃən/
Phonetic Spelling: ee·mi·si·vuh·tee kuh·rek·shuhn

  Grayscale
Pronunciation link: https://www.merriam-webster.com/dictionary/grayscale
IPA: /ˈɡreɪˌskeɪl/
Phonetic Spelling: gray·skayl

  Thermodynamic
Pronunciation link: https://www.merriam-webster.com/dictionary/thermodynamic
IPA: /ˌθɜːrmoʊdaɪˈnæmɪk/
Phonetic Spelling: thur·moh·dye·nam·ik

  Transient
Pronunciation link: https://www.merriam-webster.com/dictionary/transient
IPA: /ˈtrænʒənt/ , /ˈtrænsjənt/
Phonetic Spelling: tran·zhuhnt; tran·syuhnt