Submission ID #: 69373

Scriptwriter Name: Sulakshana Karkala

Project Page Link: https://review.jove.com/account/file-uploader?src=21158813

Title: Thermal Behavior and Power Efficiency Comparison of AC vs. DC Electrical Heating in a Distillation Column Using Infrared Thermography Analysis

Authors and Affiliations:

Gregorio Moreno-Sotelo, Adriana del C. Téllez-Anguiano, Mario Heras-Cervantes

Tecnológico Nacional de México, Instituto Tecnológico de Morelia Morelia

☒   All author names and affiliations are correct (city/state/country information not included in video title page).

Corresponding Authors:

Adriana del C. Téllez-Anguiano                           adriana.ta@morelia.tecnm.mx

Email Addresses for All Authors:

Gregorio Moreno-Sotelo                                     gregorio.ms@morelia.tecnm.mx

Adriana del C. Téllez-Anguiano                           adriana.ta@morelia.tecnm.mx

Mario Heras-Cervantes                                        mario.hc@morelia.tecnm.mx

## Author Questionnaire

1. We have marked your project as author-provided footage, meaning you film the video yourself and provide JoVE with the footage to edit. JoVE will not send the videographer. Please confirm that this is correct.

√ Correct

☐ Incorrect

2. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

3. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

4. Proposed filming date: To help JoVE process and publish your video in a timely manner, please indicate the proposed date that your group will film here: 01/17/26

Current Protocol Length

Number of Steps: 18

Number of Shots: 34

# Introduction

Answers to these questions will become interview statements that you will deliver on camera.

- Answer up to 2 introduction and up to 3 conclusion questions. No more than 5 interview statements will be included in the video.

- Enter the full name of the author who will deliver the statement.

- Speak naturally and avoid reading the lines.

- Answer in full sentences, the questions will not be displayed in the video.

- Limit the length of each statement to 20 words or fewer.

- Answers will be edited for length, clarity, and consistency with journal style guidelines.

INTRODUCTION:

What is the scope of your research? What questions are you trying to answer?

  - Gregorio Moreno Sotelo: Demonstrate the advantage of using AC and DC power supplies in a heating element within a distillation column boiler.

What are the most recent developments in your field of research?

  - Gregorio Moreno Sotelo: Although studies address thermographic analysis and AC/DC-powered distillation columns separately, no work combines both approaches.

What technologies are currently used to advance research in your field?

  - Gregorio Moreno Sotelo: Boiler flows simulation, low-efficiency zones identification, thermodynamic modeling, optimization using algorithms to minimize energy consumption and maximize distillate quality..

What are the current experimental challenges?

  - Gregorio Moreno Sotelo: Changes in emissivity, changes in ambient temperature, changes in internal pressure in the column.

CONCLUSION:

What significant findings have you established in your field?

  - Gregorio Moreno Sotelo Energy efficiency analyses showed that DC reduced losses in joules, while AC showed superior thermal stability during prolonged operations.

What research gap are you addressing with your protocol?

  - Gregorio Moreno Sotelo: Using TIR to analyze the efficiency and thermal behavior of an actuator within a distillation column boiler

What advantage does your protocol offer compared to other techniques?

  - Gregorio Moreno Sotelo: Using TIR analysis provides a clearer view of thermal behavior in relation to power sources within the process.

How will your findings advance research in your field?

  - Gregorio Moreno Sotelo: The appropriate selection of AC or DC power supply promotes optimization, minimizes energy consumption costs, and improves the quality of the distilled product.

What new scientific questions have your results paved the way for?

  - Gregorio Moreno Sotelo: Is it possible to predict actuator wear using TIR?

Is it possible to develop estimation models using TIR as input values ​​to predict future behavior based on given conditions?

What questions will future research focus on?

  - Gregorio Moreno Sotelo: This work develops a thermogram estimation model for early fault detection and analyzes thermal actuator fatigue effects on process transients.

# Protocol

Please review this section to make sure that it accurately describes your protocol. Use Track Changes when making edits or revisions.

- The two-digit steps (e.g., 2.1., 2.2.) are the narration.  JoVE is responsible for the narration of the protocol and results.

- Red italics are pronunciation guides indicating how the word will be spoken.

- Filming should take no more than 10 minutes per step. If a step takes more than 10 minutes, prepare the product for that step in advance.

- The three-digit shots (e.g., 2.1.1., 2.2.2.) are the actions that you/your videographer will capture.

- Thermographic Data Acquisition and Image Processing for Boiler Plate Temperature Measurement

Demonstrator: Gregorio Moreno Sotelo

If the same person is the demonstrator throughout, mention them once here and remove the "Demonstrator" field from the other sections; if the demonstrator changes, retain the field in the respective sections.

  - To begin, set up the thermal camera for acquisition [1].  Select the Iron color palette from the thermal camera menu [1]. Ensure the palette is correctly applied before proceeding [2].

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=21158813

    - WIDE: Talent standing in front of the experimental setup with the thermal camera and boiler plate visible.
AUTHORS: Please note that this shot was added to facilitate the WIDE angle requirement of the opening shot

    - SCREEN: Display the thermal camera menu and show the Iron color palette being selected.

  - Set the emissivity correction value to 0.95 from the thermal camera menu [1].

    - SCREEN: Show the emissivity settings menu on the thermal camera with the value adjusted to 0.95.

  - Then position the thermal camera approximately 80 centimeters away from the boiler plate [1]. Holding the thermal camera approximately 60 centimeters above the floor, orient the camera toward the boiler plate in the distillation column [2].

    - Talent positioning the thermal camera at the specified distance from the boiler plate.

    - Talent holding the thermal camera at the specified height above the floor and aligning the camera toward the boiler plate in the distillation column

  - Press the save button to record the thermogram to the thermal camera memory [1].

    - SCREEN : Talent pressing the save button on the thermal camera.

  - Capture thermograms every 5 seconds during samples 1 to 1500 on the local interface [1]. Repeat the acquisition for 20, 60, and 100 volts under alternating current and direct current conditions [2].

    - SCREEN: Display the local interface showing sample numbers increasing during acquisition.

    - SCREEN: Talent repeatedly capturing thermograms following the defined timing intervals.
AND
TEXT ON PLAIN BACKGROUND:

For 100 V (AC and DC) : Capture every 10 s (samples 1500 – 2800)
                                          Capture every 20 s ( samples 2800 until steady state)
For 20 V (AC and DC) : Capture every 10 s (samples 1500 to 5500)
                                        Capture every 20 s ( samples 5500 until steady state)
For 60 V (AC and DC) : Capture every 15 s (samples 1500 to 4000)
                                        Capture every 30 s ( samples 4000 until steady state)
Video Editor: Please play shots side by side in a split screen

  - Record the name of each captured thermogram and the corresponding boiler plate temperature sample displayed on the local interface [1].

    - SCREEN: Show the thermogram filename and temperature value on the local interface.

  - To begin acquisition of boiler plate temperature data, store the temporary files generated by the local process interface [1]. Rename and save the stored files into the current process folder [2].

    - SCREEN: Show the interface generating and listing temporary data files.

    - SCREEN: Show file renaming and saving into the process folder.

  - Collect the .csv (C-S-V) files generated during the test and generate a single file containing all acquired process data [1]. Save the consolidated file for analysis [2].

    - SCREEN: Show multiple comma-separated values files being merged.

    - SCREEN: Display the final consolidated data file.

  - To begin preparation of the data, extract thermograms from the thermal camera [1]. Transfer all image files to the analysis workstation [2]. Review the extracted thermograms and discard any unwanted images [3].

    - Talent connecting the thermal camera to the workstation.

    - SCREEN: Show thermogram files being transferred.

    - SCREEN: Display thermograms being reviewed.

  - Manually mark a reference point on each thermogram [1]. Then create a process temperature overview graph using the previously generated process data file [2].Relate the boiler temperature data from the local interface to the captured process thermograms [3].

    - SCREEN: Show a thermogram with a reference point being marked.

    - SCREEN: Show the data file being loaded into graphing software.

    - SCREEN: Show temperature data aligned with thermogram identifiers.

  - To develop an algorithm for thermogram processing, first load an image into a variable within the analysis environment [1]. Define the size of the vectors for the graph axes using the image variable [2]. Then define the area for detecting minimum and maximum temperature ranges based on the analysis image [3].

    - SCREEN: Show code or interface loading an image file.

    - SCREEN: Show vector dimensions being defined.

    - SCREEN: Display highlighted minimum and maximum regions.

  - Prepare the figure for graphing [1].

    - SCREEN: Show figure setup in the analysis environment.

  - Next, load the thermogram image into a variable for processing [1]. Define a search area for the previously marked reference point [2].

    - SCREEN: Show image loading for processing.

    - SCREEN: Display the defined search area.

  - Isolate the reference point and obtain its coordinates [1]. Then use the coordinates to indicate the analysis area [2].

    - SCREEN: Show reference point isolation.

    - SCREEN: Display the analysis area based on coordinates.

  - Now, convert the analysis area to grayscale [1]. Calculate the mean temperature value of the grayscale area [2].

    - SCREEN: Show conversion of the analysis area to grayscale.

    - SCREEN: Display calculation of the mean value.

  - Isolate the maximum and minimum temperature areas in the image [1]. Convert the graphical numbers to numeric text [2].

    - SCREEN: Show highlighted maximum and minimum temperature regions.

    - SCREEN: Display numeric text extracted from the image.

  - Convert the mean values to scaled maximum and minimum temperatures according to the image ranges [1], and store the total mean values and corresponding labels [2].

    - SCREEN: Show scaling calculations applied to mean values.

    - SCREEN: Display stored mean values with labels.

  - Lastly, plot the total mean results to visualize the processed thermogram data [1].

    - SCREEN: Show plotting of total mean results.

# Results

Please review this section to make sure that it accurately reflects your findings.

- You/Your videographer does not have to record this section. It only includes the figures/tables from your manuscript (called LAB MEDIA).

- Use Track Changes when making edits or revisions. Ensure the voiceover length is below 200 words. Current word count: 136

- Please note that the video cannot include voiceover without an accompanying visual.

- Results

  - The temperature measurements using 100 volt DC varied throughout the process, while the measurements using 100 volt AC remained more stable [1-TXT].

    - LAB MEDIA: Figure 5A. 
TXT: DC: Direct Current; AC: Alternating Current
Video editor: Please sequentially highlight the orange (VDC) and blue (VAC) lines

  - At 100-volt DC, thermal variation across the boiler section was higher than with alternating current [1]. DC presented more controlled areas due to smoother progression and increasing but not abrupt dispersion [2].

    - LAB MEDIA: Figure 6 A, B and C,D . Video editor: Please highlight 6C and D

    - LAB MEDIA: Figure 6E. Video editor: Please highlight the blue line

  - Under 60-volt conditions, temperature measurements were more stable with AC [1].

    - LAB MEDIA: Figure 5B. Video editor: Emphasize the blue smoother VAC curve

  - Thermal evolution using 60-volt AC showed a gradual temperature increase during start-up [1], whereas with 60 volt DC, the heating was more pronounced and rapid [2].

    - LAB MEDIA: Figure 9B. Video editor: Please sequentially show each image. Emphasize the box with the temperature on the top left corner of each image

    - LAB MEDIA: Figure 10B. Video editor: Please sequentially show each image. Emphasize the box with the temperature on the top left corner of each image

  - The graphical temperature trend at 20 volts showed noticeable variation DC [1], while AC resulted in a more gradual and stable profile [2].

    - LAB MEDIA: Figure 5C. Video editor: Highlight the orange VDC line with visible fluctuations.

    - LAB MEDIA: Figure 11A. Video editor: Highlight the smooth upward trend of the VAC line.

  - Thermograms under 20-volt DC showed temperatures ranging from 22.8 degrees Celsius to 26.3 degrees Celsius [1].

    - LAB MEDIA: Figure 11B. Video editor: Highlight each thermogram from IR_3068 to IR_3178, Emphasize the box with the temperature on the top left corner of each image