Submission ID #: 69405

Scriptwriter Name: Poornima G

Project Page Link: https://review.jove.com/account/file-uploader?src=21168573

Title: Peroxisome Staining in Mammalian Cells Using Peroxisome-Specific Probes

Authors and Affiliations:

Charlotte Howman1*, Tamar Cohen2*, Mor Yarshansky Shlomy2*, Melanie S. van Aerle3, Ruth E. Carmichael3, Milena Schuhmacher4, Luc Reymond5, Einat Zalckvar2, Daniel Kaganovich1, Triana Amen1

1School of Biological Sciences, University of Southampton

2The Mina and Everard Goodman Faculty of Life Sciences, Bar-Ilan University

3Biosciences, Faculty of Health and Life Sciences, University of Exeter

4Institute of Bioengineering, Faculty of Life Sciences, Ecole Polytechnique Fédérale de Lausanne (EPFL)

5Biomolecular Screening Facility, Ecole Polytechnique Fédérale de Lausanne (EPFL)

*These authors contributed equally

Corresponding Authors:

Milena Schuhmacher		milena.schuhmacher@epfl.ch

Einat Zalckvar			einat.zalckvar@biu.ac.il

Daniel Kaganovich		D.Kaganovich@soton.ac.uk

Triana Amen 			T.Amen@soton.ac.uk

Email Addresses for All Authors:

Charlotte Howman		ch12g22@soton.ac.uk

Tamar Cohen			tmr.chn.2001@gmail.com

Mor Yarshansky Shlomy	mormor1000@gmail.com

Melanie S. van Aerle		mv566@cam.ac.uk

Ruth E. Carmichael		r.carmichael@exeter.ac.uk

Luc Reymond			luc.reymond@epfl.ch

Milena Schuhmacher		milena.schuhmacher@epfl.ch

Einat Zalckvar			einat.zalckvar@biu.ac.il

Daniel Kaganovich		D.Kaganovich@soton.ac.uk

Triana Amen 			T.Amen@soton.ac.uk

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Filming location: Will the filming need to take place in multiple locations?   Yes

Two floors in the same building. The lab is located on the 3rd floor, and the microscope room and the quiet room for the interview are located on the 5th floor.

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. No

Current Protocol Length

Number of Steps: 12

Number of Shots: 17

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

Authors: The answers are edited in accordance with journal guidelines. Each answer can be around 20-25 words long.

INTRODUCTION:

NOTE: 1.1 and 1.2 were self-filmed by the authors and rest were filmed by the videographer

What is the scope of your research? What questions are you trying to answer?

  - Charlotte Howman: Our research investigates peroxisome regulation in health and disease. We develop fluorescent probes and staining protocols for live and fixed cells to visualize peroxisomes across systems. NOTE: 1.1 and 1.2 answers were changed by the authors

    - LAB MEDIA: 20260204_124049000_iOS.MP4 00:00-00:14

What are the current experimental challenges?

  - Charlotte Howman: Key experimental challenges include noninvasive peroxisome labeling, limited antibodies, real-time visualization, and obtaining functional readouts, though PeroxiSPY variability enables distinguishing distinct peroxisome functional states.

    - LAB MEDIA: 20260204_124401000_iOS.MP4 00:00-00:14

CONCLUSION:

What significant findings have you established in your field?

  - Tamar Cohen: We have developed the PeroxiSPY dye that can be used to visualize peroxisomes in both live and fixed animal cells.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

What advantage does your protocol offer compared to other techniques?

  - Tamar Cohen: Our fluorescent probe can offer a simple and quick method to study peroxisome dynamics, heterogeneity, and dysfunction in ways that were previously limited.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

How will your findings advance research in your field?

  - Tamar Cohen: This protocol will enable imaging-based peroxisome research in diverse cell types, including patient-derived cells with peroxisomal disorders.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

- Peroxisome Staining in Cells with PeroxiSPY

Demonstrator:  Tamar Cohen

  - To begin, split the cells into a plate with coverslips to achieve a 30 to 70% confluency the next day [1].

    - Talent walking into the room with the culture plate.

  - Dilute the probe with fresh media in a light-sensitive microcentrifuge tube to achieve a 4 micromolar final concentration [1].

    - Talent pipetting 2 microliters of probe into a light-sensitive microcentrifuge tube containing 0.5 milliliters of media.

  - Select the desired well to stain and label the outside using a marker pen [1].

    - Talent identifying a well and labeling it clearly.

  - Aspirate 0.5 milliliters of media from the selected well [1] and replace it with the prepared probe media drop by drop along the wall of the well [2].

    - Talent removing media from the well.

    - Shot of dispensing probe media dropwise into the well.

  - After dispensing, incubate the cells at 37 degrees Celsius for 10 to 15 minutes [1]. From this point forward, shield the cells from light using aluminum foil [2].

    - Talent placing the imaging plate in the incubator.

    - Talent covering the plate with aluminum foil.

  - Post-incubation, carefully remove the stained media and wash the cells twice with PBS [1]. Then, place the plate under a chemical hood, add 4 percent paraformaldehyde in PBS to the cells [2] and incubate at room temperature for 20 minutes [3].

    - Talent aspirating stained media and washes with PBS. Authors: We can only show one wash in the video, and mention twice in the narration

    - Talent placing the plate inside the chemical hood and adding 4% paraformaldehyde to the wells.

    - Talent placing the plate aside and starting a timer.

  - After fixation, remove the paraformaldehyde into a designated waste container [1] and wash the cells once with PBS [2-TXT].

    - Talent aspirating paraformaldehyde and discarding it into a labeled waste.

    - Talent adding the cells with PBS. TXT: Remove the PBS completely after the wash

  - Next, add 0.5 percent Triton X in PBS to permeabilize the cells and incubate for 2.5 minutes [1-TXT].

    - Talent adding Triton X solution and setting the plate aside. TXT: Any other suitable detergent can be used as required

  - After incubation, remove the Triton X and wash the cells twice with PBS for 10 minutes each [1].

    - Talent aspirating Triton X and adding PBS into the well.

  - To block the cells, incubate them with 5 percent bovine serum albumin in PBS overnight at 4 degrees Celsius [1].

    - Talent placing plate in the refrigerator. TXT: Alternatively, incubate at RT; 20 min

  - Perform the immunofluorescence staining procedure following standard protocol [1].

    - Talent removing the plate from the fridge.

  - For live cell imaging, split the cells and incubate them directly with the dye [1]. Finally, perform microscopy to acquire images of both fixed and live cells [2].

    - TEXT ON 2.12.1’s BACKGROUND:

- Start confocal software and set laser/imaging parameters

- Prepare 1 µM probe solution

- Stain the cells and incubate for 10 min

- Focus cells and position for imaging

- Optimize settings and acquire .nd2 images

    - Two talents watching the fixed cells on the screen in the microscope room and talent setting up the microscope. NOTE: If possible, show both the talents in this scene

# Results

- Results

  - Live-cell staining with PeroxiSPY (peroxy-spy) probes resulted in ATP Binding Cassette Subfamily D-dependent import into peroxisomes, producing a high peroxisome to cytoplasm fluorescence ratio in metabolically active cells [1].

    - LAB MEDIA: Figure 1B. Video editor: Highlight the WT “peroxiSPY555” image.

  - In wild-type HEK293T cells expressing GFP-SKL, PeroxiSPY555 staining showed distinct co-localization with peroxisomes [1], whereas in PEX19 knockout cells, that lack peroxisomes,  the staining was absent [1].

    - LAB MEDIA: Figure 1B. Video editor: Highlight the merge image of  WT.

    - LAB MEDIA: Figure 1B. Video editor: Highlight the merge image of PEX19 KO.

  - In human fibroblasts, PeroxiSPY staining co-localized with the peroxisomal marker GFP-SKL, confirming peroxisome targeting [1].

    - LAB MEDIA: Figure 1C. Video editor: Highlight the merged image .

  - Peroxisome density quantification revealed significant variation among different cell types [1].

    - LAB MEDIA: Figure 1D.

  - Optimized staining conditions in COS-7 (cos-7) cells using 250 nanomolar PeroxiSPY for 20 minutes significantly increased the peroxisome to cytoplasm fluorescence ratio [1] compared to the original 1 micromolar for 10 minutes protocol [2].

    - LAB MEDIA: Figure 2A. Video editor: Highlight  “20 min 250 nM” image and data points in the graph.

    - LAB MEDIA: Figure 2A. Video editor: Highlight  “10 min 1 µM” image and data points in the graph.

  - Extended incubation of HEK293T cells with PeroxiSPY for 24 hours led to an improved peroxisome to cytoplasm fluorescence intensity ratio compared to 15 minutes [1], despite reduced peroxisome staining [2].

    - LAB MEDIA: Figure 2B. Video editor: Highlight the violin plot for ‘24 hours’ in the plot on the extreme right showing ‘peroxisome to cytoplasm FI, au’ on Y-axis.

  - A fixation protocol enabled successful antibody co-staining, demonstrated by preserved PeroxiSPY labeling and anti-PEX14 (pex-14) immunostaining in Huh7 (H-U-H-7) and HEK293T cells [1].

    - LAB MEDIA: Figure 2C. Video editor: Highlight the merged images for top 2 rows.

  - Quantification showed that fixation and permeabilization procedures affected peroxisome fluorescence intensity [1] when compared to live cell staining [2].

    - LAB MEDIA: Figure 2D. Video editor: Highlight the violin plots for  “Fixed”, and “Permeabilized” conditions.

    - LAB MEDIA: Figure 2D. Video editor: Highlight the violin plots for “Live”condition.

- Confluency
Pronunciation link: https://www.merriam-webster.com/dictionary/confluency
IPA: /ˈkɑːn.flu.ən.si/
Phonetic Spelling: kahn·floo·uhn·see

- Coverslips
Pronunciation link: https://www.merriam-webster.com/dictionary/coverslip
IPA: /ˈkʌv.ərˌslɪp/
Phonetic Spelling: kuh·ver·slip

- Microcentrifuge
Pronunciation link: https://www.merriam-webster.com/dictionary/microcentrifuge
IPA: /ˌmaɪ.kroʊˈsɛn.trəˌfjuːdʒ/
Phonetic Spelling: my·kroh·sen·truh·fyooj

- Micromolar
Pronunciation link: https://www.merriam-webster.com/dictionary/micromolar
IPA: /ˌmaɪ.kroʊˈmoʊ.lɚ/
Phonetic Spelling: my·kroh·moh·ler

- Microliters
Pronunciation link: https://www.merriam-webster.com/dictionary/microliter
IPA: /ˈmaɪ.kroʊˌliː.tɚ/
Phonetic Spelling: my·kroh·lee·ter

- Aspirate
Pronunciation link: https://www.merriam-webster.com/dictionary/aspirate
IPA: /ˈæs.pəˌreɪt/
Phonetic Spelling: ass·puh·rayt

- Incubate
Pronunciation link: https://www.merriam-webster.com/dictionary/incubate
IPA: /ˈɪn.kjəˌbeɪt/
Phonetic Spelling: in·kyuh·bayt

- Paraformaldehyde
Pronunciation link: https://www.merriam-webster.com/dictionary/paraformaldehyde
IPA: /ˌper.ə.fɔːrˈmæl.dəˌhaɪd/
Phonetic Spelling: pair·uh·for·mal·duh·hyde

- Phosphate-buffered saline
Pronunciation link: https://www.merriam-webster.com/dictionary/phosphate-buffered%20saline
IPA: /ˈfɑːs.feɪt ˈbʌf.ɚd səˈliːn/
Phonetic Spelling: fahs·fayt buhf·erd suh·leen

- Triton X
Pronunciation link: https://www.merriam-webster.com/dictionary/Triton
IPA: /ˈtraɪ.tɑːn/
Phonetic Spelling: try·tahn

- Permeabilize
Pronunciation link: https://www.merriam-webster.com/dictionary/permeabilize
IPA: /ˈpɝː.mi.əˌbaɪl.aɪz/
Phonetic Spelling: per·mee·uh·bye·lize

- Immunofluorescence
Pronunciation link: https://www.merriam-webster.com/dictionary/immunofluorescence
IPA: /ɪˌmjuː.noʊ.flʊˈrɛs.əns/
Phonetic Spelling: ih·myoo·noh·floor·ess·uhns

- Confocal
Pronunciation link: https://www.merriam-webster.com/dictionary/confocal
IPA: /kɑːnˈfoʊ.kəl/
Phonetic Spelling: kahn·foh·kuhl

- Peroxisome
Pronunciation link: https://www.merriam-webster.com/dictionary/peroxisome
IPA: /pəˈrɑːk.sɪˌsoʊm/
Phonetic Spelling: puh·rahk·suh·sohm

- Cytoplasm
Pronunciation link: https://www.merriam-webster.com/dictionary/cytoplasm
IPA: /ˈsaɪ.təˌplæz.əm/
Phonetic Spelling: sigh·tuh·plaz·uhm

- PeroxiSPY
Pronunciation link: No confirmed link found
IPA: /pəˈrɑːk.siˌspaɪ/
Phonetic Spelling: puh·rahk·see·spy

- PeroxiSPY555
Pronunciation link: No confirmed link found
IPA: /pəˈrɑːk.siˌspaɪ faɪv faɪv faɪv/
Phonetic Spelling: puh·rahk·see·spy five·five·five

- ATP
Pronunciation link: https://www.merriam-webster.com/dictionary/ATP
IPA: /ˌeɪ.tiːˈpiː/
Phonetic Spelling: ay·tee·pee

- HEK293T
Pronunciation link: No confirmed link found
IPA: /hɛk tuː naɪn θriː tiː/
Phonetic Spelling: heck two·nine·three·tee

- GFP-SKL
Pronunciation link: No confirmed link found
IPA: /ˌdʒiː.ɛfˈpiː ɛs keɪ ɛl/
Phonetic Spelling: gee·ef·pee ess·kay·ell

- PEX19
Pronunciation link: No confirmed link found
IPA: /pɛks naɪnˈtiːn/
Phonetic Spelling: pex nine·teen

- Fibroblasts
Pronunciation link: https://www.merriam-webster.com/dictionary/fibroblast
IPA: /ˈfaɪ.broʊˌblæst/
Phonetic Spelling: fy·broh·blast

- COS-7
Pronunciation link: No confirmed link found
IPA: /kɑːz ˈsɛv.ən/
Phonetic Spelling: kahz sev·uhn

- Nanomolar
Pronunciation link: https://www.merriam-webster.com/dictionary/nanomolar
IPA: /ˌnæ.noʊˈmoʊ.lɚ/
Phonetic Spelling: nan·oh·moh·ler

- Violin plot
Pronunciation link: https://www.merriam-webster.com/dictionary/violin%20plot
IPA: /ˈvaɪ.ə.lɪn plɑːt/
Phonetic Spelling: vy·uh·lin plot

- PEX14
Pronunciation link: No confirmed link found
IPA: /pɛks fɔːrˈtiːn/
Phonetic Spelling: pex four·teen

- Huh7
Pronunciation link: No confirmed link found
IPA: /hʌ seɪv.ən/
Phonetic Spelling: huh sev·uhn