# Submission ID #: 69495

# Scriptwriter Name: Sulakshana Karkala

# Project Page Link: https://review.jove.com/account/file-uploader?src=21196023

# Title: Live-Cell Imaging of Lysosomal Membrane Permeabilization During Necroptosis

# Authors and Affiliations:

# Katia S.G. Andrade, John Mably, Da-Zhi Wang, Zhigao Wang

# Center for Regenerative Medicine, Heart Institute, Department of Internal Medicine, Morsani College of Medicine, University of South Florida

# Corresponding Authors:

# Katia S.G. Andrade                     kgomesandrade@usf.edu

# Email Addresses for All Authors:

# Katia S.G. Andrade                     kgomesandrade@usf.edu

John Mably                                   jmably@usf.edu

Da-Zhi Wang                                dazhiw@usf.edu

Zhigao Wang                                zhigao@usf.edu

## Author Questionnaire

1. We have marked your project as author-provided footage, meaning you film the video yourself and provide JoVE with the footage to edit. JoVE will not send the videographer. Please confirm that this is correct.

√ Correct

2. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  NO

3. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  YES, all done

4. Proposed filming date: To help JoVE process and publish your video in a timely manner, please indicate the proposed date that your group will film here: 12/08/2025

When you are ready to submit your video files, please contact our Content Manager, Utkarsh Khare.

Current Protocol Length

Number of Steps:  14

Number of Shots:  26

# Introduction

INTRODUCTION:

What is the scope of your research? What questions are you trying to answer?

  - Katia Andrade: This protocol outlines a live-cell imaging method to monitor lysosomal membrane permeabilization during necroptosis using fluorescent probes detecting integrity and acidification changes.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

What are the most recent developments in your field of research?

  - Katia Andrade: Recent advances include real-time imaging approaches revealing dynamic organelles changes during regulated cell death pathways.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

CONCLUSION:

What significant findings have you established in your field?

  - Katia Andrade: We demonstrated that lysosomal membrane permeabilization can be directly visualized in living cell undergoing necroptosis induction.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

What research gap are you addressing with your protocol?

  - Katia Andrade: This protocol shows that combining LysoTracker with fluorescein-labeled dextran beads allows real-time monitoring of lysosomal pH and cargo release dynamics.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

What advantage does your protocol offer compared to other techniques?

  - Katia Andrade: My protocol provides a unique, dual readout on both lysosomal functionality and structural integrity in live cells.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

# Protocol

NOTE: Protocol scripted from Footage

- Live-Cell Confocal Imaging for Monitoring Lysosomal Membrane Permeabilization

Demonstrator: Katia Andrade

  - To begin, plate 5,000 cells in 2 milliliters of antibiotic supplemented DMEM media into a sterile glass-bottom dish [1-TXT]. Place the dish in an incubator set to 37 degrees Celsius with 5 percent carbon dioxide and incubate the cells overnight [2].

    - LAB MEDIA: GX011140	 01:27-01:45
TXT: Media supplementation: 10% FBS, 1% Penicillin and Streptomycin

    - LAB MEDIA: GX011141 	00:12 – 00:22 
AND  GX011142	00:00 – 00:04

  - Add 20 microliters of dextran green beads to 25 micrograms per milliliter in fresh antibiotic-supplemented DMEM media to dilute it [1].

    - LAB MEDIA: GX011145 	00:21 – 01:04

  - Remove the old medium from the dish [1] and replace it with 2 milliliters of the bead-containing supplemented DMEM media [2]. Place the dish in an incubator set to 37 degrees Celsius with 5 percent carbon dioxide and incubate for 24 hours to allow internalization of beads into lysosomes [3].

    - LAB MEDIA: GX011147 	00:05 – 00:10

    - LAB MEDIA: GX011147	 00:19 – 00:30

    - LAB MEDIA: GX011147 	00:46 – 00:50 
AND	GX011148		00:00 - 00:05

  - Now, add 2 microliters of 1 millimolar Lysotracker stock solution to 2 milliliters of fresh DMEM media [1-TXT]. Replace the old medium with this staining solution [2].

    - LAB MEDIA: GX011150 	00:00 – 00:16
TXT: Final concentration: 1 µM

    - LAB MEDIA: GX011151 	00:05-00:09, 00:19 – 00:32

  - Incubate the cells at 37 degrees Celsius with 5 percent carbon dioxide for 2 hours [1].

    - LAB MEDIA: GX011152 00:05 – 00:10

  - Turn on the confocal system components by switching on the personal computer, microscope, scanner power, laser power, laser emission, and Leica power [1].

    - LAB MEDIA: GX011153 	00:00 – 00:10

  - Turn on the environmental controls including the temperature controller called The Cube and the carbon dioxide gas mixer controller called The Brick [1]. Open the carbon dioxide and oxygen supply [2].

    - LAB MEDIA: GX011153 	00:11 – 00:16

    - LAB MEDIA: GX011154	 00:00 – 00:10

  - Set the temperature controller to 37 degrees Celsius [1]. Set the gas and humidity controller to 5 percent carbon dioxide and 95 percent humidity [2].

    - LAB MEDIA: GX011155 	00:00 – 00:03

    - LAB MEDIA: GX011155 	00:17 – 00:33
AND
GX011184			00:04-00:06

  - Assemble the imaging chamber [1-TXT].

    - LAB MEDIA: GX011157 	00:30 – 00:35
AND 
TEXT ON PLAIN BACKGROUND:
Use 63X or 1.4 NA oil immersion for confocal imaging
Excitation : 488 nm (green), 555 nm (red)
Detector: Hybrid (488 nm), PMT (555nm)

  - After staining, wash the cells three times with 2 milliliters of PBS [1]. Then add 2 milliliters of fresh supplemented DMEM media [2]. Apply immersion oil onto the objective lens before imaging [3]. Mount the dish on the microscope stage [4].

    - LAB MEDIA: GX011170 	00:49 – 00:56, 01:13-01:15, 01:21-01:25

    - LAB MEDIA: GX011170	 02:50 – 03:10

    - LAB MEDIA: GX011163 	00:00 – 00:05

    - LAB MEDIA: GX011165 	00:09 – 00:20

  - Adjust the laser intensity up to 3 percent, but not more than 5 percent [1]. Then adjust the detector gain in the microscope software to optimize the signal [2]. Minimize the background by adjusting the contrast bar [3].

    - LAB MEDIA: LAS X 12_3_2025 10_31_39 AM 	00:00 – 00:08

    - LAB MEDIA: LAS X 12_3_2025 1_54_21 PM 	00:04 – 00:15

    - LAB MEDIA: LAS-X-12_3_2025-10_59_23-AM	00:00 – 00:12

  - Acquire baseline images of untreated cells, which serve as the control condition [1].

    - LAB MEDIA: LAS X 12_3_2025 11_02_12 AM Timestamps: 01:19 – 01:25

  - Return the plate to the laminar flow hood and add 1 milliliter antibiotic-DMEM supplemented with growth factors [1-TXT].

    - LAB MEDIA: GX011183 00:54 – 01:12
TXT: Media supplementation: 100 ng/mL of TNF-α (T), 500 nM of SMAC mimetic (S), 100 µM of Z-VAD-FMK (Z)

  - In the acquisition software, set the mode to xyt, format to 512 x 512 (Five-twelve-by- Five-twelve), speed to 400, zoom to 3, line average to 1, line accumulation to 3, frame accumulation to 1, time interval to 3 min (minutes), duration to 8h (hours), and enable autofocus [1]. Click Start to begin the time-lapse acquisition [2].

    - LAB MEDIA: LAS X 12_3_2025 11_02_12 AM 	00:00 – 01:10:

    - LAB MEDIA: LAS X 12_3_2025 11_02_12 AM 	01:30 – 01:34

# Results

- Results

  - Initial imaging confirmed the presence of stable puncta of LysoTracker [1] and fluorescein-labeled dextran, indicating intact lysosomes before treatment [2].

    - LAB MEDIA: Figure 3. Video editor: Highlight the red fluorescent spots labeled “LysoTracker” in the “Before” column.

    - LAB MEDIA: Figure 3. Video editor: Highlight the green fluorescent spots labeled “Dextran” in the “Before” column.

  - Four hours after treatment with T/S/Z (T-S-Z) , LysoTracker fluorescence was noticeably reduced [1-TXT], while dextran began to show diffuse cytosolic distribution, indicating initial lysosomal membrane permeabilization [2].

    - LAB MEDIA: Figure 3.
TXT: T: Tumor Necrosis Factor Alpha;
         S: SMAC mimetic;
         Z: Z-VAD-FMK
 Video editor: Highlight the visibly dimmer red fluorescence in the “4 hr after” column under “LysoTracker”.

    - LAB MEDIA: Figure 3. Video editor: Highlight the partial spread of green fluorescence in the “4 hr after” column under “Dextran”.

  - By 8 hours after treatment, LysoTracker signal had almost completely disappeared [1], while many cells retained green dextran puncta with diffuse green cytosolic signal [2].

    - LAB MEDIA: Figure 3. Video editor: Highlight the absence of red signal in the “8 hr after” column under “LysoTracker”.

    - LAB MEDIA: Figure 3. Video editor: Highlight the green puncta and surrounding diffuse green fluorescence in the “8 hr after” column under “Dextran”.