# Submission ID #: 69504

# Scriptwriter Name: Kathyayani Sridharan

# Project Page Link: https://review.jove.com/account/file-uploader?src=21198768

# Title: Single-plate Runs of Mito Stress and Glycolysis Stress Tests Using Adherent or Non-Adherent Cells

# Authors and Affiliations:

# Jane Tonello1, Salvador Gonzalez Ochoa1, Thanigaivelan Kanagasabai1,2, Muna Mohammed2, Anil Shanker1,3,4, Alla V. Ivanova1,2

1Department of Biochemistry, Cancer Biology, Neuroscience and Pharmacology, School of Medicine, Meharry Medical College

2Department of Biomedical Sciences, School of Graduate Studies, Meharry Medical College

3Host-Tumor Interactions Research Program, Vanderbilt-Ingram Comprehensive Cancer Center, Vanderbilt University School of Medicine

4Vanderbilt Institute for Infection, Immunology and Inflammation, Vanderbilt University School of Medicine

# Corresponding Authors:

# Alla V. Ivanova					aivanova@mmc.edu

# Email Addresses for All Authors:

Jane Tonello					jtonello@mmc.edu

Salvador Gonzalez Ochoa			sgonzalezochoa@mmc.edu

Thanigaivelan Kanagasabai 			tkanagasabai@mmc.edu

Muna Mohammed				mmohammed22@mmc.edu

Anil Shanker					ashanker@mmc.edu

Alla V. Ivanova					aivanova@mmc.edu

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes

If Yes, we will need you to record using screen recording software.

We recommend using the screen capture program OBS. JoVE’s tutorial for using OBS Studio is provided at this link: https://review.jove.com/v/5848/screen-capture-instructions-for-authors?status=a7854k

As these files are necessary for finalizing your script, please upload all screen-captured video files to your project page as soon as possible.

# Authors: Please create screen capture videos for shots labelled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=21198768

3. Filming location: Will the filming need to take place in multiple locations?   Yes, the same lab, different rooms

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. No

Current Protocol Length

Number of Steps: 21

Number of Shots:  52

# Interviews

Videographer: Obtain headshots for all authors available at the filming location.

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Alla Ivanova: We study how stress, sex, inflammation, and mitochondrial impairment during aging alter cell metabolism, inflammatory responses, and stress resilience.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Alla Ivanova: Existing methods poorly capture dynamic metabolic changes; this protocol simultaneously measures mitochondrial and glycolytic activity in real time.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

CONCLUSION:

Describe the most important consideration or challenge when performing this protocol.

  - Alla Ivanova: An important consideration is prior optimization of cell density to ensure accurate, reproducible, and biologically meaningful data.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Are there any additional methods or analyses that can be performed following this procedure?

  - Alla Ivanova: This protocol simultaneously evaluates the effects of compounds, drugs, and experimental conditions on the dynamics of mitochondrial and glycolytic parameters.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Alla Ivanova: Based on the results, future studies can develop strategies to restore energy metabolism and improve overall cellular health and resilience.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

Videographer: Please always record an audio slate while filming each shot mentioned in the shotlist.

- Pre-Analysis Preparation for Mito Stress and Glycolysis Stress Assays

Demonstrator: Jane Tonello

(Authors: Please specify if any sections will feature a different demonstrator during the shoot.)

  - To begin, remove the XFe96/XF Pro (X-F-E-ninety-six 0r X-F-Pro) Cell Culture Microplate containing the cells of interest, seeded at optimal density from the carbon dioxide incubator [1]. Use a multi-pipettor to remove 160 to 180 microliters of growth media from each well, working one row or column at a time to prevent the cells from drying [2]. Avoid touching the bottom of the wells during media removal [3].

    - WIDE: Talent removing the XFe96/XF Pro Cell Culture Microplate from the incubator. Videographer: Please obtain CU shot as well for this shot. (Authors: As specified, please provide the name of the cell line used during filming.

    - Talent using a multi-pipettor to aspirate media from a microplate row by row.

    - Close-up of media removal from sides of the wells without touching the well-bottom.

  - Use a single tip pipetman to remove the remaining 20 to 30 microliters media from each well without touching the cell layer [1]. To change the media, gently dispense 180 microliters of Glycolysis or Mito Stress assay media along the side walls of each well, including the cell-free corner wells used as blank controls [2].

    - CU: Show careful removal of residual media with a single tip pipetman without disturbing the cell layer.

    - Talent adding assay media into wells along the side walls of the well.

  - Place the microplate into a 37 degrees Celsius carbon dioxide-less incubator for 40 to 45 minutes prior to starting the assay [1]. Begin preparing the drugs during this time [2].

    - Talent placing the microplate into an incubator. Videographer: Please capture the digital display on the incubator.

    - Talent arranging materials for drug preparation on the bench.

  - For drug preparation and cartridge loading, use the pre-warmed Mito Stress and Glycolysis assay media [1] to dilute the compounds to the required concentrations for each cell type [2]. Prepare each compound in a separate 15 milliliter tube [3].

    - Talent pipetting assay media into tubes containing compounds.

    - TEXT ON A PLAIN BACKGROUND

Compound concentrations for Mito Stress Assay

- Oligomycin 				1 µM

- FCCP					1 µM

- Rotenone/Antimycin		0.5 µM

Compound concentrations for Glycolysis Assay

- Glucose				10 mM

- Oligomycin				1 µM

- 2-DG					50 mM

    - Shot of multiple labeled 15 milliliter tubes prepared with different compounds.

  - Remove the sensor cartridge and utility plate from the incubator [1]. After removing the lid, align the provided XF loading guide over the cartridge to ensure accurate drug injection [2].

    - Talent removing a cartridge from the incubator and opening it.

    - Talent positioning the loading guide onto the cartridge.

  - Use a multichannel pipettor and reagent reservoir to slowly add the required volume of each drug into the corresponding port while holding the pipettor upright [1].

    - Talent loading drugs into cartridge ports with a multichannel pipettor.

  - Change or reuse the loading guide after rinsing and drying for each port, as needed, and discard pipette tips after each load [1].

    - Talent rinsing and drying the loading guide and discarding used tips.

  - After loading all compounds into their respective ports, remove the guide and visually confirm that all ports are filled [1]. Secure the cartridge with its lid, ensuring drugs correspond to the assay layout [2].

    - Talent inspecting filled ports on the cartridge.

    - Talent placing the lid back onto the cartridge.

- Set up of Seahorse XFe96 Analyzer

Authors: Please create screen capture videos for shots labelled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=21198768

  - Turn on the analyzer [1] and open the WAVE software [2]. Select XF Mito Stress Test in the software [3].

    - Talent turning on the Seahorse XFe96 Analyzer.

    - SCREEN: Display the analyzer startup and opening of WAVE software.

    - SCREEN: Show selection of XF Mito Stress Test option.

  - Select the number of ‘Groups’ based on the plate setup and label each group [1]. Proceed to the next tab to assign wells for the plate map to perform both assays in the same plate [2]. Highlight wells for each group based on the plate loading [3].

    - SCREEN: Show group selection and labeling interface.

    - SCREEN: Assign wells on the plate map.

    - SCREEN: Highlight selected wells corresponding to groups.

  - Adjust the program settings according to the cell type and experimental purpose [1]. Select Run Assay and enter the required information to save the data file [2].

    - SCREEN: Modify assay program parameters.

    - SCREEN: Show clicking Run Assay and entering file details.

  - After removing the lid, load the pre-hydrated cartridge containing the drugs into the machine when prompted for initialization [1]. Confirm proper hydration and functionality of sensors during initialization [2]. Start the assay, and load and unload plates as prompted during the program [3].

    - Talent placing the cartridge into the analyzer after removing the lid.

    - SCREEN: Display initialization status indicating proper sensor hydration and readiness.

    - SCREEN: Show assay start and prompt-guided plate handling steps.

- Cellular Normalization Using Sulforhodamine B Assay

Authors: Please create screen capture videos for shots labelled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=21198768

  - After real-time metabolic analysis, remove the plate from the analyzer [1]. Add one quarter of the media volume of fixation solution to each well as per kit instructions [2] and incubate overnight at 4 degrees Celsius. [3].

    - Talent removing the plate from the analyser.

    - Talent adding fixation solution into each well.

    - Talent placing the plate at 4 °C.

  - Remove the fixative solution [1] and wash the wells three times with 200 microliters of RNase (R-N-ace) and DNase (D-N-ace)-free water without disturbing the cell layer [2]. After removing the wash solution, allow the plate to air-dry completely at room temperature [3]. Add 25 microliters of sulforhodamine B, or SRB (S-R-B) solution to each well and stain for 15 minutes at room temperature in the dark [4].

    - Talent aspirating the fixing solution.

    - Talent adding water for washing the wells.

    - Shot of plate left uncovered for drying.

    - Talent adding staining solution to wells and covering plate.

  - Remove the staining solution [1] and wash each well two to three times with 100 microliters of 1X washing solution [2]. Use vacuum suction to remove as much wash solution as possible to prevent bleaching [3].

    - Talent aspirating the staining solution.

    - Talent adding wash solution to the wells.

    - Talent using vacuum suction to rapidly remove the wash solution.

  - Add 100 microliters of 1X solubilization solution to each well [1] and place the plate on a shaker for 10 minutes at room temperature [2].

    - Talent adding 1X solubilization solution to each well.

    - Talent placing the plate on shaker.

  - Use a microplate reader to measure the optical density at 565 nanometers to determine well density based on color saturation [1]. Export the data as an Excel file for processing in WAVE software [2].

    - Talent placing plate into the microplate reader.

    - SCREEN: Show OD measurement at 565 nm and exporting the data to Excel.

- Data Normalization and Analysis

Authors: Please create screen capture videos for shots labelled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=21198768

  - Open the Excel sheet in WAVE software [1] and click on Normalize [2].

    - SCREEN: Display results panel showing the OD readings of SRB assay in WAVE software.

    - SCREEN: Click on Normalize.

  - In the open window, choose “select all” followed by “paste” to paste the data copied from the Excel spreadsheet [1], then click on Apply to automatically generate the normalized dataset [2].

    - SCREEN: Show pasting data.

    - SCREEN:  Show clicking Apply.

  - Next, click on Export tab [1] and from the Export to pop-up window, select XF Cell Mito Stress Test Report Generator [2] then save the file [3], selecting only the wells assigned to the Mito Stress Test during analysis [4].

    - SCREEN: Show clicking on the Export tab

    - SCREEN: Show selection of XF Cell Mito Stress Test Report Generator from the Export to pop-up window

    - SCREEN: Show saving the file

    - SCREEN: Show selection of wells assigned for Mito Stress Test during analysis.

  - To generate a glycolysis report, select XF Cell Glycolysis Stress Test Report Generator from the Export to pop-up window [1]. Save the file [2], selecting only the wells assigned to the Glycolysis Stress Test during analysis [3].

    - SCREEN: Show selection of XF Cell Glycolysis Stress Test Report Generator from the Export to pop-up window.

    - SCREEN: Show saving the file

    - SCREEN: Show selection of wells assigned for Glycolysis Stress Test during analysis.

# Results

- Results

  - The evaluation of metabotropic glutamate receptor 1– T-cell receptor, or mGluR1 (M-glue-R-one) – TCR (T-C-R) crosstalk using an integrated dual-assay approach revealed that blocking mGluR1 signaling in lipopolysaccharide, or LPS (L-P-S)-activated CD8+ (C-D-eight-positive) T (tee) cells significantly impacts both mitochondrial and non-mitochondrial energy production [1]. This reduced energy production is notable when compared to LPS-activated CD8+ T cells [2], while naive cells without LPS-pretreatment and mGluR1 inhibition remain metabolically unchanged [3].

    - LAB MEDIA: Figure 6A, B. Video Editor: Please emphasize the teal line in both figures.

    - LAB MEDIA: Figure 6A, B. Video Editor: Please emphasize the red line in both figures.

    - LAB MEDIA: Figure 6A, B. Video Editor: Please emphasize the orange line in both figures.

  - Specifically, Mito Stress tests revealed high basal respiration [1], maximal respiration [2], and reserve capacity in LPS pre-treated activated cells [3].

    - LAB MEDIA: Figure 6A. Video Editor: Please emphasize the red line along with the teal bar region.

    - LAB MEDIA: Figure 6A. Video Editor: Please emphasize the red line along with the green bar region.

    - LAB MEDIA: Figure 6A. Video Editor: Please emphasize the red line along with the blue bar region.

  - However, the LPS-activated cells treated with the mGluR1 antagonist showed reduced oxygen consumption [1-TXT] and diminished ATP production [2] like naive levels [3].

    - LAB MEDIA: Figure 6C. Video Editor: Please emphasize the maroon bar in the X-axis region labelled “CPCCOEt” TXT: mGluR1 antagonist CPCCOEt

    - LAB MEDIA: Figure 6C. Video Editor: Please emphasize the purple bar in the X-axis region labelled “CPCCOEt”

    - LAB MEDIA: Figure 6C. Video Editor: Please emphasize the maroon and purple bars in the X-axis region labelled “Naive”

  - Furthermore, Glycolysis assays confirmed that only untreated activated cells effectively utilize glycolytic pathways [1], as antagonist treatment reduced glycolytic activity [2] to match naive conditions, collectively demonstrating that mGluR1 signaling is vital for CD8+ T cell metabolic reprogramming during activation [3].

    - LAB MEDIA: Figure 6D. Video Editor: Please emphasize the blue, maroon and purple bars in the X-axis region labelled “Activated”

    - LAB MEDIA: Figure 6D. Video Editor: Please emphasize all the bars in the X-axis region labelled “CPCCOEt”

    - LAB MEDIA: Figure 6D. Video Editor: Please emphasize all the bars in the X-axis region labelled “Naive”