1.1.1 (0:00-0:07) - Open the QX Manager software to begin the analysis.

1.1.2 (0:07-0:14) - Click on the Browse button and open the file you want to analyze.

1.1.3 (0:19-0:25) - Once the file is loaded, the dashboard view appears on the left panel. Select your wells of interest, here C03 and G03, by holding the Ctrl key while clicking.

1.1.4 (0:27-0:31) - Click on 1D Amplitude, present on the left side of the panel. Each blue dot on the graph represents a single droplet.

1.1.5 (0:31-0:38) - Select Threshold Multiple Wells. This option allows you to apply the same threshold to more than one well.

1.1.6 (0:38-0:39) - Enter the threshold value based on where you see a clear separation between positive and negative droplets. In this example, the threshold is 4990.

1.1.7 (0:39-0:45) - View the well data section on the top right. It has multiple columns showing the sample description and concentration values of 583 and 137 copies/µL.

1.1.8 (0:48-0:50) - It also shows the number of accepted droplets, as well as positive and negative droplet counts.

1.1.9 (0:50-0:53) - Click Save to record your thresholding results.