# Submission ID #: 69742

# Scriptwriter Name: Kathyayani Sridharan

# Project Page Link: https://review.jove.com/account/file-uploader?src=21271358

# Title: Multiplexed Immunofluorescence Assay for Spatial Assessment of Senescence Markers in vivo

# Authors and Affiliations:

# Ryan Wallis, Federica Mossa, Metka Gorkič Casey, Luke Gammon, Cleo L. Bishop

# Blizard Institute, Faculty of Medicine and Dentistry, Queen Mary University of London

# Corresponding Authors:

Cleo L. Bishop				c.l.bishop@qmul.ac.uk

Ryan Wallis 				r.wallis@qmul.ac.uk

# Email Addresses for All Authors:

Federica Mossa 			f.mossa@qmul.ac.uk

Metka Gorkič Casey 			m.gorkic@qmul.ac.uk

# Luke Gammon				l.gammon@qmul.ac.uk

Cleo L. Bishop				c.l.bishop@qmul.ac.uk

Ryan Wallis 				r.wallis@qmul.ac.uk

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No.

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes

If Yes, we will need you to record using screen recording software.

We recommend using the screen capture program OBS. JoVE’s tutorial for using OBS Studio is provided at this link: https://review.jove.com/v/5848/screen-capture-instructions-for-authors?status=a7854k

As these files are necessary for finalizing your script, please upload all screen-captured video files to your project page as soon as possible.

Authors: Please create screen capture videos for shots labelled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=21271358

3. Filming location: Will the filming need to take place in multiple locations?   No

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. No

Current Protocol Length

Number of Steps: 16

Number of Shots:  48

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context

Note: The statements have been edited for length and clarity

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Dr Ryan Wallis: Spatial analysis of senescent cells is important for understanding processes like aging, cancer, immunity and normal tissue homeostasis.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Dr Ryan Wallis: Detection of senescence relies on profiling multiple biomarkers. Multiplex immunofluorescence imaging enables high throughput profiling within a single tissue section.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Dr Ryan Wallis: This protocol allows probing a combination of senescence, structural, and immune biomarkers within a single microscopy slide.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Are there any additional methods or analyses that can be performed following this procedure?

  - Dr Ryan Wallis: By detecting combination markers within a single sample, spatial analysis of tissue architecture and senescent cell distribution can be performed.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Dr Ryan Wallis: Future applications include large-scale profiling of senescence biomarkers across tissue types and integration with spatial omics to better resolve cellular heterogeneity.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

Ethics Title Card

The tissue samples used in this protocol were obtained in compliance with all relevant institutional and regulatory guidelines.

# Protocol

Videographer: Please always record an audio slate while filming each shot mentioned in the shotlist.

- Slide Preparation and Mounting Human Tissue Samples for Multiplex Immunofluorescence

Demonstrator: Dr Federica Mossa

  - To begin, verify that the five-micrometer thick formalin-fixed paraffin-embedded, or FFPE (F-F-P-E) tissue sections with a maximum size of 2 cm x 2 cm (two-by-two centimeter square) are centrally mounted on positively charged microscopy slides [1] and are clearly visible within the Coverslipless Slide Holder's viewing window [2-TXT].

    - WIDE: Talent holding up and examining the central positioning of FFPE tissue sections on charged microscopy slides

    - Talent confirming the visibility of FFPE tissue section through window. TXT: Tissue section: Cores of 34 tissue types

  - Perform [1] sample preparation steps [2] either manually using a decloaking chamber [3] or automatically using an automated stainer [4].

    - Talent preparing the samples for loading into Coverslipless Slide Holder

    - TEXT ON A PLAIN BACKGROUND
Preparation Steps:
	1) Dewaxing, rehydration and permeabilization of FFPE sections
	2) Antigen Retrieval
	3) Blocking and
	4) DAPI Counterstaining

    - Talent prepares the sample manually with a decloaking chamber.

    - Talent preparing the sample automatically using an automated stainer.

  - Place the prepared slides into the Coverslipless Slide Holder [1]. Align the top and right edges against the metal stops [2]. Lock the slide in place using both hands at the same time to turn the plastic cams to the closed padlock position [3].

    - Talent placing the prepared slides into Coverslipless Slide Holder.

    - Talent aligning the top and right edges of the slide against metal stops.

    - Talent using both hands to turn the plastic cams to lock the slide in place.

  - Verify the slide is seated completely flat and does not rock within the holder [1]. Place the Click Well insert chamber over the slide [2] and pinch the side bars with two hands to limit pressure and prevent breakages [3].

    - Talent checking if the slide is seated flat and steady without moving.

    - Talent placing the Click Well insert chamber over the slide.

    - Talent pinching the side bars with two hands to limit pressure and prevent breakages.

  - Load the Click Well with two milliliters of mounting media containing fifty percent glycerol in phosphate-buffered saline via the inlet port [1]. Add a barcode label to the left-hand side of the slide in either orientation, then proceed for imaging [2].

    - Talent loading the Click Well with 2 mL of mounting media. Videographer: Obtain multiple reusable takes of 2.5.1 as these will be used later in the protocol.

    - Talent adding barcode label to the left-hand side of the slides on both orientations.

- Multiplex Imaging

Demonstrator: Dr Luke Gammon

  - For region selection and autofluorescence analysis, open the Mx (M-X) workflow version 4.2.0.4 (four-two-zero- four) multiplex imaging acquisition software [1]. Use the protocol publisher wizard to create a protocol with the required rounds, biomarkers and exposure times [2].

    - SCREEN: Show the multiplex imaging acquisition software opening on the computer.

    - SCREEN: Show the protocol publisher wizard and enter the required rounds, biomarkers, and exposure times to create the protocol.

Authors: Please create screen capture videos for shots labelled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=21271358

  - Acquire the Cy3 (Sigh-three) channel for virtual hematoxylin and eosin or H&E (H and E) images [1]. Use the batch creator tool to add the slides to a protocol with the barcode number for creating a multiplex experiment workflow [2].

    - SCREEN: Show the acquisition settings with the Cy3 channel enabled for virtual H&E imaging.

    - SCREEN: Show the Batch Creator tool and enter the barcode number for the slides to create the multiplex experiment workflow.

  - Load the slide into the microscope using the acquisition software [1]. Follow the workflow by drawing regions of interest via 2x (two-X) and 10x (ten-X) imaging steps for each slide, ensuring each field of view has at least two neighbors to prevent stitching issues [2].

    - SCREEN: Show the acquisition software loading the slide into the microscope workflow.

    - SCREEN: Show regions of interest being drawn via 2x and 10x imaging steps for each slide ensuring each field has at least 2 neighboring fields.

  - Perform the 20x (twenty-X) autofluorescence imaging using the previously set exposure times [1]. Use quality control or QC (Q-C) wizard to scan for out of focus regions or artefacts; fail the QC if detected and reimage [2]. Generate background images for subtraction from the final biomarker signal and repeat for each multiplex round [3].

    - SCREEN: Show the 20x autofluorescence imaging step starting with the preset exposure times.

    - SCREEN: Show the QC wizard scanning for out of focus regions or artefacts.

    - SCREEN: Show background images being generated in the software for signal subtraction.

  - Prior to sequential biomarker staining, centrifuge the antibodies at 13000 g for one minute at room temperature to limit artefacts [1]. Prepare the antibody solution at a 1:200 (one-in-two hundred) dilution in phosphate-buffered saline or PBS (P-B-S) containing 0.3 (zero-point three) percent bovine serum albumin or BSA (B-S-A) [2]. Remove the mounting media by pipetting out from the Click Well port [3] and wash thrice with two milliliters of PBS containing 0.01 (zero-point zero one) percent Tween twenty or PBST (P-B-S-T) for five minutes [4].

    - Talent centrifuging the antibody sample. Videographer: Please capture the digital meter on the centrifuge showing the centrifuge speed and time.

    - Talent preparing the antibody solution in PBS containing 0.3% BSA.

    - Talent is removing the mounting media from the well. Videographer: Obtain multiple reusable takes of 3.5.3 and 3.5.4 as these will be used later in the protocol.

    - Talent washing the well with 2 mL PBST.

  - Add three hundred and fifty microliters of antibody solution into the port to ensure full sample coverage within the staining chamber [1]. Incubate for one hour at room temperature [2]. Next, pipette out the antibody solution from the Click Well port [3] and wash thrice with two milliliters of PBST for five minutes [4].

    - Talent adding 350 µL antibody solution to the well port.

    - Talent incubating the slides for one hour at RT.

    - Talent removing the antibody solution from the well port.

    - Reuse 3.5.4

  - Add two milliliters of mounting media [1]. Insert Click Well holder into the microscope [2] to initiate automatic round specific imaging step [3]. Then, progress the workflow by passing QC steps as directed on screen [4].

    - Reuse 2.5.1

    - Talent inserting the Click Well holder into the microscope. Videographer: Obtain multiple reusable takes of 3.7.2 as these will be used later in the protocol.

    - SCREEN: Show the automatic round-specific imaging step being started in the software.

    - SCREEN: Show the workflow progress and QC prompts being accepted on screen

  - Pipette out the mounting media from the Click Well port [1] and wash thrice with two milliliters of PBST for five minutes [2]. Prepare [3] the dye inactivation solution freshly before use [4].

    - Reuse 3.5.3

    - Reuse 3.5.4

    - Talent preparing the dye-inactivation solution.

    - TEXT ON A PLAIN BACKGROUND
	Double distilled water		 7 mL
	0.5 M NaHCO3 		2 mL
	30% H2O2			1 mL
	Prepare the solution freshly before use and apply immediately without delay.

  - After removing PBST, add two milliliters of dye inactivation solution for each slide [1]. Incubate for fifteen minutes in the slide holder [2]. Pipette out the dye inactivation solution from the Click Well port [3] and wash with two milliliters of PBST for five minutes [4].

    - Talent adding 2 mL dye inactivation solution.

    - Talent placing the slides for 15 minutes in slide holder.

    - Talent removing the dye-inactivation solution from the well port.

    - Reuse 3.5.4

  - Add two milliliters of mounting media and insert Click Well holder into the microscope [1] to initiate automatic round specific 20x autofluorescence imaging [2]. Progress the workflow by passing QC steps as directed on screen upon completion [3].

    - Reuse 2.5.1 and 3.7.2.

    - SCREEN: Show the automatic round-specific 20x autofluorescence imaging step being started in the software.

    - SCREEN: Show the workflow progress and QC prompts being accepted on screen

  - Repeat the sequential biomarker staining, dye inactivation, and multiplex imaging procedure for as many rounds or markers as required and continue until all required rounds are completed [1]. Use digital pathology software to perform automated HighPlex analysis and data export to finalize the study [2].

    - SCREEN: Show the workflow returning to the next round of biomarker staining and imaging and multiple completed rounds listed in the experiment workflow.

    - SCREEN: Show the data analysis using DPS software.

Authors: Please create screen capture videos for shots labelled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=21271358

# Results

- Results

  - Compared to the automated slide stainer-based method, the manual, decloaking chamber-based method resulted in a higher frequency of tissue artifacts, characterized specifically by tissue detachment [1], as well as lost [2] or slipped cores [3].

    - LAB MEDIA: Figure 3: Video Editor: Please emphasize the row “Tissue detachment” in BOND and Manual columns sequentially.

    - LAB MEDIA: Figure 3: Video Editor: Please emphasize the row “Missing Cores” in BOND and Manual columns sequentially.

    - LAB MEDIA: Figure 3: Video Editor: Please emphasize the row “Slipped Cores” in BOND and Manual columns sequentially.

  - Quantification performed with digital pathology software on representative pancreas tissue revealed that p16-positive cells were associated with lower LB-1 levels [1] and vice versa, demonstrating a profile consistent with senescence-associated marker patterns [2].

    - LAB MEDIA: Figure 6B: Video Editor: Please emphasize the orange region in the image.

    - LAB MEDIA: Figure 6C: Video Editor: Please emphasize the green region in the image.

  - A Z-score (Zee-score) normalized heatmap illustrates the distribution of the p16+/LB1-ve (P-sixteen-positive-Lamin-B-one negative) senescence phenotype across various tissues, revealing high concentrations of senescent cells in the lung, adrenal and pituitary glands [1] corresponding with elevated p16 [2] and diminished LB1 levels [3].

    - LAB MEDIA: Figure 6D: Video Editor: Please emphasize the boxes corresponding to “Lung, Adrenal and Pituitary” and the right-side label “Senescent”.

    - LAB MEDIA: Figure 6D: Video Editor: Please emphasize the boxes corresponding to “Lung, Adrenal and Pituitary” and the right-side label “p16”.

    - LAB MEDIA: Figure 6D: Video Editor: Please emphasize the boxes corresponding to “Lung, Adrenal and Pituitary” and the right-side label “Lamin B1”.

  - Conversely, the appendix, lymph node and spleen exhibited low senescence [1], maintaining high LB1 [2] and low p16 expression [3].

    - LAB MEDIA: Figure 6D: Video Editor: Please emphasize the boxes corresponding to “Appendix, Lymph node and spleen” and the right-side label “Senescent”.

    - LAB MEDIA: Figure 6D: Video Editor: Please emphasize the boxes corresponding to “Appendix, Lymph node and spleen” and the right-side label “Lamin B1”.

    - LAB MEDIA: Figure 6D: Video Editor: Please emphasize the boxes corresponding to “Appendix, Lymph node and spleen” and the right-side label “p16”.