# Submission ID #: 69847

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=21303383

# Title: A Custom-Built Recirculating Zebrafish Housing System

# Authors and Affiliations:

# Mikaela L Fairbanks1*, Kaitlyn F Poppo1*, Brian Lipscomb2, Danny C LeBert1

# 1Department of Biology, Northern Michigan University

# 2Department of Biology, Shenandoah University

# *These authors contributed equally

# Corresponding Authors:

# Mikaela L Fairbanks 	mfairban@nmu.edu

# Email Addresses for All Authors:

# Kaitlyn F Poppo 	kpoppo@nmu.edu

# Brian Lipscomb 	blipscom@su.edu

# Danny C LeBert 	dlebert@nmu.edu

# Mikaela L Fairbanks 	mfairban@nmu.edu

## Author Questionnaire

1. We have marked your project as author-provided footage, meaning you film the video yourself and provide JoVE with the footage to edit. JoVE will not send the videographer. Please confirm that this is correct.

√ Correct

2. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

3. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

4. Proposed filming date: To help JoVE process and publish your video in a timely manner, please indicate the proposed date that your group will film here: 04/26/26 or later

When you are ready to submit your video files, please contact our Content Manager, Utkarsh Khare.

Current Protocol Length

Number of Steps:  25

Number of Shots:  55

# Introduction

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context.

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Kaitlyn Poppo: We designed and tested an affordable recirculating zebrafish housing system for budget conscious research labs and education programs.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Mikaela Fairbanks: Many existing zebrafish housing systems are cost prohibitive for smaller research labs, so we designed an inexpensive housing system.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

CONCLUSION:

What does this protocol allow researchers to study or measure?

1.3., Mikaela Fairbanks: This protocol allows researchers to build a zebrafish housing system that they then can use for both research or educational purposes.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the most important consideration or challenge when performing this protocol.

  - Kaitlyn Poppo: Proper tubing connections, slope of return lines, and secure fittings are essential to maintain consistent water circulation and prevent leaks.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Kaitlyn F Poppo: Future studies can modify this system for larger zebrafish colonies and automated water quality monitoring.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Ethics Title Card

This research has been approved by the Institutional Animal Care and Use Committees at both Northern Michigan University and Shenandoah University

# Protocol

- Assembling the Vertical Return and Sump Basin

Demonstrators: Mikaela L Fairbanks and Kaitlyn F Poppo

  - To begin, assemble the housing rack by fixing support brackets at the required heights [1] and install a vertical return pipe [2].

    - WIDE: Talent showing the assembled support brackets.

    - Talent installing vertical support with drilled holes.

  - Using a saw, cut a 3.81 centimeter diameter polyvinyl chloride pipe into three pieces each measuring roughly 121.92 centimeters in length [1].

    - Talent cutting the 3.81 centimeter polyvinyl chloride pipe into three equal 121.92 centimeter sections on a work surface.

  - Then, make two lengthwise cuts along each 121.92 centimeter pipe section to create a U-shaped trough [1].

    - Talent cutting lengthwise along a pipe section to form a U-shaped trough.

  - Place each U-shaped trough onto a shelf with one end inserted into the T joint of the main vertical return [1]. Using the previously inserted string or wire, secure the opposite end of each trough to create a gentle downward slope toward the T joint of the vertical return [2]. Adjust the trough to achieve a 1.5 to 2 degree slope toward the main vertical return [3].

    - Talent positioning a U-shaped trough on a shelf and inserting one end into the T joint of the vertical return.

    - Talent tying the opposite end of the trough to the support bracket using the pre-threaded string or wire.

    - Talent adjusting the trough angle to create a slight downward slope toward the vertical return.

  - Position the sump basin beneath the assembled rack [1]. Ensure a clearance of approximately 14.29 centimeters between the top of the basin and the underside of the first shelf [2]. Adjust the basin so it protrudes slightly from the shelf and aligns with the overhanging vertical return pipe to direct water back into the basin [3].

    - Talent sliding the sump basin into position beneath the assembled rack.

    - Talent measuring and confirming the 14.29 centimeter clearance between the basin and the first shelf.

    - Talent adjusting the basin position so the vertical return pipe overhangs it.

  - Now, position the water pump inside the sump basin aligned with the support bracket of the tank return system [1].

    - Talent placing the water pump into the sump basin and aligning it with the support bracket.

- Assembling the Main Water Supply Line

  - Cut a 1.905 centimeter vinyl tubing to a length of 69.85 centimeters [1] and connect the tubing to the pump [2].

    - Talent cutting the 1.905 centimeter vinyl tubing to 69.85 centimeters in length.

    - Talent attaching the cut tubing securely to the pump outlet.

  - Connect another T coupler to the tubing [1-TXT]. Position this T coupler slightly below the second shelf and orient it toward the far side of the sump basin [2].

    - Talent inserting the tubing into a second T coupler. TXT: T coupler: 1.905 × 1.905 × 1.27 cm

    - Talent positioning the T coupler slightly below the second shelf and facing the far side of the sump basin.

  - Using a box cutter, cut a 1.905 centimeter vinyl tubing to a length of 36.195 centimeters [1] and fasten the tubing directly to the opposite end of the T coupler from the previous step [2].

    - Talent cutting the 1.905 centimeter vinyl tubing to 36.195 centimeters in length.

    - Talent securing the tubing to the open end of the T coupler.

  - Sequentially connect 36.195 and 36.83-centimeters vinyl tubing to T couplers, adding a new T coupler after each segment and positioning it slightly below the second shelf, facing the far side of the sump basin. [1].

    - Shot of the assembly after connecting 2 more vinyl tubing and T couplers and talent pointing to the connected segments. Authors: The repetitive steps have been summarized into 1 line as per our journal guidelines

  - Now, connect a 1.905 centimeter to 1.27 centimeter reducing elbow with a 1.27 centimeter male adaptor to the tubing [1]. Seat the reducing elbow and male adaptor on the top surface of the fourth shelf [2]. Orient the elbow and adaptor to connect to the vinyl pressure release line that runs back down to the sump [3].

    - Talent attaching the reducing elbow and male adaptor to the tubing.

    - Talent placing the reducing elbow and male adaptor on the top surface of the fourth shelf.

    - Talent orienting the elbow and adaptor toward the direction of the vinyl pressure release line leading back to the sump.

- Assembling the Pressure Release and Tank Water Supply Lines

  - Cut a 1.5875 centimeter vinyl tubing to a length of 7.62 centimeters [1]. Connect the tubing to the 1.27 centimeter male adaptor at the top of the shelf [2] and connect a ball valve to the free end of the tubing [3].

    - Talent cutting the 1.5875 centimeter vinyl tubing to 7.62 centimeters in length.

    - Talent attaching the tubing to the 1.27 centimeter male adaptor at the top of the shelf.

    - Talent securing a ball valve onto the end of the tubing.

  - Next, using a box cutter, cut a 1.5875 centimeter vinyl tubing to a length of 213.36 centimeters [1]. Connect the tubing to the free fitting of the ball valve [2].

    - Talent cutting the 1.5875 centimeter vinyl tubing to 213.36 centimeters in length.

    - Talent attaching the tubing to the outlet of the ball valve.

  - Route the tubing across the top of the rack [1] and then direct it down along the leg of the housing rack into the sump basin [2]. Using fastening straps, secure the tubing along the leg of the housing rack [3].

    - Talent laying the tubing across the top of the rack.

    - Talent guiding the tubing down along the leg of the housing rack into the sump basin.

    - Talent fastening the tubing to the rack leg with straps.

  - Then, using a box cutter, cut three pieces of 1.575 centimeter vinyl tubing each measuring 6.35 centimeters in length [1]. Connect each piece to one of the three free T couplers on the main water supply line [2] and connect a ball joint to the free end of each tubing section [3].

    - Talent cutting three short sections of 1.575 centimeter vinyl tubing.

    - Talent attaching each tubing section to a free T coupler on the main supply line.

    - Talent attaching a ball joint to each of the three tubing ends.

  - Next, attach three vinyl tubings with 6.35 centimeter length to the free end of a ball joint [1] and connect a 1.5875 centimeter to 0.9525 centimeter reducer to each 1.5875 centimeter tubing section [2].

    - Talent connecting each tubing section to a ball joint outlet.

    - Talent attaching a reducer to each tubing section.

  - Now, cut three pieces of 0.9525 centimeter tubing each measuring 5.715 centimeters in length [1]. Connect each piece to a reducer [2]. Connect a 0.9525 centimeter nylon T coupler to each tubing section [3].

    - Talent cutting three short sections of 0.9525 centimeter tubing.

    - Talent attaching each tubing section to a reducer outlet.

    - Talent attaching a nylon T coupler to each tube.

  - Obtain six pieces of 0.9525 centimeter vinyl tubing each measuring 5.08 centimeters in length [1] and connect each piece to the nylon T couplers [2].

    - Talent cutting six short sections of 0.9525 centimeter vinyl tubing.

    - Talent attaching each tubing piece to the outlets of the nylon T couplers.

  - Attach a hose clamp to each of the six tubing pieces [1] and slip a steel aquarium valve onto each available tubing end and tighten the hose clamps to secure the valves [2].

    - Talent placing a hose clamp onto each short tubing section.

    - Talent fitting a steel aquarium valve onto each tubing end and tightening the hose clamps.

  - Using fastening straps, secure each steel aquarium valve to the shelf above [1]. Finally, using a box cutter, cut 0.4318 centimeter vinyl tubing to the appropriate length required to reach each tank [2].

    - Talent fastening each steel aquarium valve to the underside of the shelf with straps.

    - Talent measuring and cutting 0.4318 centimeter vinyl tubing to lengths sufficient to reach individual tanks.

# Results

- Results

# Authors: The results section is slightly re-worded for better clarity

  - A three-level recirculating zebrafish housing system was created following the protocol [1]. The sump conductivity remained relatively stable over time based on weekly measurements taken across a 1-year period [2].

    - LAB MEDIA: Figure 1A.

    - LAB MEDIA: Figure 3A.

  - Over the 21-day assessment period, conductivity fluctuated but remained within the safe 600 to 1000 microsiemens per centimeter range, which is safe for the zebrafish [1].

    - LAB MEDIA: Figure 3B.

  - Nitrites remained in the low category each day during the 21-day period [1].

    - LAB MEDIA: Figure 3C. Video editor: Highlight the bar for “Nitrites”

  - Nitrates stayed in the low category for 18 of the 21 days tested [1] and were in the medium category for 3 days, remaining within the safe range for zebrafish [2].

    - LAB MEDIA: Figure 3C. Video editor: Highlight the “Nitrates” bar and focus on the larger dark “low” portion.

    - LAB MEDIA: Figure 3C. Video editor: Then highlight the smaller grey “medium” portion of the same “Nitrates” bar.

  - The reproductive success was checked in the system during the same three-week period with a total of 18 mating pairs [1]. Of the 18 total mating pairs, 11 were successful, corresponding to approximately 70 percent mating success [2].

    - LAB MEDIA: Figure 3D

    - LAB MEDIA: Figure 3D. Video editor: Highlight the bar labeled “Mating success”

  - The 24-hour embryo survival rate was nearly 70 percent [1].

    - LAB MEDIA: Figure 3D. Video editor: Highlight the bar labeled “24 h survival”

  - The average clutch size for successful matings was approximately 150 embryos [1].

    - LAB MEDIA: Figure 3E.