# Submission ID #: 69853

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=21305213

# Title: Single-Molecule Localization Microscopy of Membrane Proteins Using Single-Antibody Labeling

# Authors and Affiliations:

# Neal T. Ramseier1, Herath D.W. Herath1, Alyssa Burgess1, Hirushi Gunasekara1, Ying S. Hu1–4

# 1Department of Chemistry, University of Illinois Chicago

# 2University of Illinois Cancer Center

# 3Department of Biomedical Engineering, University of Illinois Chicago

# 4Department of Biochemistry and Molecular Genetics, College of Medicine, University of Illinois Chicago

# Corresponding Authors:

# Hirushi Gunasekara		hgunas2@uic.edu

# Ying S. Hu			yshu@uic.edu

# Email Addresses for All Authors:

# Neal T. Ramseier		nramse2@uic.edu

# Herath D.W. Herath		hherat2@uic.edu

# Alyssa Burgess			aburge6@uic.edu

# Hirushi Gunasekara		hgunas2@uic.edu

# Ying S. Hu			yshu@uic.edu

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes

If Yes, we will need you to record using screen recording software.

We recommend using the screen capture program OBS. JoVE’s tutorial for using OBS Studio is provided at this link: https://review.jove.com/v/5848/screen-capture-instructions-for-authors?status=a7854k

Please upload all screen-captured video files to your project page as soon as possible. 
Please follow the guidelines given at the end of this document

3. Filming location: Will the filming need to take place in multiple locations?   No

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. Yes

Current Protocol Length

Number of Steps: 24

Number of Shots: 45 (35 SC)

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

Authors: Answers are slightly edited in accordance with journal guidelines. 
Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context.

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Neal T Ramseier: My research focuses on the membrane of T cells and aims to reveal new nanoscale insights relevant to therapeutic targeting.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Neal Ramseier: Existing imaging methods cannot directly visualize nanoscale antibody–epitope interactions in cells; this protocol overcomes that limitation by enabling direct observation within the cellular environment.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Neal Ramseier: We can observe antibodies interacting with their membrane epitopes in a cellular environment, providing insight relevant to therapeutic antibodies.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the most important consideration or challenge when performing this protocol.

  - Neal Ramseier: The antibody concentration, non-illumination interval, and photobleaching steps are critical to optimize. Sub-optimal parameters will compromise the results.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Are there any additional methods or analyses that can be performed following this procedure?

  - Neal Ramseier: Single-Antibody Labeling can be extended to simultaneously evaluate membrane protein binding by multiple antibodies within the same cellular environment.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Videographer: Obtain headshots for all authors available at the filming location.

# `Testimonial Questions (OPTIONAL):

# Videographer: Please capture all testimonial shots in a wide-angle format with sufficient headspace, as the final videos will be rendered in a 1:1 aspect ratio. Testimonial statements will be presented live by the authors, sharing their spontaneous perspectives.

- Testimonial statements will not appear in the video but may be featured in the journal’s promotional materials.

- Provide the full name and position (e.g., Director of [Institute Name], Senior Researcher [University Name], etc.) of the author delivering the testimonial.

- Please answer the testimonial question live during the shoot, speaking naturally and in your own words in complete sentences.

What is the key takeaway you hope viewers remember after watching your video article?

  - Neal Ramseier, PhD Candidate: (authors will present their testimonial statements live)

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

How does the video format of your article help you to share your work with the broader scientific community?

  - Neal Ramseier, PhD Candidate: (authors will present their testimonial statements live)

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

# Protocol

- Image Acquisition After Gold Colloid Deposition on the Cells

Demonstrator: Neal Ramseier

  - After mounting the gold-colloid-deposited cells on the microscope, use brightfield illumination to verify that enough colloids have been deposited within the region of interest [1]. Confirm that the ideal density of 5 to 10 gold colloids is present within the region of interest before proceeding [2].

    - WIDE: Talent adjusting the microscope to brightfield illumination.

    - Talent viewing the sample.

  - For the optical configuration of the antibody probe fluorophore, open the imaging software and access the optical configuration settings [1]. Select the appropriate dichroic mirror [2] and set the laser excitation wavelength and power density [3]. Adjust the camera integration time and select the emission filter [4], then set the camera pixel binning to achieve a pixel size of 150 to 160 nanometers [5].

    - SCREEN: Display the imaging software main interface and navigate to the optical configuration setup panel.

    - SCREEN: Show the dichroic selection menu and select the appropriate dichroic mirror.

    - SCREEN: Adjust the laser excitation wavelength and set the power density for the selected fluorophore.

    - SCREEN: Set the camera integration time and choose the correct emission filter in the software.

    - SCREEN: Configure camera pixel binning and display the resulting pixel size of 150 to 160 nanometers.

Authors: Please create screen capture videos of the shots labeled as SCREEN, write a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=21305213

  - Now, create an optical configuration for the photobleaching step by selecting a photobleaching mode in the imaging software [1]. Set the laser power to 100% [2] and adjust the camera integration time to 1 second [3].

    - SCREEN: Open the photobleaching configuration panel in the imaging software.

    - SCREEN: Increase and confirm the laser power setting to 100%.

    - SCREEN: Set the camera integration time parameter to 1 second.

  - Set the imaging parameters in the image acquisition software by defining the non-illuminating interval, number of frames, and frequency of the photobleaching step [1].

    - SCREEN: Input the non-illuminating interval, total frame number, and photobleaching frequency values as required.

  - Next, prepare the imaging buffer by diluting the CD81 antibody to a concentration of 1 microgram per milliliter in 200 microliters of 5 percent BSA made in DPBS [1]. The final recommended antibody concentration is 0.5 nanomolar for Jurkat T cells or 2 nanomolar for U2OS cells [2].

    - Talent pipetting the CD81 antibody into a tube with bovine serum albumin in Dulbecco’s phosphate-buffered saline.

    - Close-up of the prepared imaging buffer labeled with the appropriate final antibody concentration.

  - Add the prepared imaging buffer to the well containing the cells [1] and start the image acquisition in the software to begin data collection [2].

    - Talent gently adding the imaging buffer to the well.

    - SCREEN: Show the acquisition control panel and initiate the start of image acquisition.

- Imaging Parameter Optimization

  - Use the live-view function in the imaging software configured for membrane single-antibody labeling to monitor antibody binding in real time [1].

    - SCREEN: Display the live-view window in the imaging software configured for mSAL, showing antibody binding events appearing on the sample.

  - Configure the imaging parameters in the image acquisition software to obtain at least a 10-frame acquisition [1]. Set the non-illuminating interval to 5 seconds [2].

    - SCREEN: Navigate to the image acquisition settings panel in the software.

    - SCREEN: Enter the total number of frames and set the non-illuminating interval to 5 seconds.

  - Start the acquisition [1] and evaluate the sparsity of single-molecule localizations in the recorded movie [2].

    - SCREEN: Click the start acquisition button in the imaging software.

    - SCREEN: Playback or display the recorded movie while observing the distribution of single-molecule localization events.

  - If multiple overlapping antibody-binding events are observed immediately after adding the imaging buffer or if single-molecule localizations accumulate over time, abort the acquisition [1]. Reduce the antibody concentration by 2-fold [2] and repeat the acquisition on a new sample [3]. Continue reducing the antibody concentration by 2-fold and reevaluating the number of events per unit area until the desired single-molecule localization density is achieved [4].

    - SCREEN: Show dense or overlapping localization events appearing in the acquisition movie.

    - Talent mixing new buffer.

    - Talent adding the adjusted antibody solution to a new sample well.

    - SCREEN: Restart acquisition and reassess event density in the new recording.

  - If the event density is sparse, increase the antibody concentration in the imaging buffer by 2-fold [1]. Repeat the acquisition on a new sample and continue increasing the antibody concentration by 2-fold while reevaluating the number of binding events per unit area until the desired single-molecule localization density is achieved [2].

    - Talent viewing the data appearing on the screen.

    - SCREEN: Display the new acquisition showing an increased but controlled density of single-molecule localization events.

  - For non-illuminating interval optimization, configure the imaging parameters in the image acquisition software to obtain a 50-frame acquisition [1].

    - Talent accessing the image acquisition settings in the imaging software.

  - After adding the antibody and starting acquisition, evaluate the density and sparsity of single-molecule localizations in the recorded movie [1]. Determine the lowest non-illuminating interval that yields an optimal single-molecule event density [2].

    - SCREEN: Playback of the recorded movie showing single-molecule localization events.

    - SCREEN: Comparison of localization densities across different non-illuminating interval settings to identify the optimal value.

  - Finally, if spatially overlapping binding events occur as the image acquisition progresses, introduce a photobleaching step at regular intervals [1]. Adjust the photobleaching frequency so that fluorescence from bound antibodies is removed before overlapping events accumulate [2].

    - SCREEN: Display overlapping binding events appearing over successive frames in the acquisition movie.

    - SCREEN: Configure and apply a photobleaching step at defined frame intervals, such as every fifth frame, in the imaging software.

- Data and Cluster Analysis

  - Set the filepath in the MATLAB software to the folder that contains the reconstructed mSAL comma-separated values file [1].

    - SCREEN: Show the MATLAB interface with the user navigating to the section of the code where the filepath is defined and selecting the folder.

  - Run the code by clicking Run in the MATLAB toolbar [1]. When prompted, select the mSAL.CSV (M-sal C-S-V) file [2] and click Open to load it into the program [3].

    - SCREEN: Cursor moving to the MATLAB toolbar and clicking the Run button.

    - SCREEN: File selection dialog opening and the mSAL comma-separated values file being highlighted.

    - SCREEN: Cursor clicking the Open button to confirm file selection.

  - Run the analysis using the previously defined inputs as a starting point [1]. Evaluating the scatter plot containing the unclustered localization data [2], choose a minimum point value that is higher than the number of noise localizations but lower than the number of localizations on the cell [3]. Adjust epsilon so that the selected number of localizations is captured within the neighborhood radius for representative on-cell regions [4].

    - SCREEN: MATLAB command window or script editor showing initial analysis parameters being used.

    - SCREEN: MATLAB Figure 1 showing the localizations on the cell and outside the cell.

    - SCREEN: Parameter field or code line where the minimum points value is entered and modified.

    - SCREEN: Adjustment of the epsilon parameter to refine the neighborhood radius.

  - Evaluate the window that depicts the clustered data [1]. Confirm that clusters are retained on the cell with the expected phenotype while localizations outside the cell are minimized [2].

    - SCREEN: Figure 2 window displayed in MATLAB showing clustered localization data.

    - SCREEN: Close-up of the plotted data highlighting clusters on the cell and reduced background localizations.

  - If the clustering parameters are too restrictive, decrease the minimum points value and increase epsilon [1]. If the clustering parameters are too inclusive, increase the minimum points value and decrease epsilon [2].

    - SCREEN: MATLAB script or parameter panel showing the minimum points value being decreased and epsilon increased.

    - SCREEN: MATLAB script or parameter panel showing the minimum points value being increased and epsilon decreased.

  - Finally, after adjusting the minimum points value and epsilon, rerun the code to obtain optimal clustering [1].

    - SCREEN: Cursor clicking Run again in the MATLAB toolbar to re-execute the analysis.

# Results

- Results

  - Jurkat T cells were immobilized with sufficient spacing to preserve membrane integrity and allow antibody access to membrane epitopes [1].

    - LAB MEDIA: Figure 1A.

  - Gold colloids were visualized and used as fiducial markers for lateral drift correction during mSAL image acquisition [1].

    - LAB MEDIA: Figure 2A AND B. Video editor: Highlight the white arrows pointing to small round gold particles.

  - Optimization of the non-illumination interval showed that a 5-second interval produced antibody binding events with minimal spatial overlap compared to shorter or longer intervals at 1 nanomolar antibody concentration [1].

    - LAB MEDIA: Figure 3A. Video editor: Highlight the panels labeled 5s.

  - Application of drift correction improved the reconstructed mSAL image compared to the uncorrected reconstruction [1].

    - LAB MEDIA: Figure 4B. Video editor: Show the reconstructed image after correction and zoom into the boxed region.

  - Density-based clustering of mSAL localizations reduced low-density background interactions and highlighted clustered CD81 binding events [1].

    - LAB MEDIA: Figure 5. Video editor: Focus on the black box in C.

NOTE to the Authors:

You have two options for filming and submitting the SCREEN footage:

Option 1: Uploading Separate Clips (Highly Recommended)

Please upload a separate video file for each SCREEN shot (tagged as SCREEN), naming the file according to the shot number (e.g., 2.4.3, 1.5.1, 3.2.1, 3.1.3, etc.).

- Each clip must be a maximum of 20 to 25 seconds to match the voice narration.

- If an action takes longer, provide only the best, most representative 20-25 second segment.

Option 2: Uploading a Single Continuous Video

If you prefer to record all steps together and upload a single video file, that is acceptable. In this case, you must write the precise timestamp for each shot within the script.

For example, if the action described in shot 2.1.2 is present in the file xxx.mp4 and occurs between timestamps 00:30 and 00:45 , then write the following in the script after each shot description or as a comment bubble.

(write the file name and timestamp after each shot): [File name] [Start Time]-[End Time]

- Example 	2.1.2  SCREEN: performing ...........action. xxx.mp4 00:30-00:45

    - SCREEN: cutting the…..........action. xxx.mp4 01:10-01:20

- The duration between the start and end times for each shot must be limited to 20 to 25 seconds.

- If a shot is longer than 25 seconds, choose two short segments (one at the beginning and one at the end) that collectively do not exceed 25 seconds.

- Example: 3.1.1 SCREEN: inserting a needle... xxx.mp4 00:00-00:10 and 02:30-02:40