# Submission ID #: 69900

# Scriptwriter Name: Kathyayani Sridharan

# Project Page Link: https://review.jove.com/account/file-uploader?src=21319548

# Title: Real-World M3-BREATHE: Toward Multimodal Mobile Monitoring of Behaviour, Respiration, and Exposures for Treatment and Health Evaluation

# Authors and Affiliations:

# Wenbo Guo

# Institute of Belt and Road & Global Governance, Fudan University

# Corresponding Authors:

# Wenbo Guo			wenbo_guo@fudan.edu.cn

# Email Addresses for All Authors:

# Wenbo Guo			wenbo_guo@fudan.edu.cn

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

Authors: Please create screen capture videos for shots labelled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible:

https://review.jove.com/account/file-uploader?src=21319548

3. Filming location: Will the filming need to take place in multiple locations?   No

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. Yes

Current Protocol Length

Number of Steps: 13

Number of Shots: 59 (41 SC)

# Interviews

Videographer: Obtain headshots for all authors available at the filming location.

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context

Note: The statements have been edited for length and clarity

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Wenbo Guo: I develop M3-BREATHE application to integrate behavior, respiration, and environment, examining how mobility influences real-time health risks and outcomes.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Wenbo Guo: Existing studies use static locations and miss mobility; this protocol captures high-resolution, synchronized, individual-level data across behavior, physiology, and exposures.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Wenbo Guo: M3-BREATHE enables real-time measurement of personalized exposure, inhaled doses of environmental pollutants, and physiological responses during daily activities and travel.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the most important consideration or challenge when performing this protocol.

  - Wenbo Guo: The key challenge is reliable device calibration, synchronization, and sustained participant compliance in real-world, long-term monitoring.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Wenbo Guo: Future work can integrate AI interventions, genomics, and cross-city deployments to study causal pathways and improve precision environmental health.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

`Testimonial Questions (OPTIONAL):

# Videographer: Please capture all testimonial shots in a wide-angle format with sufficient headspace, as the final videos will be rendered in a 1:1 aspect ratio. Testimonial statements will be presented live by the authors, sharing their spontaneous perspectives.

- Testimonial statements will not appear in the video but may be featured in the journal’s promotional materials.

- Provide the full name and position (e.g., Director of [Institute Name], Senior Researcher [University Name], etc.) of the author delivering the testimonial.

- Please answer the testimonial question live during the shoot, speaking naturally and in your own words in complete sentences.

What problem in your field of research does your JoVE article address?

  - Wenbo Guo, Associate Research Professor, Fudan University: (authors will present their testimonial statements live)

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

How does the video format of your article help you to share your work with the broader scientific community?

  - Wenbo Guo, Associate Research Professor, Fudan University: (authors will present their testimonial statements live)

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

# Ethics Title Card

# This research was approved by Peking University’s Human Research Ethics Committee

# Protocol

Videographer: Please always record an audio slate while filming each shot mentioned in the shotlist.

- Setup and Configuration of Wearable Physiological and Portable Environmental Sensors

Demonstrator: Wenbo Guo

  - To begin, instruct the participant to wear the sensor-integrated shirt or chest strap, directly against their skin beneath their clothing to ensure optimal contact [1]. Adjust the fit so that it is snug and comfortable without constraining breathing or movement [2]. Confirm stable skin contact by checking that the sensor does not slip during deep inhalation [3].
2.1.1. WIDE: Talent assisting participant in wearing the sensor-integrated shirt or chest strap under clothing.

Videographer: Please obtain CU shot as well for this shot.
2.1.2. Close-up of talent adjusting the shirt or strap for a snug fit.
2.1.3. Close-up of participant taking a deep breath while the sensor remains stable.

  - Ensure the sensor is fully charged and pair it with the smartphone via Bluetooth for real-time monitoring [1]. Confirm synchronization of the device clock with the smartphone system time to enable accurate temporal alignment across multimodal data streams [2]. Verify that respiratory rate, tidal volume, and heart rate are being recorded on the smartphone [3].
2.2.1. SCREEN: Show the sensor’s battery status.
2.2.2. SCREEN: Show Bluetooth pairing between the sensor and smartphone and confirm connection.
2.2.3. SCREEN: Display real-time recordings of respiratory rate, tidal volume, and heart rate on the application.

  - To set up the smart wristband, choose the non-dominant wrist and secure the device proximal to the ulnar styloid process [1]. Adjust the strap to ensure proper sensor-skin contact while allowing insertion of one fingertip [2]. Confirm that the photoplethysmography or PPG (P-P-G) sensor maintains constant and flush contact with the skin [3].

    - WIDE: Talent placing the wristband on the participant’s non-dominant wrist.

    - Close-up of strap adjustment demonstrating snug fit with fingertip insertion.

    - Close-up of the underside of the wristband showing sensor contact with skin.

  - Open the companion application on the smartphone and connect the wristband via Bluetooth [1]. Navigate to the real-time display and configure continuous monitoring for all physiological parameters [2].

    - SCREEN: Show opening the application and selecting the device for Bluetooth connection.

    - SCREEN: Navigate to settings and enable continuous monitoring options.

  - Monitor the cardiovascular metrics including heart rate, or HR (H-R) in beats per minute [1], heart rate variability, or HRV (H-R-V) in milliseconds [2], blood pressure, or BP (B-P) in millimeters of mercury or mmHg (M-M-H-G) [3], electrocardiogram, or ECG (E-C-G) in millivolts or mV (M-V) [4].

    - SCREEN: Show the heart rate readings in beats/min.

    - SCREEN: Show the HRV values being recorded in milliseconds.

    - SCREEN: Show the BP values in mmHg.

    - SCREEN: Show the ECG being recorded in mV.

  - Similarly, monitor the respiratory and metabolic parameters like algorithmically estimated respiratory rate, or RR (R-R) in breaths per minute [1], blood oxygen saturation, or SpO2 (S-P-O-two) in percentage [2] and metabolic equivalents ratio, or METs (M-E-tees) [3].

    - SCREEN: Show algorithmically estimated respiratory rate values being determined.

    - SCREEN: Show the SpO2 values in percentage.

    - SCREEN: Show the MET ratio being recorded.

  - Assess the peripheral skin temperature in degrees Celsius [1] and motion data from accelerometer, or ACC (A-C-C) in gravitational acceleration, or g (gee) units [2].

    - SCREEN: Show the peripheral skin temperature being recorded in °C

    - SCREEN: Show the motion data being updated.

  - Confirm that all metrics including cardiovascular, respiratory, metabolic, temperature, and motion data are visible [1].

    - SCREEN: Display dashboard with all listed physiological parameters updating in real time.

  - For the environmental sensor setup, connect environmental sensing devices to the portable power bank using appropriate USB (U-S-B) cables for twenty four-hour operations [1-TXT]. Ensure all connections are stable and cables are securely arranged to prevent accidental disconnection during movement [2].

    - WIDE: Talent connecting multiple sensors to a portable power bank. TXT: Powerbank: 10,000 mAh capacity

    - Close-up of neatly arranged and secured cables.

  - Place the multi-air-pollutant monitor, noise and temperature sensors in the fishnet side pocket of the participant’s backpack [1].

    - Talent placing the air pollutant, noise and temperature sensors in the fish net side pocket of the backpack.

  - Monitor air-pollution parameters like particulate matters, PM2.5 (P-M-two-point-five) and PM10 (P-M-ten) in micrograms per cubic meters [1], carbon monoxide, or CO (C-O) in parts per million, or ppm (P-P-M) [2], nitrogen dioxide [3], sulfur dioxide [4] and volatile organic compounds, or VOCs (V-O-seas) in parts per billion, or ppb (P-P-B) [5].

    - SCREEN: Show PM2.5 and PM10 values in μg/m³

    - SCREEN: Show CO values in ppm.

    - SCREEN: Show NO2 values.

    - SCREEN: Show SO2 values.

    - SCREEN: Show the value of VOCs in ppb.

  - Assess the noise pollution by measuring the noise values in decibels, or dB (D-B) [1] and temperature from the temperature sensor in degrees Celsius [2]. Next, attach the light exposure tracker to the shoulder strap facing outward [3] to measure the illuminance in lux or lx (/lʌks/) and circadian light A, or CLA (C-L-A) values as 0-1 (zero-to-one) [4].

    - SCREEN: Show the noise values in dB.

    - SCREEN: Show the temperature in °C.

    - Talent attaching light tracker on shoulder strap.

    - SCREEN: Show the illuminance value and CLA values.

  - Instruct the participant to carry the backpack during daily activities for twenty-four hours and charge the power bank every night [1]. Provide a contact number for troubleshooting and ensure all sensors are calibrated strictly according to their respective manufacturers’ specifications before being handed to the participants [2]. Confirm that all sensors are powered on and transmitting real-time data to the smartphone [3].

    - WIDE: Participant wearing backpack and preparing to leave.

    - Close-up of talent handing over contact information.

    - SCREEN: Show active data streams from environmental sensors in the application.

  - Install the M3-BREATHE (M-three-Breathe) mobile application custom-developed for Android and iOS (eye-O-S) on the participant’s smartphone [1]. Enable global positioning system, GPS (G-P-S), Bluetooth, and background data synchronization [2]. Configure the application to record GPS location data at one-minute intervals and sync sensor data every five minutes [3]. Set the data sampling rate to one sample per second for each wearable physiological monitors and portable environmental sensors [4].

    - SCREEN: Show installation of the mobile application.

    - SCREEN: Enable GPS, Bluetooth, and background sync settings.

    - SCREEN: Configure the app to record data at regular time intervals.

    - SCREEN: Adjust sampling rate and synchronization intervals in settings.

  - Confirm that all application settings are saved before starting data recording [1]. Perform a short test recording and review the collected data for completeness and accuracy [2]. Verify correct time-stamping and plausible values across all data streams [3].

    - SCREEN: Show saving configuration settings.

    - SCREEN: Display short test recording in progress.

    - SCREEN: Review recorded dataset showing multiple parameters with timestamps.

Authors: Please create screen capture videos for shots labelled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible:

https://review.jove.com/account/file-uploader?src=21319548

- Data Acquisition, Synchronization and Backup

Demonstrator: Wenbo Guo

  - Recruit participants using a stratified sampling method to ensure representativeness of the target population [1]. Provide instructional materials to the participants including manuals and videos for device usage [2]. Conduct a baseline health survey within the application to collect demographic and medical information [3].

    - WIDE: Research team interacting with participants during recruitment.

    - Close-up of participant reviewing instructional materials.

    - SCREEN: Display survey form being completed within the application.

  - Assess the demographic data parameters like gender, age, weight, height and marital status of the participants [1]. Evaluate medical history including non-communicable diseases, or NCDs (N-C-dees) [2] and respiratory health status such as smoking pack-years, chronic cough, or dyspnea [3]. Additionally, assess self-reported health data such as physical and mental well-being, alcohol consumption, diet, physical activity, and sleep quality [4].

    - SCREEN: Show the demographic parameters field with sub-fields like age, gender, height, weight and marital status in the survey form.

    - SCREEN: Show the medical history field including the NCDs sub-field in the survey form.

    - SCREEN: Show the respiratory health status field showing sub-fields like smoking pack-years, chronic cough etc.

    - SCREEN: Show the self-reported health information page with parameters like physical and mental well-being, alcohol consumption, diet, physical activity and sleep quality.

  - Instruct the participant to wear physiological sensors and carry environmental devices continuously for 24 hours [1]. During sleep, position the environmental sensors to maintain exposure assessment, with particular attention to the light sensor orientation. [2]. Remind the participant to recharge the power bank daily to maintain uninterrupted operation and data [3].

    - WIDE: Participant wearing devices during daily routine.

    - Medium shot of sensors placed in bedroom environment.

    - Close-up of participant connecting power bank to charger.

  - Monitor the data stream through four-hourly manual checks or via automated backend alerts triggered after sync interruption of thirty minutes [1]. Contact the participant if disconnection exceeds 30 minutes [2]. Encourage completion of daily activity-travel diary surveys each evening after receiving automated reminder at 8 p.m. (P-M) to promote consistent compliance [3].

    - SCREEN: Show monitoring dashboard with live data streams.

    - Medium shot of researcher making a phone call to participant.

    - SCREEN: Display diary survey prompt within the application.

  - Sync all collected real-time and activity-travel diary data to a secure cloud server configured with regular automated backups via network connection [1]. Store data temporarily on the smartphone’s internal memory [2] and clear the synced data from the device every day after successful verification by the cloud server [3]. Ensure all data are anonymized and securely stored according to institutional data security and privacy policies [4].

    - SCREEN: Show data syncing process to cloud server.

    - SCREEN: Display showing data stored in phone’s internal memory.

    - SCREEN: Data clearing from smartphone storage and syncing to the cloud server.

    - Medium shot of researcher reviewing secure data storage system.

Authors: Please create screen capture videos for shots labelled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible:

https://review.jove.com/account/file-uploader?src=21319548

# Results

- Results

  - Average PM2.5 concentrations and subjective well-being or SWB (S-W-B) scores varied significantly across daily activities, with exposure differing markedly between indoor versus outdoor and sedentary versus active settings [1]. The highest average exposure exceeding 20 micrograms per cubic meter was observed during sleep, surfing the internet, entertainment, studying and taking a walk [2].

    - LAB MEDIA: Figure 1: Video editor: Please emphasize the X-axis and all the blue bars.

    - LAB MEDIA: Figure 1: Video editor: Please emphasize the blue bars labelled “Sleep”, “Surfing the Internet”, “Entertainment”, “Study” and “Taking a Walk”.

  - Mid-range exposure concentrations in the range of fifteen to twenty micrograms per cubic meter were found for activities such as work, physical activity, caregiving and trips [1]. The lowest exposures in the range of five to fifteen micrograms per cubic meter were recorded during personal affairs, seeing a doctor, dining and personal care [2].

    - LAB MEDIA: Figure 1: Video editor: Please emphasize the blue bars labelled “Work”, “Physical Activity”, “Taking care of someone” and “Trips”.

    - LAB MEDIA: Figure 1: Video editor: Please emphasize the blue bars labelled “Personal affairs”, “Seeing a doctor”, “Dining”, “Personal care”.

  - The cognitive SWB scores showed less variability across activities, ranging narrowly between one-point seven and two-point three on a one-to five dissatisfaction scale [1]. The lowest satisfaction scores were associated with seeing a doctor, trips, personal affairs and work [2].

    - LAB MEDIA: Figure 1: Video editor: Please emphasize the Y-axis and the brown line.

    - LAB MEDIA: Figure 1: Video editor: Please emphasize the points on the brown line corresponding to “Seeing a doctor”, “Trips”, Personal affairs” and “Work”.

  - The highest satisfaction scores were linked to physical activity, entertainment and social activity [1]. The activity types associated with higher PM2.5 exposure concentrations did not always correspond to lower cognitive SWB, suggesting decoupling patterns between PM2.5 exposure and self-reported cognitive SWB [2].

    - LAB MEDIA: Figure 1: Video editor: Please emphasize the points on the brown line corresponding to “Physical activity”, “Entertainment”, and “Social activity”.

    - LAB MEDIA: Figure 1: Video editor: Please emphasize the X-axis and blue bars and Y-axis and the brown line sequentially.