Replace Jove transcript 2.1 to 2.18.2 with below

- Smart Controller and Aerosolized Hydrogen Peroxide Disinfecting Fogger Setup, Configuration, and Operation

  - Insert controller on cart bracket and install controller cover with two screws

  - Plug USB cable into controller and cart charge port

  - Install USB cable on cart handle using clips

  - Install Breezy blue on cart by plugging device into cart power cord and setting device in cart pocket

  - Fill breezy blue tank

  - Plug Breezy Buggy power cord into wall outlet in room to be disinfected

  - Power on controller (volume up  + ON/OFF buttons)

Option 1 – Site already exists at location

  - PC SCREEN Site administrator invites new user to the site from cloud.breezymed.com Site Settings & Users

  - PC SCREEN If the site administrator is unknown, email Breezy Med at info@breezymed.com for more information

  - PC SCREEN Using the Breezy Cloud invitation email link, setup account on cloud.breezymed.com

  - PC SCREEN Use the verification code sent to the email address to complete account setup

  - Controller SCREEN Use new account to login to controller

Option 2 – no site exists at location

  - Controller SCREEN If no site exists at the location, create a user account on the controller

  - Controller SCREEN Use the verification code sent to the email address to verify the account on the controller

  - Controller SCREEN Create site on the controller

  - Controller SCREEN Verify the Breezy Blue on the cart shows up in scanning list

  - Controller SCREEN Press +Add Favorite Device button, select the Breezy Blue on the cart from the list and press Submit

  - Controller SCREEN Select the Breezy Blue device shown as the favorite on the scanning screen

  - Controller SCREEN Press the Pair button

  - Disinfect the room by pressing the appropriate room size button on the fogging screen

  - After fogging and settling has completed, re-enter the room, unplug breezy buggy from the wall outlet, wrap the cord around the handle and roll device out of room

  - PC SCREEN Login into cloud.breezymed.com

  - PC SCREEN view the site fogging logs (Home -> VIEW LOGS)

  - PC SCREEN view the site devices (Devices ->Breezy Blue , Devices -> Smart Controller)