# Submission ID #: 69923

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=21326563

# Title: Dissolving Microneedle Array Patches Manufactured by Solvent Casting Technique and Essential Characterization of Microneedle-Based Biomedical Devices

# Authors and Affiliations:

# Antonio José Guillot1, Miquel Martínez-Navarrete1,2, Jose Alejandro Bernabeu-Martínez1,2, Miguel Ángel Rubio1, Ana Melero1,2

# 1Department of Pharmacy and Pharmaceutical Technology and Parasitology, University of Valencia

# 2Interuniversity Research Institute for Molecular Recognition and Technological Development (IDM), University of Valencia-Polytechnic University of Valencia

# Corresponding Authors:

# Antonio José Guillot 			antonio.guillot@uv.es

# Email Addresses for All Authors:

# Miquel Martínez-Navarrete		miquel.martinez-navarrete@uv.es

# Jose Alejandro Bernabeu-Martínez	bermajo@alumni.uv.es

# Miguel Ángel Rubio			mirulo@alumni.uv.es

# Ana Melero				ana.melero@uv.es

# Antonio José Guillot 			antonio.guillot@uv.es

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

Authors: For 3.2 section, the videographer will film the talent viewing the sample under the microscope

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Filming location: Will the filming need to take place in multiple locations?   No

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. No

Current Protocol Length

Number of Steps: 24

Number of Shots: 51

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

Authors: Please note that the answers are slightly edited and the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context.

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Antonio Guillot: Our research mainly focuses on the optimization of skin drug delivery using nano- or micro-sized systems, such as microneedle-based devices.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Antonio Guillot: This study offers a feasible and reproducible method for preparing efficient dissolving microneedles.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Miquel Navarrete: This protocol provides a foundational guideline for fabricating dissolving microneedle devices, enabling researchers to explore their design, performance, and biomedical application potential.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the most important consideration or challenge when performing this protocol.

  - Miquel Navarrete: A key challenge is ensuring cargo compatibility with the polymer matrix, as certain cargos may interfere with material properties or compromise microneedle performance.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Antonio Guillot: Future studies can adapt this protocol to specific drugs and polymers, optimizing microneedle properties and expanding applications across diverse therapeutic and biomedical uses.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Videographer: Obtain headshots for all authors available at the filming location.

Ethics Title Card

This research has been approved by the University of Valencia Human and Animal Research Ethics Committee

# Protocol

- Manufacture of Dissolving Microneedle Array Patches (DMAPs)

Demonstrators: Miquel Navarrete, Antonio Guillot

  - To begin, prepare the aqueous colloidal dispersion of the constituent materials of DMAPs by dissolving them in the appropriate amount of water [1-TXT]. Stir the mixture vigorously to obtain a uniform dispersion [2].

    - WIDE: Talent adding the constituent materials into a container with water. TXT: DMAPs: Dissolving Microneedle Array Patches

    - Talent stirring the mixture vigorously to form an aqueous colloidal dispersion.

  - Allow the polymers to stir minimally overnight to achieve complete dispersion [1] and let the air bubbles disperse during this prolonged stirring period [2].

    - Talent adjusting the stirrer to minimal speed.

    - Shot of the sample with minimal bubbles.

  - Centrifuge the polymeric dispersion if air bubbles persist by placing it in a centrifuge [1]. Set the centrifugation to 1000 g for 5 minutes at 25 degrees Celsius [2].

    - Talent placing the container with polymeric dispersion into the centrifuge.

    - Show the centrifuge control panel with the settings beings adjusted to 1000 g, 5 minutes, and 25 degree Celsius.

  - Add the cargo at the desired concentration into the polymeric dispersion [1] and stir the mixture moderately until it becomes homogeneous without incorporating new air bubbles [2].

    - Talent adding the cargo into the polymeric dispersion.

    - Talent stirring the mixture at moderate speed while monitoring for air bubble formation.

  - Now, to accommodate the molds into the wells, use modeling clay or dental cement [1] and dispose the PDMS molds into the 12-well plate [2].

    - Talent adding the supporting material inside the wells.

    - Talent placing PDMS molds into a 12-well plate.

  - Next, cast 20 microliters of the drug-loaded polymeric blend onto the PDMS molds [1]. Place the plate in the centrifuge [2] and spin at 3000 g for 5 minutes at 25 degrees Celsius [3]. Now, turn the plate 180 degrees [4] and repeat the centrifugation to ensure filling of all needle-like cavities [5].

    - Talent pipetting 20 microliters of the drug-loaded polymeric blend onto the PDMS molds.

    - Talent placing the plate into the centrifuge.

    - Show the centrifuge set to 3000 g, 5 minutes, and 25 degree Celsius.

    - Talent removing the plate and rotating it 180 degrees.

    - Talent placing the plate back into the centrifuge for the second run.

  - Then, carefully remove the excess drug-loaded polymeric dispersion from the molds [1]. Using a pipette, transfer the polymeric dispersion if it is liquid [2-TXT].

    - Talent inspecting the PDMS molds for excess polymeric dispersion.

    - Talent removing excess liquid dispersion using a pipette. TXT: Use a spatula if the dispersion is highly viscous

  - Dry the drug-loaded polymeric dispersion in the molds by placing them under vacuum at 600 millibars for 30 minutes at 25 degrees Celsius [1].

    - Talent placing the plate with molds into a vacuum chamber.

  - Prepare a highly concentrated polymeric dispersion free of drug to form the baseplate of the DMAPs device [1]. Ensure the dispersion is homogeneous before use [2].

    - Talent adding the sample to a container.

    - Talent mixing the dispersion until uniform.

  - Next, cast 50 microliters of the free-drug polymeric dispersion onto the PDMS molds [1]. Place the plate in the centrifuge and spin twice, including a midpoint plate rotation of 180 degrees [2-TXT].

    - Talent pipetting 50 microliters of free-drug polymeric dispersion onto the molds.

    - Talent placing the late into the centrifuge. TXT: 500 x g; 5 min; 25 °C

  - After centrifugation, add 100 microliters of dispersion to each mold [1], followed by an additional 50 microliters to the baseplate after 12 hours [2].

    - Talent pipetting 100 microliters of free-drug polymeric dispersion onto the molds.

    - Talent adding 50 microliters of free-drug polymeric dispersion onto the molds.

  - Leave the DMAPs in the PDMS molds to dry for 3 to 5 days at room temperature in a dry-seal desiccator [1].

    - Talent placing the plate into a dry-seal desiccator.

  - Demold the DMAPs carefully from the PDMS molds using forceps [1]. Alternatively, peel the DMAPs off the molds using scotch cellulose tape [2].

    - Talent gently removing DMAPs from the PDMS molds with forceps.

    - Talent peeling DMAPs from the molds using scotch cellulose tape.

  - For the positive pressure method, cast 150 microliters of the drug-loaded polymeric blend onto the PDMS molds [1] and place them in a pressure tank [2].

    - Talent pipetting 150 microliters of the drug-loaded polymeric blend onto the PDMS molds.

    - Talent placing the PDMS molds into the pressure tank.

  - Fill the pressure tank with air until it reaches a pressure between 3 and 4 bar. Maintain this pressure for at least 15 minutes [1].

    - Talent closing the pressure tank and initiating air filling.

- Characterization of DMAPs

  - Visualize the DMAPs using a micro-camera with appropriate magnification to distinguish the needle-like projections [1]. Adjust the focus to clearly resolve individual microneedles [2].

    - Talent viewing through the micro-camera.

    - Talent adjusting the focus.

  - Next, attach the DMAPs to a support device to visualize the morphology [1] and length of the microneedle-like projections by optical microscopy [2].

    - Talent mounting the DMAPs onto a support device.

    - Talent viewing through the optical microscope.

  - Evaluate the mechanical properties of the DMAPs by placing the device into a commercially available DMAPs applicator [1]. Compress the applicator against a stainless-steel flat surface for 30 seconds [2].

    - Talent loading the DMAPs device into the applicator.

    - Talent pressing the applicator against a stainless-steel flat surface.

  - After compression, record the length of the microneedle-like projections [1] and calculate the deformation percentage by comparing the length before and after compression [2].

    - Talent noting down the length in a book.

    - TEXT ON PLAIN BACKGROUND:

Compression %=ho-hf ho·100

  - Then, determine the preliminary insertion capacity of the DMAPs using a surrogate artificial skin method based on thermoplastic sheets made of olefin-type material [1]. Compress the DMAPs device against eight bound sheets using the same compression conditions applied to the stainless-steel surface [2].

    - Talent aligning the DMAPs device with the stacked thermoplastic sheets.

    - Talent compressing the applicator against the bound sheets.

  - Now, determine the ex vivo insertion capacity of the DMAPs using human, porcine, or rodent skin explants that have been previously excised and carefully trimmed [1]. Clean the skin explant by soaking it in abundant water for 1 to 2 minutes [2].

    - Talent handling trimmed skin explants on a work surface.

    - Talent soaking the skin explant in water for cleaning.

  - Place the cleaned skin explant onto an aluminum foil-wrapped foam material with the stratum corneum side facing upwards [1]. Proceed with insertion using the same compression conditions previously described [2].

    - Talent positioning the skin explant on aluminum foil-wrapped foam.

    - Talent compressing the DMAPs device into the skin explant.

  - Leave the DMAPs inserted into the skin until dissolution of the microneedle tips occurs [1] and observe the device remaining in contact with the skin during dissolution [2].

    - SCOPE: Close-up view of DMAPs inserted into the skin explant.

    - Show a device after the microneedle dissolution.

- Franz-Diffusion Cell (FDC) Setup

  - Determine the ex vivo drug skin deposition and absorption using a Franz diffusion cell setup [1]. Prepare the skin explant using the previously described trimming and cleaning procedure [2], and insert the DMAPs into the skin structure for 30 seconds [3].

    - Talent arranging the Franz diffusion cell components and skin explant on the bench.

    - Talent positioning the prepared skin explant for insertion.

    - Talent inserting the DMAPs into the skin explant for 30 seconds.

  - Glue the donor chamber of the Franz diffusion cell to the skin using an appropriate amount of cyanoacrylate [1]. Allow the adhesive to dry completely [2], then attach the skin–donor chamber complex to the receptor chamber using appropriate clamps [3]. Finally, proceed with sampling according to the standard diffusion cell protocol [4].

    - Talent applying cyanoacrylate to bond the donor chamber to the skin.

    - Shot showing the assembled donor chamber and skin resting while the adhesive dries.

    - Talent securing the donor–skin assembly to the receptor chamber with clamps.

    - Talent initiating the sampling procedure from the assembled Franz diffusion cell.

# Results

- Results

# Authors: Please check if narration in the results are correct and if they match with the corresponding figure and the requested highlights

  - Manufactured DMAPs displayed a homogeneous appearance with successful formation of microneedle-like projections, and the colored dye was localized within the projections, with the baseplate remaining free of drug [1].

    - LAB MEDIA: Figure 2A.

  - Optical microscopy revealed that microneedle-like projections had sharp tip ends and no deformation before compression [1]. After compression, microneedle-like projections showed visible deformation of structure [2].

    - LAB MEDIA: Figure 2B. Video editor: Zoom in on the needle tips.

    - LAB MEDIA: Figure 2C. Video editor: Highlight the bent and collapsed structure of the projections.

  - In an artificial skin model, DMAPs successfully penetrated the thermoplastic sheet, creating visible holes [1]. The number of holes created in different layers of the thermoplastic model correlated with microneedle insertion depth [2].

    - LAB MEDIA: Figure 2D. Video editor: Highlight the grid of puncture holes in the right panel showing successful microneedle insertion.

    - LAB MEDIA: Figure 2E. Video editor: Highlight the decreasing red line on the graph showing fewer holes at greater insertion depths.

  - Ex vivo murine skin explants showed progressive dyeing of the inner skin layers after insertion, consistent with DMAP dissolution over time [1]. Histological examination with eosin-hematoxylin staining confirmed insertion of microneedle-like projections into skin sections [2].

    - LAB MEDIA: Figure 2F. Video editor: Highlight the image columns sequentially.

    - LAB MEDIA: Figure 2G. Video editor: Point to the black arrows marking the insertion points in the pink-stained skin tissue section.

  - A Franz-diffusion cell setup was used to evaluate the diffusion of drug through skin explants, with clear visualization [1] of the donor chamber, skin, and receptor chamber [2].

    - LAB MEDIA: Figure 2I

    - LAB MEDIA: Figure 2I. Video editor: Highlight labels 1 (donor chamber), 2 (skin explant), and 3 (receptor chamber) in the image.