TITLE:

Cryosectioning of Synthetic Biocompatible Implants Using UV-Curable Resin for Enhanced Immunofluorescence Analysis

AUTHORS AND AFFILIATIONS:

Amanda M. Richards*1, Andrea Cheslea*1, Cheryl E. Myers1, Jonah D. Reichenbach1, David G. Lott1,2

1Head and Neck Regenerative Medicine Laboratory, Mayo Clinic, Arizona.

2Division of Laryngology, Department of Otolaryngology Head and Neck Surgery, Mayo Clinic, Arizona.

*Contributed equally.

Email addresses of co-authors:

Amanda M Richards		(richards.amanda@mayo.edu)

Andrea Cheslea		(cheslea.andrea@mayo.edu)

Cheryl E Myers		(myers.cheryl@mayo.edu)

Jonah D Reichenbach		(jdreichenbach06@gmail.com)

Corresponding author:

David G. Lott 			(lott.david@mayo.edu)

SUMMARY:

This work presents a cryosectioning protocol that uses UV-curable resin stabilization to maintain the integrity of the implant-tissue interface while preserving antigen accessibility, enabling reliable, high-resolution immunofluorescence analysis of synthetic polymer implants.

ABSTRACT:

Histological evaluation of synthetic, biocompatible implants poses significant technical challenges due to the incompatibility between conventional tissue processing methods and the physicochemical properties of these materials. Current approaches utilizing complete resin embedding provide adequate structural preservation but severely compromise epitope accessibility, thereby limiting immunohistochemical applications essential for characterizing host immune responses. Conversely, conventional paraffin embedding exhibits poor adhesion to synthetic substrates and limits antibody selection for comprehensive immunological profiling. Standard cryosectioning protocols enhance immunohistochemical compatibility by improving antigen preservation; however, these methods often result in inadequate structural integrity of implant-containing specimens, compromising morphological assessment and sectioning quality. To address these limitations, a novel processing protocol was developed that incorporates a UV-curable resin for selective stabilization of implant-tissue constructs prior to cryosectioning. This methodology employs a photopolymerizable resin to provide mechanical support while preserving epitope accessibility and antigenic determinants, which are typically compromised by full resin polymerization. The protocol enables reproducible generation of high-quality sections at optimal thickness for immunofluorescence evaluation, achieving superior structural preservation compared to standard frozen sectioning without the extensive crosslinking that limits antibody penetration in conventional resin-embedded specimens. This approach represents a practical, cost-effective solution for histological processing of synthetic implants, offering enhanced analytical capabilities for biocompatibility assessment and facilitating detailed characterization of local immune responses at implant-tissue interfaces.

INTRODUCTION:

Immunofluorescence microscopy is an essential analytical technique in regenerative medicine research, facilitating visualization of tissue architecture and cellular composition using fluorophore-conjugated antibodies. This methodology integrates the molecular specificity of immunochemical recognition with the sensitivity and versatility of fluorescence detection, enabling detailed observation of cellular target distributions within their tissue microenvironments. Immunofluorescence approaches encompass both direct and indirect labeling strategies, with indirect immunofluorescence protocols providing enhanced signal amplification through secondary antibody systems conjugated to diverse fluorophores spanning the visible and near-infrared spectrum1. The methodology's inherent ability for simultaneous multi-target detection through spectral separation facilitates comprehensive phenotypic characterization of heterogeneous cellular populations and their spatial relationships within complex tissue microenvironments2.

The technical capabilities of immunofluorescence microscopy extend beyond conventional protein localization to encompass quantitative analytical applications, including fluorescence intensity measurements, colocalization analyses, and spatial distribution assessments critical for understanding cellular function and tissue organization3. In the context of implant biocompatibility research, immunofluorescence analysis provides critical insights into host immune responses, including foreign body reactions, inflammatory cascades, and tissue integration processes that determine implant acceptance or rejection at the cellular and molecular levels4.

The successful implementation of immunofluorescence protocols requires careful consideration of tissue processing methodologies that preserve morphological integrity while maintaining fluorescent signal and minimizing background autofluorescence. Traditional embedding media selection depends on tissue type, downstream analytical requirements, and the specific antigens of interest. For synthetic materials and implants, current literature primarily recommends complete resin embedding to maintain structural stability during sectioning procedures5. However, this approach presents significant limitations due to the requirement for specialized ultramicrotomes and blades for sectioning polymerized resin blocks, representing substantial equipment investment and technical expertise. Paraffin embedding, while providing adequate structural support for sectioning, introduces conformational changes to antigenic determinants during fixation and processing through extensive dehydration, chemical hardening, and infiltration with organic solvents that can denature proteins and mask epitopes. Paraffin processing also produces intrinsic autofluorescence interference that compromises signal detection and quantitative analysis. These alterations necessitate antigen retrieval procedures, typically involving heat-induced epitope retrieval (HIER) or proteolytic-induced epitope retrieval (PIER), to restore antibody accessibility6. Despite these interventions, numerous commercially available antibodies remain incompatible with paraffin-embedded specimens, and the harsh retrieval conditions can further compromise fluorescent signal quality and introduce photobleaching artifacts.

Cryosectioning offers an alternative processing methodology for immunofluorescence applications. Two cryopreservation methods can be used for tissue preparation. The first approach involves rapid cryopreservation by direct immersion of fresh tissue specimens in an isopentane bath pre-cooled with liquid nitrogen to achieve temperatures below –20 °C, followed by sectioning using cryostat microtomes equipped with temperature-controlled cutting chambers. The second protocol uses a short fixation period in paraformaldehyde (PFA) or formalin, followed by cryoprotection via graded sucrose infiltration before freezing. The cryopreservation process creates a rigid matrix that maintains tissue integrity while preserving native protein conformations essential for antibody recognition. Frozen sectioning maintains tissues in a near-native state with minimal chemical modifications7, thereby reducing the need for harsh antigen retrieval procedures and enabling the use of a broader range of primary antibodies, including those incompatible with paraffin-embedded tissues due to epitope masking or denaturation8.

The cryosectioning process typically utilizes optimal cutting temperature (OCT) compound, a water-soluble embedding medium composed of polyvinyl alcohol and polyethylene glycol that provides structural support during sectioning while remaining transparent and compatible with fluorescence detection systems. OCT embedding facilitates the generation of thin, uniform sections ranging from 5–15 μm in thickness, depending on the specific analytical requirements and tissue characteristics9. The preservation of native protein conformations in frozen sections maintains optimal signal-to-noise ratios essential for quantitative immunofluorescence analysis, while also retaining intact epitopes that allow the use of a broader range of primary antibodies, including those incompatible with paraffin-embedded tissues8.

However, frozen tissue sectioning presents unique technical challenges, particularly when processing synthetic implant materials that possess distinct physicochemical properties compared to native biological tissues. The differential thermal expansion between synthetic polymers and biological tissue interfaces during freezing can result in localized mechanical stress that compromises structural integrity and creates sectioning artifacts10. Furthermore, the inherent rigidity of many synthetic materials relative to frozen biological tissues can cause blade deflection during sectioning, resulting in uneven section thickness and damage to both the specimen and cutting equipment. These technical limitations necessitate specialized processing protocols that can accommodate the unique requirements of implant-containing specimens while maintaining the advantages of frozen sectioning for immunofluorescence applications.

To address these limitations, this study developed a cryosectioning protocol that combines selective UV-curable resin stabilization with controlled cryostat sectioning parameters to enable high-resolution, multi-parameter immunofluorescence analysis of implant-tissue constructs.

PROTOCOL:

This research was performed in compliance with Mayo Clinic Institutional Animal Care and Use Committee (IACUC) guidelines under approval number A00003765-18-24. All procedures involving synthetic biocompatible implants were conducted under sterile conditions, in accordance with appropriate biosafety protocols.

- Specimen processing and cryoprotection

NOTE: Handle formalin exclusively in a chemical fume hood and wear appropriate personal protective equipment (PPE), because formalin contains formaldehyde, a potent irritant, sensitizer, and carcinogen.

  - Fix tissue-integrated biocompatible synthetic implant specimens in 10% neutral buffered formalin overnight at 4° C using a minimum of 10 volumes of fixative relative to tissue volume to ensure complete immersion and adequate penetration. The objective of this limited fixation period is to achieve minimal cross-linking while preserving tissue morphology. (See Table of Materials for materials used in Protocol.)

  - Transfer specimens to 15% sucrose solution prepared in phosphate-buffered saline (PBS) for overnight incubation at 4° C with gentle agitation. Ensure complete sucrose infiltration as evidenced by tissue sinking.

  - Transfer specimens to 30% sucrose solution in PBS for a second overnight incubation at 4° C. Confirm complete equilibration by tissue sinking to the container bottom, indicating adequate sucrose infiltration and water displacement.

  - Prepare a 1:1 volumetric ratio of 30% sucrose solution and OCT compound. Incubate specimens in this transitional medium for 2 h at 4° C, then transfer to 100% OCT for 1 h to achieve complete equilibration, as indicated by a uniformly opaque appearance of the tissue.

  - Position specimens carefully in standard cryomolds filled with fresh OCT compound, ensuring optimal orientation for sectioning planes. Orient with the implant-tissue interface accessible for subsequent procedures. Eliminate air bubbles using pre-cooled forceps.

  - Place filled cryomolds immediately on the dry ice platform to create a temperature gradient promoting rapid ice crystal formation from bottom upward. Monitor for 10–15 min until complete solidification, confirmed by the characteristic opaque white appearance of frozen OCT compound. Transfer frozen blocks to –80° C storage or directly to the cryostat chamber for immediate sectioning.

- Cryostat setup and initial sectioning

  - Maintain the cryostat temperature-controlled chamber at –15° C. Verify temperature using integrated digital monitoring and equilibrate overnight prior to specimen introduction.

  - Install new disposable blades. Set the blade angle for combined synthetic-biological material sectioning according to the manufacturer's specifications.

  - The desired section thickness (50–100 μm) is programmed into the cryostat by selecting the Thickness setting on the control panel, adjusting using the arrow keys, and confirming the parameter by pressing the Set key.

  - Advance through the embedded specimen to reach the predetermined depth corresponding to the implant-tissue interface. Monitor advancing section surface to identify optimal plane for UV-resin application, characterized by subtle changes in tissue opacity and initial implant-periphery exposure, and depth is confirmed when these features remain consistently visible across successive trimmed sections.

NOTE: Halt sectioning immediately upon reaching the critical interface to prevent advancement beyond the target region.

- UV-Curable resin application

NOTE: Handle UV-curable resin in a well-ventilated area while wearing appropriate PPE and protect eyes and skin from LED-UV curing light. These resins contain irritants that can cause skin and eye reactions, and LED-UV exposure can result in UV-induced eye injury.

  - Cut parafilm using scissors to dimensions 2–3 mm exceeding the implant perimeter. Carefully separate from the protective paper backing using sterile forceps.

  - Dispense 5–10 μL UV-curable resin onto exposed parafilm surface (Figure 1A). Resin volume is optimized by conducting preliminary test applications in which incremental volumes are dispensed, and the resulting spread is visually evaluated to ensure uniform coverage of the specimen surface without overflow or excessive resin accumulation that would interfere with sectioning.

  - Replace protective paper backing immediately over resin-loaded parafilm. Apply a polyurethane seam roller (2 inches) with consistent pressure in overlapping, parallel passes, followed by perpendicular passes to achieve a thin, uniform resin layer (Figure 1B).

  - Remove protective paper layer carefully, revealing uniformly distributed resin film. Position resin-parafilm directly over the exposed implant surface, ensuring precise alignment with the implant-tissue interface (Figure 1C).

  - Apply polyurethane seam roller from implant center outward to eliminate air bubbles and ensure intimate contact between resin and both synthetic and biological surfaces (Figure 1D).

  - Position 405 nm wavelength UV LED light source at 2–3 cm distance directly at the implant surface. Expose for 90 s to ensure complete polymerization, verified by tactile assessment of resin hardness and non-tacky characteristics (Figure 1E). Place UV-protective cover over cryostat chamber window prior to UV photopolymerization to shield operator from harmful UV exposure during resin curing procedure.

- Section recovery and slide preparation

  - Remove parafilm using a single continuous motion to prevent mechanical disruption of the resin-tissue interface. Complete resin adherence is confirmed by inspecting the exposed surface under cryostat lighting for uniform resin coverage, absence of lifting at the tissue edge, and lack of visible parafilm residue.

  - Section specimens to a predetermined 8 μm thickness using a calibrated microtome advance mechanism. Monitor each section for consistent thickness and absence of artifacts (compression, tearing, incomplete sectioning). Position the anti-roll plate optimally to prevent curling (Figure 1F).

NOTE: Hydrophilic, positively charged adhesive glass slides are used because they provide superior retention of challenging specimens, improving adherence during section transfer and subsequent processing. Label slides individually prior to section transfer.

  - Position slide adhesive surface, ensuring initial contact at section edge before progressive contact across entire area. Ensure the UV-cured resin surface adheres to the slide while maintaining the tissue surface upward for immunofluorescence (Figure 1G).

  - Press the section-slide assembly gently to eliminate air bubbles. Verify proper orientation under cryostat lighting, confirming the intact implant-tissue interface.

- Storage and preservation

  - Place completed slides in pre-labeled storage boxes at –20° C for applications requiring processing within 72 h.

  - Utilize – 80° C conditions for preservation periods exceeding one week.

- Immunofluorescence staining protocol

NOTE: Handle pepsin with appropriate PPE, as it is a skin and respiratory irritant.

  - Retrieve slides from storage and transfer to humidity chambers for 15–20 min for room temperature equilibration.

  - Perform three sequential 2-minute PBS washing cycles.

  - Antigen retrieval:

    - Heat-induced epitope retrieval (HIER) method: Position slides horizontally in microwave-safe containers with 1x target retrieval solution. Microwave (1000-watt output) at high power, for 15 min, followed by cooling to room temperature.

    - Proteolytic-induced epitope retrieval (PIER) method: Apply 100–150 μL ready-to-use pepsin solution per section. Cover with temporary plastic coverslips and incubate at 37° C for 10 min in a humidified chamber.

  - Perform three sequential PBS washing cycles (2 min each).

  - Apply 1 mL blocking solution containing 0.1% saponin and 2.5% normal goat serum in PBS. Incubate 30 min at room temperature in a humidified chamber.

  - Perform three sequential PBS washing cycles (2 min each).

  - Apply 100 μL primary antibody solutions at optimized concentrations:

    - Anti-Collagen IV rabbit polyclonal at 1:100 dilution.

    - Anti-Laminin rabbit polyclonal at 1:500 dilution.

    - Anti-F4/80 rabbit monoclonal antibody at 1:500 dilution.

  - Cover with temporary plastic coverslips and incubate overnight at 4° C in humidified chambers.

  - Perform three PBS washing cycles (2 min each).

  - Apply secondary antibodies at a 10 μg/mL concentration.

    - Donkey Anti-Rabbit IgG Alexa Fluor Plus 555

    - Goat Anti-Rabbit IgG Alexa Fluor 488

  - Incubate 60 min at room temperature in darkened conditions.

  - Optional wheat germ agglutinin (WGA) staining:

    - Apply WGA-Alexa Fluor 647 at 5 μg/mL in PBS with 2.5% normal goat serum.

    - Cover and incubate at 37° C for 30 min. Perform three sequential 2 min PBS washing cycles.

  - Apply 80 μL hard-setting mounting media with nuclear counterstain per section.

  - Cover the coverslip with the cover glass to eliminate air bubbles.

  - Allow slides to cure horizontally at room temperature for 2 h in darkness.

  - Store at 4° C for up to one week for optimal imaging quality.

- Fluorescence microscopy and image acquisition

  - Perform imaging using an inverted fluorescence microscope with manufacturer filter sets: 360/460 nm excitation/emission, 470/525 nm excitation/emission, 545/605 nm excitation/emission, 620/700 nm excitation/emission.

  - Images are captured by selecting each fluorescence channel within the Filter menu of the microscope software. Prior to exposure optimization, a black balance procedure is performed by selecting Adjust Black Balance to ensure accurate baseline correction for each channel. Exposure parameters are then optimized by opening the Image Capture window, enabling Live mode, and adjusting the Exposure and Light Intensity controls until a high signal-to-noise ratio is achieved without pixel saturation. Saturation is monitored using the software’s Histogram display and the Highlight Saturated Pixels tool. Each channel is adjusted independently prior to image acquisition.

  - Perform automated tile acquisition with the manufacturer’s 10% overlap between adjacent fields. Maintain automated focus adjustment throughout the acquisition sequence.

REPRESENTATIVE RESULTS:

Section quality assessment and optimization

Following UV-resin stabilization and sectioning at 8 μm thickness, collect sections onto adhesive slides and evaluate for quality control. High-quality sections demonstrate intact morphology without significant cracks or fracturing artifacts, with continuous tissue architecture and a preserved implant-tissue interface (Figure 2A). Sections exhibiting substantial structural damage, including fracturing or tissue separation from the resin-stabilized implant surface, are excluded from further analysis (Figure 2B). Notably, UV-curable resin application did not compromise the OCT block, as serial sections of uniform thickness were reliably obtained throughout the full depth of the block, indicating that resin penetration and UV curing did not adversely affect cut quality. Retain sections meeting quality criteria for subsequent immunofluorescence staining protocols.

Immunofluorescence staining performance across specimen types

Immunofluorescence staining was successfully performed across diverse specimen types, including mouse subcutaneous tissue containing implants and isolated implant materials only. The abbreviated formalin fixation protocol (overnight at 4° C) necessitated antigen retrieval procedures to restore epitope accessibility for optimal antibody binding. Standard immunofluorescence protocols were effectively applied after antigen retrieval, including blocking with 0.1% saponin and 2.5% normal goat serum, temporary coverslip application to ensure uniform reagent distribution, and sequential primary and secondary antibody incubations. The implant material exhibited autofluorescence; however, its distinctive uniform morphology and geometric structure enabled clear differentiation from specific immunofluorescence signals in both isolated implants (Figure 3A) and tissue-integrated constructs (Figures 3B–C). Nuclear counterstaining contributed measurably to implant autofluorescence, with maximal fluorescence intensity in the DAPI filter channel.

Antigen-specific staining results

Collagen IV immunofluorescence using rabbit polyclonal anti-collagen IV antibody with Alexa Fluor 488-conjugated secondary antibody identified extracellular matrix (ECM) collagen IV distribution throughout tissue sections. High-magnification imaging at 40X revealed fibrillar patterns consistent with native collagen IV architecture, demonstrating that the UV-resin stabilization (Figures 4A–C) protocol maintained structural preservation comparable to conventional formalin fixed paraffin embedded (FFPE) processing methods (Figures 4D–F).

Macrophage populations within and around implant materials were visualized using an anti-F4/80 antibody with Alexa Fluor 555-conjugated secondary antibody. Positive F4/80 staining revealed immune cell infiltration patterns at implant-tissue interfaces, providing a critical assessment of local immune responses to synthetic biomaterials (Figures 5A–C). The distribution and density of F4/80-positive macrophages indicated active immune engagement with implant surfaces, validating the protocol's effectiveness for biocompatibility studies as compared to FFPE samples (Figures 5D–F).

Laminin immunofluorescence using a rabbit polyclonal anti-laminin antibody with an Alexa Fluor 555-conjugated secondary antibody demonstrated clear detection of basement membrane components within mouse subcutaneous implants. Laminin signal showed the expected linear

distribution along implant–tissue interfaces, consistent with preserved extracellular matrix organization and comparable to FFPE controls (Figures 6A–F). Complementary multi-parameter imaging combining wheat germ agglutinin (Alexa Fluor 647), laminin (Alexa Fluor 488), and nuclear counterstaining further highlighted plasma membrane localization and extracellular matrix architecture (Figures 7A–F). Following heat-induced epitope retrieval, WGA staining appropriately outlined membrane structures surrounding implant materials, while laminin and nuclear signals provided additional spatial definition. Together, these assays demonstrated intact matrix organization and robust multiplex antigen detection across distinct cellular and extracellular compartments, underscoring the protocol’s versatility and compatibility with diverse fluorescent probes. To ensure consistency across all imaging experiments, all immunofluorescent staining was processed in batches, and microscope exposure parameters were maintained constant throughout acquisition. However, exposure times varied slightly depending on the specific fluorescent probe used to optimize signal detection. A black balance procedure was also applied during image acquisition to minimize background noise and improve contrast.

Negative control validation

Negative control specimens processed without primary antibody exhibited no detectable fluorescence in the TRITC, GFP, and Cy5 channels, with only nuclear counterstaining visible (Figures 3B–C). This confirmed that the secondary antibody did not generate a non-specific signal and that the fluorescence observed in experimental specimens reflects primary antibody-dependent binding. Specimens that displayed fluorescence patterns comparable to the negative controls, defined as counterstain-only images without a corresponding signal in the target fluorescence channel despite primary antibody application, were classified as unsuccessful stains and prompted protocol troubleshooting or antibody optimization.

Comparative analysis with paraffin embedding

Paraffin-embedded murine tissue sections containing implants were processed using identical immunofluorescence protocols for collagen IV, laminin, and F4/80 detection (Figures 4D–F, 5D–F, 6D–F). High-magnification imaging at 40X revealed morphological patterns and staining intensities comparable between UV-resin stabilized cryosections and paraffin-embedded specimens in tissue regions distant from synthetic materials. However, critical differences emerged at implant-tissue interfaces. Paraffin-embedded specimens demonstrated compromised tissue retention with visible gaps and tissue discontinuities that impeded comprehensive interface analysis. In contrast, UV-resin stabilized cryosections maintained continuous tissue architecture throughout implant regions, enabling successful immunofluorescence detection across entire specimens, including challenging implant-tissue boundary zones. Multi-parameter immunofluorescence using collagen IV, laminin, F4/80 antibodies, and WGA with counterstaining demonstrated preserved antigen accessibility and facilitated detailed characterization of basement membrane structures, extracellular matrix organization, and immune cell infiltration patterns directly at synthetic material surfaces.

Quantitative results confirm stable and reproducible sectioning performance.

Across all three biocompatible polymers evaluated (5–6 blocks per material and 15–20 sections per block; total n = 287 sections), 94% of sections were successfully produced at an 8 µm thickness under optimized UV-curing conditions. This resulted in 17 failed sections, with 9 failures in polymer 1, 8 in polymer 2, and none in polymer 3. Reducing the section thickness to 6–7 µm decreased reliability, with approximately 15% of sections becoming compromised. In contrast, increasing the thickness to 9–10 µm restored performance, yielding the same success rate as 8 µm sections. Optimal UV-curing parameters that generated a firm, non-tacky resin surface resulted in consistently usable sections. Insufficient curing, resulting in a soft, tacky surface layer, led to smearing, material drag, and an inability to cut sections, rendering the samples unusable. Conversely, excessive curing or exposure beyond 2 min caused all samples to fracture due to excessive brittleness, resulting in complete section failure.

FIGURES AND TABLE LEGENDS:

Figure 1: UV-curable resin application and cryosectioning workflow. Sequential steps for UV-resin stabilization of synthetic implant specimens. (A) Resin dispensing onto parafilm following removal of protective backing. (B) Uniform resin distribution using a seam roller with protective backing replaced. (C) Alignment of a thin layer of resin on parafilm with the exposed implant surface. (D) Application of controlled pressure for complete parafilm-resin adhesion. (E) UV photopolymerization at 405 nm wavelength. (F) Post-curing sectioning with the anti-roll plate positioned and parafilm removed. (G) Section transfer showing resin-stabilized surface adhered to adhesive glass slide.

Figure 2: Section quality assessment in UV-resin stabilized cryosections. Representative examples of sectioning outcomes for implants in mouse subcutaneous tissue. (A) High-quality section demonstrates intact morphology without fracturing artifacts. (B) The section exhibiting fracturing was excluded from subsequent analysis. Yellow arrow, epidermis; Blue arrow, silicone sheeting; Black arrow, implant.

Figure 3: Negative control validation across processing methods. Autofluorescence and non-specific binding assessment in specimens processed without primary antibody. (A) Isolated implant shows intrinsic material autofluorescence on the DAPI channel. (B) Paraffin-embedded implant with surrounding murine subcutaneous tissue demonstrating minimal background fluorescence. (C) UV-resin stabilized cryosection of implant with surrounding tissue showing comparable background levels to paraffin processing. All images: 4X magnification, tiled composite, DAPI filter. Scale bars: A–C = 500 μm. Astrix, Biocompatible material # 2; Arrow, tissue-implant interface; Dagger, silicone sheet. Abbreviations; DAPI = 4′,6-diamidino-2-phenylindole.

Figure 4: Collagen IV immunofluorescence in UV-resin stabilized cryosection versus paraffin-embedded specimens. Extracellular matrix collagen IV distribution in mouse subcutaneous tissue containing implants. (A–C) UV-resin stabilized cryosections: (A) overlay, (B) DAPI channel, (C) GFP channel showing collagen IV (Alexa Fluor 488). (D–F) Paraffin-embedded sections: (D) overlay, (E) DAPI channel, (F) GFP channel. All images are acquired at 40X magnification. Scale bar = 20 μm. Abbreviations; DAPI = 4′,6-diamidino-2-phenylindole; GFP = green fluorescent protein.

Figure 5: F4/80 immunofluorescence reveals macrophage distribution at implant-tissue interfaces. Macrophage detection using anti-F4/80 antibody in murine subcutaneous tissue containing implants. (A–C) UV-resin stabilized cryosections: (A) overlay, (B) DAPI channel, (C) TRITC channel showing F4/80-positive macrophages (Alexa Fluor 555). (D–F) Paraffin-embedded sections: (D) overlay, (E) DAPI channel, (F) TRITC channel. All images acquired at 40× magnification. Scale bar = 20 μm. Abbreviations; DAPI = 4′,6-diamidino-2-phenylindole; TRITC = tetramethyl rhodamine isothiocyanate.

Figure 6: Laminin immunofluorescence in UV-resin cryosectioned versus paraffin-embedded specimens. Comparative laminin distribution in mouse subcutaneous tissue containing implants. (A–C) UV-resin stabilized cryosections: (A) overlay, (B) DAPI channel, (C) TRITC channel showing laminin (Alexa Fluor 555). (D–F) Paraffin-embedded sections: (D) overlay, (E) DAPI channel, (F) TRITC channel. All images are acquired at 40X magnification. Scale bar = 20 μm. Abbreviations; DAPI = 4′,6-diamidino-2-phenylindole; TRITC = tetramethyl rhodamine isothiocyanate.

Figure 7: Multi-parameter immunofluorescence visualization of implant-tissue interface. Representative UV-resin stabilized cryosection of mouse subcutaneous tissue containing an implant. (A) Low magnification overlay composite image (4X) showing complete specimen with laminin (Alexa Fluor 555), WGA (Alexa Fluor 647), and Hoechst 33342 counterstaining. High-magnification regional images (40X): (B) DAPI blue channel (nuclear counterstain), (C) TRITC red channel (laminin), (D) Cy5 green channel (WGA), (E) merged overlay includes blue (nuclear counterstain), red (laminin), green (WGA). The overlapping of red, green, and blue colors generates secondary colors such as magenta, yellow, and white. Scale bars: A = 200 μm, B–E = 20 μm. Abbreviations; DAPI = 4′,6-diamidino-2-phenylindole; TRITC = tetramethyl rhodamine isothiocyanate; Cy5 = cyanine; WGA = wheat germ agglutinin.

Table 1: Troubleshooting table for resin-stabilized cryosectioning. Summary of common issues observed during UV-curable resin stabilization and cryosection of implant-tissue constructs, including potential causes and recommended solutions.

DISCUSSION:

This UV-curable resin stabilization method provides an alternative approach for cryosectioning OCT-embedded tissue specimens containing synthetic materials that are otherwise difficult to process using standard frozen sectioning techniques. Although a variety of adhesive options, such as charged slides, tissue-bonding tapes, and other stick-on supports, are available to help prevent tissue sections or fragile samples from detaching during handling, these products offer slide adhesion and do not supply the mechanical reinforcement needed at the implant–tissue interface during cutting. Moreover, there are currently no spreadable, UV-curing resins designed for thin, uniform application directly onto biological specimens to improve structural stability during cryosectioning. In this context, the UV-curable resin protocol presented here fills a methodological gap by providing targeted mechanical support while preserving epitope accessibility for antibodies that require frozen processing, enabling reliable sectioning of specimens containing synthetic polymeric components11.

Several procedural elements require careful attention for successful implementation. Resin selection must prioritize minimal autofluorescence and appropriate viscosity to maintain structural support during sectioning while avoiding brittleness or incomplete curing. The high-viscosity formulation selected for this protocol demonstrated minimal background interference across multiple fluorescence channels. Sectioning thickness requires material-specific optimization. Testing across 6–10 μm thickness ranges revealed that 8 μm provided adequate structural preservation. Standard biological tissues without synthetic components were sectioned successfully at 6 μm. Initial thickness optimization for each new material type is recommended. UV curing parameters critically impact section quality. A 90-second exposure at an irradiance of 42.44–95.5 mW/cm2 (405 nm), delivered from 2–3 cm, achieved complete polymerization of 5–10 µL resin volumes. Under these specified curing parameters, the climate-controlled cryostat chamber temperature remained stable and did not change during the resin curing time. Over-curing produces brittleness, causing fracturing during sectioning, while under-curing leaves tacky resin that compromises cutting quality12,13. Tactile assessment should confirm a hard, non-tacky resin surface before proceeding to sectioning.

Sharp blade condition is critical for optimal sectioning; monitor blade performance continuously and replace when cutting resistance increases or section quality deteriorates. Resin-stabilized specimens can accommodate antigen retrieval modalities through horizontal slide positioning rather than standard upright orientation, which additionally prevents specimen detachment from the adhesive slide during processing. For HIER protocols, position slides horizontally in microwave-safe containers to prevent heat-induced tissue detachment from slides. PIER protocols similarly benefit from horizontal positioning, facilitating uniform enzyme distribution while maintaining structural stability.

[Place Table 1 here]

The parafilm application system serves dual purposes: creating a uniform thin resin layer through controlled spreading and providing a removable barrier for post-curing handling. Alternative spreading methods (direct application, brush application) resulted in inconsistent resin thickness, compromising both sectioning quality and slide adherence during immunofluorescence staining, and causing difficulties with coverslip application. WGA staining required HIER for successful membrane visualization, while the laminin antibody performed adequately with either HIER or PIER. Antibody-specific optimization of retrieval methods is recommended for new targets. Multi-parameter immunofluorescence panels combining up to three fluorescent probes (illustrated with laminin, WGA, and DAPI) were successfully performed, indicating compatibility with complex staining protocols required for comprehensive biocompatibility characterization.

Equipment requirements include a standard cryostat with temperature control, a UV LED light source (405 nm), and a seam roller. Resin materials incompatibility has not been fully evaluated across all synthetic polymer types; therefore, researchers working with new biomaterials should conduct preliminary optimization to assess resin behavior and optimize sectioning parameters. As part of the evaluation, additional biocompatible materials were examined, and under comparable microscope settings, biocompatible material # 3 surgical implant exhibited no detectable autofluorescence, whereas 3D-printed biocompatible material # 1 demonstrated pronounced autofluorescence (images not shown).

Under standard OCT-embedded cryosectioning conditions, all specimens containing biocompatible material # 1 and # 2 consistently exhibited mechanical failure, with the biomaterial fracturing into multiple pieces during sectioning. In contrast, biocompatible material # 3 maintained complete morphological stability throughout processing, demonstrating no evidence of fragmentation or compromised morphology. All tested materials were successfully sectioned following paraffin embedding, and both biocompatible materials # 1 and # 2 showed no evidence of structural fracturing under paraffin-based processing conditions. Biocompatible material # 3 likewise retained stable morphology, exhibiting no detectable changes. These observations highlight material-specific differences in mechanical stability during cryosectioning and underscore the need for stabilization strategies when processing synthetic polymers that demonstrate reduced structural integrity under cryogenic conditions.

Processing time requires approximately 3–4 days from fixation through mounted slides (overnight fixation, two overnight sucrose incubations, equilibration periods, sectioning, and staining). This extended timeline compared to paraffin processing (24–48 h) represents a practical limitation for time-sensitive applications. Reproducibility depends on careful attention to UV curing parameters, consistent resin application technique, and appropriate blade condition.

Potential applications extend beyond synthetic implant biocompatibility studies to include investigation of bone-implant interfaces, dental material integration, and other tissue-biomaterial systems where conventional histological processing proves inadequate. The protocol's compatibility with standard immunofluorescence reagents suggests broad utility across diverse research applications requiring preserved material-tissue interface analysis. Systematic evaluation of protocol compatibility with additional synthetic polymers, ceramics, and metallic biomaterials would expand applicability guidance for researchers working with diverse material systems.

This UV-curable resin stabilization approach provides researchers with a practical method for comprehensive immunofluorescence analysis of challenging specimen types where conventional processing methods compromise either structural integrity or antigen accessibility, facilitating detailed biocompatibility assessment and tissue engineering applications in preclinical research settings.

DISCLOSURES:

The authors declare that they have no competing financial interests.

ACKNOWLEDGEMENTS:

Research reported in this publication was supported by the National Institute on Deafness and Other Communications of the National Institutes of Health under award number R01DC019114.

REFERENCES:

1.	Moreno, V., Smith, E. A., Pina-Oviedo, S. Fluorescent immunohistochemistry. Methods Mol Biol. 2422, 131–146 (2022).

2.	Stack, E. C., Wang, C., Roman, K. A., Hoyt, C. C. Multiplexed immunohistochemistry, imaging, and quantitation: a review, with an assessment of Tyramide signal amplification, multispectral imaging, and multiplex analysis. Methods. 70 (1), 46–58 (2014).

3.	Wilson, C. M. et al. Challenges and opportunities in the statistical analysis of multiplex immunofluorescence data. Cancers (Basel). 13 (12), doi:10.3390/cancers13123031 (2021).

4.	Dievernich, A., Achenbach, P., Davies, L., Klinge, U. Characterization of innate and adaptive immune cells involved in the foreign body reaction to polypropylene meshes in the human abdomen. Hernia. 26 (1), 309–323 (2022).

5.	Rousselle, S. D., Wicks, J. R., Tabb, B. C., Tellez, A., O'Brien, M. Histology strategies for medical implants and interventional device studies. Toxicol Pathol. 47 (3), 235–249 (2019).

6.	Im, K., Mareninov, S., Diaz, M. F. P., Yong, W. H. An introduction to performing immunofluorescence staining. Methods Mol Biol. 1897, 299–311 (2019).

7.	Szunyogova, E. P., S. Histological and Histochemical Methods, Theory and Practice, 5th Ed. 5th edn,  (2015).

8.	Osborn, M., Brandfass, S. Immunocytochemistry of Frozen and of Paraffin Tissue Sections. 1 (3), 563–569 (2006).

9.	Fischer, A. H., Jacobson, K. A., Rose, J., Zeller, R. Cryosectioning tissues. CSH Protoc. 2008, 4991, doi:10.1101/pdb.prot4991, (2008).

10.	Steif, P. S., Noday, D. A., Rabin, Y. Can thermal expansion differences between cryopreserved tissue and cryoprotective agents alone cause cracking? Cryo Letters. 30 (6), 414–421 (2009).

11.	Serowoky, M. A., Patel, D. D., Hsieh, J. W., Mariani, F. V. The use of commercially available adhesive tapes to preserve cartilage and bone tissue integrity during cryosectioning. Biotechniques. 65 (4), 191–196 (2018).

12.	Bouchareb, S., Doufnoune, R. Effect of UV post-curing on the mechanical properties of photopolymer resin in stereo-lithographic 3D printing. Materials Letters. 394, doi:ARTN 138587 10.1016/j.matlet.2025.138587 (2025).

13.	Stowe, R. W. Key factors in the UV curing process - The relationship of exposure conditions and measurement in UV process design and process control: Part I - Introduction. Metal Finishing. 100 (4), 62–71 (2002).