# Submission ID #: 70075

# Scriptwriter Name: Pallavi Sharma

# Project Page Link: https://review.jove.com/files_upload.php?src=21372923

Title: Metagenomic Next-Generation Sequencing of Cerebrospinal Fluid for the Detection of Central Nervous System Pathogens

# Authors and Affiliations:

Sumanth P. Reddy, Jarod Morgenroth-Rebin, Shiyin Wang, Michael R. Wilson

University of California, San Francisco Weill Institute for Neurosciences, Department of Neurology, University of California, San Francisco

# Corresponding Authors:

Sumanth P. Reddy			(Sumanth.Reddy@ucsf.edu)

# Email Addresses for All Authors:

Jarod Morgenroth-Rebin		(jarod.morgenrothrebin@ucsf.edu)

Shiyin Wang				(shiyin.wang2@ucsf.edu)

Michael R. Wilson			(michael.wilson@ucsf.edu)

Sumanth P. Reddy			(Sumanth.Reddy@ucsf.edu)

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  yes

3. Filming location: Will the filming need to take place in multiple locations?   no

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. Yes

Current Protocol Length

Number of Steps:  27

Number of Shots:  53

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Sumanth Reddy: This is a research-based protocol that allows for metagenomic next-generation sequencing of cerebrospinal fluid samples for the detection of pathogens.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Sumanth Reddy: Existing mNGS methods struggle with low RNA yield in CSF, affecting extraction and enzyme efficiency; this protocol optimizes workflows for low-biomass samples.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B roll: 3.4.1

CONCLUSION:

Describe the most important consideration or challenge when performing this protocol.

  - Jarod Morgenroth-Rebin: A key challenge is precise pipetting due to low volumes, along with time-sensitive bead drying, where under- or overdrying can compromise results.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B roll: 3.7.2

Are there any additional methods or analyses that can be performed following this procedure?

  - Sumanth Reddy: In addition to pathogen detection, host RNA transcripts obtained from this protocol can be further analyzed for various downstream applications.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Sumanth Reddy: Future studies can compare emerging library preparation protocols for sparse samples to evaluate their performance in both pathogen detection and bulk transcriptomic analyses.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

# `Testimonial Questions (OPTIONAL):

# Videographer: Please capture all testimonial shots in a wide-angle format with sufficient headspace, as the final videos will be rendered in a 1:1 aspect ratio. Testimonial statements will be presented live by the authors, sharing their spontaneous perspectives.

What is the key takeaway you hope viewers remember after watching your video article?

  - Sumanth Reddy, MD Neuroimmunology Fellow, University of California San Francisco, : (authors will present their testimonial statements live)

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

How does the video format of your article help you to share your work with the broader scientific community?

  - Sumanth Reddy, MD Neuroimmunology Fellow, University of California San Francisco, : (authors will present their testimonial statements live)

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

# Ethics Title Card

This research has been approved by the Institutional Review Board (IRB) at the University of California, San Francisco

# Protocol

Videographer: Please always record an audio slate while filming each shot mentioned in the shotlist.

- Extraction of Cerebrospinal Fluid (CSF) Nucleic Acid

Demonstrator: Jarod Morgenroth-Rebin

  - To begin, thaw the CSF (C-S-F) samples on ice [1]. Transfer 100 microliters to 1 milliliter of each sample to a 1.5-milliliter tube [2] and centrifuge at 16000 g for 10 minutes at 4 degrees Celsius [3].

    - WIDE: Talent thawing the cerebrospinal fluid samples on ice.

    - Talent transferring each sample to a 1.5 milliliter tube.

    - Talent placing the tubes in the centrifuge.

  - Then, carefully pipette off the supernatant, leaving behind the pellet and 100 microliters of supernatant [1].

    - Talent pipetting off the supernatant from the tube.

  - Add 100 microliters of DNA and RNA Shield to the remaining 100 microliters of supernatant and pellet and mix thoroughly [1].

    - Talent adding DNA/RNA Shield to the tube and mixing it. VIDEO NOTE: Use Take 2

  - Then, add 600 microliters of DNA and RNA lysis buffer and mix by pipetting until the solution is homogeneous [1].

    - Talent adding deoxyribonucleic acid/ribonucleic acid lysis buffer to the tube and mixing it.

  - Perform the DNA and RNA extraction protocol as per the manufacturer’s instructions [1]. Perform all centrifugation steps for 1 minute at maximum speed, except the final DNA or RNA wash, which should be centrifuged for 3 minutes [2].

    - Talent adding a reagent to the tube with the labelled vials of all the reagents in the frame.

    - Talent places the tube in the centrifuge and setting the parameters.

  - To elute the sample, add 22 microliters of nuclease-free water directly onto the column matrix, then incubate for 3 minutes at room temperature [1].

    - Talent adding nuclease-free water directly onto the column matrix and leaving it to incubate.

  - After elution, pipette the flow-through back onto the column matrix [1]. Centrifuge again for 1 minute at maximum speed before transferring the sample to a new 1.5-milliliter storage tube [2]. NOTE: The VO is edited for the deleted step

    - Talent pipetting the flow-through back onto the column matrix.

    - Talent placing the column in the centrifuge again.

    - Talent transferring the sample to a new 1.5 milliliter storage tube.    NOTE: Not filmed

- CSF RNA Library Preparation

  - Pipette the components required for RNA fragmentation and priming in a PCR tube [1]. Incubate the tube in a thermocycler with the heated lid set to 105 degrees Celsius [2].

    - TEXT ON A PLAIN BACKGROUND

RNA sample: 3.5 µL

ERCC spike-in (1:2500): 0.5 µL

First-strand reaction buffer (5x): 4 µL

Random primers: 1 µL

rRNA (1:100): 1 µL

    - Talent placing the polymerase chain reaction tube into the thermocycler.

and

TEXT ON A PLAIN BACKGROUND

75 °C for 2 min

70 °C for 2 min

65 °C for 2 min

60 °C for 2 min

55 °C for 2 min

37 °C for 5 min

25 °C for 5 min Video editor: Please use split screen to show both the shots simultaneously

  - Pipette mix the fragmented and primed RNA with 8 microliters of nuclease-free water and 2 microliters of first-strand synthesis enzyme mix [1]. Incubate the tube in a thermocycler with the heated lid set to 105 degrees Celsius [2-TXT].

    - Talent pipetting and mixing the fragmented and primed ribonucleic acid with reagents.

    - Talent placing the tube into the thermocycler. TXT: 25 °C for 10 min; 42 °C for 15 min; 70 °C for 15 min

  - Then mix the first-strand synthesized DNA with 48 microliters of nuclease-free water, 8 microliters of second-strand synthesis reaction buffer, and 4 microliters of second-strand synthesis enzyme mix [1]. Place the tube in a thermocycler and incubate for 1 hour at 16 degrees Celsius with the heated lid turned off [2].

    - Talent pipetting and mixing the first-strand synthesized deoxyribonucleic acid with reagents.

    - Talent placing the tube into the thermocycler.

  - Next, perform magnetic bead purification using a 1.8 times ratio of solid-phase reversible immobilization magnetic beads [1]. After incubation, elute the sample into 53 microliters of nuclease-free water [2]. Transfer 50 microliters of the final supernatant containing purified double-stranded cDNA to a clean nuclease-free PCR tube [3].

    - Talent adding solid-phase reversible immobilization magnetic beads to the sample.

    - Talent adding nuclease-free water to elute the sample.

    - Talent transferring the supernatant to a clean polymerase chain reaction tube.

  - If the sample was stored at minus 20 degrees Celsius overnight, thaw on ice before restarting [1]. Pipette mix 50 microliters of purified double-stranded cDNA with 7 microliters of end-preparation reaction buffer and 3 microliters of end-preparation enzyme mix [2].

    - Talent thawing the sample on ice.

    - Talent pipetting and mixing the purified double-stranded complementary deoxyribonucleic acid with reagents.

  - Place the samples in a thermocycler and incubate at 20 degrees Celsius for 30 minutes and 65 degrees Celsius for 30 minutes with the heated lid turned off [1].

    - Talent placing the tubes into the thermocycler.

  - Next, perform the adapter ligation step on ice by first creating a 1 to 100 dilution of adapter in nuclease-free water [1]. Pipette the end-preparation reaction mixture into a tube containing 30 microliters of ligation master mix, 1 microliter of ligation enhancer, and 2.5 microliters of the diluted adapter [2-TXT].

    - Talent preparing a diluted adapter solution on ice.

    - Talent pipetting the reaction mixture, ligation master mix, ligation enhancer, and adapter sequentially into a tube.

  - Place the samples in a thermocycler and incubate at 20 degrees Celsius for 15 minutes with the heated lid turned off [1].

    - Talent placing the tubes into the thermocycler.

  - After that, proceed immediately to magnetic bead purification using a 0.9 times ratio of solid-phase reversible immobilization magnetic beads [1].

    - Talent adding solid-phase reversible immobilization magnetic beads to the sample.

  - Once the sample is incubated, elute into 17 microliters of nuclease-free water [1]. Transfer 15 microliters of the final supernatant to a clean nuclease-free PCR tube [2].

    - Talent adding nuclease-free water to elute the sample.

    - Talent transferring the supernatant to a clean polymerase chain reaction tube.

  - Now, pipette purified adapter-ligated cDNA into a tube with 3 microliters of USER (User) enzyme, 25 microliters of Q5 Master Mix, and 10 microliters of unique barcoded primers [1]. Place the tube in a thermocycler with the heated lid set to 105 degrees Celsius [2] and incubate at the presented conditions [3].

    - Talent pipetting and mixing the complementary deoxyribonucleic acid with USER enzyme, Q5 Master Mix, and barcoded primers.

    - Talent placing the tube in the thermocycler.

    - TEXT ON A PLAIN BACKGROUND

37 °C for 15 min

98 °C for 30 s

98 °C for 10 s and 65 °C for 75 s for 19 cycles

65 °C for 5 min.

  - Perform a final magnetic bead purification using a 0.8 times ratio of magnetic beads  [1]. Then, elute the sample into 23 microliters of nuclease-free water [2]. Transfer 20 microliters of the final supernatant to a clean nuclease-free PCR tube [3].

    - Talent adding magnetic beads to the sample.

    - Talent adding nuclease-free water to elute the sample.

    - Talent transferring the supernatant to a clean polymerase chain reaction tube.

  - Quantify the concentration of the sample libraries using a DNA quantification kit and the corresponding fluorometer [1]. Verify that water controls have substantially lower or unquantifiable concentrations compared to the other samples [2].

    - Talent places the sample in the flurometer.

    - Display fluorometer readings showing low or undetectable signal for water controls compared to sample libraries.

NOTE: 3.13.1. and 3.13.2 are combined in a single shot

  - Assess the sample library sizes by running the samples on an automated capillary electrophoresis machine [1-TXT].

    - Talent loading the samples into the automated capillary electrophoresis machine. TXT: Perform CSF DNA library preparation following a similar workflow

- Data Analysis for Pathogen Detection

Demonstrator: Sumanth Reddy

  - Use the open-source web-based platform Chan Zuckerberg ID to perform metagenomic analysis for pathogen detection [1].

    - SCREEN: 70075_screenshot_1.mp4: 00:00-00:02

  - Log into Chan Zuckerberg ID and click Upload [1]. Select Metagenomics as the analysis type and choose the input FASTQ (Fast-Q) files from the sequencing run [2]. Enter the required sample information, including sample name, sample type CSF, and nucleotide type RNA or DNA [3].

    - SCREEN: 70075_screenshot_1.mp4:  00:03-00:09, 00:17-00:19

    - SCREEN: 70075_screenshot_1.mp4:   00:49-01:02

    - SCREEN: 70075_screenshot_2.mp4:  00:00-00:20

  - Once processing is complete, click the project folder containing the samples and navigate to the summary dashboard [1].

    - SCREEN: 70075_screenshot_3.mp4

  - Scroll down to examine the number of reads per sample [1]. Review the percentage of reads passing quality control filtering [2] and the duplicate compression ratio to assess for biased PCR overamplification [3].

    - SCREEN: 70075_screenshot_4.mp4: 00:03-00:05 Video editor: Highlight the left hand side graph

    - SCREEN: 70075_screenshot_4.mp4: 00:03-00:05 Video editor: Highlight the right hand side graph

    - SCREEN: 70075_screenshot_4.mp4: 00:09-00:11 Video editor: Highlight the left-hand side graph

  - To determine any Foreign DNA contamination, within the project folder, select each water control sample and click Background model to generate the model [1]. During analysis, apply the model and use Threshold filters to set an NTZ (N-T-Z) score greater than 1 to remove likely contaminants. Then, analyse the data using appropriate statistical software [2].

    - SCREEN: 70075_screenshot_5.mp4

    - SCREEN: 70075_screenshot_6.mp4

  - Utilize predefined reads per million thresholds by comparing sample reads per million values to control averages [1]. Download taxon-specific reads per million data by selecting samples, clicking Download, and choosing Combined Sample Taxon Results [2]. Analyze the resulting comma-separated values file using statistical software such as R or Stata [3].    NOTE: Not filmed

    - SCREEN: Selecting samples and clicking Download followed by Combined Sample Taxon Results.

    - SCREEN: Displaying the downloaded comma-separated values file.

    - SCREEN: Opening and analyzing the file in statistical software.

# Results

- Results

  - The capillary electrophoresis and electropherogram data demonstrated a cDNA sample library that was over-fragmented and contained adapter dimers not suitable for sequencing [1].

    - LAB MEDIA: Figure 2A and B.  Video editor: Highlight the small peak region at low size values indicating adapter dimers

  - The cDNA library without significant adapter dimer contamination was ready for pooling and sequencing [1].

    - LAB MEDIA: Figure 2C and D. Video editor: Show the smooth main peak representing the cDNA library

  - CSF mNGS detected diverse pathogens in Cohort A and B, with at least one pathogen in 73.5 percent of Cohort A samples [1] and 51.2 percent of Cohort B samples [2]

    - LAB MEDIA: Figure 3

    - LAB MEDIA: Figure 3

- Cerebrospinal
Pronunciation link: https://www.merriam-webster.com/dictionary/cerebrospinal
IPA: /səˌriː.broʊˈspaɪ.nəl/
Phonetic Spelling: suh·ree·broh·spy·nuhl

- Centrifuge
Pronunciation link: https://www.merriam-webster.com/dictionary/centrifuge
IPA: /ˈsen.trəˌfjuːdʒ/
Phonetic Spelling: sen·truh·fyooj

- Supernatant
Pronunciation link: https://www.merriam-webster.com/dictionary/supernatant
IPA: /ˌsuː.pɚˈneɪ.tənt/
Phonetic Spelling: soo·per·nay·tuhnt

- Pellet
Pronunciation link: https://www.merriam-webster.com/dictionary/pellet
IPA: /ˈpel.ɪt/
Phonetic Spelling: pel·it

- Deoxyribonucleic
Pronunciation link: https://www.merriam-webster.com/dictionary/deoxyribonucleic%20acid
IPA: /diˌɑːk.siˌraɪ.boʊ.nuːˈkliː.ɪk/
Phonetic Spelling: dee·ok·see·ry·boh·noo·klee·ik

- Ribonucleic
Pronunciation link: https://www.merriam-webster.com/dictionary/ribonucleic%20acid
IPA: /ˌraɪ.boʊ.nuːˈkliː.ɪk/
Phonetic Spelling: ry·boh·noo·klee·ik

- Homogeneous
Pronunciation link: https://www.merriam-webster.com/dictionary/homogeneous
IPA: /ˌhoʊ.məˈdʒiː.ni.əs/
Phonetic Spelling: hoh·muh·jee·nee·uhs

- Nuclease
Pronunciation link: https://www.merriam-webster.com/dictionary/nuclease
IPA: /ˈnuː.kli.eɪs/
Phonetic Spelling: noo·klee·ays

- Elute
Pronunciation link: https://www.merriam-webster.com/dictionary/elute
IPA: /iˈluːt/
Phonetic Spelling: ee·loot

- Thermocycler
Pronunciation link: No confirmed link found
IPA: /ˈθɝː.moʊˌsaɪ.klɚ/
Phonetic Spelling: ther·moh·sy·kler

- Fragmentation
Pronunciation link: https://www.merriam-webster.com/dictionary/fragmentation
IPA: /ˌfræɡ.mənˈteɪ.ʃən/
Phonetic Spelling: frag·muhn·tay·shuhn

- Priming
Pronunciation link: https://www.merriam-webster.com/dictionary/priming
IPA: /ˈpraɪ.mɪŋ/
Phonetic Spelling: pry·ming

- Polymerase
Pronunciation link: https://www.merriam-webster.com/dictionary/polymerase
IPA: /pəˈlɪm.əˌreɪs/
Phonetic Spelling: puh·lim·uh·rays

- Electrophoresis
Pronunciation link: https://www.merriam-webster.com/dictionary/electrophoresis
IPA: /iˌlek.troʊ.fəˈriː.sɪs/
Phonetic Spelling: ih·lek·troh·fuh·ree·suhs

- Electropherogram
Pronunciation link: No confirmed link found
IPA: /iˌlek.troʊˈfer.əˌɡræm/
Phonetic Spelling: ih·lek·troh·fer·uh·gram

- Metagenomic
Pronunciation link: https://www.merriam-webster.com/dictionary/metagenomics
IPA: /ˌmet.ə.dʒəˈnoʊ.mɪk/
Phonetic Spelling: met·uh·juh·noh·mik

- FASTQ
Pronunciation link: No confirmed link found
IPA: /ˈfæst.kjuː/
Phonetic Spelling: fast·kyoo

- Pathogen
Pronunciation link: https://www.merriam-webster.com/dictionary/pathogen
IPA: /ˈpæθ.ə.dʒən/
Phonetic Spelling: path·uh·juhn