TITLE:

Wire Myography for Dorsal Aorta and Mesenteric Resistance Arteries from Mice and Rats

AUTHORS AND AFFILIATIONS:

Joanna Glaser*, Arturo Grano de Oro*, Sanjana Kumariya*, Islam Osman

*Equally contributed to this work

Department of Physiology and Pharmacology, Center for Hypertension and Personalized Medicine, University of Toledo, College of Medicine and Life Sciences, Toledo, Ohio, USA

Email addresses of the co-authors:

Joanna Glaser					Joanna.stuck@utoledo.edu

Arturo Grano de Oro				Arturo.GranoDeOro@rockets.utoledo.edu

Sanjana Kumariya				sanjana.kumariya@rockets.utoledo.edu

Corresponding author:

Islam Osman					islam.osman@utoledo.edu

SUMMARY:

We provide a detailed protocol for side-by-side wire myography to assess vascular reactivity in conduit versus resistance arteries isolated from mice and rats.

ABSTRACT:

Wire myography is a widely used ex vivo technique for the quantitative assessment of vascular function in isolated arteries. Because large conduit and small resistance arteries differ fundamentally in structure and physiological role, analyzing both vessel classes in parallel using arteries harvested from the same mouse or rat enables paired comparisons that reduce inter-animal variability and reveal vessel-specific functional changes. This article presents a step-by-step protocol for assessing vascular reactivity in isolated murine conduit and resistance arteries using isometric wire myography. We describe the preparation of fresh, oxygenated physiological buffer; careful dissection, cleaning, and mounting of arterial segments; and normalization to set vessels near their optimal length for active force generation. A troubleshooting guide and an example experimental sequence for generating cumulative concentration-response curves to standard vasoconstrictors and vasodilators are provided. This protocol allows quantitative measurement of both endothelial and smooth muscle function and facilitates detection of vessel-specific alterations in vascular reactivity associated with vascular disease, such as hypertension.

INTRODUCTION:

Hypertension is characterized by altered vascular reactivity, such as endothelial dysfunction and vascular smooth muscle hyperreactivity to vasoconstrictors1. These functional changes, together with microvascular remodeling and rarefaction, contribute to impaired tissue perfusion and dysregulated blood pressure2-4. Ex vivo assays of isolated vessels are powerful tools for dissecting the cellular and molecular mechanisms that control vascular tone in health and disease, as they allow precise control of the vascular microenvironment and selective pharmacological manipulation of smooth muscle and endothelial signaling pathways.

Vascular reactivity in isolated arteries is commonly assessed using two complementary ex vivo techniques: wire myography and pressure myography5,6. In wire myography, vessel rings or segments are mounted on pins or wires, and isometric force is recorded7,8. In pressure myography, intact vessel segments are cannulated and pressurized to a defined intraluminal pressure, and vessel diameter is tracked by video microscopy9. Unlike wire myography, pressure myography permits control of transmural pressure and intraluminal flow. Thus, it more closely recapitulates the in vivo hemodynamic environment of resistance arteries and is commonly used to study myogenic tone, flow-mediated responses, and vascular structural or mechanical properties9,10. Conversely, wire myography is technically easier to implement (no cannulation or video microscopy), supports higher throughput using commercial multi-chamber systems, and is well-suited for generating reproducible pharmacological concentration-response curves in small-to-medium arteries with diameters on the order of tens to several hundred micrometers5,6. Accordingly, wire myography is often preferred for side-by-side pharmacological profiling of vascular reactivity in conduit and resistance arteries isolated from mice or rats.

Conduit (large elastic) and resistance (small muscular) arteries differ fundamentally in structure and function, and these differences dictate distinct experimental considerations and endpoints. Conduit arteries (e.g., descending thoracic aorta) contain large amounts of elastic tissue and substantial smooth muscle mass; resistance arteries (e.g., mesenteric arteries) have relatively greater smooth-muscle-to-lumen ratios and are major determinants of peripheral resistance and local autoregulation11,12. Because the same agonist can activate different molecular responses in these vascular beds, side-by-side comparison of conduit and resistance vessels from the same animal provides useful complementary information. For instance, in conduit arteries, endothelium-derived nitric oxide release is the predominant relaxant pathway by which acetylcholine (ACh) induces vasorelaxation13,14. In contrast, in small resistance arteries, ACh induces relaxation via nitric oxide, prostacyclin, and, importantly, endothelium-derived hyperpolarizing factors (EDHF)15,16. Accordingly, side-by-side wire myography of conduit and resistance arteries harvested from the same mouse or rat enables paired comparisons that facilitate detection of vessel-specific functional changes while reducing inter-animal variability.

Accurate interpretation of wire myography data depends on careful attention to a set of technical variables: composition and oxygenation of the physiological salt solution (PSS), tissue handling and dissection, mounting technique, optimal resting pretension (normalization), bath temperature, and selection of appropriate pharmacological controls6. Normalization refers to the technical procedure of setting the mechanical operating point of the vessel so that active force development is optimized and approximates physiological operating conditions5. Practically, for aortic rings, it is common practice to use a fixed pretension, often determined empirically to avoid overdistension while giving stable responses. For small resistance arteries, normalization is performed by first determining the internal circumference corresponding to an equivalent transmural pressure of 100 mmHg (IC₁₀₀). The vessel is then set to a defined fraction of this value to establish the normalized internal circumference (IC₁). In practice, many protocols set IC₁ near 0.9 × IC₁₀₀ for rat MRA and around 1.0 × IC₁₀₀ for mouse MRA, although these values may be adjusted based on vessel type, species, and experimental goals5,17.

PROTOCOL:

The following protocol adheres to the guidelines set and has been approved by The University of Toledo Health Science Campus Animal Care and Use Committee.

CAUTION: Before you begin, please follow Institutional Animal Care and Use Committee (IACUC) and Institutional Biosafety Committee (IBC) guidelines at all times; use appropriate personal protective equipment (PPE); exercise care when using surgical or other sharp instruments; consult and comply with relevant Safety Data Sheets (SDS) when handling chemicals and biological tissues; handle carbogen as a pressurized gas cylinder using approved regulators and secure storage; and dispose of biological and chemical waste according to institutional procedures.

- Preparation of PSS/modified Krebs bicarbonate (Krebs) buffer

NOTE: Krebs buffer composition (final concentrations, pH 7.4): 130 mM NaCl, 4.7 mM KCl, 1.17 mM MgSO4, 1.18 mM KH2PO4, 1.6 mM CaCl2, 25 mM NaHCO3, 5.5 mM glucose, and 0.03 mM EDTA.

  - Prepare large-volume stock: dissolve all constituents except NaHCO3, glucose, and CaCl₂ in double-distilled water to make a stock. Store this stock at 4 °C.

  - On the day of the experiment, aliquot the required volume of stock (e.g., 1 L), sequentially add NaHCO3, glucose, and CaCl₂, and continuously oxygenate with carbogen (95% O₂/5% CO₂).

NOTE: To minimize calcium salt precipitation, prepare and store a 1 M CaCl₂ stock at 4 °C and add the required volume slowly to the buffer only after NaHCO₃ is fully dissolved, while stirring continuously. Maintain carbogen bubbling during CaCl₂ addition (see step 1.3) to reduce calcium carbonate formation. If visible precipitation occurs, discard and prepare fresh buffer.

  - For pH equilibration, bubble the buffer with carbogen for at least 20 min, then measure pH and adjust to pH 7.4 if necessary.

NOTE: Typical prebubbling pH ≈ 7.7; continuous bubbling reduces pH to physiological ≈7.4.

  - Prepare myograph chambers by filling them with freshly prepared Krebs buffer and begin gentle bubbling with carbogen. Adjust the bubbling rate to ensure adequate gas exchange without vigorous agitation.

  - Reserve an aliquot of freshly prepared, oxygenated Krebs buffer and chill it on ice for tissue dissection. Warm the remaining oxygenated buffer to 37 °C (with continuous carbogen bubbling) for use during experiments.

- Dissection of the descending thoracic aorta and mesenteric arcade

  - Euthanize the animal according to IACUC guidelines. Standardize the timing of animal euthanasia and tissue harvest within a 2 h window to minimize potential circadian effects.

  - Immediately open the thoracic and abdominal cavities via a midline incision and reflect the chest wall laterally to expose the heart and great vessels.

NOTE: Do not perform systemic perfusion under standard conditions, as this may inadvertently cause endothelial damage. Furthermore, retained blood in the mesenteric vessels enhances the visual discrimination between arteries and veins and facilitates assessment of arterial integrity. However, in some pathological models (e.g., thrombotic disorders), gentle, low-pressure perfusion with heparinized, ice-cold physiological buffer may be advisable to clear obstructing thrombi while minimizing endothelial injury.

  - Excise the descending thoracic aorta. To facilitate dissection, remove the lungs and esophagus to improve visualization. Do not directly grasp the descending thoracic aorta; Instead, gently hold the aortic arch or periadventitial fat, lift slightly, and carefully sever the intercostal branch vessels from below the aorta. Excise the descending thoracic aorta by cutting just before it passes through the diaphragm and place it in a Sylgard-coated Petri dish containing ice-cold oxygenated Krebs buffer.

NOTE: Maintain cold temperature to minimize spontaneous tone and preserve viability. Black Sylgard increases contrast for vessel dissection.

  - Excise the whole mesenteric arcade. Lift the mesenteric arcade gently to locate the superior mesenteric artery branching from the abdominal aorta caudal to the diaphragm. Cut the superior mesenteric artery proximally and free the mesenteric arcade from the surrounding connective tissue without direct vessel manipulation. Place the mesenteric arcade in a separate Sylgard-coated Petri dish containing ice-cold oxygenated Krebs buffer.

- Isolation and mounting of aortic rings on pin myographs

  - Pin the proximal and distal ends of the descending thoracic aorta with dissection pins to straighten the vessel. Avoid overstretching to prevent mechanical damage.

NOTE: Place pins in the periadventitial fat rather than through the vessel wall to minimize damage.

  - Under a stereomicroscope with an external light source, carefully remove periadventitial fat and connective tissue using fine Vannas scissors and forceps. Avoid damage to the vessel wall.

NOTE: It is common practice to remove the periadventitial fat unless the goal is to study the role of perivascular adventitial tissues in the regulation of vascular tone.

  - Cut 2 mm aortic rings, making clean, perpendicular cuts. Discard rings that are punctured or visibly damaged. Whenever possible, sample comparable locations across animals to minimize variability.

  - Disconnect one chamber containing oxygenated Krebs buffer from the myography machine and carefully transport it to the stereomicroscope.

NOTE: Jostling the chamber can affect the transducer and may alter results, so hold the chamber carefully and place it down gently.

  - Under the stereomicroscope, use the micrometer to bring the myograph pins together and slide the aortic ring onto the pins, minimizing manipulation of the luminal surface.

  - Note the micrometer reading with the ring fully relaxed; apply a slight stretch to secure the tissue for transport.

  - Connect the chamber to the instrument and restore the vessel to the original micrometer reading.

  - After all rings are mounted, heat the chambers to 37 °C and monitor temperature with the supplied probe (typically 20–30 min to reach 37 °C).

  - Zero the force readings on the user interface, launch the data acquisition software, and begin recording.

  - Incrementally apply pretension to aortic rings to a vessel-appropriate fixed value using the micrometer.

NOTE: A typical pretension value for mouse aortic rings is 5 mN5. While reported pretension values for rat aortic rings vary across studies, prior optimization supports a pretension of 10 mN in this protocol18.

  - Equilibrate aortic rings for 45–60 min at the selected pretension, with repeated buffer changes using 37 °C Krebs.

NOTE: It is common for aortic rings to partly lose tension and thus require fine readjustment of pretension several times using the micrometer before the tension stabilizes.

  - Zero the force readings on the user interface before beginning experimental protocols.

- Isolation and mounting of MRA on wire myographs

  - Pin the mesenteric arcade in a clockwise pattern to straighten the mesenteric branches. Begin by securing the cecum, then pin successive stretches of small intestine clockwise to gently tension the mesenteric vasculature without overstretching. Replace the buffer with fresh Krebs as needed to remove blood or debris.

  - Identify the superior mesenteric artery at the center of the dish where arcades radiate.

  - Select an appropriate MRA for isolation; typical targets: internal diameter 200–250 μm (determined later during normalization).

NOTE: In adult experimental animals, this generally corresponds to 2nd-order branches in mice and 3rd-4th-order branches in rats. Branch order is defined by sequential bifurcations from the superior mesenteric artery. 1st-order branches arise directly from the superior mesenteric artery, 2nd-order branches arise from 1st-order branches, and so forth.

  - Under a stereomicroscope, dissect away the accompanying vein and remove periadventitial fat and connective tissue using Vannas scissors and fine forceps.

NOTE: MRAs can be distinguished from veins by their thicker walls and smaller lumens, which are readily visualized by the presence of residual intraluminal blood.

  - Relax the nearby gut-pin(s) that are stretching the gut wall adjacent to the selected vessel before excising the artery to prevent vessel recoil.

  - Excise the whole length of the selected MRA (should be >2 mm long), making clean, perpendicular cuts. Using fine forceps, grasp one end of the artery and transfer it to a clean Petri dish containing ice-cold Krebs buffer (regular, non-Sylgard-coated dishes facilitate subsequent mounting). Inspect the artery for damage; the presence of intraluminal blood facilitates visual assessment of integrity.

  - Excise an approximately 2 mm segment from an undamaged region. Discard any areas that were handled during transport or show visible damage.

NOTE: Using transparent, non-Sylgard-coated dishes allows for the placement of a ruler underneath the dish to accurately measure the vessel length.

  - Prepare two 2.5 cm lengths of 40 μm stainless steel wire for mounting.

  - Using fine forceps, carefully thread the wires, one at a time, through the lumen so there is sufficient wire projecting from both ends for mounting. Minimize endothelial trauma. After inserting both wires, record the exact length with a ruler for use in the normalization procedure (see step 5.2).

NOTE: This is one of the most technically challenging and time-consuming steps of the protocol and requires patience and repeated practice to minimize endothelial damage. After inserting the first wire, it is often easier to clamp both wires together and advance them through the vessel.

  - Disconnect one chamber containing oxygenated Krebs buffer from the myography machine and carefully transport it to the stereomicroscope.

  - Under the stereomicroscope, transfer the MRA (holding wires at both ends) to the myograph chamber. Using the micrometer, bring the myograph jaws together to clamp the wire ends between the jaws so the artery is suspended freely between them.

NOTE: At this stage, it is advisable to clamp the wires such that they lie on top of one another. This secures the wires in place and prevents them from twisting.

  - Secure the top wire under the mounting screws at the near micrometer side by wrapping the ends in a clockwise fashion; repeat for the second wire on the transducer side.

NOTE: Tighten the screws gently but firmly. If the screws are too loose, the vessel may slip; if over-tightened, the wire or transducer may be damaged. After fastening the first end, slightly open the gap between the myograph jaws before securing the opposite end to ensure even wire tension.

  - Slightly move the jaws apart and align the wires to lie parallel and coplanar, nearly touching.

NOTE: Perform all wire adjustments away from the mounted vessel to minimize damage.

  - Repeat mounting for all MRAs.

  - Heat chambers to 37 °C and monitor temperature (20–30 min typical).

  - Zero the force readings on the user interface before initiating the normalization procedure.

- Normalization procedure for the MRA

NOTE: Aortic rings are mounted at a fixed, predefined pretension. Therefore, a separate normalization step is not performed.

  - Set normalization parameters in the data acquisition software: eyepiece calibration = 1; target pressure = 13.3 kPa (≈100 mmHg); normalized internal circumference (IC₁) = 0.9 for rat MRAs and 1.0 for mouse MRAs; averaging time = 2 s; delay = 60 s.

  - Enter the vessel-specific parameters into the normalization module as follows:

    - For tissue endpoints (mm): set a1 = 0 and a2 = the measured length of the MRA in millimeters (the exact length recorded during excision).

    - Enter wire diameter: 40 μm.

    - Micrometer reading: enter the baseline micrometer value recorded with the vessel mounted and fully relaxed.

  - Start the normalization procedure by stepwise stretching the mounted MRA in 50 μm increments. After each stretch, enter the new micrometer reading into the normalization module and wait 60 s between the increments.

NOTE: Sudden drops in tension after a micrometer adjustment usually indicate vessel slippage. Mounting screws should be tightened gently but firmly.

  - Wait for the normalization module to plot resting wall tension (mN/mm) versus internal circumference (µm) and fit an exponential curve. Using the micrometer, continue stepwise distension until the tension-circumference curve intersects the 100 mmHg isobar. When near the intersection, reduce the step size to 10–30 µm to avoid overstretching the vessel.

  - When the curve intersects the 100 mmHg isobar, note the software-calculated internal circumference and set the micrometer to the corresponding value calculated by the module so the preparation rests at the target normalized internal circumference (IC₁).

NOTE: Poor curve fitting usually indicates incorrect parameters entered into the software, such as tissue length, wire diameter, or micrometer readings.

  - Repeat the normalization procedure for all mounted MRAs, then allow vessels to equilibrate for 30–45 min with repeated buffer changes using 37 °C Krebs.

  - Zero the force readings on the user interface immediately before beginning experiments.

NOTE: Key parameter differences between mice and rats are summarized in Table 1.

Place Table 1 here.

- Experimental protocol (example sequence)

  - Viability check: Elicit a contractile response with a depolarizing high-K⁺ challenge (e.g., 120 mM KCl).

NOTE: The high-K⁺ buffer (KPSS) is prepared by equimolar substitution of NaCl with KCl from the standard Krebs buffer to maintain osmolarity and is continuously oxygenated with 95% O₂/5% CO₂. To avoid cold shock, warm the KPSS to 37 °C before use. KPSS buffer composition (final concentrations, pH 7.4): 120 mM KCl, 14.7 mM NaCl, 1.17 mM MgSO₄, 1.18 mM KH₂PO₄, 1.6 mM CaCl₂, 25 mM NaHCO₃, 5.5 mM glucose, and 0.03 mM EDTA.

  - Perform at least three consecutive washes with fresh 37 °C oxygenated Krebs buffer until a stable passive basal tension is restored.

NOTE: All wash steps are performed by completely removing the bathing solution and replacing it with fresh 37 °C oxygenated Krebs buffer. Commercial myographs commonly have a built-in wash function that drains the myography chambers.

  - Generate a cumulative concentration-response curve (CRC) to phenylephrine (PE), adding increasing concentrations (example range 10⁻⁹ to 10⁻4 M). Allow each concentration to plateau before adding the next concentration until a stable maximal plateau is achieved.

  - Perform at least three consecutive washes with fresh 37 °C oxygenated Krebs buffer until a stable passive basal tension is restored.

  - Assess endothelial function: Preconstrict with a submaximal concentration of PE, allow response to stabilize, and then assess endothelium-dependent relaxation with ACh (example range 10⁻⁹ to 10⁻⁵ M). Allow each concentration to plateau before adding the next concentration until a stable maximal relaxation is achieved. Record percent relaxation relative to the preconstricted tone (See step 7.4). Preparations exhibiting <70% relaxation are excluded.

NOTE: Vascular wall diseases, such as hypertension, are commonly associated with endothelial dysfunction, which often leads to impaired endothelium-dependent vasorelaxation response to ACh19. Under these conditions, preparations exhibiting <70 % relaxation should not be excluded automatically, as this may represent true disease-related endothelial dysfunction rather than technical failure.

  - Perform at least three consecutive washes with fresh 37 °C oxygenated Krebs buffer until a stable passive basal tension is restored.

  - Test other vasoactive agents, with and without pathway-specific inhibitors, as required by the experimental question.

  - At the end of the experiment, remove all the mounted vessels, clean all chambers with 8% acetic acid, followed by three washes of water, and finally turn off the gas tank and the machine.

  - Dispose of biological and chemical waste according to institutional procedures.

- Data analysis and reporting

  - Baseline definition and recording. Record the baseline tension immediately before the start of any vasoconstrictor challenge (i.e., high-K⁺ challenge, vasoconstrictor CRC, or preconstriction before a vasodilator CRC).

NOTE: For subsequent analysis, it is practically easier to zero the force readings on the user interface after adequate washing of any previous stimulation, immediately before the start of any vasoconstrictor challenge (i.e., displayed baseline tension = 0).

  - High-K⁺ challenge. Compute the active force as: Active Force = Maximum Tone after KCl - Baseline tension (units: millinewtons “mN”). Express the active force normalized to either unit length (mN/mm) or cross-sectional area (mN/mm2) of the mounted arterial segment. Measurement of cross-sectional area requires subsequent histological analysis of the mounted artery segments to determine wall thickness.

NOTE: Because aortic rings often produce a non-plateauing response, measure the maximum high-K⁺ response within a pre-specified time (e.g., 10 min) across all preparations, to minimize variability. In MRAs, maximum high-K⁺ response is usually achieved within seconds after stimulation.

  - Contractile CRCs. For each concentration in a contractile cumulative CRC (e.g., PE), calculate the active force as follows. Express the active force at each concentration as a percentage of the response elicited by the high-K⁺ challenge. Use these % values for fitting contractile CRCs (See step 7.5).

Active Force = Maximum Tone after drug application — Baseline tension (units: mN)

NOTE: Certain conditions may cause variability in the maximum high-K⁺ responses across preparations (e.g., changes in smooth muscle mass or differentiation state resulting from pathology). Under these circumstances, express the active force at each concentration normalized to either unit length (mN/mm) or cross-sectional area (mN/mm2) of the mounted arterial segment.

  - Relaxant CRCs. For relaxation CRCs after a submaximal preconstriction (preconstriction tone), compute percent relaxation at each concentration as follows. Use these % values for fitting relaxation CRCs (See step 7.5).

% Relaxation = (Preconstriction tone - Tone after drug application) / (Preconstriction tone - Baseline tension) × 100.

  - Curve fitting. For either contractile or relaxant cumulative CRCs, plot log [agonist concentration] on the x-axis versus the normalized response on the y-axis. Fit the plotted curve with a four-parameter logistic model using non-linear regression. Extract Emax (maximal effect) and pEC₅₀ (−logEC₅₀) for statistical comparisons, where EC₅₀ is the agonist concentration producing 50% of Emax.

  - Data presentation: Present the normalized CRCs. For each of the data points on the curve, use group mean ± standard error of the mean (SEM) or standard deviation of the mean (SD). Include representative original traces and provide a parameter table summarizing the Emax and pEC₅₀ values (mean ± SEM or SD).

  - Statistical analysis:

    - Compare CRCs between experimental groups using two-way repeated measures analysis of variance (ANOVA) with an appropriate post-hoc test (e.g., Bonferroni). However, check assumptions (sphericity, normality) and use alternative approaches (e.g., mixed-effects models) when data are missing or unbalanced.

7.7.,2. Compare pEC₅₀ and Emax between two experimental groups using an unpaired t-test, or one-way ANOVA if comparing more than two experimental groups. Alternatively, use non-parametric tests when variance homogeneity and normality assumptions are violated. Set statistical significance at p < 0.05.

REPRESENTATIVE RESULTS:

A schematic depicting the overall workflow of wire myography in conduit and resistance arteries isolated from mice or rats is shown in Figure 1. Representative images of isolation and mounting of a mouse thoracic aortic ring and a 2nd-order MRA are shown in Figure 2 and Figure 3, respectively. Software screenshots depicting pretensioning and normalization procedures using a rat aortic ring and an MRA are shown in Figure 4A,B, respectively.

Following the protocol, an adult male C57BL/6J mouse (Figure 5A,B) or an adult male Wistar Kyoto rat (Figure 5C,D) was sacrificed for assessment of vascular function using an endothelium-intact descending thoracic aortic ring and an MRA, studied simultaneously by isometric wire myography. Each of the representative original traces depicts a contractile response with a depolarizing high-K⁺ challenge, followed by a cumulative contractile CRC to PE, and finally a relaxant cumulative CRC to ACh after precontraction with PE.

Because of their greater smooth muscle mass, contractile responses are typically higher in arteries isolated from wild-type rats compared to mice. In both aortic rings and MRAs of adult wild-type mice or rats, high-K⁺ elicits a biphasic contraction: an initial rapid (phasic) rise in force that peaks within seconds, followed by a sustained (tonic) phase. Notably, aortic rings often exhibit a non-plateauing response to high-K⁺ stimulation, whereas MRAs typically achieve a stable plateau. Consequently, for aortic rings, the maximum high-K⁺ response is measured within a pre-specified time window (e.g., 10 min) across all preparations to minimize inter-animal variability. PE induces concentration-dependent contractions in both vessel types, with responses reaching a stable maximal plateau at higher concentrations. Similarly, ACh induces concentration-dependent relaxations that plateau at higher concentrations. Representative CRCs to PE and ACh from a mouse aortic ring are presented in Figure 6.

FIGURE AND TABLE LEGENDS:

Figure 1: Schematic depicting the overall workflow of wire myography in conduit and resistance arteries isolated from mice or rats.

Figure 2: Representative images of isolation and mounting of a mouse aortic ring. A 12-week-old male C57BL/6J mouse was sacrificed for isolation and mounting of an endothelium-intact descending thoracic aortic ring on a pin myograph. (A,B) Dissection of the heart and the descending thoracic aorta. (C) Removal of the surrounding perivascular adventitial fat. (D) Partially cleaned descending thoracic aorta. Note the remaining perivascular adventitial fat at the distal end of the vessel (left side of the panel). (E) An isolated 2-mm descending thoracic aortic ring. (F) A pin myograph chamber suitable for mouse aortic rings. (G) Mounting procedure. (H) Successfully mounted aortic ring.

Figure 3: Representative images of isolation and mounting of a mouse MRA. A 12-week-old male C57BL/6J mouse was sacrificed for isolation and mounting of an endothelium-intact MRA on a wire myograph. (A) Dissection of the mesenteric arcade. (B) Pinning down of the mesenteric arcade to straighten the mesenteric vasculature and visualize the SMA and SMV. Numbering depicts vascular branch order. (C) Close-up view of the SMA and SMV from which mesenteric arteries and veins radiate, respectively. (D) Close-up view of a mesenteric artery versus vein. Note the thicker wall and smaller lumen of the artery. (E) An isolated 2-mm 2nd order MRA. (F-H)  Threading the wires, one at a time, through the lumen of the MRA. (I) A wire myograph chamber suitable for mouse MRA. (J) First wire secured under the mounting screws on the micrometer side. (K) Second wire secured under the mounting screws on the transducer side. (L) Successfully mounted MRA. Abbreviations: MRA = Mesenteric resistance artery; SMA = Superior mesenteric artery; SMV = Superior mesenteric vein.

Figure 4: Pretensioning and normalization procedures in rat arteries. (A) Representative trace depicting pretension adjustment and equilibration of a descending thoracic aortic ring form an adult male Wistar Kyoto rat. Note the repeated fine adjustments of pretension before the tension stabilizes. (B) Screenshot of the normalization software module illustrating the normalization procedure for a rat MRA. Abbreviations: MRA = Mesenteric resistance artery. IC₁₀₀ = The internal circumference corresponding to an equivalent transmural pressure of 100 mmHg. IC₁ = Normalized internal circumference.

Figure 5: Representative original traces of aortic rings and mesenteric resistance arteries isolated from mice or rats. (A,B) A 12-week-old male C57BL/6J mouse was sacrificed for assessment of vascular function by isometric wire myography using (A) an endothelium-intact descending thoracic aortic ring, or (B) an endothelium-intact MRA. (C,D) A 16-week-old male Wistar Kyoto rat was sacrificed for assessment of vascular function by isometric wire myography using (C) an endothelium-intact descending thoracic aortic ring, or (D) an endothelium-intact MRA. Each of the traces depict high-K⁺ (120 mM) induced contraction (red line), a cumulative CRC to PE (10-9-10-4 M; blue line), and a cumulative CRC to ACh (10-9-10-5 M; green line) after preconstriction with a submaximal concentration of PE. Blue and green arrows denote the application of cumulative increasing concentrations of PE and ACh, respectively. PE CRC and PE preconstriction were initiated after repeated washing with fresh 37°C oxygenated Krebs buffer till a stable passive basal tension was restored. Abbreviations: CRC = Concentration Response Curve; PE = Phenylephrine; ACh = Acetylcholine; MRA = Mesenteric Resistance Artery.

Figure 6: Representative original concentration response curves of PE and ACh using a mouse aortic ring. A 12-week-old male C57BL/6J mouse was sacrificed for assessment of vascular function by isometric wire myography using an endothelium-intact descending thoracic aortic ring. (A) A cumulative CRC to PE (10-9-10-4 M). Active force at each concentration is calculated as a percentage of the maximum response elicited by the high-K⁺ challenge. (B) A cumulative CRC to ACh (10-9-10-4 M) after preconstriction with a submaximal concentration of PE. Percent relaxation at each concentration is calculated relative to the PE preconstriction level. N = 1.

Table 1: Key parameter differences between mice and rats.

DISCUSSION:

Wire myography remains a powerful, high-resolution approach for interrogating vascular function ex vivo. When executed carefully, this method provides reliable quantitative endpoints that address distinct biological questions while allowing rigorous comparison of macro- and microvascular beds within the same animal. Conduit artery studies reveal changes in global arterial mechanics and large-vessel reactivity relevant to pulse pressure and aortic stiffness, whereas resistance artery experiments probe microvascular tone and mechanisms that directly control peripheral resistance and tissue perfusion12,20. In addition to key differences in structure and physiological role, conduit and resistance arteries also exhibit important functional differences, including distinct contributions of nitric oxide, prostanoids, and EDHFs to endothelium-dependent vasorelaxation across vascular beds and species13-16. Accordingly, conducting myography assays in parallel using vessels obtained from the same animal enables direct comparison of bed-specific effects of genetic or pharmacologic manipulations while minimizing inter-animal variability. This experimental design is statistically powerful, reduces sample size requirements, and enhances detection of regional differences in vascular reactivity.

Compared with alternative methods, wire myography offers several distinct advantages for vascular reactivity studies. While pressure myography more closely preserves physiological conditions by maintaining intraluminal pressure and flow, it typically requires longer intact vessel segments, is technically more demanding, and is less amenable to high-throughput pharmacological screening. In vivo imaging and hemodynamic approaches (e.g., ultrasound or photoacoustic imaging) provide essential physiological context but generally lack the pharmacological precision and direct access to the vessel wall afforded by ex vivo assays21. Wire myography, therefore, serves a complementary role to alternative methods. It delivers high sensitivity for detecting small changes in force, supports detailed concentration-response analyses under tightly controlled conditions, and allows multiple arterial segments (conduit and resistance) from the same animal to be interrogated side-by-side in a relatively high-throughput manner, which is particularly valuable for mechanistic pharmacology and bed-specific drug profiling.

Critical steps in the protocol warrant emphasis because they disproportionately affect data quality and interpretation, and several are explicitly species-specific. First, buffer composition and oxygenation must be consistent and physiologically appropriate (e.g., bicarbonate-based solution equilibrated with carbogen and maintained at pH 7.4), as small deviations in pH, temperature, or gas tension can markedly alter basal tone and agonist sensitivity in both mouse and rat preparations. Second, endothelial integrity is a major source of experimental variability, particularly in small mouse mesenteric arteries, which are more fragile and prone to luminal damage during isolation and mounting than larger rat arteries. Tissue trauma, especially at the luminal surface, should be minimized by gentle dissection, limited manipulation, and careful threading of wires. Successful endothelial preservation should be confirmed in every segment by robust ACh-induced vasorelaxation after standardized preconstriction. Third, pretension/normalization sets the mechanical operating point of the vessel and is inherently species- and bed-specific. Following empiric procedures (e.g., determining IC₁ relative to IC₁₀₀ for MRAs) maximizes active force and enhances reproducibility. It’s important to note that the chosen pretension values and IC₁ settings for mouse versus rat arteries in this protocol are empirically-derived, historically validated values rather than physiological constants5,17. Given that optimal IC₁/IC₁₀₀ ratios and resting tensions differ between species and vessel types, investigators should validate the pretension value and IC₁/IC₁₀₀ ratio for each vessel type and species studied22. Fourth, tissue viability and preserved endothelial function should always be verified in each arterial preparation prior to testing the experimental hypothesis (e.g., by demonstrating robust contraction to high-K⁺ depolarization and reproducible relaxation to ACh, respectively).  Segments that fail these criteria should be excluded from analysis to avoid misinterpreting technical failure as true biological effect.

In addition to providing a detailed protocol for side-by-side wire myography to assess vascular reactivity in conduit versus resistance arteries isolated from mice or rats, this protocol outlines the typical causes of common technical problems and offers troubleshooting strategies for each. Several recurrent technical issues may arise during wire myography and can substantially affect data quality if not recognized. A weak or absent contractile response to high K⁺ typically reflects compromised tissue viability, such as ischemic injury caused by prolonged dissection, but may also result from incorrect preparation or composition of physiological salt solution (PSS) or high-K⁺ solution (KPSS), suboptimal bath temperature below 37 °C, inadequate oxygenation, excessive vessel overstretching during pretensioning or normalization, or insufficient equilibration following these steps.

Poor endothelium-dependent relaxation to acetylcholine (ACh) is most often attributable to mechanical endothelial damage, including truncated, crushed, or unevenly cut vessel segments, excessive manipulation during dissection or mounting, or trauma caused by aggressive wire threading, particularly in mesenteric resistance arteries. Inadequate equilibration after mounting or normalization, cold shock, or excessive preconstriction before ACh application can further impair relaxation responses. Importantly, in pathological or disease models, attenuated endothelium-dependent relaxation may reflect genuine biological dysfunction rather than a technical artifact and should be interpreted in the context of appropriate controls.

Instability of baseline tension is frequently observed during the initial pretensioning of aortic rings and often reflects gradual viscoelastic relaxation before stabilization. Additional contributors include insufficient equilibration after mounting or fluctuations in bath temperature. Slippage of aortic rings after mounting may occur if the chamber is moved before the vessel is adequately seated on the wires, or if excessive bubbling induces mechanical agitation; careful handling and controlled gas flow reduce this risk. In mesenteric resistance arteries, slippage most commonly manifests as a sudden loss of tension during or after normalization and is typically caused by inadequate tightening of mounting screws or misalignment of the wires. Recognizing these pitfalls facilitates distinction between technical failure and biologically meaningful variation in vascular reactivity.

To improve reproducibility across laboratories, authors should adhere to community guidelines for experimental design, normalization, and reporting and provide sufficient detail to enable direct replication and meta-analysis. This includes clear documentation of experimental variables, including animal factors (e.g., age, sex, strain, circadian timing, anesthesia), anatomical sampling sites with anatomical landmarks, species- and vessel-specific pretension/normalization values, viability criteria and exclusion rates, and statistical methods used for normalization and analysis. Such transparency facilitates meaningful cross-study comparisons and helps identify sources of heterogeneity when results differ across studies.

Several limitations of this protocol should be considered when interpreting results. Wire myography is an ex vivo assay that isolates the vascular wall from hemodynamic forces, such as shear stress and pulsatile pressure, as well as systemic neural and hormonal inputs that modulate endothelial function and smooth muscle tone23. Consequently, results from wire myography should be integrated with in vivo data, such as telemetric blood pressure monitoring or ultrasound-based flow assessments, when drawing physiological or translational conclusions21 . Animal factors (e.g., age, sex, strain, circadian timing, anesthesia) can all influence vascular outcomes and should be prospectively controlled, reported in detail, and, where possible, incorporated into experimental design and analysis as biological variables rather than treated solely as confounders24,25. Furthermore, the arterial segments chosen for wire myography require careful selection and consistent sampling of anatomical sites across animals, because vascular structure and function vary regionally even within a single vessel bed26. The aortic tree shows clear, reproducible regional differences in vascular reactivity between the descending thoracic and the abdominal aorta27-29. In the mesenteric circulation, vascular reactivity can vary according to branch order and vessel caliber, reflecting differences in wall structure, smooth muscle content, and endothelial signaling mechanisms30.

The protocol described here is broadly applicable to mechanistic studies of vascular disease and pharmacology. Side-by-side myography of the descending thoracic aorta and MRAs is particularly useful in preclinical drug screening and in investigating vascular wall changes in hypertension, atherosclerosis, and metabolic diseases, including endothelial dysfunction and altered vasoreactivity to various vasoactive substances, because it reveals whether therapeutic interventions normalize macrovascular reactivity, microvascular tone, or both. Combining wire myography with complementary approaches, such as pressure myography, cellular Ca²⁺ imaging, molecular profiling, and in vivo hemodynamic measurements, will enable integrated, multi-scale characterization of vascular pathophysiology and facilitate the development of bed-specific therapeutic strategies. This framework can also be extended to cross-species translational studies comparing mouse, rat, and human small arteries to identify conserved and divergent mechanisms.

ACKNOWLEDGMENTS:

The work was supported by an NHLBI award (R00 HL153896 to I.O.) and startup funds from the University of Toledo.

DISCLOSURES:

The authors have no conflicts of interest to disclose.

REFERENCES:

- De Oliveira, M. G., Nadruz, W., Jr., Mónica, F. Z. Endothelial and vascular smooth muscle dysfunction in hypertension. Biochem Pharmacol. 205, 115263 (2022).

- Gallo, G., Volpe, M., Savoia, C. Endothelial dysfunction in hypertension: current concepts and clinical implications. Front Med. 8 (2022).

- Touyz, R. M. et al. Vascular smooth muscle contraction in hypertension. Cardiovasc Res. 114 (4), 529–539 (2018).

- Serné, E. H., de Jongh, R. T., Eringa, E. C., Ijzerman, R. G., Stehouwer, C. D. A. Microvascular dysfunction. Hypertension. 50 (1), 204–211 (2007).

- Wenceslau, C. F. et al. Guidelines for the measurement of vascular function and structure in isolated arteries and veins. Am J Physiol Heart Circ Physiol. 321 (1), H77–H111 (2021).

- Del Campo, L., Ferrer, M. Wire myography to study vascular tone and vascular structure of isolated mouse arteries. Methods Mol Biol. 1339, 255–276 (2015).

- Mulvany, M. J., Halpern, W. Mechanical properties of vascular smooth muscle cells in situ. Nature. 260 (5552), 617–619 (1976).

- Sun, J. et al. Isometric contractility measurement of the mouse mesenteric artery using wire myography. J Vis Exp. 138, e58064 (2018).

- Halpern, W., Osol, G., Coy, G. S. Mechanical behavior of pressurized in vitro prearteriolar vessels determined with a video system. Ann Biomed Eng. 12 (5), 463–479 (1984).

- Jadeja, R. N., Rachakonda, V., Bagi, Z., Khurana, S. Intravital assessment of vascular function. J Vis Exp. 101, e50997 (2015).

- Intengan, H. D., Schiffrin, E. L. Structure and mechanical properties of resistance arteries in hypertension. Hypertension. 36 (3), 312–318 (2000).

- Lee, R. M. K. W., Dickhout, J. G., Sandow, S. L. Vascular structural and functional changes in hypertension: models, remodeling and relevance. Hypertens Res. 40 (4), 311–323 (2017).

- Furchgott, R. F., Zawadzki, J. V. The obligatory role of endothelial cells in arterial smooth muscle relaxation. Nature. 288 (5789), 373–376 (1980).

- Palmer, R. M., Ferrige, A. G., Moncada, S. Nitric oxide release accounts for endothelium-derived relaxing factor activity. Nature. 327 (6122), 524–526 (1987).

- Brandes, R. P. et al. An endothelium-derived hyperpolarizing factor distinct from nitric oxide and prostacyclin. Proc Natl Acad Sci U S A. 97 (17), 9747–9752 (2000).

- Garland, C. J., Hiley, C. R., Dora, K. A. EDHF: spreading the influence of the endothelium. Br J Pharmacol. 164 (3), 839–852 (2011).

- Mulvany, M. J., Halpern, W. Contractile properties of resistance vessels in hypertensive rats. Circ Res. 41 (1), 19–26 (1977).

- Grano De Oro, A. et al. Spontaneous vascular dysfunction in Dahl salt-sensitive rats without high-salt diet. Physiol Rep. 12 (14), e16165 (2024).

- Deanfield, J. E., Halcox, J. P., Rabelink, T. J. Endothelial function and dysfunction: testing and clinical relevance. Circulation. 115 (10), 1285–1295 (2007).

- Mulvany, M. J. Structure and function of small arteries in hypertension. J Hypertens Suppl. 8 (7), S225–S232 (1990).

- Huda, K., Lawrence, D. J., Thompson, W., Lindsey, S. H., Bayer, C. L. In vivo noninvasive systemic myography in pregnant mice. Nat Commun. 14 (1) (2023).

- Choi, R. et al. Optimization of resting tension for wire myography in rat pulmonary arteries. Physiol Rep. 12 (1), e15911 (2024).

- Lu, X., Kassab, G. S. Assessment of endothelial function across vessel sizes using a unified myograph. Am J Physiol Heart Circ Physiol. 300 (1), H94–H100 (2011).

- Hou, T., Guo, Z., Gong, M. C. Circadian variations of vasoconstriction and blood pressure. Curr Opin Pharmacol. 57, 125–131 (2021).

- Li, T. et al. Age- and sex-dependent vascular responsiveness mediated by estrogen receptor pathways. Am J Physiol Heart Circ Physiol. 306 (8), H1105–H1115 (2014).

- Schubert, R., Gaynullina, D., Shvetsova, A., Tarasova, O. S. Myography of isolated blood vessels. Front Physiol. 14, 1176748 (2023).

- Ruddy, J. M., Jones, J. A., Spinale, F. G., Ikonomidis, J. S. Regional heterogeneity within the aorta. J Thorac Cardiovasc Surg. 136 (5), 1123–1130 (2008).

- Ameer, O. Z. et al. Regional abnormalities within the aorta in metabolic syndrome. Exp Physiol. 106 (3), 771–788 (2021).

- Mezhenskyi, O. R., Philyppov, I. B. Regional differences in endothelium-dependent relaxation of rat thoracic aorta. Physiol Rep. 12 (6) (2024).

- Zhang, L., Rong, S., Dong, H. Functional heterogeneity of endothelium-dependent vasorelaxation in mesenteric arteries. Microvasc Res. 158, 104777 (2025).