TITLE:

DNA Viral Size Fraction Metagenomics for Human Stool Samples

AUTHORS AND AFFILIATIONS:

Elizabeth R. Denison, Luke S. Hillary, Haley A. Bolanos, Hazel I. Anagu, Joanne B. Emerson

Department of Plant Pathology, University of California, Davis

Email addresses of the co-authors:

Elizabeth R. Denison		erdenison@ucdavis.edu

Luke S. Hillary			lhillary@ucdavis.edu

Haley A. Bolanos		habolanos@ucdavis.edu

Hazel I. Anagu			hazelanagu@gmail.com

Corresponding authors:

Joanne B. Emerson		jbemerson@ucdavis.edu

SUMMARY:

We present a flexible protocol for generating high-quality viral size fraction metagenomes (viromes) from human stool samples. Originally developed for soil viromics, here this protocol was successfully applied to stool samples, enabling robust characterization of the human gut virosphere.

ABSTRACT:

Understanding the healthy human virosphere (the viral component of the microbiome) requires accurate measurements of viral community composition across a diverse range of viral types. Building on prior experience with soil viral community ecology methods, here we demonstrate a series of laboratory approaches for enriching and extracting DNA from extracellular DNA viruses in human stool samples. A working primary protocol is presented, along with options for deviations at different steps. The general approach involves adding a liquid buffer (default: protein-enhanced phosphate buffered saline, PPBS) to facilitate removal of free viral particles from the stool matrix, centrifugation to separate the liquid fraction containing viral particles, filtration (default: 0.2 µm pore size) to remove most cells, concentration of viral particles (default: ultracentrifugation), removal of free nucleic acids with nucleases prior to virion lysis, and then DNA extraction for sequencing. Alternative techniques, including different buffers, filter sizes, and concentration methods, are also noted. Overall, multiple options for generating high-quality viromic DNA for sequencing are offered. Rather than tailoring the approach to specific equipment and resources, the protocol's flexibility should make it broadly applicable across labs with varying standard molecular biology equipment.

INTRODUCTION:

Viromics, or the untargeted metagenomic sequencing of the viral size fraction of mixed communities, can provide insights into the diversity, evolution, and ecology of uncultured viruses. Since the early efforts to catalog gut viral communities over 20 years ago, which notably used methods developed for environmental samples like seawater1, viromics approaches have continued to enable discoveries in viral ecology within the human microbiome2. However, there is no single universal viromics protocol, and variations in techniques can impact sequencing results3,4, ultimately influencing the interpretation of viral community dynamics and hindering inter-study comparisons. Optimized protocols, combined with a thorough understanding of how methodological choices shape virome profiles, are needed to robustly characterize the human virosphere.

While there is no universal viromics protocol, the general basis of viromics methods involves physically separating and concentrating extracellular virus-like particles (virions) based on their size and/or density, removing free nucleic acids (derived in part from cellular debris or compromised viral capsids) prior to virion lysis, and extracting DNA and/or RNA for sequencing. Some viromics techniques select for or against specific viral types, whether as an intentional choice by researchers or an unintended methodological limitation. Syringe or vacuum filtration is frequently used to remove microbial cells and other debris, and the filter size inherently affects the quality and composition of the recoverable virome. For example, filtration can increase virome purity5, but it may exclude larger viral particles6,7. Virion concentration methods, such as ultracentrifugation, polyethylene glycol (PEG) precipitation, density gradient ultracentrifugation, and ultrafiltration, also have trade-offs among purity, throughput, and equipment costs3,8. Multiple buffer chemistries are known to be effective for viromics. They may not have outsized impacts on virome characterization9, but storage and resuspension buffers can still affect virus recovery and the virome profiles generated from bioinformatic analysis10,11. Lastly, as viral nucleic acid concentrations can be low in natural samples, many studies opt to amplify viral DNA before library construction (e.g., by multiple displacement amplification (MDA) or sequence-independent single-primer amplification (SISPA)). Yet some of these techniques introduce significant bias12,13. Overall, while many viromics methods can successfully generate viromes, optimized protocols that acknowledge biases and are somewhat standardized will enable accurate and reproducible virome research.

This article presents a flexible workflow for generating viromes from human stool samples. Adapted from proven soil viromics techniques14–16, this protocol includes several steps that facilitate modification, enabling researchers to adapt it to their own equipment and research needs. The standard soil viromics method outlined below, successfully applied to both soil and frozen stool samples, produces high-quality viromes without a viral DNA amplification step prior to library construction. The suggestions for protocol modifications have been tested to varying degrees in other studies, but their specific, downstream effects on human stool virome sequences were not examined here.

PROTOCOL:

All subjects recruited for stool sample collection provided informed consent. Sample collection protocols were approved by the University of California, Davis Institutional Review Board, and the studies conform to the Declaration of Helsinki. This protocol is adapted from Emerson et al. (2022)17.

- Preparation of materials

  - Prepare protein-enhanced phosphate-buffered saline (PPBS, 10x PBS, 1% K-citrate, 150 mM MgSO4, 2% bovine serum albumin (BSA))18.

NOTE: Other commonly used buffers in viromics protocols are saline magnesium (SM)19 and amended potassium-citrate prime (AKC’) buffer11.

    - Add 10 mL 10x PBS, 10 g K-citrate, 18.05 g anhydrous MgSO4 to a 1 L autoclavable, glass bottle and fill to 1 L with ultra-pure laboratory water. Adjust the pH to 6.5 (typically requires about 50 μL of 6 M HCl or about 100 μL of 2 M HCl). Leave a stir bar inside the bottle for use in Step 1.1.2.

    - Autoclave the bottle with buffer and one empty 1 L autoclavable glass bottle. After autoclaving, once the buffer cools to room temperature, place it on a magnetic stir plate and slowly add 20 g BSA directly into the bottle, adding about 5 g of BSA at a time (to avoid foaming, minimize excessively agitating the solution, e.g., start with a slow rate of stirring and gradually increase).

    - Filter the entire buffer volume through a sterile 0.22 μm vacuum filter into the empty autoclaved 1 L bottle from Step 1.1.2. Store the buffer at 4 °C until use.

NOTE: PPBS buffer can be stored at 4 °C for about 3 months, but it is not stable long-term when stored above freezing.

  - Prepare the ultracentrifuge tubes.

NOTE: This protocol uses ultracentrifugation to concentrate virions. Other common approaches include polyethylene glycol (PEG) precipitation, density gradient ultracentrifugation (e.g., cesium chloride gradients), and ultrafiltration.

    - To clean the ultracentrifuge tubes, first, scrub the tubes using ultra-pure laboratory water. Then rinse the tubes with ultra-pure laboratory water.

    - Fill the tubes with 1 M NaOH and let soak for at least 1 hour (can let soak overnight). Clean the plastic/rubber ultracentrifuge caps and their o-rings by immersing them in 1 M NaOH.

NOTE: Do not immerse the metal screw top caps in NaOH.

    - After soaking, rinse three times with ultra-pure laboratory water, followed by one rinse using nuclease-free, molecular biology grade water. Dry the tubes by inverting and gently tapping, or allowing them to dry completely overnight. Label each ultracentrifuge tube with a sample identifier.

  - Pre-cool the ultracentrifuge rotor at 4 °C by leaving the rotor in the fridge or cold room overnight.

- Virion purification and concentration

  - Weigh out the stool into a sterile, DNase-, RNase-, and DNA-free 50 mL conical tube.

NOTE: The amount of stool used may depend on the amount of sample material available. Here, 2 g of stool was used. If unused sample material needs to be stored frozen, consider aliquoting the entire sample before freezing to minimize future freeze-thaw cycles.

  - Add 8 mL of PPBS buffer directly into the conical tubes containing the stool (conical #1). Thoroughly resuspend by vortexing.

  - Shake the tubes at 300 RPM at 4 °C for 10 min.

NOTE: If necessary, the shaking can be done at room temperature, but 4 °C is recommended.

  - Centrifuge samples in conical #1 at 4,000 x g at 4 °C for 10 min.

NOTE: Use a swinging bucket rotor, if possible, to facilitate vertical density separation to make the next steps easier.

  - Carefully decant the supernatant into a new sterile 50 mL conical (conical #2) and store at 4 °C (the supernatant here contains the virions). Do not discard the conical tube that contains the pellet (conical #1), as it is used again in Step 2.6.

  - Repeat steps 2.2-2.5 two more times (so that the sample in conical #1 has gone through a total of 3x resuspension with buffer, 3x shaking, and 3x centrifugation) and continue to pool the supernatant into conical #2.

NOTE: Three rounds should accumulate ~24 mL supernatant total.

  - Centrifuge the 50 mL conical tube containing accumulated supernatant (conical #2) for 8 min at 10,000 x g at 4 °C.

  - Perform a second round of centrifugation. First, decant the supernatant from conical #2 into a new 50 mL conical tube (conical #3). Centrifuge conical #3 for 8 min at 10,000 x g at 4 °C.

  - Using syringe filtration, sequentially filter the supernatant from conical #3, avoiding the pellet at/near the bottom of the tube. Pre-filter using a sterile 5.0 μm polyethersulfone (PES) syringe filter, followed by a sterile 0.20 μm PES syringe filter, directly into an ultracentrifuge tube.

NOTE: The purpose of pre-filtering with 5.0 μm filters is to reduce clogging of the 0.20 μm filters. It may still be necessary to discard and replace the syringe filters if they become clogged. Filtration can be performed in a single step by stacking the 5 μm filter on top of the 0.20 μm filter, attached to a single syringe, and directly into an ultracentrifuge tube. PES membranes are preferred, as other membrane materials can retain virions (e.g., glass fiber) or clog faster (e.g., polyvinylidene fluoride, PVDF). 0.45 μm is another common filter size.

  - Check the mass of all ultracentrifuge tubes with their sample contents, and pair tubes in the ultracentrifuge based on mass. If necessary, add PPBS to the lighter tube of each pair to equalize weight (within a 0.05 g margin of error for paired tubes).

  - Ultracentrifuge at 112,000 x g at 4 °C under vacuum for 2 h 25 min.

NOTE: Depending on the rotor used, the spin duration will differ (according to the sediment coefficient (s) = 58). Refer to the Table of Materials for the rotor used here. Before removing the tubes from the ultracentrifuge, note the orientation of each tube in the rotor to aid in locating the viral pellet. Sometimes, after ultracentrifugation, the viral pellet is not visible, and extra care should be taken when removing the supernatant in the following step.

- Viral pellet resuspension and DNase treatment

  - Remove the supernatant either by decanting or using a pipette, taking care not to disturb the pellet, which can be fragile (the pellet contains the concentrated virions).

NOTE: As noted in Step 2.11, the pellet may or may not be visible, but a 'hole punch-sized' brown or yellow pellet can often be seen.

  - Treat the samples with DNase.

NOTE: If opting not to DNase treat samples, proceed to Step 4 and follow “If DNase treatment was not performed”. Refer to the Discussion for considerations on performing DNase treatment.

    - Resuspend the pellet in 100 μL of nuclease-free, molecular biology grade water into a labeled 1.5 mL microcentrifuge tube.

    - Make the DNase treatment master mix by adding 10 μL of DNase reaction buffer and 10 μL of DNase for each sample to be treated. Pipette to mix thoroughly.

NOTE: Prepare enough DNase treatment master mix for extra reactions to account for potential volume loss during pipetting in Step 3.2.3.

    - Add 20 μL of the DNase treatment master mix to the resuspended virions. Mix well by gentle pipetting. Incubate at 37 °C for 30 min.

    - Add 10 μL of DNase stop solution to inactivate the nucleases. Briefly vortex.

- Extraction of viral nucleic acids

  - Perform the DNA extraction using the DNA extraction kit listed in the Table of Materials.

NOTE: Make two modifications to the manufacturer’s instructions. 1) After Step 1 in the kit procedure, heat the bead tube containing the virions in buffer CD1 at 65 °C for 10 min, then proceed with the extraction according to the manufacturer’s instructions. 2) During the DNA elution step (step 16 in the kit procedure), incubate the columns at room temperature for 5 min after adding C6.

    - If DNase treatment was performed: After the nucleases have been inactivated (in Step 3.2.4), use the entire volume as input for the DNA extraction (i.e., in step 1 of the DNA extraction kit procedure, add this volume to the bead tube containing 800 μL of CD1). Make sure not to leave any solution behind (this solution contains the virions).

    - If DNase treatment was not performed: Add 100 μL of nuclease-free, molecular-grade water to the ultracentrifuge tube containing the pellet from Step 3.1. Gently scrape the pellet and resuspend. Use the resuspended pellet as input for DNA extraction.

- DNA quantification

  - Quantify the extracted DNA with the fluorescence-based method of choice.

NOTE: Virome DNA yields are rarely high enough for absorbance-based quantification; however, this can still be helpful for identifying chemical contaminants.

  - For short-term storage, store DNA at -20 °C after quantification. For longer-term storage, store at -80 °C. The DNA is now ready for library preparation and sequencing. Refer to the Table of Materials for the library preparation kit and sequencing platform used here.

REPRESENTATIVE RESULTS:

An exploratory effort applying this protocol to a single frozen stool sample (processed in technical triplicate for viromes) alongside two soil samples demonstrated its efficacy in producing high-quality viromes from different sample types. Using quality-filtered sequencing data and de novo-assembled contigs generated by this protocol, putative viral sequences were identified using an open-source virus identification tool20. Reads mapped to the identified viral contigs were then used as an estimate for viral relative abundance in each sample. The stool viromes were enriched in viral sequences, where viral reads comprised, on average, 74% of all sequenced reads in the viromes, compared to only 4% in total metagenomes generated from the same sample (Figure 1A). Furthermore, high-confidence viral genomes were generated from the stool samples (Figure 1B). Similar to results from the soil viromes, mean virus confidence scores from the stool viromes were high, with a mean score of 0.97 (with 1 being the highest likelihood a sequence is viral) (Figure 1B), indicating that the recovered contigs were indeed likely viral. Notably, the viral contigs assembled from stool virome replicates had significantly higher mean virus confidence scores than those from total metagenomes (two-sample t-test, p < 0.001). Overall, this protocol produced stool viromes enriched in viral reads and high-confidence viral genomic sequences for use in downstream genomic and ecological analyses.

Initial results suggest that this protocol can be modified to meet user needs and generate sufficient DNA for sequencing viromes. Testing modified versions of the protocol on additional frozen stool samples using different buffers, filter sizes, and virion concentration methods commonly used in other viromics protocols resulted in successful DNA extraction from the viral size fraction (Figure 2). For each protocol tested, all three stool samples (except for one sample in the buffer test) displayed significant differences in DNA yield based on method (ANOVA, p < 0.05) (Figure 2). Since each stool sample was collected from either a different individual or the same individual at a different time point, differences in DNA yield across samples may be due to inherent differences in stool properties and/or interpersonal virosphere variation. It is worth noting that, while DNA was successfully extracted by all methods, DNA quantification does not discriminate between viral and non-viral DNA. Additional measures, such as analyzing sequencing data or qPCR of bacterial marker genes, would be required to assess the extent of non-viral contamination.

FIGURE LEGENDS:

Figure 1: Viromes generated with this protocol were enriched in viral sequences and can be used to assemble high-confidence viral contigs. (A) Percent of total reads that mapped to predict viral contigs de novo assembled from viromes generated from two soil and one stool sample. The stool virome and metagenome were processed in technical triplicate. (B) Mean virus confidence scores for the de novo assembled contigs from the technical triplicates. Virus confidence scores were generated by an open-source virus identification tool20. Scores range from 0.7 to 1 (0.7 is the default minimum score for a contig to be classified as viral20). Significant differences in mean virus contig confidence scores between groups were tested by two-sample t-tests. All samples were DNase-treated at Step 3 in the Protocol.

Figure 2: Successful extraction of viral-size fraction DNA (in ng/μL) from frozen stool samples across three protocol modifications. (A) Resuspension buffer type: protein-enhanced phosphate-buffered saline (PPBS), saline magnesium (SM), amended potassium citrate prime buffer (AKC’). (B) Syringe filter pore size: 0.2 μm, 0.45 μm, 0.8 μm. (C) Virion concentration method: ultracentrifugation (UC), polyethylene glycol precipitation (PEG), ultrafiltration (UF). Protocol variations were performed with three biological replicates (i.e., stool samples from three different subjects), each processed in technical triplicate (shown as points). For each biological replicate (stool #), statistical differences in DNA yield across protocol variations were assessed using ANOVA. All samples were DNase-treated at Step 3 in the Protocol.

DISCUSSION:

This protocol presents a relatively straightforward and adaptable method for generating viromes. Proven in soils14-16,21, it is successfully applied here to human stool samples. This protocol will best recover extracellular, intact DNA viruses and is not designed to intentionally recover intracellular viruses or prophage (though some may be recovered from contaminating cells), very large DNA viruses, or RNA viruses. Generating paired cellular fractions or bulk metagenomes can be useful if examining these groups is desired. Its basis is simple, and while the protocol outlines the standard workflow that has been successful for us, key aspects of the protocol are flexible based on individual needs and research questions. Modifications to filter size, buffer chemistry, and the virion concentration method all yielded detectable DNA concentrations, which, in most cases, we presume is mostly viral21. While viromic DNA yields were significantly different between modified protocols, future sequencing and analysis is required to determine the effects on viral community recovery.

Using a smaller filter size (0.2 μm over 0.45 μm) can result in considerably less contamination from non-viral sequences18. However, smaller sizes can exclude some virus particles due to their size (e.g., giant viruses)22 and/or filter clogging. Additionally, work in aquatic systems has demonstrated that different filter size fractions can recover different viral community members7,23, thus any conclusions drawn from virome sequence analysis should be done with consideration of filter size. The higher DNA yields observed here when using larger filter sizes (particularly 0.8 μm) is likely due to increased cellular DNA contamination, but that requires validation with sequencing and bioinformatic analysis, as DNA quantification used here cannot discriminate between viral and non-viral DNA. Similarly, while filtration is an effective method for reducing cellular contamination in viromes5, intact cells may still pass through filters, leading to non-viral DNA being sequenced. Bacterial contamination could be assessed through qPCR24 and estimating the fraction of contaminating bacteria and human reads after sequencing is an important and recommended step in viromics analysis. Regarding the choice of buffers, PPBS is commonly used in soil viromics because the salt components stabilize pH and maintain virus particle integrity, while the protein components disrupt phage-soil particle interactions11,18. SM buffer has also been recommended for stool viromics3. We note that buffer chemistry has been shown to have only a small effect on virome profiles in soils, including when comparing SM and PPBS buffers9.

DNase treatment is commonly performed in viromics protocols to reduce contaminating free DNA prior to virion lysis and DNA extraction. However, frozen sample storage may compromise virions, leading to DNase treatment degrading viral genomic DNA inside compromised capsids. For example, in soil samples DNase treatment of frozen samples can yield viromic DNA below detection limits, so avoiding DNase can be advantageous for soils stored frozen. Comprehensive testing of DNase treatment with freezing has yet to be performed; however, tests using a single frozen stool sample showed that DNase treatment still produced sufficient DNA for sequencing, and there was little to no effect on viral richness or composition in virome libraries prepared with and without DNase treatment25. These results are similar to those from soils processed fresh, where ecological patterns were essentially the same between DNase-treated and untreated samples, though DNase treatment resulted in fewer contaminating microbial sequences and greater recovery of viral sequences15. Additionally, DNA was recovered from different frozen stool samples treated with DNase (Figure 2). Taken together, preliminary results suggest that omitting DNase treatment is potentially not necessary for stool samples stored frozen, but additional sequencing data and fresh versus frozen stool sample storage comparisons are needed to confirm that. If stool viromic DNA yields are below detection limits after DNase treatment, omitting DNase treatment may be advisable to obtain sufficient yields for sequencing, possibly at the cost of greater contaminating non-viral DNA.

An advantage of this protocol is that the resulting viromes are both enriched in viral sequences and free of biases introduced by DNA amplification prior to library construction. While viromics reduces cellular contamination and enriches viral sequences compared to total metagenomes26, the resulting DNA yields can be low. Therefore, many studies have reported amplifying extracted DNA after viral enrichment but before library construction (e.g., performing multiple-displacement amplification (MDA) is common in stool viromics)27, but this can substantially bias results and skew abundances for specific viral types12,13,28. The protocol reported here does not rely on DNA amplification, and the enrichment steps alone yielded detectable DNA concentrations for sequencing. As expected, based on results from soil and other sample types26,29, the stool viromes also outperformed the total metagenome approach for enriching viral sequences. Alternate methods may still be appropriate depending on the research goals, as total metagenomes can detect prophages and intracellular viruses that are excluded by viromics approaches.

This protocol can also be easily adapted for RNA viromics or for simultaneous extraction of RNA and DNA from the same viral pellet. This is particularly advantageous when sample material is limited or where parallel recovery of RNA and DNA viruses from the same sample is desired. Alternatively, total nucleic acids can be purified, followed by cDNA synthesis and sequencing of both DNA and RNA viral genomes. However, issues related to the overrepresentation of viral genomes of different nucleic acid forms30 remain unresolved. Additionally, ribosomal RNA contamination can vary widely, and further optimization of ribosomal RNA depletion is required31. In summary, this straightforward and flexible protocol provides a robust foundation for exploring viral communities of the human gut.

ACKNOWLEDGMENTS:

We thank Drs. Sean Adams, Trina Knotts, and Mohamed R. Ali (University of California, Davis School of Medicine) for collecting and providing the stool sample in adherence with IRB protocols and ethical standards. We thank Dr. Matthew R. Olm (University of Colorado, Boulder) for collaboration on analyses of the stool sample. This research was supported by the NIH Common Fund (Human Virome Program), Award # U01DE034198. LSH was also partially supported by the U.S. Department of Energy (DOE), Office of Science, Office of Biological and Environmental Research (BER), Genomic Science Program, award number DE-SC0023127 (grant to JBE, grant PI Sydney Glassman).

DISCLOSURES:

The authors have no conflicts of interest to declare.

REFERENCES:

- Breitbart, M. et al. Metagenomic Analyses of an Uncultured Viral Community from Human Feces. J Bacteriol. 185 (20), 6220–6223, doi: 10.1128/jb.185.20.6220-6223.2003 (2003).

- Liang, G., Bushman, F.D. The human virome: assembly, composition and host interactions. Nat Rev Microbiol. 19 (8), 514–527 (2021).

- Callanan, J., Stockdale, S.R., Shkoporov, A., Draper, L.A., Ross, R.P., Hill, C. Biases in Viral Metagenomics-Based Detection, Cataloguing and Quantification of Bacteriophage Genomes in Human Faeces, a Review. Microorganisms 9 (3), 524 (2021).

- Garmaeva, S., Sinha, T., Kurilshikov, A., Fu, J., Wijmenga, C., Zhernakova, A. Studying the gut virome in the metagenomic era: challenges and perspectives. BMC Biol. 17 (1), 84 (2019).

- Lewandowska, D.W. et al. Optimization and validation of sample preparation for metagenomic sequencing of viruses in clinical samples. Microbiome 5 (1), 94, doi: 10.1186/s40168-017-0317-z (2017).

- Conceição-Neto, N. et al. Modular approach to customise sample preparation procedures for viral metagenomics: a reproducible protocol for virome analysis. Sci Rep. 5 (1), 16532, doi: 10.1038/srep16532 (2015).

- Palermo, C.N., Shea, D.W., Short, S.M. Analysis of Different Size Fractions Provides a More Complete Perspective of Viral Diversity in a Freshwater Embayment. Appl Environ Microbiol. 87 (11), e00197-21, doi: 10.1128/AEM.00197-21 (2021).

- Kleiner, M., Hooper, L.V., Duerkop, B.A. Evaluation of methods to purify virus-like particles for metagenomic sequencing of intestinal viromes. BMC Genomics 16 (1), 7 (2015).

- Fudyma, J.D. et al. Exploring viral particle, soil, and extraction buffer physicochemical characteristics and their impacts on extractable viral communities. Soil Biol Biochem. 194, 109419, doi: 10.1016/j.soilbio.2024.109419 (2024).

- Zhai, X. et al. The impact of storage buffer and storage conditions on fecal samples for bacteriophage infectivity and metavirome analyses. Microbiome 11 (1), 193, doi: 10.1186/s40168-023-01632-9 (2023).

- Trubl, G., Solonenko, N., Chittick, L., Solonenko, S.A., Rich, V.I., Sullivan, M.B. Optimization of viral resuspension methods for carbon-rich soils along a permafrost thaw gradient. PeerJ. 4, e1999 (2016).

- Roux, S. et al. Towards quantitative viromics for both double-stranded and single-stranded DNA viruses. PeerJ. 4, e2777, doi: 10.7717/peerj.2777 (2016).

- Kim, K.H., Bae, J.W. Amplification methods bias metagenomic libraries of uncultured single-stranded and double-stranded DNA viruses. Appl Environ Microbiol. 77 (21), 7663–7668, doi: 10.1128/AEM.00289-11 (2011).

- ter Horst, A.M. et al. Minnesota peat viromes reveal terrestrial and aquatic niche partitioning for local and global viral populations (vol 9, 233, 2021). Microbiome 9 (1), doi: 10.1186/s40168-021-01210-x (2021).

- Sorensen, J., Zinke, L., ter Horst, A., Santos-Medellín, C., Schroeder, A., Emerson, J. DNase Treatment Improves Viral Enrichment in Agricultural Soil Viromes. mSystems. 6 (5), e00614-21, doi: 10.1128/msystems.00614-21 (2021).

- Santos-Medellín, C., Estera-Molina, K., Yuan, M., Pett-Ridge, J., Firestone, M.K., Emerson, J.B. Spatial turnover of soil viral populations and genotypes overlain by cohesive responses to moisture in grasslands. Proc Natl Acad Sci U S A 119 (45), e2209132119, doi: 10.1073/pnas.2209132119 (2022).

- Emerson, J.B., Geonczy, S.E., Santos-Medellín, C., ter Horst, A.M., Fudyma, J.D. Soil Viromics Protocol - Emerson Lab v1. Protocols.io (2022).

- Göller, P.C., Haro-Moreno, J.M., Rodriguez-Valera, F., Loessner, M.J., Gómez-Sanz, E. Uncovering a hidden diversity: optimized protocols for the extraction of dsDNA bacteriophages from soil. Microbiome 8 (1), 17, doi: 10.1186/s40168-020-0795-2 (2020).

- Larsen, F. SM buffer. Protocols.io (2021).

- Camargo, A.P. et al. Identification of mobile genetic elements with geNomad. Nat Biotechnol (2023).

- Santos-Medellín, C., Blazewicz, S.J., Pett-Ridge, J., Firestone, M.K., Emerson, J.B. Viral but not bacterial community successional patterns reflect extreme turnover shortly after rewetting dry soils. Nat Ecol Evol. 7 (11), 1809–1822, doi: 10.1038/s41559-023-02207-5 (2023).

- Halary, S., Temmam, S., Raoult, D., Desnues, C. Viral metagenomics: are we missing the giants? Curr Opin Microbiol. 31, 34–43, doi: 10.1016/j.mib.2016.01.005 (2016).

- Williamson, S.J. et al. Metagenomic exploration of viruses throughout the Indian Ocean. PLoS One 7 (10), e42047, doi: 10.1371/journal.pone.0042047 (2012).

- Liang, G. et al. The stepwise assembly of the neonatal virome is modulated by breastfeeding. Nature 581 (7809), 470–474, doi: 10.1038/s41586-020-2192-1 (2020).

- Hillary, L.S., Knotts, T.A., Adams, S.H., Ali, M.R., Olm, M.R., Emerson, J.B. DNA extraction and virome processing methods strongly influence recovered human gut viral community characteristics. bioRxiv (2025).

- Santos-Medellín, C., Zinke, L.A., ter Horst, A.M., Gelardi, D.L., Parikh, S.J., Emerson, J.B. Viromes outperform total metagenomes in revealing the spatiotemporal patterns of agricultural soil viral communities. ISME J 15 (7), 1956–1970, doi: 10.1038/s41396-021-00897-y (2021).

- Gregory, A.C., Zablocki, O., Zayed, A.A., Howell, A., Bolduc, B., Sullivan, M.B. The Gut Virome Database Reveals Age-Dependent Patterns of Virome Diversity in the Human Gut. Cell Host Microbe 28 (5), 724–740, doi: 10.1016/j.chom.2020.08.003 (2020).

- Yilmaz, S., Allgaier, M., Hugenholtz, P. Multiple displacement amplification compromises quantitative analysis of metagenomes. Nat Methods 7 (12), 943–944 (2010).

- Kosmopoulos, J.C., Klier, K.M., Langwig, M.V., Tran, P.Q., Anantharaman, K. Viromes vs. mixed community metagenomes: choice of method dictates interpretation of viral community ecology. Microbiome 12 (1), 195, doi: 10.1186/s40168-024-01905-x (2024).

- Zhai, X., Gobbi, A., Kot, W., Krych, L., Nielsen, D.S., Deng, L. A single-stranded based library preparation method for virome characterization. Microbiome 12 (1), 219, doi: 10.1186/s40168-024-01935-5 (2024).

- Hillary, L.S., Adriaenssens, E.M., Jones, D.L., McDonald, J.E. RNA-viromics reveals diverse communities of soil RNA viruses with the potential to affect grassland ecosystems across multiple trophic levels. ISME Commun. 2, 34, doi: 10.1038/s43705-022-00110-x (2022)