# Submission ID #: 70202

# Scriptwriter Name: Pallavi Sharma

# Project Page Link:  https://review.jove.com/account/file-uploader?src=21411658

# Title: Long-Read Plasmid Sequencing and Assembly using Nanopore Sequencing-Based Workflows

# Authors and Affiliations:

Gerardo Cortés-Cortés*, Caison Warner*, Juliet Ellenbach, Samara Katz, Jay Kim, and Manel Camps

Department of Microbiology and Environmental Toxicology, University of California, Santa Cruz

*These authors contributed equally.

# Corresponding Authors:

Manel Camps 			(mcamps@ucsc.edu)

# Email Addresses for All Authors:

Gerardo Cortés-Cortés 	(gccortes@ucsc.edu)

Caison Warner 		(cpwarner@ucsc.edu)

Juliet Ellenbach 		(jcstephe@ucsc.edu)

Samara Katz 			(Saakatz@ucsc.edu)

Jay Kim 			(jaykim@soe.ucsc.edu)

Manel Camps 			(mcamps@ucsc.edu)

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes

3. Filming location: Will the filming need to take place in multiple locations?   No

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. No

Current Protocol Length

Number of Steps:  35

Number of Shots:  58

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Caison Warner: We investigate the replication, maintenance, and dissemination of self-replicating circular DNA elements, known as plasmids, which necessitates the determination of their sequences.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera Suggested B roll: 2.3.1

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Caison Warner: This protocol addresses coverage and accuracy issues associated with long-read sequences by enriching plasmid DNA and optimising assembly methods.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the research contexts or systems in which this protocol can be applied.

  - Caison Warner: Our protocol is designed to sequence plasmids in situations where the chromosomal sequence is either already known or irrelevant.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera Suggested B roll: 4.6.2

CONCLUSION:

Describe the most important consideration or challenge when performing this protocol.

  - Caison Warner: The most important consideration is whether the chromosomal sequence is of interest; if so, standard whole-genome sequencing protocols are better.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Caison Warner: Our protocol’s increased coverage opens the door to the study of heteroplasmy, i.e. plasmid sequence variation within a given population.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

Videographer: Please always record an audio slate while filming each shot mentioned in the shotlist.

- Plasmid DNA Extraction from Escherichia coli

Demonstrator: Juliet Ellenbach

  - To begin, streak the samples onto LB agar plates supplemented with the appropriate antibiotic to select for the respective plasmid-borne resistance marker [1]. Incubate the plates at 37 degrees Celsius for 24 hours [2].

    - WIDE: Talent streaking samples onto Luria-Bertani agar plates supplemented with antibiotics.

    - Talent placing the plates in the incubator.

  - The next day, inoculate 3 to 5 millilitres of LB medium supplemented with the appropriate antibiotic with a single isolated colony [1]. Incubate the culture with shaking at 37 degrees Celsius for approximately 8 hours, or until the optical density at 600 nanometers reaches 1 to 2 [2].

    - Talent inoculating 3 to 5 milliliters of Luria-Bertani medium with a single isolated colony.

    - Talent placing the culture into a shaking incubator.

  - Then, inoculate 400 millilitres of LB medium supplemented with the appropriate antibiotic with 400 microliters of the starter culture and incubate as demonstrated earlier [1].

    - Talent inoculating 400 milliliters of Luria-Bertani medium with 400 microliters of starter culture.

  - Then, transfer the culture into centrifuge bottles appropriate for the culture volume [1]. Centrifuge at 4200 g for 10 minutes [2] and resuspend the pellet using the reagent volumes recommended by the manufacturer [3-TXT].

    - Talent transferring the culture into centrifuge bottles appropriate for the culture volume.

    - Talent placing the centrifuge bottles into the centrifuge.

    - Talent resuspending the pellet with the recommended reagent volume. TXT: 
Vol. mL=Culture Volume mL ×OD60050

  - Following cell lysis and neutralisation, centrifuge the lysate at 4200 g for 30 minutes [1]. Transfer the supernatant to a new tube and centrifuge it again [2-TXT].

    - Talent placing the lysate into the centrifuge.

    - Talent transferring the supernatant to a new tube.

  - Perform equilibration, washing, and DNA elution according to the manufacturer’s instructions [1].

    - Talent at the working bench with the components required for washing in the frame.

  - Then, resuspend the precipitated DNA in 100 microliters of nuclease-free water to maximize concentration for Nanopore library preparation [1]. Next, prepare the assay tubes as per the presented table [2].

    - Talent adding nuclease-free water to the DNA pellet and resuspending.

    - LAB MEDIA: Table 1

  - Vortex each tube for a few seconds and incubate the tubes at room temperature for 2 minutes [1].  NOTE: This step is not filmed

    - Talent vortexing assay tubes and placing the tubes on the bench for incubation

  - After calibrating the fluorometer, insert the sample tubes into the fluorometer [1] and record the readings according to the instrument manual [2].

    - Talent inserting sample tubes into the fluorometer.

    - Talent recording readings from the fluorometer display.

- Library Preparation for Plasmid Sequencing

  - Program the thermal cycler to 30 degrees Celsius for 2 minutes, followed by 80 degrees Celsius for 2 minutes [1].

    - Show the thermal cycler interface while entering 30 degrees Celsius for 2 minutes and 80 degrees Celsius for 2 minutes.

  - Spin and mix the thawed kit components as per the presented table [1].

    - LAB MEDIA: Table 2

  - Incubate the PCR tubes prepared for barcording using the programmed thermal cycler [1]. Then, place the tubes on ice to cool [2].

    - Talent placing PCR tubes into the thermal cycler.

AND 
LAB MEDIA: Table 3   Video editor: Use split screen and show this information

    - Talent placing tubes on ice.

  - Pool the barcoded samples in a clean 1.5-millilitre Eppendorf tube, ensuring the total volume does not exceed 1000 microliters [1].

    - Talent transferring barcoded samples into a 1.5 millilitre Eppendorf tube.

  - Once the beads are vortexed, add an equal volume of beads to the pooled barcoded samples, mix by flicking the tube followed by a 10-minute incubation [1]. Incubate the tubes at room temperature for 10 minutes on a Hula mixer [2]. NOTE: The VO for the deleted shot I added concisely in the previous shot

    - Talent adding beads to the pooled sample and flicking the tube to mix.

    - WIDE: Talent placing tubes onto a Hula mixer for incubation at room temperature. NOTE: Not filmed

  - Then, wash the beads with 1 millilitre of 80 percent ethanol while on the magnet [1]. Remove the ethanol and repeat once [2].

    - Talent adding 1 milliliter of 80 percent ethanol to the beads on the magnet.

    - Talent removing ethanol from the tube while on the magnet.

  - Remove the tube from the magnet [1], resuspend the beads in elution buffer using the appropriate volume based on the number of barcodes in use and incubate for 10 minutes at room temperature [2].

    - Talent removing the tube from the magnetic rack.

    - Talent resuspending beads in elution buffer and keeping it aside on the bench.

  - Quantify the DNA concentration using 1 microliter of the eluted sample on the fluorometer to confirm successful library preparation [1].

    - Talent loading 1 microliter of sample into the fluorometer for measurement.

  - For adapter ligation, transfer 11 microliters of the DNA library into a new tube and label it [1].

    - Talent transferring 11 microliters of DNA library into a new tube and labeling the tube.

  - Then, mix the presented components in a fresh tube to dilute the thawed Rapid Adapter [1].

    - LAB MEDIA: Table 5

  - Next, add 1 microliter of diluted Rapid Adapter to the 11 microliters of DNA library and flick the tube to mix the contents [1]. Spin down the tube briefly to ensure the complete volume collects at the bottom, and incubate the tube on the bench for 5 minutes [2]

    - Talent adding diluted Rapid Adapter to the DNA library tube and flicking the tube.

    - Talent removes the tube from the centrifuge and keeps it on the bench.

- Priming and Loading of the Flow Cell

Demonstrator: Caison Warner

  - Prepare the flow cell priming mix in a fresh DNA tube and mix by inversion and pipetting [1].

    - Talent adding reagents into a fresh DNA tube

AND
LAB MEDIA: Table 6 Video editor: Use split screen and show this information

  - Lift the lid of the device [1] and insert the flow cell into it while applying gentle pressure to ensure proper placement [2].

    - Talent lifting the lid of the device.

    - Talent inserting the flow cell into the device.

  - After checking for air bubbles within or adjacent to the priming port, draw back 20 to 30 microliters of fluid to remove bubbles [1].

    - Talent withdrawing 20 to 30 microliters of fluid from the priming port.

  - Then, draw up 800 microliters of priming mix and dispense it into the priming port while avoiding air bubbles, and let the device sit for 5 minutes [1].

    - Talent drawing 800 microliters of priming mix into a pipette and dispensing the priming mix into the priming port.

  - Mix the DNA library with the appropriate volume of library beads in a new tube as presented here [1].

    - LAB MEDIA: Table 7

  - Load 200 microliters of priming mix into the flow cell priming port [1]. Adjust the sample port cover to reveal the port [2].  Dispense 75 microliters of the DNA library with beads into the sample port one drop at a time, allowing each drop to absorb before adding the next [3]. NOTE: The VO is edited  for the inverted shots

4.6.2  Talent loading 200 microliters of priming mix into the priming port. NOTE: 4.6.1 and 4.6.2 are inverted

    - Talent adjusting the sample port cover to open the port.

    - Talent carefully dispensing the DNA library with beads into the sample port dropwise.

  - Then, reapply the sample port cover and close the priming port [1].

    - Talent placing the sample port cover back onto the device and closing the priming port.

  - Once the device lid is closed, begin the sequencing run [1]. Set the run time limit based on the number of plasmids being multiplexed, with a default of 48 hours and a maximum of 72 hours [2].

    - SCREEN: SCREEN_4_8_1.mp4; 00:06-00:09

    - SCREEN: SCREEN_4_8_2.mp4;  00:20-00:35

  - Select the super-accurate basecalling mode and the appropriate rapid-barcode kit option [1].

    - SCREEN: SCREEN_4.9.1_AND_4_10_1_AND_4_10_2.mp4: 00:07-00:25

  - Then, select the raw FAST5 (Fast-Five) or POD5 (POD-Five) data [1] and start basecalling [1].

    - SCREEN: SCREEN_4.9.1_AND_4_10_1_AND_4_10_2.mp4:  00:32-00:36

    - SCREEN: SCREEN_4.9.1_AND_4_10_1_AND_4_10_2.mp4:  00:38-00:40

  - To generate a filtered FASTQ file for downstream assembly, trim long reads using Filtlong to remove approximately 5 to 10 percent of the lowest-quality reads while retaining approximately 90 percent of the total yield [1].

    - SCREEN: 4_11_1_AND_4_11_2.mp4

  - Subsample reads using Autocycler according to recommended settings [1]. Assemble each read subset using Raven, Flye, Canu, miniasm, Myloasm, NECAT, Plassembler, and other selected assemblers [2]. Record contig lengths and circularisation status for each assembly. Prioritise assemblies reporting circularisation for plasmid reconstruction [3].

    - SCREEN: 4_12_1.mp4;  00:00-00:05, 00:36-00:40

    - SCREEN: 4_12_1.mp4: 00:43-01:00

    - SCREEN: Show reviewing contig lengths and circularization status in assembly outputs. NOTE: Not filmed

  - Visualize each assembly graph file using Bandage. Evaluate contigs for circularity, size consistency across assemblers, and evidence of chimeric joints [1]. Remove linear contigs lacking support across multiple assemblers and retain circular contigs with consistent size estimates [2]. Remove minor unsupported contigs before consensus generation when major plasmids are circularized. Proceed to consensus generation after manual curation [3].

    - SCREEN: 4_13_1_REUPLOAD.mp4_fixed: 00:06-00:30.

    - SCREEN: 4_13_2_REUPLOAD.mp4_fixed: 00:00-00:38

    - SCREEN: 4_13_3_REUPLOAD.mp4_fixed

  - Generate the consensus sequence using Autocycler with the curated assemblies [1]. Execute the remaining Autocycler commands to produce the final consensus plasmid sequence [2].

    - SCREEN: 4_14_1_AND_4_14_2.mp4:  00:08-00:24.

    - SCREEN: 4_14_1_AND_4_14_2.mp4:  00:32-00:45

  - At last, polish the final assembly using trimmed long reads with Medaka to improve consensus accuracy [1].

    - SCREEN: 4_15_1.mp4: 00:00-00:20.

# Results

- Results

  - All conjugation assays produced transconjugants, confirming the presence of a conjugative plasmid in the donors [1]. The sizes of assembled plasmid sequences were congruent with estimates based on pulse-field gel electrophoresis mobility [2].

    - LAB MEDIA: Table 8. Video editor: Show the table listing donor strains and plasmids to indicate the presence of conjugative plasmids across samples

    - LAB MEDIA: Table 8. Video editor: Highlight the column “Estimated PFGE Size” and “Assembly Length”

  - Nanopore sequencing produced sufficient read output and quality for all samples analyzed [1]. Chromosomal contamination rates ranged between 72.1 percent and 28.0 percent with an average of 46.9 percent [2].

    - LAB MEDIA: Table 9.

    - LAB MEDIA: Table 10. Video editor: Highlight the column “% Chromosomal Reads”

  - Pulse-field gel electrophoresis analysis of strain pUPEC271 (plasmid-U-PEC-Two-Seven-One) revealed a band corresponding to a plasmid of approximately 145 kilobase pairs [1]. DNA from one of the transconjugants showed a sequence identical to the 145 kilobases [2].

    - LAB MEDIA: Figure 3A and B. Video editor: Highlight the band near 145 kilobases in the donor lane in 3B

    - LAB MEDIA: Figure 3A and B. Video editor: Highlight the corresponding band in the transconjugant lane in 3B

  - Pulse-field gel electrophoresis analysis of strain Blood_10_0913 (Blood-Ten-Zero-Nine-One-Three) identified two plasmids of approximately 70 and 110 kilobase pairs [1].

    - LAB MEDIA: Figure 4A. Video editor: Highlight the two bands near 70 kilobases and 110 kilobases indicated by arrows

  - Nanopore sequencing confirmed two plasmid assemblies of 71,182 base pairs [1] and 111,226 base pairs [2].

    - LAB MEDIA: Figure 4B. Video editor: Zoom into the circular map labeled “pBlood_10_0913_1 71,182 bp”

    - LAB MEDIA: Figure 4B. Video editor: Zoom into the circular map labeled “pBlood_10_0913_2 111,226 bp”

- Luria-Bertani
Pronunciation link: https://www.merriam-webster.com/dictionary/Luria-Bertani
IPA: /ˈlʊr.i.ə bərˈtɑː.ni/
Phonetic Spelling: LOOR·ee·uh ber·TAA·nee

- Plasmid
Pronunciation link: https://www.merriam-webster.com/dictionary/plasmid
IPA: /ˈplæz.mɪd/
Phonetic Spelling: PLAZ·mid

- Antibiotic
Pronunciation link: https://www.merriam-webster.com/dictionary/antibiotic
IPA: /ˌæn.ti.baɪˈɑː.tɪk/
Phonetic Spelling: an·tee·by·AA·tik

- Incubate
Pronunciation link: https://www.merriam-webster.com/dictionary/incubate
IPA: /ˈɪŋ.kjəˌbeɪt/
Phonetic Spelling: ING·kyuh·bayt

- Inoculate
Pronunciation link: https://www.merriam-webster.com/dictionary/inoculate
IPA: /ɪˈnɑː.kjəˌleɪt/
Phonetic Spelling: ih·NAA·kyuh·layt

- Centrifuge
Pronunciation link: https://www.merriam-webster.com/dictionary/centrifuge
IPA: /ˈsen.trəˌfjuːdʒ/
Phonetic Spelling: SEN·truh·fyooj

- Supernatant
Pronunciation link: https://www.merriam-webster.com/dictionary/supernatant
IPA: /ˌsuː.pərˈneɪ.tənt/
Phonetic Spelling: soo·per·NAY·tuhnt

- Resuspension
Pronunciation link: https://www.merriam-webster.com/dictionary/resuspension
IPA: /ˌriː.səˈspen.ʃən/
Phonetic Spelling: ree·suh·SPEN·shuhn

- Neutralization
Pronunciation link: https://www.merriam-webster.com/dictionary/neutralization
IPA: /ˌnuː.trə.ləˈzeɪ.ʃən/
Phonetic Spelling: noo·truh·luh·ZAY·shuhn

- Equilibration
Pronunciation link: https://www.merriam-webster.com/dictionary/equilibration
IPA: /ɪˌkwiː.ləˈbreɪ.ʃən/
Phonetic Spelling: ih·KWEE·luh·BRAY·shuhn

- Elution
Pronunciation link: https://www.merriam-webster.com/dictionary/elution
IPA: /iˈluː.ʃən/
Phonetic Spelling: ee·LOO·shuhn

- Conjugation
Pronunciation link: https://www.merriam-webster.com/dictionary/conjugation
IPA: /ˌkɑːn.dʒəˈɡeɪ.ʃən/
Phonetic Spelling: kaan·juh·GAY·shuhn

- Nuclease
Pronunciation link: https://www.merriam-webster.com/dictionary/nuclease
IPA: /ˈnuː.kliˌeɪs/
Phonetic Spelling: NOO·klee·ays

- Kilobase
Pronunciation link: https://www.merriam-webster.com/dictionary/kilobase
IPA: /ˈkɪl.əˌbeɪs/
Phonetic Spelling: KIL·uh·bays

- Circularization
Pronunciation link: https://www.merriam-webster.com/dictionary/circularization
IPA: /ˌsɝː.kjə.lɚ.əˈzeɪ.ʃən/
Phonetic Spelling: sur·kyuh·lur·uh·ZAY·shuhn

- Basecalling
Pronunciation link: No confirmed link found
IPA: /ˈbeɪsˌkɔː.lɪŋ/
Phonetic Spelling: bays·kaw·ling

- Reconstruction
Pronunciation link: https://www.merriam-webster.com/dictionary/reconstruction
IPA: /ˌriː.kənˈstrʌk.ʃən/
Phonetic Spelling: ree·kuhn·STRUK·shuhn

- Contamination
Pronunciation link: https://www.merriam-webster.com/dictionary/contamination
IPA: /kənˌtæm.əˈneɪ.ʃən/
Phonetic Spelling: kuhn·tam·uh·NAY·shuhn

- Ligation
Pronunciation link: https://www.merriam-webster.com/dictionary/ligation
IPA: /laɪˈɡeɪ.ʃən/
Phonetic Spelling: ly·GAY·shuhn

- Incubation
Pronunciation link: https://www.merriam-webster.com/dictionary/incubation
IPA: /ˌɪŋ.kjəˈbeɪ.ʃən/
Phonetic Spelling: ING·kyuh·BAY·shuhn