TITLE:

# A Workflow for Visualizing Protein Conformational Dynamics from Cryo-EM Maps Using Human Asparagine Synthetase

AUTHORS AND AFFILIATIONS:

Yuichiro Takagi

Department of Biochemistry, Molecular Biology and Pharmacology, Indiana University School of Medicine, Indianapolis, IN 46202, USA

E-mail address of the author:

Yuichiro Takagi	ytakagi@iu.edu

SUMMARY:

This protocol outlines how to perform 3D variability analysis (3DVA) in CryoSPARC and variability refinement in Phenix to analyze conformational heterogeneity in cryo-EM data. Using human asparagine synthetase as an example, it outlines how to generate variability maps, visualize principal components, and refine corresponding atomic models.

ABSTRACT:

This article presents a comprehensive workflow for analyzing conformational heterogeneity in single-particle cryo-electron microscopy (cryo-EM) data using 3D variability analysis (3DVA) in CryoSPARC and variability refinement in Phenix. The protocol describes how to compute variability maps along principal components derived from particle images, visualize principal modes of structural variation, and refine corresponding ensembles of atomic models against individual variability frames. The resulting outputs include a series of 3D variability maps, refined multi-model atomic structures representing discrete conformational states, and visualization files suitable for comparative structural analysis and movie generation. This workflow is broadly applicable to cryo-EM reconstructions in the near-atomic resolution range (typically ~2.5–4.5 Å) where conformational heterogeneity can be meaningfully interpreted at the backbone and side-chain levels. It is particularly suited for capturing continuous domain motions, hinge-bending movements, and coordinated local rearrangements that are not easily resolved by discrete classification alone. The combined use of variability maps and refined model ensembles enables direct structural comparison of conformational extremes, supports quantitative analyses such as RMSF calculations, and provides a framework for linking dynamic structural features to biochemical or functional hypotheses. Human asparagine synthetase (ASNS) is used as an example to demonstrate the practical implementation of the workflow.

INTRODUCTION:

Characterizing conformational heterogeneity in macromolecular structures is an increasingly important aspect of single-particle cryo-electron microscopy (cryo-EM) analysis. The goal of this protocol is to provide a practical workflow for extracting, visualizing, and refining dominant modes of structural variability directly from cryo-EM datasets using three-dimensional variability analysis (3DVA) in CryoSPARC1 combined with variability refinement in Phenix2. This article is intended to guide readers in determining whether this computational approach is appropriate for their data and in understanding the types of outputs generated by the workflow.

Traditional experimental techniques (e.g., hydrogen–deuterium exchange, NMR spectroscopy, mass spectrometry) provide valuable information on protein motions3–5 but are often limited by molecular size, sample requirements, or experimental complexity. Advances in cryo-EM have enabled high-resolution structure determination for large and heterogeneous systems, allowing analysis of conformational variability within particle populations. 3D variability analysis (3DVA), implemented in CryoSPARC1, decomposes cryo-EM particle datasets into principal components that describe dominant, continuous modes of structural variation. When combined with variability refinement in Phenix2, this approach produces atomic models corresponding to discrete positions along each principal component, enabling visualization and comparison of alternative conformations derived from experimental data.

This protocol is best suited for cryo-EM datasets reconstructed at near-atomic to intermediate resolution (typically ~3–6 Å). At higher resolutions, the workflow can capture backbone rearrangements and limited side-chain variability6, whereas at lower resolutions it is more appropriate for identifying domain-level or secondary-structure movements2,7. The method is suited to capture dominant, continuous modes of motion that are broadly populated in the dataset, such as hinge motions, domain breathing, or coordinated rearrangements between domains or subunits. Adequate particle numbers and angular coverage are essential, and interpretation may be limited in flexible or weakly resolved regions where side-chain positions remain ambiguous. Compared with focused classification8 or extensive subclassification strategies, the 3D variability refinement workflow represents conformational heterogeneity as continuous trajectories rather than discrete classes. This makes the method particularly useful for the exploratory analysis of enzymes, molecular machines, and multi-domain complexes that are expected to sample multiple conformations.

Human asparagine synthetase (ASNS) is used as an illustrative example to demonstrate the protocol's implementation. ASNS is a glutamine-dependent amidotransferase composed of two functional domains connected by an intramolecular ammonia tunnel9,10. Previous studies have suggested that conformational changes involving residues such as Arg-142 contribute to the modulation of this tunnel6. In this article, ASNS serves solely as a representative system for demonstrating how 3DVA and variability refinement generate variability maps, refined atomic model ensembles, and visualizations. Detailed mechanistic interpretation is deferred to the Results and Discussion sections. Although molecular dynamics (MD) simulations are often used as a complementary approach to explore conformational landscapes11,12, MD is not required for this protocol. Comparisons with MD simulations may be performed as an optional downstream analysis, but the workflow described here (Figure 1) focuses exclusively on extracting and visualizing conformational heterogeneity from cryo-EM data.

Place Figure 1 here

PROTOCOL:

- Data preparation

  - Generation of consensus EM map

    - Use single-particle cryo-electron microscopy (cryo-EM) to determine the consensus 3D structure of the macromolecular complex.

    - Vitrify samples on cryo-grids and image using a high-voltage transmission electron microscope equipped with a direct electron detector.

    - Process the collected movies to correct beam-induced motion, estimate imaging parameters, and select particle images.

    - Perform iterative classification and refinement of the particles to generate a consensus EM map.

    - Perform data processing using CryoSPARC (v4 or higher)13, since 3D variable analysis (3DVA) will be implemented.

  - Generate a consensus model

    - Build an atomic model and refine it against the consensus map. Validate the refined model quality using standard criteria

NOTE: For ASNS, the structure was determined using a template-based particle-picking strategy. Template derived from the X-ray structure of DON-modified human ASNS (PDB 6GQ3)9 was used to efficiently select high-quality particles in CryoSPARC13. These particles underwent iterative rounds of 2D classification, Ab initio reconstruction, and heterogeneous and non-uniform refinement with C2 symmetry imposed, yielding a 3.5 Å consensus map. The atomic model was generated by using the crystal structure9 into the EM map, followed by real-space refinement in Phenix14,15 and manual adjustment in Coot16, producing the final consensus ASNS structure as described6.

- Performing 3D variability analysis (3DVA)

  - Use the particle stack and mask (not a final refined map) for a final refinement

NOTE: Often, the results from a non-uniform refinement job will be used for 3DVA.

  - Create a 3D Variability job in CryoSPARC. Input particle stack and mask from the non-uniform refinement job.

  - Set Number of modes to solve: number of principal components (PCs) to 5.

NOTE: Because macromolecular structures exist in three-dimensional space, at least three PCs are generally required to describe their major spatial modes of motion. To ensure that additional, potentially smaller but functionally relevant conformational variations were not overlooked, two more PCs were included.

  - Set Filter resolution to 4.0 Å, and Highpass resolution to 20. Leave all other parameters as default. Start the job.

NOTE: “Filter resolution” of 4.0 Å was set to match the overall quality of the dataset while avoiding overfitting to high-frequency noise. Since the final consensus map was determined at 3.5 Å resolution, applying a slightly lower filter resolution (4.0 Å) ensured that the variability analysis focused on genuine structural movements rather than noise near the resolution limit.

  - Obtain a set of variability volumes as output (Figure 2A)

- Performing 3D variability display

  - Create a 3D Variability Display job in CryoSPARC to visualize the continuous conformational changes identified by the 3DVA.

  - Input the particle stack and the variability volumes generated from the 3DVA job.

  - Select Simple as output mode (see below).

  - Set Filter resolution to 4.0 and set Highpass resolution to 20. Leave all other parameters at their default values. Queue the job.

  - Obtain 20 variability maps as outcome (frame 0 to 19) corresponding to 5 Principal components (PC) from 0 to 4 (Figure 2B)

NOTE: The “simple mode” in the 3D Variability Display program provides a straightforward visualization of conformational changes along each principal component. This mode generates linear trajectories between the extreme states identified by 3DVA, enabling a clear, direct interpretation of the motions within the particle population.

- Data preparation for movies and Phenix 3D variable refinement

  - Download 3DVA variability maps:  Download the zip files containing variability maps for PCs 0–4 (five zips total) from CryoSPARC.

  - Organize directories: Create a parent folder (e.g., 3DVA/). Inside, create one subfolder for each principal component (comp0, comp1, comp2, comp3, comp4).

  - Move each zip into its corresponding folder and unzip the contents.

  - Confirm that each folder contains 20 variability maps (frames 0–19) and repeat for comp1–comp4.

- NOTE: Maintain separate folders for each principal component to prevent mixing frames across PCs.

- Prepare quick movies in UCSF Chimera (Optional)17

  - Open the Volume Series tool:  Launch Chimera, then navigate to Tools ▸ Volume Data ▸ Volume Series.  A new Volume Series window will open.

  - Load the maps: Click Open and select the 20 maps corresponding to one principal component (e.g., comp0).

  - Adjust visualization parameters: Open Volume Viewer, set the appropriate density threshold, and assign a color to the map.

- NOTE: In this example, sky blue was used (Figure 3).

  - Preview motion: In the Volume Series window, set Play direction to Oscillate, then click Play to display a continuous transition from frame 0 → 19 → 0.

  - Repeat for additional components: Repeat Steps 5.2–5.4 for other principal components as needed.

  - Generate a movie file using command-line input: Export a movie by opening the Chimera command line and running Supplementary Coding File 1.

NOTE: The per-component, 20-frame mrc series (e.g., compX_frame000–019.mrc) are the inputs for Phenix 3D variable refinement. Keep component folders separate (comp0–comp4) to avoid mixing frames across PCs.

### Model generation by variability refinement in phenix

NOTE: Phenix.varref can be executed from any directory as long as the full paths to the input maps and model are specified. However, for organizational clarity, it is recommended to perform the refinement in a dedicated subdirectory for each PC.

  - Copy the final refined consensus model file into the subdirectory corresponding to each PC.

  - From within each subdirectory, run Phenix.varref from the command line using the following syntax:

- phenix.varref consensus_model.pdb *frame_*.mrc resolution=4 nproc=20 models_per_map=50

  - Set Resolution to 4.0 Å to match the effective resolution of the 3DVA maps

  - Set nproc to the number of processors available (20 in this example, but this can vary depending on available computational resources)

  - Set models_per_map to 50 models (default).

NOTE: For each map, 50 models are generated with restraints adjusted to the resolution of the 3DVA data. The best-fitting model for each map is automatically retained. The resulting output file contains 20 refined models corresponding to the 20 variability maps. The output consists of a PDB bundle containing 20 refined models, along with individual map files.

  - Do not apply additional map sharpening during variability refinement. Use the variability maps generated by CryoSPARC (filtered at 4.0 Å) directly as inputs to phenix.varref.

### Visualization

### Open the PDB bundle in ChimeraX18 to load all 20 refined models simultaneously.

### Use the Models panel to selectively display individual frames or specific models of interest for comparison and analysis (Figure 4). Alternatively, each refined model can also be opened individually in ChimeraX or Chimera.

- Data preparation for analysis

NOTE: Running the syntax above generates a PDB bundle containing 20 variable models and each individual map. For most analyses, only two models—corresponding to frame 0 and frame 19—are required, as they represent the two extreme conformational states along the selected principal component (PC).

### Open the PDB bundle in ChimeraX, select model 1.1 (frame 0) and model 1.20 (frame 19), and save each model as a separate PDB file (Figure 4).

### Edit the resulting PDB files to isolate residues of interest. This can be done manually or using Phenix.pdb.tools.

### Compare the two modified PDB files to identify conformational changes in backbone or side chains.

### To compare the two PDB models, superimpose the modified structures using the MatchMaker function in Chimera.

### NOTE: For reliable alignment, MatchMaker typically requires at least 10–15 overlapping residues. In our case, only 6 residues were analyzed; however, successful superposition was achieved by leveraging the homodimeric nature of ASNS—while only 6 residues remained in chain A, chain B was intact and served as the reference for alignment.

RESULTS:

This section presents representative outputs generated by the 3D variability analysis (3DVA) and variability refinement workflow and provides guidance on their interpretation. Human asparagine synthetase (ASNS) is used as an illustrative dataset to demonstrate successful outcomes, common artifacts, and comparative analyses enabled by the protocol. The complete set of 3DVA-derived variability maps and the corresponding refined models generated from the ASNS datasets have been deposited in the Zenodo public repository. Detailed access information is provided in the Supplementary Table 1.

3DVA variability maps and variability movies.
The primary output of 3DVA is a set of variability maps describing dominant modes of structural variation1, visualized as ordered map series along each principal component (PC). For the ASNS dataset, map series corresponding to principal component 0 (PC0) were generated for both the wild-type (WT) and R142I variant datasets (Supplementary Videos 1 and 2). These movies provide a qualitative view of domain motions and coordinated structural rearrangements present in the particle population. These movies should be interpreted as representations of dominant, continuous variability rather than time-resolved trajectories.

Comparison of visualized movies between datasets (e.g., WT versus mutant) assumes comparable data quality, particle numbers, and processing parameters.  Yet it should be noted that the primary goal of the 3DVA-derived movies is to illustrate global, continuous conformational variability along a principal component. They are not intended to resolve or demonstrate specific side-chain rearrangements, particularly given the compressed and qualitative nature of movie representations. Residue-level analysis requires frame-by-frame inspection of the individual variability maps together with their corresponding variability-refined models.

Variability-refined atomic model ensembles.
Variability refinement in Phenix 2 produces ensembles of atomic models corresponding to each frame in a variability map. For each PC, refinement against 20 variability maps yields refined models representing discrete positions along the variability mode. Inspection of these ensembles allows identification of coordinated backbone movements and side-chain rearrangements supported by the maps. In the ASNS example, comparison of models corresponding to frames 0 and 19 highlights side-chain displacements of approximately 2–5 Å in well-resolved regions (Figure 5). These values should be interpreted as approximate structural differences rather than precise atomic coordinates.

The current implementation of phenix.varref does not automatically generate validation statistics for each variability map–model pair. Therefore, statistical validation must be performed manually if needed using standard Phenix validation tools. Because the largest conformational differences are typically observed at the extreme frames, validation analyses were conducted for frame 00 and frame 19 models of the ASNS datasets. These results are reported as representative validation of the variability refinement procedure. The corresponding validation statistics for the 3DVA-derived maps and refined models are provided in Supplementary Table 2.

RMSF profiles as quantitative descriptors of variability.
Per-residue root-mean-square fluctuation (RMSF) profiles calculated from variability-refined model ensembles using the CPPTRAJ software package19 provide a quantitative summary of residue-level variability (Figure 6). RMSF values of ~0.5–1.5 Å generally correspond to modest backbone flexibility, whereas higher values often reflect regions of poor local resolution or incomplete model definition. Apparent RMSF spikes, such as those observed for residues 207–209, 221–223, 464, and 478–479 in the ASNS dataset, arise from poorly defined neighboring density and serve as diagnostic indicators of map quality rather than biologically meaningful motions 6.

Comparative analyses and interpretation assumptions.
Comparisons between datasets rely on the use of equivalent processing pipelines, resolution cutoffs, and refinement parameters. Differences in variability patterns should be evaluated in the context of these assumptions and supported by consistent trends across multiple outputs, including variability movies, model ensembles, and RMSF profiles. Variability-derived metrics describe relative structural variability within the analyzed datasets and do not directly report on kinetics, energetics, or functional consequences.

Together, these outputs illustrate how the 3DVA–variability refinement workflow generates complementary qualitative and quantitative descriptors of conformational heterogeneity from cryo-EM data. By integrating visualization, model comparison, and residue-level fluctuation analysis, the protocol provides a structured framework for interpreting dominant modes of structural variability while distinguishing meaningful signals from artifacts inherent to experimental data.

FIGURE AND TABLE LEGENDS:

### Figure 1. Workflow overview. A consensus cryo-EM map is reconstructed from single-particle data and subjected to three-dimensional variability analysis (3DVA) to generate variability maps along principal components. Variability refinement (Phenix.varref) uses the consensus atomic model to generate corresponding atomic models for visualization and comparison.

### Figure 2. CryoSPARC outputs for variability analysis. (A) Completed 3D Variability Analysis job showing generated variability volumes (red circle). (B) Completed 3D Variability Display job showing variability maps for principal components PC0–PC4 (red rectangle).

### Figure 3. Visualization of variability maps in UCSF Chimera. Representative view of the Volume Series tool displaying sequential variability maps (frames 0–19) for principal component 0 (PC0), illustrating continuous structural variation along a single PCA axis.

### Figure 4. Visualization of variability-refined models in ChimeraX. Bundle of 20 variability-refined atomic models for principal component 0 (PC0) displayed in ChimeraX. Individual frames (Model 1.1–1.20) can be shown or hidden using the Model Panel. All models are shown in a uniform color.

Figure 5. Variability-derived conformations at PCA extremes. (A) Cartoon representation of the consensus ASNS structure (PDB: 8SUE) illustrating domain organization. The glutaminase domain (residues 1–209) is colored in forest green, and the synthetase domain (residues 211–539) is colored in sky blue. Arg-142 is shown as red spheres and is positioned at the interface between the two domains. (B) Tunnel-forming residues displayed in the consensus ASNS model, with Arg-142 highlighted for reference. The corresponding consensus EM map (EMD-40765) is shown as a mesh density. (C–G) Superposition of variability-refined models corresponding to frame 0 and frame 19 (extreme conformations along each principal component) for PCs 0–4. EM map density is displayed in mesh format. Qualitative descriptors (e.g., “open,” “closed”) refer to relative side-chain orientations determined by visual inspection of the refined atomic models in the context of the supporting EM density. For PC1, PC3, and PC4, the most significant conformational displacement of the Arg-142 side chain was quantified by measuring the distance between the terminal heavy atoms (NH1) in two PCA extremes. These distances (3.47 Å, 5.16 Å, and 2.17 Å, respectively) are indicated in the figure by magenta-dashed double lines. Data were previously reported6. This figure was generated by the author for this publication.

Figure 6. RMSF profile from variability-refined models. Normalized per-residue root-mean-square fluctuation (RMSF) values for ASNS (Chain A) calculated from 100 variability-refined models derived from PCs 0–4 after alignment to the consensus model. RMSF values were normalized to the maximum RMSF observed. Elevated RMSF values indicate regions of limited local structural definition. Data were previously reported6. This figure was generated by the author for this publication.

Supplementary Table 1: The complete set of 3DVA-derived variability maps and the corresponding refined models generated from the ASNS datasets are available in the Zenodo public repository. Detailed access information is provided in the file.

Supplementary Table 2: Validation analyses were conducted using Phenix validation tool for frame 00 and frame 19 models of the ASNS datasets. The validation statistics for the 3DVA-derived maps and refined models are provided.

### Supplementary Coding File 1: UCSF Chimera command-line script used to export movies from 3DVA-derived variability maps.

### Supplementary Video 1: Variability maps for wild-type ASNS along principal component 0 (PC0). The movie contains 20 frames (frames 0–19) and represents variation along a PCA axis, not time-resolved dynamics.

### Supplementary Video 2: Variability maps for ASNS R142I along principal component 0 (PC0). The movie contains 20 frames (frames 0–19) and represents variation along the same PCA axis used for the wild-type dataset.

DISCUSSION:

Critical protocol steps and decision points
Effective use of this protocol depends on several key steps in the workflow. The quality of the particle stack and mask used for final cryo-EM refinement is critical, as 3D variability analysis (3DVA) operates directly on these inputs rather than on a post-processed map. Selection of the number of principal components (PCs) should reflect dataset size and expected complexity; too few components may underrepresent variability, whereas too many may introduce noise-dominated modes. Resolution parameters used in both 3DVA and variability refinement should match the effective resolution of the data. Downstream choices, such as which frames to visualize or compare, should be guided by the structural features of interest.

Troubleshooting and common pitfalls
Several issues can complicate the interpretation of variability results. Apparent motions may arise from weak or poorly resolved density rather than meaningful conformational variation. Inspection of local map quality and comparison across multiple PCs can help distinguish signal from noise. Limited particle numbers, preferred orientation, or uneven angular coverage may result in poorly defined variability modes. Although validation statistics for each variability map–model pair can be generated manually using standard Phenix validation tools, these metrics alone are not sufficient. Careful manual inspection remains essential to ensure that interpreted motions are supported by well-resolved density and are structurally plausible.

Methodological limitations and interpretation boundaries
Interpretation of structural variability is inherently resolution dependent. Around 3.5 Å or better, side-chain conformations can generally be assigned with reasonable confidence when supported by clear and continuous density. In the ~3.5–4.0 Å range, backbone tracing and secondary-structure elements are typically reliable; however, side-chain positioning—particularly for flexible or solvent-exposed residues—must be interpreted cautiously and evaluated in the context of local density quality. At resolutions poorer than ~4.0 Å, interpretation is generally limited to secondary-structure features and domain-level movements2,7. Because the variability maps in this study were filtered at 4.0 Å, residue-level interpretations were made only when supported by well-defined density and are discussed within these resolution constraints.

Comparison with alternative techniques
Compared with focused classification8 or extensive subclassification, the 3DVA–variability refinement workflow represents conformational heterogeneity as continuous modes rather than discrete classes, reducing manual parameter tuning. Unlike molecular dynamics (MD) simulations, which explore conformational space using computational models, 3DVA is derived directly from experimental cryo-EM data and is constrained by data quality and sampling. These approaches are complementary: 3DVA identifies dominant experimentally supported modes of variability, while MD simulations may be used as an optional downstream tool. MD is not required for the application of this protocol.

Broader applications and future use cases
This workflow is applicable to cryo-EM studies of enzymes, molecular machines, and multi-domain assemblies exhibiting continuous conformational heterogeneity. It is particularly useful for systems involving large-scale domain movements, coordinated rearrangements, or ligand-dependent structural variability. Beyond enzymatic systems, the protocol can be applied to transporters, chaperones, and regulatory complexes to characterize dominant modes of structural variation. As cryo-EM datasets continue to increase in size and resolution, this approach provides a general framework for incorporating conformational variability analysis into routine structural workflows.

ACKNOWLEDGMENTS:

I thank members of the Takagi, Zhu, Vos, Chaptal, Alcaro, and Richards laboratories for their contributions to the publication on which this article is based. The author gratefully acknowledges the Indiana University School of Medicine Electron Microscopy Facility and the NIH/NIGMS (S10 OD028723) for supporting this work. Additional funding was provided by the Indiana University School of Medicine (Y.T.), in part by the NIGMS (R01GM111695 to Y.T.), and the American Cancer Society (DBG-23-1038947-01-IBCD to Y.T.).

DISCLOSURES:

The author has no conflict of interest to declare.

REFERENCES:

1	Punjani, A. & Fleet, D. J. 3D variability analysis: Resolving continuous flexibility and discrete heterogeneity from single particle cryo-EM. J Struct Biol. 213 (2), 107702, doi:10.1016/j.jsb.2021.107702, (2021).

2	Afonine, P. V. et al. Conformational space exploration of cryo-EM structures by variability refinement. Biochim Biophys Acta Biomembr. 1865 (4), 184133, doi:10.1016/j.bbamem.2023.184133, (2023).

3	Schwartz, S. D. Protein Dynamics and Enzymatic Catalysis. J Phys Chem B. 127 (12), 2649–2660, doi:10.1021/acs.jpcb.3c00477, (2023).

4	Xu, D., Meisburger, S. P. & Ando, N. Correlated Motions in Structural Biology. Biochemistry. 60 (30), 2331–2340, doi:10.1021/acs.biochem.1c00420, (2021).

5	Gao, S. & Klinman, J. P. Functional roles of enzyme dynamics in accelerating active site chemistry: Emerging techniques and changing concepts. Curr Opin Struct Biol. 75, 102434, doi:10.1016/j.sbi.2022.102434, (2022).

6	Coricello, A. et al. 3D variability analysis reveals a hidden conformational change controlling ammonia transport in human asparagine synthetase. Nat Commun. 15 (1), 10538, doi:10.1038/s41467-024-54912-9, (2024).

7	Gobet, A. et al. Rhodamine6G and Hoechst33342 narrow BmrA conformational spectrum for a more efficient use of ATP. Nat Commun. 16 (1), 1745, doi:10.1038/s41467-025-56849-z, (2025).

8	Barchet, C. et al. Focused classifications and refinements in high-resolution single particle cryo-EM analysis. J Struct Biol. 215 (4), 108015, doi:10.1016/j.jsb.2023.108015, (2023).

9	Zhu, W. et al. High-resolution crystal structure of human asparagine synthetase enables analysis of inhibitor binding and selectivity. Commun Biol. 2, 345, doi:10.1038/s42003-019-0587-z, (2019).

10	Richards, N. G. & Schuster, S. M. Mechanistic issues in asparagine synthetase catalysis. Adv Enzymol Relat Areas Mol Biol. 72, 145–198, doi:10.1002/9780470123188.ch5, (1998).

11	Matsumoto, S., Ishida, S., Terayama, K. & Okuno, Y. Quantitative analysis of protein dynamics using a deep learning technique combined with experimental cryo-EM density data and MD simulations. Biophys Physicobiol. 20 (2), e200022, doi:10.2142/biophysico.bppb-v20.0022, (2023).

12	Vant, J. W. et al. Exploring cryo-electron microscopy with molecular dynamics. Biochem Soc Trans. 50 (1), 569–581, doi:10.1042/BST20210485, (2022).

13	Punjani, A., Rubinstein, J. L., Fleet, D. J. & Brubaker, M. A. cryoSPARC: algorithms for rapid unsupervised cryo-EM structure determination. Nat Methods. 14 (3), 290–296, doi:10.1038/nmeth.4169, (2017).

14	Adams, P. et al. PHENIX: a comprehensive Python-based system for macromolecular structure solution. Acta Crystallogr D Biol Crystallogr. 66, 213–221 (2010).

15	Afonine, P. V. et al. New tools for the analysis and validation of cryo-EM maps and atomic models. Acta Crystallogr D Struct Biol. 74 (Pt 9), 814–840, doi:10.1107/S2059798318009324, (2018).

16	Emsley, P. & Cowtan, K. Coot: model-building tools for molecular graphics. Acta Crystallogr D Biol Crystallogr. 60 (Pt 12 Pt 1), 2126–2132 (2004).

17	Pettersen, E. et al. UCSF Chimera--a visualization system for exploratory research and analysis. J Comput Chem. 25, 1605–1612 (2004).

18	Pettersen, E. F. et al. UCSF ChimeraX: Structure visualization for researchers, educators, and developers. Protein Sci. 30 (1), 70–82, doi:10.1002/pro.3943, (2021).

19	Roe, D. R. & Cheatham, T. E., 3rd. PTRAJ and CPPTRAJ: Software for Processing and Analysis of Molecular Dynamics Trajectory Data. J Chem Theory Comput. 9 (7), 3084–3095, doi:10.1021/ct400341p, (2013).