# Submission ID #: 70334

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=21451918

# Title: Environmental DNA Sampling from Whale-Watching Vessels for Cetacean Monitoring

# Authors and Affiliations:

# Belén G. Ovide1, Lauren K. Rodriguez2, Eleonora Barbaccia3, Rebecca Niklas2, Philipp Hobmeier2, Anna G. Iacono4, Emer Leahy5, Enrico Villa5, Caterina Lanfredi6, Arianna Azzellino3, Marianne H. Rasmussen1, Bettina Thalinger 2,7

# 1 Húsavík Research Center, University of Iceland

# 2 Applied Animal Ecology Research Unit, Department of Zoology, University of Innsbruck

# 3 Politecnico di Milano DICA Civil and Environmental Engineering Department

# 4 Department of Life and Environmental Science DISVA, Marche Polytechnic University

# 5  Cetacean Watching Lda

# 6  Tethys Research Institute 
7   WasserCluster Lunz - Biologische Station GmbH

# Corresponding Authors:

# Bettina Thalinger 	bettina.thalinger@uibk.ac.at

# Email Addresses for All Authors:

# Belén García Ovide 		bgo4@hi.is

# Lauren Kelly Rodriguez 	lauren.rodriguez@uibk.ac.at

# Eleonora Barbaccia 		eleonora.barbaccia@polimi.it

# Rebecca Niklas 		rebecca.niklas99@gmail.com

# Philipp Hobmeier 		philipp.hobmeier@student.uibk.ac.at

# Anna Gaia Iacono 		iaconoannagaia@gmail.com

# Emer Leahy 			emerleahyy@gmail.com

# Enrico Villa 			enrico.villa@cwazores.com

# Caterina Lanfredi 		lanfredicaterina@gmail.com

# Arianna Azzellino 		arianna.azzellino@polimi.it

# Marianne Helene Rasmussen 	mhr@hi.is

## Author Questionnaire

1. We have marked your project as author-provided footage, meaning you film the video yourself and provide JoVE with the footage to edit. JoVE will not send the videographer. Please confirm that this is correct.

√ Correct

2. Interview statements: Which interview statement filming option is the most appropriate for your group? Please select one.

☒ 	Interviewees self-record interview statements. JoVE can provide support for this option.

3. Proposed interview filming date: Please indicate the proposed date that your group will self-film interviews: 08/04/2026

When you are ready to submit your video files, please contact our Content Manager, Utkarsh Khare.

Current Protocol Length

Number of Steps:  25

Number of Shots:  48

# Introduction

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context.

We have chosen 5 best answers

INTRODUCTION:

Author’s NOTE: I made a single video for 1.1, 1.2 and 1.3 - A001_04081827_C107_intro

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Belén García Ovide: This protocol provides a standardized approach to whale eDNA sampling during whale watching cruises.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Belén García Ovide: Many different eDNA sampling methods are available. This hinders comparative data analysis. Our protocol has been optimized across environmental conditions and safeguards against contamination.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the research contexts or systems in which this protocol can be applied.

  - Belén García Ovide: With the support of an eDNA-ready laboratory, this protocol can be carried out by non-experts and citizen scientist during whale-watching activities around the globe.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Belén García Ovide: This protocol enables the non-invasive detection and monitoring of cetaceans and co-occurring taxa via eDNA contained in flukeprints collected from whale-watching vessels.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera FILE: A001_04081836_C112_conclusion_1

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Belén García Ovide: This protocol can be used for future large-scale eDNA sampling campaigns to investigate marine biodiversity or whale population genetics.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera A001_04081836_C112(1)_conclusion_2

# Protocol

- Equipment Preparation and Sterilization

Demonstrators: Belén García Ovide and Anna Gaia Iacono

  - To begin, prepare field documentation materials [1], including the field data sheets and the laminated step-by-step field protocol for cetacean environmental DNA sampling [2].

    - WIDE: Talent standing in the lab and looking at the field document.

    - LAB MEDIA:  1.1. Preparing docs. 00:00-00:04

  - Put spare gloves, spare filters with one filter per sample replicate, a waterproof marker, and a dedicated waste container in the box [1].

    - LAB MEDIA:  1.2 Preparing box.MOV Video editor: Please show the initial 5 seconds and the last 5 seconds

  - Prepare one or more rigid buckets suitable for the vessel type and onboard configuration [1], a secondary environmental DNA container [2] and a portable peristaltic or vacuum water pump [3].

    - LAB MEDIA:  1.3 Preparing buckets.MOV

    - LAB MEDIA:  1.3.1 Preparing additional eDNA container.MOV 00:10–00:20

    - LAB MEDIA:  1.4 preparing pump.

  - Wear gloves during all equipment handling and change gloves between tasks [1].

    - LAB MEDIA:  1.5 Putting gloves_caution.MOV

  - To sterilize the equipment, apply a diluted household bleach solution by splashing evenly across all surfaces of the buckets, lids, containers, and pump tubing [1].

    - LAB MEDIA:  2.4 rinsing bucket, 2.3 Cleaning container; 2.3 Cleaning tubes.MOV Use first few seconds in all these footage.

  - Rinse thoroughly with tap water three times [1].

    - LAB MEDIA:  2.4 Rinsing container

- Flukeprint Water Collection

  - Bring all the materials onboard before passengers arrive [1] and store them in a designated clean area [2].

    - LAB MEDIA:  3.1 Bringing materials onboard.

    - LAB MEDIA: 3.1 SamplingMaterials on zodiac

  - Coordinate sampling timing and positioning of the vessel with the vessel crew [1]. Observe surfacing patterns on the water to anticipate whale dives [2].

    - LAB MEDIA:  3.2 GX010109.MOV;00:15–00:25

    - LAB MEDIA:  3.3 GX010158.MOV

  - Collect 20 liters of seawater from the flukeprint as soon as possible, ensuring no more than a 10 minute delay after the whale dives [1]. Generate two 10 liter sample replicates [2]. Obtain samples from various regions using customized logistical conditions appropriate to each location [3].

    - LAB MEDIA:  3.4 GX010159.MOV

    - LAB MEDIA:  3.4 GX010166.MOV 00:00-00:10

    - LAB MEDIA: 3.4. water collection from zodiak azores.MOV  and  3.4. Water collection from motorsailor Italy.MOV. 00:00-00:20 Video editor: Play the 2 videos side by side using split screen

  - Now, transport the collected water to a designated filtration station on board, avoiding spillage [1].

    - LAB MEDIA:  3.5 GX010167.MOV

  - Record species identity, geographic positioning system coordinates, time, and sample replicate identification in the data sheet [1]. Change gloves immediately after reporting [2].

    - LAB MEDIA:  3.6 GX010124.MOV

    - LAB MEDIA:  3.7 GX010168.MOV

- Environmental DNA Filtration Onboard

  - Open the filter bag and handle the filter only by the outlet [1]. Insert the inlet into the snorkel [2].

    - LAB MEDIA:  4.1 GX010129.MOV 00:00-00:06

    - LAB MEDIA:  4.1 GX010129.MOV 00:312–00:20

  - Then, connect the pump tubing to the outlet [1].

    - LAB MEDIA:  4.2 GX010133.MOV 00:10–00:25

  - Submerge the snorkel in the water sample without submerging the filter housing [1] and set the pump to a fast, steady flow rate [2].

    - LAB MEDIA:  4.2 GX010133.MOV 00:25–00:30

    - LAB MEDIA:  4.3;4.4 GX010138.MOV 00:30–00:40

  - Filter 10 liters continuously while monitoring flow stability [1]. Next, invert the filter and run the pump for 30 to 60 seconds to remove residual water [2]. Customise the filtering pump setup according to the field conditions [3].

    - LAB MEDIA:  4.5 GX010141.MOV

    - LAB MEDIA:  4.6 GX010145.MOV 00:20–00:30

    - LAB MEDIA: 4.5 Filtering set-up Azores.MOV and  4.5 Filtering set up in Italy,(2) .MOV   Video editor: Play the 2 videos side by side using split screen Authors: I have added this as per your suggestion

  - Switch off the pump and disconnect the tubing [1]. Place the filter in its original bag [2] and label it immediately [3]. Change gloves and prepare to perform filtration for the second replicate [2].

    - LAB MEDIA:  4.7 GX010149.MOV 00:00–00:08

    - LAB MEDIA:  4.7 GX010149.MOV 00:20–00:30

    - LAB MEDIA:  4.8 GX010171.MOV

    - LAB MEDIA:  4.10 GX010168.MOV

- Filter Processing and Lysis

  - Filter one field negative control after every five flukeprint samples. Use target DNA-free water immediately after returning to shore and before equipment cleaning [1]. Store self-preserving environmental DNA filters in a safe, dry container in a shaded, ambient-temperature room or in a refrigerator if ambient temperature exceeds 20 degrees Celsius, for no longer than 3 months before shipping [2].

    - LAB MEDIA:  5.1;5.2 GX010190.MOV 00:44–00:55

    - LAB MEDIA:  6.1 GX010178.MOV

  - Perform all laboratory procedures in dedicated ultraviolet-sterilized rooms with adequate ventilation [1] and clean the workbenches after donning DNA-free gloves and protective clothing [2].

    - LAB MEDIA:  7. IMG_0939.MOV

    - LAB MEDIA: 7. IMG_0946.MOV

  - Using DNA-free forceps, fold the filter membrane into triangles [1] and transfer it to a 2-milliliter reaction tube with the tip oriented downward [2]. Add 380 microliters of TES buffer and 20 microliters of Proteinase K to each reaction tube to obtain 400 microliters of lysis buffer [3].

    - LAB MEDIA:  7.1.3 IMG_1079.MOV 00:05-00:10 and 00:25-00:30

    - LAB MEDIA:  7.1.3 IMG_1083.MOV

    - LAB MEDIA:  7.1.4 IMG_1090.MOV 00:15-00:25

  - Vortex briefly and incubate at 56 degrees Celsius for at least 3 hours with agitation [1]. After incubation, centrifuge the tubes at 18626 g for 10 minutes at room temperature [2].

    - LAB MEDIA:  7.1.5 IMG_1092.MOV 00:016-00:25

    - LAB MEDIA:  7.1.6 IMG_1097.

- DNA Extraction and Quantitative PCR

  - Pipette extraction reagents into wash plates and elution plates [1].

    - LAB MEDIA:  7.2.2 IMG_0959.MOV 00:20–00:30

  - Transfer 300 microliters of lysate into each well in the bind plate [1]. Vortex and add 30 microliters of magnetic particles per well in the first Bind plate [2].

    - LAB MEDIA:  7.2.4 IMG_0989.MOV 00:05-00:10

    - LAB MEDIA:  7.2.4 IMG_0990.MOV

  - Run the preliminary DNA uptake program for two bind plates and the subsequent DNA extraction program on the robot. [1].

    - LAB MEDIA:  7.2.6 IMG_0998.MOV 00:2–00:16

  - Next, prepare the master mix for PCR [1]. In a dedicated space for preparing PCRs, add the extracted DNA standards to each well containing master mix [2] and then add 3 microliters of DNA extract to each subsequent well [3].

    - LAB MEDIA:  7.3.4 IMG_1039.MOV 00:28–00:30 Authors: This step is moved here

    - LAB MEDIA:  7.3.5 IMG_1053.MOV 00:10–00:24

    - LAB MEDIA:  7.3.5 IMG_1051 00:03–00:15

  - After sealing the prepared PCR plate, centrifuge it at 3500 g for 10 seconds [1] and load the plate into the real-time quantitative PCR thermocycler [2]. Finally, run the assay under optimized cycling conditions [3].

    - LAB MEDIA:  7.3.7 IMG_1058.MOV 00:00-00:10

    - LAB MEDIA: 7.3.7 IMG_1060.MOV 00:05-00:15

    - LAB MEDIA:  7.3.8 IMG_1061.MOV 00:08-00:17

# Results

- Results

  - 247 Environmental DNA or eDNA samples were collected in Skjálfandi Bay, Iceland and in the Azores involving the collection of up to 10 liters of seawater with two self-preserving eDNA filters per flukeprint [1].

    - LAB MEDIA: Figure 9. Video editor: Show the map and highlight the black dots marking sampling locations in Iceland and the Azores

  - Among the 44 eDNA filters analyzed for humpback whale eDNA, 20 yielded amplification signals [1], whereas 24 showed no amplification [2].

    - LAB MEDIA: Table 4. Video editor: Highlight the rows with yellow-orange background

    - LAB MEDIA: Table 4. Video editor: Highlight the rows with white background

  - Extraction blanks, field controls, and quantitative PCR controls showed no detectable amplification, indicating absence of contamination [1].

    - LAB MEDIA: Table 4. Video editor: Highlight the rows towards the end with grey background

  - Among positive reactions, 9 eDNA filters yielded at least one cycle threshold value below the limit of detection and were classified as reliable detections [1].

    - LAB MEDIA: Figure 10. Video editor: Highlight the dark blue bars

  - The remaining 11 eDNA filters produced cycle threshold values above the limit of detection, indicating plausible detections [1].

    - LAB MEDIA: Figure 10. Video editor: Highlight the light blue bars

  - The mean cycle threshold value was marginally lower in the Azores eDNA filters at about 34.7 [1] compared to the Iceland filters with a value of approximately 36.9 [2].

    - LAB MEDIA: Figure 11. Video editor: Highlight the Azores box plot

    - LAB MEDIA: Figure 11. Video editor: Highlight the Iceland box plot

  - The difference in cycle threshold values between Azores and Iceland eDNA filters was not statistically significant [1].

    - LAB MEDIA: Figure 11.

- Cetacean
Pronunciation link: https://www.merriam-webster.com/dictionary/cetacean
IPA: /sɪˈteɪʃən/
Phonetic Spelling: sih·tay·shuhn

- Peristaltic
Pronunciation link: https://www.merriam-webster.com/dictionary/peristaltic
IPA: /ˌpɛrəˈstɔltɪk/, /ˌpɛrəˈstæltɪk/
Phonetic Spelling: pehr·uh·stawl·tik, pehr·uh·stal·tik

- Ultraviolet
Pronunciation link: https://www.merriam-webster.com/dictionary/ultraviolet
IPA: /ˌʌltrəˈvaɪələt/
Phonetic Spelling: uhl·truh·vye·uh·luht

- Forceps
Pronunciation link: https://www.merriam-webster.com/dictionary/forceps
IPA: /ˈfɔrsɛps/, /ˈfɔrsəps/
Phonetic Spelling: for·seps, for·suhps

- Membrane
Pronunciation link: https://www.merriam-webster.com/dictionary/membrane
IPA: /ˈmɛmbreɪn/
Phonetic Spelling: mem·brayn

- Proteinase
Pronunciation link: https://www.merriam-webster.com/dictionary/proteinase
IPA: /ˈproʊtəˌneɪs/, /ˈproʊtin/, /ˈproʊtiənˌeɪz/
Phonetic Spelling: პროht·uh·nays, პროh·teen, პროh·tee·uhn·ayz

- Lysis
Pronunciation link: https://www.merriam-webster.com/dictionary/lysis
IPA: /ˈlaɪsəs/
Phonetic Spelling: lye·suhs

- Lysate
Pronunciation link: https://www.merriam-webster.com/dictionary/lysate
IPA: /ˈlaɪˌseɪt/
Phonetic Spelling: lye·sayt

- Centrifuge
Pronunciation link: https://www.merriam-webster.com/dictionary/centrifuge
IPA: /ˈsɛntrəˌfjudʒ/
Phonetic Spelling: sen·truh·fyooj

- Quantitative
Pronunciation link: https://www.merriam-webster.com/dictionary/quantitative
IPA: /ˈkwɑn(t)əˌteɪtɪv/
Phonetic Spelling: kwahn·tuh·tay·tiv

- Amplification
Pronunciation link: https://www.merriam-webster.com/dictionary/amplification
IPA: /ˌæmpləfəˈkeɪʃən/
Phonetic Spelling: am·pluh·fuh·kay·shuhn

- Threshold
Pronunciation link: https://www.merriam-webster.com/dictionary/threshold
IPA: /ˈθrɛʃhoʊld/, /ˈθrɛʃəld/
Phonetic Spelling: thresh·hohld, thresh·uhld